# THE AIDC CONTROL FUNCTION

## Introduction

### Recommendation. ¾ An implementation of the AIDC-AE should behave as if there existed a Control Function as defined in this chapter.

- ¾ This section specifies the AIDC Control Function (CF) in terms of sequence diagrams, state definitions, and service mapping’s.

### With the permissible exception of abort service primitives, the AIDC-AE shall process service primitives in the order in which they are received: this ensures that the AE will, with the exception of aborts, guarantee message sequencing.

## Sequence Diagrams

- ¾ The sequence diagrams in this section do not mandate the user sequencing of primitives. Nor do they cover all possible sequences.

- ¾ The following figures, show the sequence of AIDC-User primitives and the CF mapping of those primitives to/from the primitives of the AIDC-ASE.

- ¾ As the User-confirmation service is common to all services, except the abort services, the invocation of the primitives are shown as a dotted line.

- ¾ The “+” symbol preceding the User-confirmation primitive indicates that the user has validated and accepted the contents of the received message.

- ¾ The “-” symbol preceding the User-confirmation primitive indicates that the user has rejected the contents of the received message.

- ¾ The timers shown in the diagrams are technical timers and are defined in section 6 of this document.

### Notify Service Primitives

Figure 5-1: Sequence diagram and CF mapping for the Notify service

### Coordinate-start Service Primitives

Figure 5-2: Sequence diagram and CF mapping for the Coordinate-start service and Coordinate-end service

Figure 5-3: Sequence diagram and CF mapping for the Coordinate-start service with AIDC-User rejection of Coordinate-end indication

### Coordinate-end Service Primitives

Figure 5-4: Sequence diagram and CF mapping for the Coordinate-end service with Coordinate-end request primitive invoked by C-ATSU AIDC-User

Figure 5-5: Sequence diagram and CF mapping for the Coordinate-end service with Coordinate-end request invoked by D-ATSU AIDC-User

### Coordinate-negotiate Service Primitives

Figure 5-6: Sequence diagram and CF mapping for the Coordinate-negotiate service

Figure 5-7: Sequence diagrams and CF mapping for the Coordinate-negotiate service with AIDC-User rejection of Coordinate-negotiate indication

Figure 5-8: Sequence diagram and CF mapping for the Coordinate-negotiate service with Coordinate-negotiate service response

### Coordinate-standby Service Primitives

Figure 5-9: Sequence diagram and CF mapping for the Coordinate-standby service

Figure 5-10: Sequence diagram and CF mapping for the Coordinate-standby service with AIDC-User rejection of Coordinate-standby indication

Figure 5-11: Sequence diagram and CF mapping for the Coordinate-standby service followed by Coordinate-negotiate service

Figure 5-12: Sequence diagram and CF mapping for the Coordinate-standby service used in response to the Coordinate-negotiate service

### Transfer-initiate Service Primitives

Figure 5-13: Sequence diagram and CF mapping for the Transfer-initiate service

### Transfer-request Service Primitives

Figure 5-14: Sequence diagram and CF mapping for Transfer-request service

Figure 5-15: Sequence diagram and CF mapping for Transfer-request service with AIDC-User rejection of Transfer-initiate indication

### Transfer-conditions-proposal Service Primitives

Figure 5-16: Sequence diagram and CF mapping for the Transfer-conditions-proposal service

Figure 5-17: Sequence diagram and CF mapping for the Transfer-conditions-proposal service with Transfer-conditions-accept service response

Figure 5-18: Sequence diagram and CF mapping for the Transfer-conditions-proposal service with AIDC-User rejection of Transfer-conditions-accept indication

### Transfer-control Service Primitives

Figure 5-19: Sequence diagram and CF mapping for the Transfer-control service

Figure 5-20: Sequence diagram and CF mapping for the Transfer-control service with AIDC-User rejection of Transfer-control confirmation

Figure 5-21: Sequence diagram and CF mapping for the Transfer-control service

### Transfer-communication Service Primitives

Figure 5-22: Sequence diagram and CF mapping for the Transfer-communication service with Transfer-communication-assume service used as response

Figure 5-23: Sequence diagram and CF mapping for the Transfer-communication service with AIDC-User rejection of Transfer-communication-assume service

### Transfer-communication-assume Service Primitives

Figure 5-24: Sequence diagram and CF mapping for the Transfer-communication-assume service

### Info-transfer Service Primitives

Figure 5-25: Sequence diagram and CF mapping for the Info-transfer service

Figure 5-26: Sequence diagram and CF mapping for the Info-transfer service used in Coordinating Regime

Figure 5-27: Sequence diagram and CF mapping for the Info-transfer service used in Transferring Regime

Figure 5-28: Sequence diagram and CF mapping for the Info-transfer service used between user confirmation of Coordinate-start service

### End Service Primitives

Figure 5-29: Sequence diagram and CF mapping for the End service.

### User-abort Service Primitives

Figure 5-30: Sequence diagram and CF mapping for the User-abort service.

### Provider-abort Service Primitives

Figure 5-31: Sequence diagram and CF mapping for the Provider-abort service - ATN Service Provider abort

Figure 5-32: Sequence diagram and CF mapping for the Provider-abort service -  AIDC-AE abort.

## AIDC-AE CF State Definitions

### The AIDC-AE CF shall be in one of the following states at a given time:

- Null  (STA0) –  This is the state of the CF when there is no association in existence.

- Association Pending  (STA1) –  The CF enters this state when or the AIDC-User has invoked certain primitives or an indication has been received that the peer has made a request to establish an association.

- Data Transfer  (STA2) –  The CF enters this state once the establishment phase is complete.  An association has successfully been established and the communicating partners are free to send and receive data.

- Release Pending  (STA3) –  The CF enters this state when either the AIDC-User has requested the termination of the AIDC service or an indication has been received that the peer has made a request to terminate the association.

## AIDC-AE CF Service Mappings

- ¾ Figure 5-33  indicates which sections in this chapter specifies the behaviour of the CF in response to events at the various service interfaces.

Figure 5-33: Elements of the AIDC-AE

### AIDC-User Services Primitives Submitted to the CF

- ¾ This section specifies the actions of the CF in response to events which occur at the upper service boundary of the AIDC-AE: specifically, request and response primitives generated by the AIDC-User.

#### Implicit Association

##### The invocation of a Notify request primitive or a Coordinate-start request primitive shall implicitly cause the establishment of an association, if one does not exist, between two peer AIDC-AEs.

##### The association establishment and release between peer AIDC-AE shall be performed by invoking the primitives of ACSE.

##### Upon the receipt of a Notify request or Coordinate-start request primitive, the CF shall:

- invoke an A-ASSOCIATE Request primitive with the parameters specified in Table 5-1; and

- enter the ASSOCIATION PENDING state.

Table 5-1: A-ASSOCIATE Request Primitive Parameters

| A-ASSOCIATE Request Parameter | Value |
| --- | --- |
| Mode | Not used (default value) |
| Application Context Name | {iso (1) identified-organization (3) icao (27) atn-ac (3) 1 } |
| Application Context Name List | Not used |
| Calling AP Title | {iso (1) identified-organization (3) icao (27) atn-end-system-ground (2) <end-system-id> (n) operational (0) } |
| Calling AE Qualifier | idc (6) |
| Calling AP Invocation-identifier | Not used |
| Calling AE Invocation-identifier | Not used |
| Called AP Title | Not used |
| Called AE Qualifier | Not used |
| Called AP Invocation-identifier | Not used |
| Called AE Invocation-identifier | Not used |
| ACSE Requirements | Not used |
| Authentication-mechanism Name | Not used |
| Authentication-value | Not used |
| User Information | Not used |
| Calling Presentation Address | Local Implementation |
| Called Presentation Address | Local Implementation |
| Presentation Context Definition List | Not used |
| Default Presentation Context Name | Not used |
| Quality of Service | Not used. |
| Presentation Requirements | Not used (default value) |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not used |
| Initial Assignment of Tokens | Not used |
| Session-connection Identifier | Not used |

#### User-confirmation Request Primitive

##### When Invoked

###### It shall be valid to invoke the User-confirmation request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, and the User-confirmation request primitive is invoked, then appropriate error recovery action should be taken.

###### Action Upon Invocation

The CF shall invoke an AIDC-ucf request primitive with:

- the User-confirmation Result parameter as the AIDC-ucf Result parameter;

- the User-confirmation Referenced Number parameter as the AIDC-ucf Reference ID parameter; and

- if present, the User-confirmation Reason parameter as the AIDC-ucf Reason parameter.

#### Notify Request Primitive

##### When Invoked

###### It shall be valid to invoke the Notify request primitive when the CF is in the NULL state, or the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### If in the NULL state the CF shall:

- store the value of the Notify request primitive parameters;

- set the Boolean variable nfy-assc; and

- perform the implicit association function as specified in section 5.4.1.1.

###### If in the DATA TRANSFER state, the CF shall invoke an AIDC-nfy request primitive with:

- the Notify request primitive Notification Information parameter as the AIDC-nfy request primitive User Data  parameter; and

- the Notify request primitive Message Number parameter as the AIDC-nfy request primitive Msg Number parameter.

#### Coordinate-start Request Primitive

##### When Invoked

###### It shall be valid to invoke the Coordinate-start request primitive when the CF is in the NULL state, or the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### If in the NULL state the CF shall:

- store the value of the Coordinate-start request primitive parameters;

- set the Boolean variable crd-assc; and

- perform the implicit association function as specified in section 5.4.1.1.

###### If in the DATA TRANSFER state, the CF shall invoke an AIDC-crd-start request primitive with:

- the Coordinate-start request primitive Coordination Start Information parameter as the AIDC-crd-start request primitive User Data  parameter; and

- the Coordinate-start request primitive Message Number parameter as the AIDC-crd-start request primitive Msg Number parameter.

#### Coordinate-end Request Primitive

##### When Invoked

###### It shall be valid to invoke the Coordinate-end request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Coordinate-end request primitive, the CF shall invoke an AIDC-crd-end request primitive with:

- the Coordinate-end request primitive Coordinate End Information  parameter as the AIDC-crd-end request primitive User Data parameter;

- the Coordinate-end request primitive Result parameter as the AIDC-crd-end request primitive Result parameter; and

- the Coordinate-end request primitive Message Number parameter as the AIDC-crd-end request primitive Msg Number parameter.

#### Coordinate-negotiate Request Primitive

##### When Invoked

###### It shall be valid to invoke the Coordinate-negotiate request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Coordinate-negotiate request primitive, the CF shall invoke an AIDC-crd-ngtt request primitive with:

- the Coordinate-negotiate request primitive Coordinate Negotiate Information  parameter as the AIDC-crd-ngtt request primitive User Data parameter; and

- the Coordinate-negotiate request primitive Message Number parameter as the AIDC-crd-ngtt request primitive Msg Number parameter.

#### Coordinate-standby Request Primitive

##### When Invoked

###### It shall be valid to invoke the Coordinate-standby request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Coordinate-standby request primitive, the CF shall invoke an AIDC-crd-stndby request primitive with:

- the Coordinate-standby request primitive Coordinate Standby Information  parameter as the AIDC-crd-stndby request primitive User Data parameter; and

- the Coordinate-standby request primitive Message Number parameter as the AIDC-crd-stndby request primitive Msg Number parameter.

#### Transfer-initiate Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-initiate request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-initiate request primitive, the CF shall invoke an AIDC-tfr-init request primitive with:

- the Transfer-initiate request primitive Transfer Initiate Information  parameter as the AIDC-tfr-init request primitive User Data parameter; and

- the Transfer-initiate request primitive Message Number parameter as the AIDC-tfr-init request primitive Msg Number parameter.

#### Transfer-request Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-request request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-request request primitive, the CF shall invoke an AIDC-tfr-rqst request primitive with:

- the Transfer-request request primitive Transfer Request Information  parameter as the AIDC-tfr-rqst request primitive User Data parameter; and

- the Transfer-request request primitive Message Number parameter as the AIDC-tfr-rqst request primitive Msg Number parameter.

#### Transfer-conditions-proposal Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-conditions-proposal request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-conditions-proposal request primitive, the CF shall invoke an AIDC-tfr-prpsl request primitive with:

- the Transfer-conditions-proposal request primitive Transfer Conditions Proposal Information  parameter as the AIDC-tfr-prpsl request primitive User Data parameter; and

- the Transfer-conditions-proposal request primitive Message Number parameter as the AIDC-tfr-prpsl request primitive Msg Number parameter.

#### Transfer-conditions-accept Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-conditions-accept request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-conditions-accept request primitive, the CF shall invoke an AIDC-tfr-accept request primitive with:

- the Transfer-conditions-accept request primitive Transfer Conditions Accept Information  parameter as the AIDC-tfr-accept request primitive User Data parameter; and

- the Transfer-conditions-accept request primitive Message Number parameter as the AIDC-tfr-accept request primitive Msg Number parameter.

#### Transfer-control Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-control request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-control request primitive, the CF shall invoke an AIDC-tfr-cntrl request primitive with:

- the Transfer-control request primitive Transfer Control Information  parameter as the AIDC-tfr-cntrl request primitive User Data parameter; and

- the Transfer-control request primitive Message Number parameter as the AIDC-tfr-cntrl request primitive Msg Number parameter.

#### Transfer-control Response Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-control response primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-control response primitive, the CF shall invoke an AIDC-tfr-cntrl response primitive with:

- the Transfer-control response primitive Transfer Control Information  parameter as the AIDC-tfr-cntrl response primitive User Data parameter;

- the Transfer-control response primitive Result parameter as the AIDC-tfr-cntrl response primitive Result parameter; and

- the Transfer-control response primitive Message Number parameter as the AIDC-tfr-cntrl response primitive Msg Number parameter.

#### Transfer-communication Request Primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-communication request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-communication request primitive, the CF shall invoke an AIDC-tfr-comm request primitive with:

- the Transfer-communication request primitive Transfer Communication Information  parameter as the AIDC-tfr-comm request primitive User Data parameter; and

- the Transfer-communication request primitive Message Number parameter as the AIDC-tfr-comm request primitive Msg Number parameter.

#### Transfer-communication-assume Request primitive

##### When Invoked

###### It shall be valid to invoke the Transfer-communication-assume request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Transfer-communication-assume request primitive, the CF shall invoke an AIDC-tfr-comm-assm request primitive with:

- the Transfer-communication request primitive Transfer Communication Assume Information  parameter as the AIDC-tfr-comm-assm request primitive User Data parameter; and

- the Transfer-communication request primitive Message Number parameter as the AIDC-tfr-comm-assm request primitive Msg Number parameter.

#### Info-transfer Request primitive

##### When Invoked

###### It shall be valid to invoke the Info-transfer request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a Info-transfer request primitive, the CF shall invoke an AIDC-inf-tfr request primitive with:

- the Info-transfer request primitive Information  parameter as the AIDC-inf-tfr request primitive User Data parameter; and

- the Info-transfer request primitive Message Number parameter as the AIDC-inf-tfr request primitive Msg Number parameter.

#### End Request Primitive

##### When Invoked

###### It shall be valid to invoke the End request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a End request primitive, the CF shall invoke an AIDC-end request primitive with:

- the End request primitive Cancel Information  parameter, if present, as the AIDC-end request primitive User Data parameter; and

- the End request primitive Message Number parameter as the AIDC-end request primitive Msg Number parameter.

#### User-abort Request Primitive

##### When Invoked

###### It shall be valid to invoke the User-abort request primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a User-abort request primitive, the CF shall:

- invoke an AIDC-usr-abrt request primitive; and

- enter the RELEASE PENDING state.

### AIDC-ASE Service Primitives Delivered to the CF

- ¾ This section specifies the actions of the CF in response to events which occur at the upper service boundary of the AIDC-ASE: specifically, indication and confirmation primitives which are generated by the AIDC-ASE protocol machine.

#### AIDC-ucf Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-ucf indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-ucf indication primitive, the CF shall invoke a User-confirmation indication primitive with:

- the AIDC-ucf indication primitive Result  parameter as the User-confirmation indication primitive Result parameter;

- if present, the AIDC-ucf indication primitive Reason parameter as the User-confirmation primitive Reason parameter; and

- the AIDC-ucf indication Reference ID parameter as the User-confirmation indication Referenced Number parameter.

#### AIDC-nfy Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-nfy indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-nfy indication primitive, the CF shall invoke the Notify indication primitive with:

- the stored Calling ICAO Facility Designator as the Notify indication primitive Calling ICAO Facility Designator parameter;

- the AIDC-nfy indication primitive User Data parameter as the Notify indication primitive Notification Information parameter; and

- the AIDC-nfy indication primitive Msg Number parameter as the Notify indication primitive Message Number parameter.

#### AIDC-crd-start Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-crd-start indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-crd-start indication primitive, the CF shall invoke the Coordinate-start indication primitive with:

- the stored Calling ICAO Facility Designator as the Coordinate-start indication primitive Calling ICAO Facility Designator parameter;

- the AIDC-crd-start indication primitive User Data parameter as the Coordinate-start indication primitive Coordinate Start Information parameter; and

- the AIDC-crd-start indication primitive Msg Number parameter as the Coordinate-start indication primitive Message Number parameter.

#### AIDC-crd-end Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-crd-end indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-crd-end indication primitive the CF shall invoke a Coordinate-end indication primitive with:

- the AIDC-crd-end indication primitive User Data parameter as the Coordinate-end indication primitive Coordinate End Information parameter; and

- the AIDC-crd-end indication primitive Msg Number parameter as the Coordinate-end indication primitive Message Number parameter.

#### AIDC-crd-ngtt Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-crd-ngtt indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-crd-ngtt indication primitive the CF shall invoke a Coordinate-negotiate indication primitive with:

- the AIDC-crd-ngtt indication primitive User Data parameter as the Coordinate-negotiate indication primitive Coordinate Negotiate Information parameter; and

- the AIDC-crd-ngtt indication primitive Msg Number parameter as the Coordinate-negotiate indication primitive Message Number parameter.

#### AIDC-crd-stndby Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-crd-stndby indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-crd-stndby indication primitive the CF shall invoke a Coordinate-standby indication primitive with:

- the AIDC-crd-stndby indication primitive User Data parameter as the Coordinate-standby indication primitive Coordinate Standby Information parameter; and

- the AIDC-crd-stndby indication primitive Msg Number parameter as the Coordinate-standby indication primitive Message Number parameter.

#### AIDC-tfr-init Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-init indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-init indication primitive the CF shall invoke a Transfer-initiate indication primitive with:

- the AIDC-tfr-init indication primitive User Data parameter as the Transfer-initiate indication primitive Transfer Initiate Information parameter; and

- the AIDC-tfr-init indication primitive Msg Number parameter as the Transfer-initiate indication primitive Message Number parameter.

#### AIDC-tfr-rqst Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-rqst indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-rqst indication primitive the CF shall invoke a Transfer-request indication primitive with:

- the AIDC-tfr-rqst indication primitive User Data parameter as the Transfer-request indication primitive Transfer Request Information parameter; and

- the AIDC-tfr-rqst indication primitive Msg Number parameter as the Transfer-request indication primitive Message Number parameter.

#### AIDC-tfr-prpsl Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-prpsl indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-prpsl indication primitive the CF shall invoke a Transfer-conditions-proposal indication primitive with:

- the AIDC-tfr-prpsl indication primitive User Data parameter as the Transfer-conditions-proposal indication primitive Transfer Conditions Proposal Information parameter; and

- the AIDC-tfr-prpsl indication primitive Msg Number parameter as the Transfer-conditions-proposal indication primitive Message Number parameter.

#### AIDC-tfr-accept Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-accept indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-accept indication primitive the CF shall invoke a Transfer-conditions-accept indication primitive with:

- the AIDC-tfr-accept indication primitive User Data parameter as the Transfer-conditions-accept indication primitive Transfer Conditions Accept Information parameter; and

- the AIDC-tfr-accept indication primitive Msg Number parameter as the Transfer-conditions-accept indication primitive Message Number parameter.

#### AIDC-tfr-cntrl Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-cntrl indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-cntrl indication primitive the CF shall invoke a Transfer-control indication primitive with:

- the AIDC-tfr-cntrl indication primitive User Data parameter as the Transfer-control indication primitive Transfer Control Information parameter; and

- the AIDC-tfr-cntrl indication primitive Msg Number parameter as the Transfer-control indication primitive Message Number parameter.

#### AIDC-tfr-cntrl Confirmation Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-cntrl confirmation primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-cntrl confirmation primitive the CF shall invoke a Transfer-control confirmation primitive with:

- the AIDC-tfr-cntrl confirmation primitive User Data parameter as the Transfer-control confirmation primitive Transfer Control Information parameter;

- the AIDC-tfr-cntrl confirmation primitive Result parameter as the Transfer-control confirmation primitive Result parameter; and

- the AIDC-tfr-cntrl confirmation primitive Msg Number parameter as the Transfer-control confirmation primitive Message Number parameter.

#### AIDC-tfr-comm Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-comm indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-comm indication primitive the CF shall invoke a Transfer-communication indication primitive with:

- the AIDC-tfr-comm indication primitive User Data parameter as the Transfer-communication indication primitive Transfer Communication Information parameter; and

- the AIDC-tfr-comm indication primitive Msg Number parameter as the Transfer-communication indication primitive Message Number parameter.

#### AIDC-tfr-comm-assm Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-tfr-comm-assm indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-tfr-comm-assm indication primitive the CF shall invoke a Transfer-communication-assume indication primitive with:

- the AIDC-tfr-comm-assm indication primitive User Data parameter as the Transfer-communication-assume indication primitive Transfer Communication Assume Information parameter; and

- the AIDC-tfr-comm-assm indication primitive Msg Number parameter as the Transfer-communication-assume indication primitive Message Number parameter.

#### AIDC-inf-tfr Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-inf-tfr indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-inf-tfr indication primitive the CF shall invoke a Info-transfer indication primitive with:

- the AIDC-inf-tfr indication primitive User Data parameter as the Info-transfer indication primitive Information parameter; and

- the AIDC-inf-tfr indication primitive Msg Number parameter as the Info-transfer indication primitive Message Number parameter.

#### AIDC-end Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-end indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-end indication primitive the CF shall invoke a End indication primitive with:

- the AIDC-end indication primitive User Data parameter, if present, as the End indication primitive Cancel Information parameter; and

- the AIDC-end indication primitive Msg Number parameter as the End indication primitive Message Number parameter.

#### AIDC-usr-abrt Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-usr-abrt indication primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-usr-abrt indication primitive the CF shall invoke a User-abort indication primitive.

#### AIDC-pvd-abrt Indication Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-pvd-abrt indication primitive when the CF is in the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, or the ASSOCIATION PENDING state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of  a AIDC-pvd-abrt indication primitive the CF shall:

if the CF is in the DATA TRANSFER state:

- invoke a User-abort indication primitive; and

- enter the RELEASE PENDING state.

if the CF is in the RELEASE PENDING state invoke a User-abort indication primitive.

### AIDC-ASE Service Primitives Submitted to the CF

- ¾ This section specifies the actions of the CF in response to events which occur at the lower service boundary of the AIDC-ASE: specifically, request primitives which are generated by the AIDC-ASE protocol machine.

#### AIDC-DATA Request primitive

##### When Invoked

##### It shall be valid to invoke the AIDC-DATA request primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

##### Upon the receipt of a AIDC-DATA request primitive the CF shall:

- encode the AIDC-DATA Request User Data parameter using the definition of presentation-user-data in [x, section 3.2.6]; and

- invoke a P-DATA request primitive with the resulting encoding from a) above, as the User Data parameter.

#### AIDC-ABORT Request Primitive

##### When Invoked

###### It shall be valid to invoke the AIDC-ABORT request primitive when the CF is in the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, or the ASSOCIATION PENDING state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of a AIDC-ABORT request primitive the CF shall:

If the CF is in the DATA TRANSFER state:

- invoke an A-ABORT request primitive with no parameters; and

- enter the RELEASE PENDING state.

If the CF is in the RELEASE PENDING state, invoke an A-ABORT request primitive with no parameters.

### ACSE Service Primitives Delivered to the CF

- ¾ This section specifies the action of the CF in response to events which occur at the upper service boundary of ACSE: specifically, indication and confirmation primitives which are generated by the ACSE Protocol Machine (ACPM).

#### A-ASSOCIATE Indication Primitive

##### When Invoked

###### It shall be valid to invoke the A-ASSOCIATE indication primitive when the CF is in the ASSOCIATION PENDING state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

##### Upon the receipt of an A-ASSOCIATE indication primitive, the CF shall:

- extract and store the encoded Calling  ICAO Facility Designator from Calling AP Title parameter; and

- invoke an A-ASSOCIATE response primitive with the parameters as shown in Table 5-2;

Table 5-2: A-ASSOCIATE Response Primitive Parameters

| Parameter | Value |
| --- | --- |
| Mode | Not used (default value) |
| Application Context Name | {iso (1) identified-organization (3) icao (27) atn-ac (3) 1 } |
| Application Context Name List | Not used |
| Responding AP Title | Not used |
| Responding AE Qualifier | Not used |
| Responding AP Invocation-identifier | Not used |
| Responding AE Invocation-identifier | Not used |
| ACSE Requirements | Not used |
| Authentication-mechanism Name | Not used |
| Authentication-value | Not used |
| User Information | Not used |
| Result | Not used |
| Diagnostic | Not used |
| Responding Presentation Address | Local Implementation |
| Presentation Context Definition Result List | Not used |
| Default Presentation Context Result | Not used |
| Quality of Service | Not used |
| Presentation Requirements | Not used (default value) |
| Session Requirements | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | Not used |
| Initial Assignment of Tokens | Not used |
| Session-connection Identifier | Not used |

#### A-ASSOCIATE Confirmation Primitive

##### When Invoked

###### It shall be valid to invoke the A-ASSOCIATE confirmation primitive when the CF is in the ASSOCIATION PENDING state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an A-ASSOCIATE confirmation primitive, the CF shall:

- if the Result parameter has the abstract value: accepted, then:

  - if the nfy-assc variable is set:- invoke an AIDC-nfy request primitive with:

    - the stored Notification Information as the User Data parameter;

    - the stored Message Number parameter as the Msg Number parameter; and

    - enter the DATA TRANSFER state; or

  - if the crd-assc variable is set:- invoke an AIDC-crd-start request primitive with:

    - the stored Coordinate Start Information as the User Data parameter;

    - the stored Message Number parameter as the Msg Number parameter; and

    - enter the DATA TRANSFER state.

- if the Result parameter has the abstract value “rejected (permanent)” or “rejected (transient)”, then:

  - invoke a Provider-abort indication primitive with the A-ASSOCIATE confirmation primitive Result Source parameter as the Provider-abort indication primitive Provider Abort Reason parameter; and

  - enter the NULL state.

#### A-ABORT Indication primitive

##### When Invoked

###### It shall be valid to invoke the A-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an A-ABORT indication primitive, the CF shall:

if the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state:

- invoke an AIDC-ABORT indication primitive; and

- enter the RELEASE PENDING state.

if the CF is in the RELEASE PENDING state, enter the NULL state.

#### A-P-ABORT Indication Primitive

##### When Invoked

###### It shall be valid to invoke the A-P-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an A-P-ABORT indication primitive, the CF shall:

- if in the ASSOCIATION PENDING state:

  - invoke a Provider-abort indication with the A-P-ABORT indication primitive Provider Reason parameter as the Provider-abort indication primitive Provider Abort Reason parameter; and

  - enter the NULL state.

- if in the DATA TRANSFER state, or the RELEASE PENDING state:

  - invoke a AIDC-P-ABORT indication primitive, and discard any Provider Reason parameter in the A-P-ABORT indication; and

  - enter the RELEASE PENDING state.

### ACSE Service Primitives Submitted to the CF

- ¾ This section specifies the actions of the CF in response to events at the lower service boundary of ACSE: specifically, request and response primitives generated by the ACMP.

- ¾ ACSE (Edition 2) mandates the mapping between ACSE and the underlying Presentation service provider. Invocations of Presentation service primitives by ACSE are “intercepted” by the CF and re-mapped to the “actual” Presentation service as appropriate.

#### P-CONNECT Request Primitive

##### When Invoked

###### It shall be valid to invoke the P-CONNECT request primitive when the CF is in the ASSOCIATION PENDING state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-CONNECT request primitive, the CF shall invoke the equivalent Presentation service primitive of the ATN service provider.

#### P-CONNECT Response Primitive

##### When Invoked

###### It shall be valid to invoke the P-CONNECT response primitive when the CF is in the ASSOCIATION PENDING state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-CONNECT response primitive, the CF shall:

- transparently invoke the equivalent presentation service primitive; and

- enter the DATA TRANSFER state.

#### P-U-ABORT Request Primitive

##### When Invoked

###### It shall be valid to invoke the P-U-ABORT request primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-U-ABORT request primitive, the CF shall:

- invoke a P-U-ABORT request primitive without any parameters; and

- ¾  This will abort the connection to the underlying ATN Service Provider .

- enter the NULL state.

### Presentation Services Primitives Delivered to the CF

- ¾ This section specifies the actions of the CF in response to events which occur at the lower service boundary of the AIDC-AE: specifically, indication and confirmation primitives which are delivered by the Presentation service.

- ¾ ACSE (Edition 2) mandates the mapping between ACSE and the underlying Presentation service provider. Presentation service primitives are “intercepted” by the CF.

#### P-CONNECT Indication Primitive

##### When Invoked

###### It shall be valid to invoke the P-CONNECT indication primitive when the CF is in the NULL state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-CONNECT indication primitive, the CF shall:

- enter the ASSOCIATION PENDING state; and

- invoke the equivalent Presentation service primitive at the lower ACSE service boundary.

#### P-CONNECT Confirmation Primitive

##### When Invoked

###### It shall be valid to invoke the P-CONNECT confirmation primitive when the CF is in the ASSOCIATION PENDING state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-CONNECT confirmation primitive, the CF shall invoke the equivalent Presentation service primitive at the lower ACSE service boundary.

#### P-U-ABORT Indication Primitive

##### When Invoked

###### It shall be valid to invoke the P-U-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-U-ABORT indication primitive, the CF shall invoke the equivalent Presentation service primitive at the lower ACSE service boundary.

#### P-P-ABORT Indication Primitive

##### When Invoked

###### It shall be valid to invoke the P-P-ABORT indication primitive when the CF is in the ASSOCIATION PENDING state, or the DATA TRANSFER state, or the RELEASE PENDING state.

###### Recommendation. ¾ If the CF is in the NULL state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-P-ABORT indication primitive, the CF shall invoke the corresponding Presentation service primitive at the lower ACSE service boundary.

#### P-DATA Indication Primitive

##### When Invoked

###### It shall be valid to invoke the P-DATA indication primitive when the CF is in the DATA TRANSFER state.

###### Recommendation. ¾ If the CF is in any other state, then appropriate error recovery action should be taken.

##### Action Upon Invocation

###### Upon the receipt of an P-DATA indication primitive, the CF shall:

- decode the presentation user data as indicated in [x, section 3.2.6] to determine the destination ASE of the APDU, and extract the Presentation data value.

- invoke an AIDC-DATA indication primitive with the extracted AIDC-PDU as the AIDC-DATA AIDC Data parameter.

## AIDC-CF State Table

### The AIDC-AE shall behave as if it has a CF which functions in accordance with the following state table.

- ¾The table shows the state transitions and actions performed by the AIDC-CF in response to service primitives submitted to the AIDC-CF.  The source of the service primitive invocation is shown in column one of the table and the service primitive invocations are shown in the second column.

### In the event of discrepancies between the state table and the text in the sections above, the text shall take precedence.

### Each cell in the state table shows:

- the action, if any, which the CF shall perform; and

- the new state that the CF shall enter.

### Blank cells shall indicate error conditions.

#### Recommendation.¾The error handling shall result in the association being aborted, if one exists, and a notification being given to the AIDC-User.

Table 5-3:  AIDC CF State Table

| Source | State--> Primitive Submitted | STA0 Null | STA1 Association Pending | STA2 Data Transfer | STA3 Release Pending |
| --- | --- | --- | --- | --- | --- |
| From AIDC-User | Notify Req | - Store Notification Information - set nfy-assc - A-ASSOCIATE Req - STA1 |  | - AIDC-nfy Req |  |
|  | Coordinate-start Req | - Store Coordination Start Information - set crd-assc - A-ASSOCIATE Req - STA1 |  | - AIDC-crd-start Req |  |
|  | End Req |  |  | - set end = Message Number - AIDC-end Req |  |
|  | User-abort Req |  | - A-ABORT Req - STA3 | - AIDC-usr-abrt Req - STA3 | - STA0 |
|  | All other Req |  |  | - equivalent AIDC-ASE Req |  |
| From AIDC-ASE (upper) | AIDC-ucf Ind+ |  |  | if end = Reference ID - User-confirmation Ind+ - A-ABORT Req - STA3 if end = NULL - User-confirmation Ind+ |  |
|  | AIDC-ucf Ind- |  |  | if end = Reference ID - User-confirmation Ind- - set end = NULL if end = NULL - User-confirmation Ind- |  |
|  | AIDC-usr-abrt Ind |  | - User-abort Ind - STA0 | - User-abort Ind - STA0 | - User-abort Ind - STA0 |
|  | AIDC-pvd-abrt Ind |  | - Provider-abort Ind - STA0 | - Provider-abort Ind - STA0 | - Provider-abort Ind - STA0 |
|  | All other Ind |  |  | - equivalent AIDC-User service invocation |  |
| From AIDC-ASE (lower) | AIDC-DATA Req |  |  | - P-DATA Req (APDU) |  |
|  | AIDC-ABORT Req |  | - A-ABORT Req - STA3 | - A-ABORT Req - STA3 |  |
|  | A-ASSOCIATE Ind |  | - A-ASSOCIATE Rsp+ |  |  |
|  | A-ASSOCIATE Cnf+ |  | - if nfy-assc: AIDC-nfy Req ¦ crd-assc: AIDC-crd-start Req - STA2 |  |  |
| From ACSE (upper) | A-ASSOCIATE Cnf- |  | - Provider-abort Ind - STA0 |  |  |
|  | A-ABORT Ind |  | - Provider-abort Ind - STA0 | - AIDC-ABORT Ind - STA3 | AIDC-ABORT Ind |
|  | A-P-ABORT Ind |  | - Provider-abort Ind - STA0 | - AIDC-P-ABORT Ind - STA3 | AIDC-P-ABORT Ind |
|  | P-CONNECT Req |  | - P-CONNECT Req |  |  |
|  | P-CONNECT Rsp+ |  | - P-CONNECT Rsp+ - STA2 |  |  |
| From ACSE (lower) | P-CONNECT Rsp- |  | - P-CONNECT Rsp- - STA0 |  |  |
|  | P-U-ABORT Req |  | - P-U-ABORT Req - STA0 | - P-U-ABORT Req - STA0 | - P-U-ABORT Req - STA0 |
|  | P-CONNECT Ind | - P-CONNECT Ind - STA1 |  |  |  |
|  | P-CONNECT Cnf+/- |  | - P-CONNECT Cnf |  |  |
| From ATN Service Provider | P-DATA Ind |  |  | - P-DATA Ind |  |
|  | P-U-ABORT Ind |  | - P-U-ABORT Ind | - P-U-ABORT Ind | - P-U-ABORT Ind |
|  | P-P-ABORT Ind |  | - P-P-ABORT Ind | - P-P-ABORT Ind | - P-P-ABORT Ind |