# COMMUNICATION REQUIREMENTS

## Encoding Rules

### The AIDC application shall use PER as defined in reference [1], using the Basic Unaligned variant to encode/decode the ASN.1 message structure and content specified in section X, or a functionally equivalent means which provides the same result.

## Quality-of-Service Requirements

#### The application service priority for AIDC shall be have the abstract value of “normal priority flight safety messages”.

#### The application service RER for AIDC is required to be 10-7 or better.

#### Routing Policy

##### The Class of Communications for AIDC shall not be specified.  This corresponds to no routing preference, and “best effort” transit delay times by the internet.

### Application Entity Qualifier

#### The title of the AIDC application used as the AE-Qualifier for the AIDC application shall be the IA5 character string “IDC”.