# THE AIDC-ASE Abstract Service

## Introduction

- ¾ This section defines the abstract service interface used by an AIDC-ASE user to access the AIDC-ASE services and the services assumed to be supporting the AIDC-ASE.

## Standard Parameters

- ¾ The following service primitive parameters are defined here rather than repeated in each section.

### User Data

#### The User Data parameter, if provided, shall contain data provided by the user of the AIDC-ASE service.

- ¾ The User Data parameter conforms to one of the following ASN.1 abstract syntaxes:

- Notiy,

- CoordinateStart,

- CoordinateEnd,

- CoordinateNegotiate,

- CoordinateStandby,

- TransferInitiate,

- TransferRequest,

- TransferConditionsProposal,

- TransferConditionsAccept,

- TransferControl,

- TransferControlAssume,

- TransferControlReject,

- TransferCommunication,

- TransferCommunicationAssume,

- GeneralExecutiveData,

- GeneralPoint,

- SurveillanceData,

- GeneralFreeText,

- EmergencyFreeText,

- Cancel.

### Msg Number

#### The Msg Number parameter shall be provided by the AIDC-ASE user accept when invoking the AIDC-usr-abort or AIDC-ucf request primitives.

#### The Msg Number parameter shall consist of a unique identifier for reference in the AIDC-ucf primitives.

- ¾ The Msg Number parameter conforms to the ASN.1 abstract syntax MessageNumber.

## AIDC-ASE User Services

### List of AIDC-ASE Services

#### An implementation of the AIDC-AE shall exhibit behaviour consistent with having implemented an AIDC-ASE with the following abstract services:

- AIDC-ucf service as defined in section 4.3.2

- AIDC-nfy service as defined in section Note7.

- AIDC-crd-start service as defined in section 4.3.4

- AIDC-crd-end service as defined in section 4.3.5

- AIDC-crd-ngtt service as defined in section Note8.

- AIDC-crd-stndby service as defined in section 4.3.7

- AIDC-tfr-init service as defined in section 4.3.8

- AIDC-tfr-rqst service as defined in section 4.3.9

- AIDC-tfr-prpsl service as defined in section 4.3.10

- AIDC-tfr-accept service as defined in section 4.3.11

- AIDC-tfr-cntrl service as defined in section 4.3.12

- AIDC-tfr-comm service as defined in section Note9.

- AIDC-tfr-comm-assm service as defined in section 4.3.14

- AIDC-inf-tfr service as defined in section 4.3.15

- AIDC-end service as defined in section 4.3.16

- AIDC-usr-abrt service as defined in section 4.3.17

- AIDC-pvd-abrt service as defined in section 4.3.18

### AIDC-ucf Service

#### Service Primitives

##### The AIDC-ucf service shall be an unconfirmed service.

##### Table 4-1 specifies the parameters that shall be passed when the primitives of the AIDC-ucf service are invoked.

Table 4-1: AIDC-ucf Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Result | M | M(=) |
| Reason | U | C(=) |
| Reference ID | M | M(=) |

##### Result

###### The Result parameter shall be provided by the AIDC-ASE user.

###### The Result parameter shall indicate the acceptance or rejection of the service primitive.

- ¾ The Result parameter conforms to the ASN.1 abstract syntax Result.

##### Reason

###### The Reason parameter shall be provided by the AIDC-ASE user.

- ¾ The Reason parameter conforms to the ASN.1 abstract syntax ApplicationErrorData.

##### Reference ID

###### The Reference ID parameter shall be provided by the AIDC-ASE user.

###### The Reference ID parameter shall contain the Msg Number of the service that is being accepted or rejected.

- ¾ The Reference ID parameter conforms to the ASN.1 abstract syntax ReferencedNumber.

### AIDC-nfy Service

#### Service Primitives

##### The AIDC-nfy service shall be an unconfirmed service.

##### Table 4-2 specifies the parameters that shall be passed when the primitives of the AIDC-nfy service are invoked.

Table 4-2: AIDC-nfy Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-crd-start Service.

#### Service Primitives

##### The AIDC-crd-start service shall be an unconfirmed service.

##### Table 4-3 specifies the parameters that shall be passed when the primitives of the AIDC-crd-start service are invoked.

Table 4-3: AIDC-crd-start Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-crd-end Service

#### Service Primitives

##### The AIDC-crd-end service shall be an unconfirmed service.

##### Table 4-4 specifies the parameters that shall be passed when the primitives of the AIDC-crd-end service are invoked.

Table 4-4: AIDC-crd-end Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Result | M | M(=) |
| Msg Number | M | M(=) |

##### Result

###### The Result parameter shall be provided by the AIDC-ASE user.

###### The Result parameter shall be used to indicate acceptance or rejection of the ending of the Coordinating Regime.

- ¾ The Result parameter conforms to the ASN.1 abstract syntax Result.

### AIDC-crd-ngtt Service

#### Service Primitives

##### The AIDC-crd-ngtt service shall be an unconfirmed service.

##### Table 4-5 specifies the parameters that shall be passed when the primitives of the AIDC-crd-ngtt service are invoked.

Table 4-5: AIDC-crd-ngtt Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number |  |  |

### AIDC-crd-stndby Service

#### Service Primitives

##### The AIDC-crd-stndby service shall be an unconfirmed service.

##### Table 4-6 specifies the parameters that shall be passed when the primitives of the AIDC-crd-stndby service are invoked.

Table 4-6: AIDC-crd-stndby Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-init Service

#### Service Primitives

##### The AIDC-tfr-init service shall be an unconfirmed service.

##### Table 4-7 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-init service are invoked.

Table 4-7: AIDC-tfr-init Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-rqst Service

#### Service Primitives

##### The AIDC-tfr-rqst service shall be an unconfirmed service.

##### Table 4-8 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-rqst service are invoked.

Table 4-8: AIDC-tfr-rqst Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-prpsl Service

#### Service Primitives

##### The AIDC-tfr-prpsl service shall be an unconfirmed service.

##### Table 4-9 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-prpsl service are invoked.

Table 4-9: AIDC-tfr-prpsl Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-accept Service

#### Service Primitives

##### The AIDC-tfr-accept service shall be an unconfirmed service.

##### Table 4-10 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-accept service are invoked.

Table 4-10: AIDC-tfr-accept Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-cntrl Service

#### Service Primitives.

##### The AIDC-tfr-cntrl service shall be a confirmed service.

##### Table 4-11 below specifies the parameters that shall be passed when the primitives of the AIDC-tfr-cntrl service are invoked.

Table 4-11: AIDC-tfr-cntrl Service Primitive Parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| User Data | U | C(=) | U | C(=) |
| Result |  |  | M | M(=) |
| Msg Number | M | M(=) | M | M(=) |

##### Result

###### The Result parameter, shall be provided by the AIDC-ASE user.

- ¾ The Result parameter conforms to the ASN.1 abstract syntax Result.

### AIDC-tfr-comm Service

#### Service Primitives

##### The AIDC-tfr-comm service shall be an unconfirmed service.

##### Table 4-12 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-comm service are invoked.

Table 4-12: AIDC-tfr-comm Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-tfr-comm-assm Service

#### Service Primitives.

##### The AIDC-tfr-comm-assm service is an unconfirmed service.

##### Table 4-13 specifies the parameters that shall be passed when the primitives of the AIDC-tfr-comm-assm service are invoked.

Table 4-13: AIDC-tfr-comm-assm Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-inf-tfr Service

#### Service Primitives.

##### The AIDC-inf-tfr service shall be an unconfirmed service.

##### Table 4-14 specifies the parameters that shall be passed when the primitives of the AIDC-inf-tfr service are invoked.

Table 4-14: AIDC-inf-tfr Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-end Service

#### Service Primitives.

##### The AIDC-end service shall be an unconfirmed service.

##### Table 4-15 specifies the parameters that shall be passed when the primitives of the AIDC-end service are invoked.

Table 4-15: AIDC-end Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| User Data | U | C(=) |
| Msg Number | M | M(=) |

### AIDC-usr-abrt Service

#### Service Primitives.

##### The AIDC-usr-abrt service shall be an unconfirmed service.

##### The AIDC-usr-abrt service primitives shall have no parameters.

### AIDC-pvd-abrt Service

- ¾ The purpose of the AIDC-pvd-abrt service is to allow the AIDC-ASE to inform its user that it can no longer support the AIDC service.

#### The AIDC-pvd-abrt service shall be an AIDC-ASE service-provider initiated service.

#### Service Primitive.

##### The AIDC-pvd-abrt service shall be an unconfirmed service.

##### Table 4-16 specifies the parameter that shall be passed when the primitive of the AIDC-pvd-abrt service is invoked.

Table 4-16: AIDC-pvd-abrt Service Primitive Parameter

| Parameter Name | Ind |
| --- | --- |
| Reason | M |

##### Reason

###### The Reason parameter shall be provided by the AIDC-ASE.

###### The Reason parameter shall be used to identify the reason for the abort.

- ¾ The Reason parameter conforms to the ASN.1 abstract syntax Reason.

## Services Supporting the AIDC-ASE

- ¾ The AIDC-ASE may be incorporated in any application entity that provides the following services.

### List of Supporting Services

#### An implementation of the AIDC-AE shall exhibit the behaviour consistent with having implemented an AIDC-ASE supported by the following abstract services:

- AIDC-DATA service as defined in section 4.4.2

- AIDC-ABORT service as defined in section Note13.

- AIDC-P-ABORT service as defined in section 4.4.4

### AIDC-DATA Service

#### Service Primitives

#### The AIDC-DATA service shall be an unconfirmed service.

##### Table 4-17 specifies the parameter that shall be passed when the primitives of the AIDC-DATA service are invoked.

Table 4-17: AIDC-DATA Service Primitive Parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| AIDC Data | U | C(=) |

##### AIDC Data

###### The AIDC Data parameter shall be provided by the AIDC-ASE.

- ¾ The AIDC Data parameter is the PER encoded AIDC-PDU.

### AIDC-ABORT Service

#### Service Primitives

##### The AIDC-ABORT service is an unconfirmed service.

##### The AIDC-ABORT service primitives shall have no parameters.

### AIDC-P-ABORT Service

#### Service Primitives

##### The AIDC-P-ABORT service shall be an unconfirmed service.

##### The AIDC-P-ABORT service primitives shall have no parameters.