# APPLICATION OVERVIEW

## Introduction

### Purpose

#### Recommendation.¾ AIDC is an ATN application which should be employed by two Air Traffic Service (ATS) units when exchanging Air Traffic Control (ATC) information for an active flight.

- ¾ This relates to flight notification, flight coordination, transfer of control, surveillance data and free (i.e. unstructured) text data.

#### The Aeronautical Telecommunication Network (ATN) shall provide the media and protocols to conduct data communications for the AIDC application up to, and including, the presentation layer.

#### Compliance with these standards is a means of ensuring that an implementaion of the AIDC application shall perform its intended functions, and that the AIDC application is implemented in a globally uniform and interoperable manner.

### Background

- ¾ The AIDC application exchanges information between ATS Units (ATSUs) for support of critical Air Traffic Control (ATC) functions, such as notification of flights approaching a Flight Information Region (FIR) boundary, coordination of boundary conditions and transfer of control or communication.

- ¾ AIDC is strictly an ATC application for exchanging tactical control information between ATS units, not with other offices or facilities.

#### In performing this role, the AIDC application shall interact with the following:

- the Aeronautical Telecommunication Network (ATN); and

- the AIDC user.

### Structure of Document

- ¾ Chapter 1: APPLICATION OVERVIEW identifies the document’s purpose and structure, and contains a summary of the operational requirements that relate to AIDC.

- ¾ Chapter 2: GENERAL REQUIREMENTS contains performance, time accuracy, security, backwards compatibility, and error processing requirements.

- ¾ Chapter 3: THE AIDC-AE ABSTRACT SERVICE contains the description of the primitives used by the AIDC-user when invoking the AIDC service.

- ¾ Chapter 4: THE AIDC-ASE SERVICE describes the AIDC-ASE service primitives.

- ¾ Chapter 5: THE AIDC CONTROL FUNCTION describes the Control Function (CF) mapping of the AIDC-user invoked primitives, the AIDC-ASE service, the ACSE service and the service provided by the communications service provider.

- ¾ Chapter 6: THE AIDC-ASE PROTOCOL DEFINITION describes the exchanges of messages allowed by the AIDC protocol, as well as time constraints and AIDC-ASE protocol descriptions.

- ¾ Chapter 7: COMMUNICATION REQUIREMENTS contains the requirements that the AIDC application imposes on the underlying communication system.

- ¾ Chapter 8 : AIDC USER REQUIREMENTS defines the requirements that the user of the AIDC service must meet.

### Explanation of Terms

#### Acronyms

| AE | Application Entity |
| --- | --- |
| AIDC | ATS Interfacility Data Communication |
| APDU | Application Protocol Data Unit |
| ASE | Application Service Element |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Air Traffic Control |
| ATM | Air Traffic Management |
| ATN | Aeronautical Telecommunication Network |
| ATS | Air Traffic Services |
| ATSU | Air Traffic Services Unit |
| C-ATSU | Controlling ATS Unit |
| CF | Control Function |
| CNS | Communication Navigation Surveillance |
| D-ATSU | Downstream ATS Unit |
| IA5 | International Alphabet No. 5 |
| ICAO | International Civil Aviation Organization |
| ICC | Inter Centre Communications |
| ID | Identification |
| ISO | International Organization for Standardization |
| OSI | Open Systems Interconnection |
| PDU | Protocol Data Unit |
| PER | Packed Encoding Rules |
| QOS | Quality of Service |
| R-ATSU | Receiving ATS Unit |
| RCP | Required Communication Performance |
| RER | Residual Error Rate |
| SARPs | Standards and Recommended Practices |
| UTC | Coordinated Universal Time |

- (not used)

#### General Definitions

##### For the purpose of this document the following definitions shall apply:

- AE Qualifier: that part of the AE title that uniquely identifies the particular application entity.

- AE Title: a unique name for an application entity.

- AIDC abstract service interface: the abstract interface between the AIDC-AE and the AIDC-User.

- AIDC-ASE: an abstract part of a system which performs the communication related functions of AIDC.

- AIDC-AE CF: that abstract part of the application entity which performs the mapping of all service primitives at all service interfaces of the AIDC-AE.

- AIDC-User: the abstract part of a system which performs the non communication related functions of AIDC.

- AIDC service provider: the AIDC-service provider is composed of two peer AIDC AEs, all underlying data communication protocol entities and the physical media.

- Application Entity (AE): a model of those aspects of an application process that are significant from the viewpoint of accessing OSI capabilities.

- Application Process: an element within an open system which performs information processing tasks for a particular application.

- APDU: basic unit of information exchanged between the AIDC-AE peers.

- ASE: an abstract element of a system providing service to other parts of the system.

- ATSU1: Used to denote an arbitrary ATS unit.

- ATSU2: Used to denote an arbitrary ATS unit other than ATSU1.

- C-ATSU: The ATS unit currently responsible for control (i.e. the controlling ATS unit).

- D-ATSU: a downstream ATSU is an ATS unit responsible for control of an airspace region through or near which the aircraft will progress at some future time.

- ICAO Facility Designator (ICAO AFTN Addressee Indicator): An eight-letter code group formulated in accordance with rules prescribed by ICAO and assigned to the ATS end system executing an application process.

- Message: The basic unit of information exchanged between the Initiating AIDC users.

- R-ATSU: the ATS unit that will assume control responsibility (i.e. the receiving ATS unit).

- Service primitives: the actions at the service interfaces of the AE or ASE.

#### Conventions For Expressing Requirements

##### The following conventions shall apply for expressing requirements in this document:

- shall - used to state a mandatory requirement.

- should - used to state a recommended practice.

### References

- Draft ICAO Manual of ATS Data Link Applications, March 1996.

- ISO/IEC DIS 8825-2, ITU-T Recommendation X.691, Packed Encoding Rules (PER)

- ISO/IEC 8824-1, ITU-T Recommendation X.682, Abstract Syntax Notation One (ASN.1)

- ISO 8824-1/Amd.1, Information Technology - Open Systems Interconnection -- Abstract Syntax Notation One (ASN.1) - Specification of Basic Notation - Amendment 1: Rules for Extensibility.

- Draft SARPs  for ATN Upper Layers for the CNS/ATM-1 Package, (as proposed in February 1995).

- TBD

## Application Functionality supported by these SARPs

### This section lists the ADSP derived operational services that shall be supported by these SARPs .

- Approval for a Flight to Enter ADS-ATC Airspace.

- Automatic Transfer of Control and Communications Between Airspaces Using Digital Data Interchange.

### Descriptions of AIDC Related Functions

#### Flight Notification

##### This function shall allow the Controlling ATS Unit (C-ATSU) to notify the Downstream ATS Unit (D-ATSU) of a flight’s cleared profile some time before the flight enters the D-ATSU’s area of interest.  This function may be initiated a multiple number of times for the same flight, depending on the number and type of changes made to the flight’s cleared profile.

#### Flight Coordination

##### This function shall allow the C-ATSU to coordinate the conditions of transfer for a flight with a D-ATSU..

#### Transfer of Control

##### This function shall allow the C-ATSU to transfer control authority for a flight to the R-ATSU and allows the R-ATSU to accept the control authority for the flight.

#### Transfer of Communications

##### This function shall allow one of the following to take place:

- the C-ATSU to transfer the control and communications authority for a flight to the R-ATSU and the R-ATSU to accept the control and communications authority for the flight; or

- the R-ATSU to take the control authority and communications authority for a flight.

#### Transfer of Surveillance Data

##### This function shall allow an ATSU1 to transfer surveillance data to an ATSU2.

#### Transfer of General Data

##### This function shall allow an ATSU1 to exchange flight related free text messages (i.e. unstructured) with an ATSU2.

## The AIDC Functional Model

### Figure 1-1 shows the functional model of the AIDC Application.

### The functional elements identified in this model are the following :

- the AIDC-User,

- the AIDC Application Entity (AIDC-AE) Service Interface,

- the AIDC-AE,

- the AIDC Control Function (CF),

- the AIDC Application Service Element (AIDC-ASE) Service Interface,

- the AIDC-ASE,

- the Association Control Service Element, and

- the ATN Service Provider Interface.

Figure 1-1: Functional Model of the AIDC Application

- ¾ The AIDC-User represents the operational part of the AIDC system.  This user does not perform the communication functions but relies on a communication service provided by the AIDC-AE through the AIDC-AE service interface.

- ¾ The AIDC-ASE is the element in the communication system which executes the AIDC specific protocol.  In other words, it enforces the AIDC specific primitive sequencing actions, timer management, and error handling.

- ¾ The AIDC-AE CF is responsible for mapping primitives received from one element to another within the AIDC functional model.

## Modelling Conventions

- ¾ In modelling the AIDC application, each service description includes a table listing both the service primitives and the parameters of the service.

- ¾ For a given primitive, the presence of each parameter is described by one of the following values in the parameter tables.

- blank	not applicable;

- C	conditional upon some predicate explained in the text;

- C(=)	conditional upon the value of the parameter to the left being present.  In addition, the value of the parameter is equal to the value of the parameter to the left;

- M	mandatory;

- M(=)	mandatory.  In addition, the value of the parameter is equal to the value of the parameter to the left;

- U	user option.

- ¾ The following abbreviations are used in this document:

- Req - request; data is input by the user initiating the service.

- Ind - indication; data is indicated by the service to its respective user.

- Rsp - response; data is input by the service user.

- Cnf - confirmation; data is confirmed by the service to its respective user.

- ¾ A confirmed service is requested, indicated, responded and confirmed between peer service users.

- ¾ An unconfirmed service is only requested and indicated between peer users.