|  | ATNP/WG3/IP/16-01 11 May 1999 |
| --- | --- |

- AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

- WORKING GROUP 3

- Naples, May 1999

- Potential future Operational Requirements for the Generic ATN Service - GACS

- (Information Paper)

- Prepared by  Danny Van Roosbroek

- Presented by: Danny Van Roosbroek

- SUMMARY

This information paper provides a preview of emerging Operational Requirements which can be considered as candidates to be supported by the Generic ATN Communications Service (GACS).

# Introduction

The ATNP/WG3 and its Sub Group on Upper layers is developing draft SARPs for the Generic ATN Communications Service.  The validation of the draft SARPs has started and the results can be expected at the upcoming ATNP/WG3 September meeting,  in time for ATNP/3.

At the last Working Group 3 meeting it was noted that GACS is a building block of the ATN infrastructure providing services to new ATS Applications or legacy AOC applications.

While the Operational Requirements definition for ATS Applications is the responsibility of the ADS Panel, the ATN Panel is responsible – amongst others -  to develop the SARPs for the supporting communication mechanisms.

It can be expected that the ADSP will develop new operational services from January 2000 onwards.  Within Europe, such new services are being discussed and specified within the Eurocontrol EATMP (European ATM Programme) ODIAC subgroup for submission to the ADSP during next year.  This document provides a selection of these services which can be considered as candidate Operational Requirements for GACS subject to the discussions in the ADS Panel.  This document provides a preview of two such candidate Services : Common Trajectory Co-ordination (COTRAC  and Pilot Preference Downlink (PPD).

Chapter 3 of the document describes how the Airlines Electronics Engineering Committee (AEEC) intents to use GACS within the context of AOC applications.

# References

[1]	Operational Requirements for Common Trajectory Co-ordination (COTRAC)
Attachment O, ODIAC/24 (16-18 February 1999)

[2]	Operational Requirements for Pilot Preference Downlink (PPD)
ODIAC 25 – WP6/25

# Potential Future Operational Services

The ATS service descriptions below are working draft versions of the Eurocontrol ODIAC sub-group and are provided as a status of the work in progress by that group.  While the two services relate to A/G exchange of information, new G/G exchanges in the context of Collaborative Decision Making (CDM) and more advanced Flight Data Processing co-ordination are being considered for possible use of GACS but are at this moment not mature.

The text below provides a short overview of the COTRAC and PPD Services.  The complete description of these services is contained in the attachment which is only distributed in soft copy.

## Common Trajectory Negotiation (COTRAC) service.

The purpose of COTRAC is to establish and agree trajectory contracts between aircrew and controllers in real time using graphical interfaces, air and ground data communications and automation systems, in particular the FMS, by means of a structured negotiation method in order to significantly enhance ATM capacity and flexibility.

The coordination of trajectories can be performed more effectively by involving Airline operations directly or through Collaborative Decision Making and maximizing ground/ground data communications to reduce costs.

Ground automation tools such as conflict detection, arrival managers, monitoring aids, can be enhanced through the availability of accurately predictable trajectories.

COTRAC will also contribute to the optimal use of the Flexible Use of Airspace (FUA) and Route Navigation (R-NAV) concepts.

Airborne situational awareness is improved through the display of flight path intent of aircraft affecting own trajectory, e.g. in case of parallel approaches.

Note:	It is not the purpose of COTRAC to determine how a trajectory can be used to enhance automation tools.

## Pilot Preferences Downlink (PPD)

The objective of the PPD service is to enhance ATC planning through the provision of Aircrew preferred parameters to the Controller.

The PPD service allows Aircrew to provide the Controller with information not available in the filed flight plan (e.g.  maximum flight level) as well as requests for modification of some flight plan elements (e.g.  requested flight level).

This information forms an important addition to Controller decision making, enabling a better understanding of Aircrew needs and improving the Controller’s assessment of the traffic situation.  Therefore the control process is enhanced by becoming more responsive to Aircrew needs and improving its efficiency.

The PPD service is foreseen to be initially used in continental airspace.

# Status of GACS use by AEEC

The Airlines Electronics Engineering Committee (AEEC) reviewed the draft GACS Sarps during its 637A adhoc working group meeting in April; and decided to adopt GACS as the end state for ACARS Applications using the ATN based on an AE (Application Entity) approach.  The AEEC intends to refer to the GACS SARPS once ICAO has completed the validation process and ATNP/3 has approved the draft SARPs.  In the meantime the draft GACS SARPs will included in the AEEC 637 material.

# Recommendations

The Working Group is invited to note the information in this paper.

# Attachment 1 : COTRAC (Only in Soft Copy)

# Attachment 2 : PPD (Only in Soft Copy)