ATNP WG3/SG3 WP16-34

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

WG3 – Applications and upper layers

SG3 – Upper Layer Architecture

Napoli, May the 17th, May the 28nd, 1999

Sv4.8

Prepared by G. Mittaux-Biron

Summary

ICAO is in the process of standardising the requirements for ATN Security. This will allow the development of a secured environment of data transfer between airborne and ground ATN systems, taking into account the various local legislation constraints.

This document presents the upper layers SARPs chapter 4.8 which specifies the implementation of security in the ATN Upper Layers.

- Security Application Service Object description

  - Scope and Objectives

    - Implementations of the Security ASO, together with the UL elements which provide a secured Dialogue Service (DS), shall exhibit the behaviour defined in this chapter.

    - The Security ASO is included in the Application Layer of the ATN, under the Dialogue Service.

This chapter specifies how ATN Upper Layers use OSI standardised mechanisms in order to secure elements of the various ATN ASEs. It also describes how the ATN upper layers use the OSI Directory authentication framework in order to provide support for peer entities authentication in the ATN environment.

Figure 4.8.1 presents the model for the secured ATN Upper Layers Architecture.

- Overview of secured ATN Application Entity

    - A secured Application Entity (AE) contains the following functional components:

- An ATN Application ASE.

- A Dialogue Control Function.

- An Association Control Service Element (ACSE), in charge of managing the association establishment and release.

- A Security ASO, which includes:

- A Security Control Function in charge of the co-ordination of the various security related service activation,

- A Security Exchange Service Element (SESE), in charge of the incorporation of the security related information in the exchanges.

- the SSO (System Security Object) in charge of the processing of the security related functions. The SSO implements a set of  System Security Functions (SSF). A System Security Function (SSF) is a capability of the system to perform security related processing.

- The Security ASO provides a mean to transparently secure the communications which take place between peer ATN-App ASEs. The ATN-App ASEs specify their willingness in term of security through the Security Requirements of the D-START service, and receive, in response, the level of security which is actually implemented by the Dialogue.

- No encryption mechanisms other than those involved in authentication, in data integrity , or in the confidential exchange of keys are provided in the security ASO.

- Security mechanisms are part of the AE protocol: in case of any security related problem during authentication or data integrity check, the  dialogue shall be aborted using the exception handling procedure of the Dialogue Control function.

- for security reason, nor shall tThe ATN-App ASEs involved in which originated the dialogue, in case of security problem occurring during the authentication phase, or both ATN-App ASEs, in case of security problem occurring during  the data integrity check, shall be aware of the abortion of the Dialogue via the be aware of security problems which could happen during the exchangereception of a D-PABORT indication.

- Appropriate security error related events shall be logged and their processing, together with the appropriate recovery of security problems actions shall take place under System Management activities.

- Neither Key management related activities nor their transfer is described in the Security ASO.

  - Services offered by the Security ASO

    - Implementations of the ATN-App ASE, together with the UL elements which provide the Dialogue Service (DS) including the Security ASO, shall exhibit the behaviour defined in this abstract service definition.

- The Secured Association Service (SA Service) is the abstract service which is used by a Security ASO user at its lower service boundary.  There is no requirement to implement the SA Service in any product.  ATN end systems will in general be designed in such a way that it is impossible to detect (from external access) whether or not an interface corresponding to the SA Service has been built.

- The SA Service is described from the viewpoint of its user, using abstract service definition conventions. The abstract service definition is a descriptive technique used to specify the behaviour exhibited by part of the ATN application layer.  Specifications of application service elements (ASEs), may include common functionality by reference to the SA Service.  The SA Service allows ASEs to be specified without the need to consider the complexities of the security mechanisms implemented in the underlying communications.

- The SA Service supports a communication relationship between two peers for a duration which exists until the peers agree to terminate the relationship or the relationship is aborted.

- The SA Service defines a service which may be used to support the Dialogue Control function at its “lower” service boundary.  The Dialogue Control function is denoted the SA-User.   The SA-User can be specified to use the SA Service in a variety of ways that can be defined in terms of reliability characteristics.

- An implementation of the SA Service provider will typically be responsible for detection of errors such as:

- Invalid primitive (primitive unknown or error in parameter(s))

- Invalid sequence (primitive issued at inappropriate time)

- Insufficient resources on submission

- Invalid or unreachable recipient on submission

- Data field too large on receive (local implementation constraint exceeded)

- Invalid or unreachable recipient on receive,

- Failure in authentication,

- Failure in data integrity check,,

- Invalid security information.

- An implementation of an ATN application which makes use of the SA Service has to be designed with error handling procedures for local error conditions.

  - Service Primitives

    - Implementations which claim to support the SA Service functionality shall exhibit the behaviour defined by the service primitives in Table 4.8-1.

    - Summary of SA Service primitives

| Service | Description |
| --- | --- |
| SA-AUTHENTICATE | This is a confirmed service used to authenticate two communicating entities and optionally provide data integrity. |
| SA-SIGNED | This unconfirmed service is used by a SA-User to sign a message to be sent. |
| SA-TERMINATE | This unconfirmed service can be invoked to terminate the authentication process. |

- -Table 4.8-2 lists the parameters used when invoking the SA services.

    - Parameters of the SA Service primitives

| Service | Parameters |
| --- | --- |
| SA-AUTHENTICATE | Security Requirements To be signed Check result |
| SA-SIGNED | To be signed Check results |
| SA-TERMINATE | Origin |

  - Security Requirements

    - An implementation of an ATN application which makes use of the SA Service shall provide the following security services:

- Peer entities authentication: this service shall protect ATN communication systems from threats that fall in the scope of identity interception.

- Peer entities authentication takes place during Dialogue establishment takes place during Dialogue establishment.

- Data integrity: this service shall protect ATN communication systems from threats that fall in the scope of masquerade or of data manipulation.

- Protection of data integrity takes place both during Dialogue establishment, release and during data transfer.

- Confidentiality: this service shall protect ATN communication systems from threats that fall in the scope of unauthorised disclosure of security information.

- Protection of data confidentiality takes place during Dialogue establishment, and its only target is the confidential exchange of secret keys used to implement peer entities authentication and data integrity.

    - Security services abstract values and object identifiers:

      - The object identifier associated to Unsecured Dialogue Service shall be:

unsecuredDS OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3)icao (27) atn-security-requirements (4)  security-services (1) unsecured-dialogue-service (1)}

- If unsecured dialogue service is chosen, then no protection is provided on the dialogue neither for its establishment, nor during the data exchanges.

      - The object identifier associated to Secured Dialogue Service shall be:

securedDS OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27)  atn-security-requirements (4)  security-services (1) secured-dialogue-service (2)}

- If Secured Dialogue Service is chosen, then peer entities authentication is carried out during the dialogue establishment and maintenance. No security service is provided during the data exchanges.

      - The object identifier associated to the Forward Path Secured Application Dialogue Service shall be:

forwardPathSecuredDS OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1) forward-path-secured-application-dialogue-service (3)}

- If forward path secured application dialogue service is chosen then peer entities authentication is carried out during the dialogue establishment, and data integrity is checked for all data issued by the initiator of the dialogue.

      - The object identifier associated to the Return Path Secured Application Dialogue service shall be:

returnPathSecuredDS OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1) return-path-secured-application-dialogue (4)}

- If return path secured application dialogue service is chosen then peer entities authentication is carried out during the dialogue establishment and maintenance, and data integrity is checked for all data issued by the acceptor of the dialogue.

      - The object identifier associated to the Secured Application Dialogue service shall be:

securedApplication OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1)  secured-application-dialogue-service (5)}

- If secured application dialogue service is chosen, then peer entities authentication is carried out during the dialogue establishment and maintenance, and data integrity is checked for all data issued by both entities involved in the dialogue.

- The Security Requirements parameter allows the SA Service Users to exchange and agree on requirements for security.  The parameter is mandatory in the SA-AUTHENTICATE primitives.

      - In order to provide flexibility and adaptability to various legislation and operational conditions, the initially required level of security shall be negotiated during the establishment of the dialogue by the ATN-App ASEs involved in the Dialogue establishment. This negotiation shall based on the use of the Security Requirements parameter of the D-START service: if the requirements specified by the ATN-App ASE initiating the dialogue can be fulfilled by the peer ATN-App ASE, then the value returned in the response shall be identical to the one received. Otherwise, the initially required value shall be downgraded by the receiving ATN-App ASE in order to conform with its capabilities.

      - In order to provide flexibility and adaptability to various legislation and operational conditions, the initially required level of security can be negotiated during the establishment of the dialogue by the Security ASO. This negotiation shall be based on the use of the Security Requirements parameter of the D-START service: if the requirements specified by the SA Service user can be fulfilled by the SA Service provider, then the value provided to the receiving ATN-App ASE shall be identical to the one required by the initiating ATN-App ASE. Otherwise, the value provided to the receiving ATN-App ASE in the Security Requirements parameter shall be downgraded by the SA Service provider in order to conform with its capabilities.

      - The negotiation shall take place according to the following rules:

- If the initial requirement abstract value was "Unsecured Dialogue Service" then, in any case, the dialogue shall be established without implementing any security mechanisms.

- If the initial requirement abstract value was "Secured Dialogue Service" then, according to the SA Service Provider and the peer ATN-App ASE, the dialogue shall be established implementing the security mechanisms associated with one of the following:

- a dialogue implementing all the security mechanisms required for secured dialogue service (the abstract value "Secured Dialogue Service" shall be returned in the D-START confirm),

- a dialogue without any security mechanism (the abstract value "Unsecured Dialogue Service" shall be returned in the D-START confirm).

- If the initial requirement abstract value was "Forward path secured application dialogue" then, according to the SA Service Provider and the peer ATN-App ASE, the dialogue shall be established implementing the security mechanisms associated with one of the following:

- a dialogue implementing security on the forward path only (the abstract value "Forward path secured application dialogue" is returned in the D-START confirm),

- a dialogue implementing all the security mechanisms required for secured dialogue service (the abstract value "Secured Dialogue Service" is returned in the D-START confirm),

- a dialogue without any security mechanism (the abstract value "Unsecured Dialogue Service" is returned in the D-START confirm).

- If the initial requirement abstract value was "Return path secured application dialogue" then, according to the SA service provider and the peer ATN-App ASE, the dialogue shall be established implementing the security mechanisms associated with one of the following:

- a dialogue implementing security on the return path only (the abstract value "Return path secured application dialogue" shall be returned in the D-START confirm),

- a dialogue implementing all the security mechanisms required for secured dialogue service (the abstract value "Secured Dialogue Service" shall be returned in the D-START confirm),

- a dialogue without any security mechanism (the abstract value "Unsecured Dialogue Service" shall be returned in the D-START confirm).

- If the initial requirement abstract value was "Secured Application Dialogue" then, according to the SA Service Provider and the peer ATN-App ASE, the dialogue shall be established implementing the security mechanisms associated with one of the following:

- a dialogue implementing all the security mechanisms required for secured application dialogue (the abstract value "Secured Application Dialogue" shall be returned in the D-START confirm),

- a dialogue implementing security on the forward path only (the abstract value "Forward path secured application dialogue" shall be returned in the D-START confirm),

- a dialogue implementing security on the return path only (the abstract value "Return path secured application dialogue" shall be returned in the D-START confirm),

- a dialogue implementing all the security mechanisms required for secured dialogue service (the abstract value "Secured Dialogue Service" shall be returned in the D-START confirm),

- a dialogue without any security mechanism (the abstract value "Unsecured Dialogue Service" shall be returned in the D-START confirm).

  - SA Service definition

    - Sequence of primitives

      - Implementations that claim to support the Security ASO functionality shall exhibit behaviour allowing two users to:

- mutually authenticate during association establishment,

- exchange data with the provision for proof of integrity.

- consistent with the appropriate use of the corresponding service primitive.

- For the purposes of this service definition, it is only valid for the SA Service User to issue and be prepared to receive primitives for one association according to the permitted sequences of SA Service primitives shown in Table 4.8-3, where intersections marked “Y” show possible primitives which may occur after the primitive in the column heading.

    - Sequence of SA Service primitives at one AS Service User

| The AS Service primitive X -> may be followed by the AS Service primitive Y | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - SA-AUTHENTICATE req |  |  |  |  | Y | Y |  |  |
| - SA-AUTHENTICATE cnf | Y |  |  |  |  |  |  |  |
| - SA-AUTHENTICATE ind |  |  |  |  | Y | Y |  |  |
| - SA-AUTHENTICATE rsp |  |  | Y |  |  |  |  |  |
| - SA-TERMINATE req | Y |  | Y |  |  |  |  |  |
| - SA-TERMINATE ind | Y |  | Y |  |  |  |  |  |
| - SA-SIGNED req |  | Y |  | Y |  |  | Y | Y |
| - SA-SIGNED ind |  | Y |  | Y |  |  | Y | Y |

- For compactness, each SA Service primitive is given a number in the column headings in Table 4.8-3; the numbers have the meanings assigned in the row headings. For simplicity, Table 4.8-3 only shows the case where the primitive activation are accepted by the peer. If a SA-AUTHENTICATE request is rejected, then a SA Service User may not issue or receive any other primitives apart from SA-AUTHENTICATE request or indication. A SA-AUTHENTICATE request results in a new instance of communication with the peer SA Service User, so it could occur at any time. A SA-SIGNED request can occur at any time and does result in a new instance of communication. Table 4.8-3 only applies to a single instance of communication.

    - The SA-AUTHENTICATE service

      - The behaviour defined by the SA-AUTHENTICATE service primitive shall be provided to enable the authentication process between two SA Service Users.

- SA-AUTHENTICATE is a confirmed service which is invoked by a SA Service User, the initiator, to start a secured association with a peer SA Service User. SA-AUTHENTICATE request, indication, response and confirmation primitives are defined, as illustrated in Figure 4.8.2.

SA-AUTHENTICATE sequence diagram

- The initiating SA Service User issues a SA-AUTHENTICATE request primitive.  It is not then valid to issue any other primitives (except SA-TERMINATE) until a SA-AUTHENTICATE confirmation is received. When the responding SA Service User receives the SA-AUTHENTICATE indication primitive, it must decide whether or not to accept this instantiation of the SA Service. It may issue only a SA-AUTHENTICATE response or a SA-TERMINATE request primitive.

- The SA-AUTHENTICATE primitives contain a Security Requirements parameter which is used on request and response primitives, in order to specify the user requirements on security and on indication and confirmation, in order to specify the actual selected level of security. The parameters of the SA-AUTHENTICATE primitives are specified in Table 4.8-4.

    - SA-AUTHENTICATE parameters

| Parameter Name | Req | Ind | Rsp | Cnf |
| --- | --- | --- | --- | --- |
| Security Requirements | M | M(<=) | M (<=) | M(=) |
| To be signed | C | - | C | - |

- The To be signed parameter allows the peer SA Service Users to sign the optional user data exchanged during the dialogue establishment.

    - The SA-SIGNED service

      - The behaviour defined by the SA-SIGNED service primitive shall be provided to enable the secured exchange of information between two SA Service Users.

- SA-SIGNED is an unconfirmed service which provides data transfer between peer SA Service Users with data integrity check.  Request and indication primitives are defined, as illustrated in Figure 4.8.3.

SA-SIGNED sequence diagram

- The parameters of the SA-SIGNED primitives are specified in Table 4.8-5.

    - SA-SIGNED parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| To be signed | M | M |

- The To be signed parameter contains the data to be signed.

    - The SA-TERMINATE service

      - The behaviour defined by the SA-TERMINATE service primitive shall be provided to enable the termination of an authentication process, by either SA Service user or by the service provider.

- When issued by the SA Service user, the SA-TERMINATE service request and indication primitives are as illustrated in Figure 4.8.4.

-

- User originated SA-TERMINATE sequence diagram

- When issued by the SA Service provider, the SA-TERMINATE service request and indication primitives are as illustrated in Figure 4.8.5.

Provider originated SA-TERMINATE sequence diagram

- The parameters of the SA-TERMINATE primitives are specified in Table 4.8-6

    - -

    - SA-TERMINATE parameters

| Parameter Name | Req | Ind |
| --- | --- | --- |
| Originator | U | M |

- The Originator parameter is used to distinguish the source of the termination. Its presence in the indication primitive is conditional upon it being specified by the SA Service user in the request primitive.  It can take one of the following abstract values:

- User — the termination originated from the SA Service user; or

- Provider — the termination originated in the -SA Service Provider (including the -SESE).

- If the SA-TERMINATE Originator parameter is not specified, the default value “Provider” is assumed.

  - Application level naming and context definition

    - Naming hierarchy

- The names assigned to upper layers security objects are incorporated in the ATN naming tree. The resulting sub-tree is illustrated in Figure 4.8.6

Naming hierarchy for ATN security

    - Presentation Context Identification

      - The value of the Presentation Context Identification assigned to the Security ASO shall be 2.

  - Control Function Specification

    - Security ASO State Definitions

      - The Security ASO shall behave as if it has a Control Function which can exist only in one of the following states:

- STA0 - This is the initial state of the CF. The CF shall come enter this state

- From any state, and after

- the SESE has validly issued a SE-ABORT indication, and

- the CF has issued a SA-TERMINATE indication, or

- the SA Service user has validly issued a SA-TERMINATE request and

- the CF has issued a SE-UABORT request.

- From state STA05 and after the CF has issued SA-AUTHENTICATE confirmation,

- From state STA15 and after the CF has issued a SATR PDU with the receiver credentials and optional signature for user data, or

- From STA21 and after the CF has issued the SATR with the signed user data

- From STA22 and after the CF has issued a SA-SIGNED indication.

      - Description of states for authentication on initiator side

- Outgoing Signature pending (STA01) - The CF shall enter in this state from STA0 and after

- the SA Service user has issued a SA-AUTHENTICATE request in order to start a peer entity authentication coupled with user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct user data signature.

- Authentication pending (STA02) - The CF shall enter in this state from

- STA01 and after:

- the SESE has issued a SETR PDU in response to a SE-TRANSFER request, with a request to construct user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct a initiator credentials.

- STA0 and after:

- the SA Service user has issued a SA-AUTHENTICATE request in order to start a peer entity authentication coupled without user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct initiator credentials.

- Issued credential (STA03) - The CF enters in this state from STA02 and after

- the SESE has issued a SETR PDU in response to a SE-TRANSFER request, with initiator credentials,

- the CF has issued a SATR PDU in response to a SA-AUTHENTICATE request in order to start a peer entity authentication coupled with optional user data signature.

- Incoming signature pending (STA04) - The CF shall enter in this state from STA03 and after

- the CF has received a SATR PDU containing the receiver's credential together with signature for user data.

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the user data signature.

- Received credentials (STA05) - The CF shall enter in this state

- from STA03 and after

- the CF has received a SATR PDU containing the receiver's credential together without signature for user data.

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the receiver credentials.

- from STA04 and after

- the SESE has issued a SE-TRANSFER indication, with the user data signature,

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the receiver credentials.

      - Description of states for authentication on responder side

- Incoming signature pending (STA11) - The CF shall enter in this state from STA0 and when:

- the CF has received a SATR PDU containing the initiator's credential together with signature for user data.

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the user data signature.

- Authentication pending (STA12) - The CF shall enter in this state from:

- STA0 and after

- the CF has received a SATR PDU containing the receiver's credential without signature for user data.

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the initiator credentials.

- STA11 and after

- the SESE has issued a SE-TRANSFER indication, with the user data signature,

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the initiator credentials.

- Received credentials (STA13) - The CF shall enter in this state from STA12 and when:

- the SESE has issued a SETR PDU in response to a SE-TRANSFER request, with receiver credentials,

- the CF has issued a SA-AUTHENTICATE indication in response to the initial SATR in order to indicate an incoming start of peer entity authentication coupled with optional user data signature.

- Outgoing signature pending (STA14) - The CF shall enter in this state from STA12 and when:

- the SA Service user has issued a SA-AUTHENTICATE response in order to respond to the incoming peer entity authentication, coupled with user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct user data signature.

- Sent credentials (STA15) - The CF shall enter in this state from

- STA13 and after:

- the SA Service user has issued a SA-AUTHENTICATE response in order to respond to the incoming peer entity authentication coupled without user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct initiator credentials.

- STA14 and after:

- the SESE has issued a SETR PDU in response to a SE-TRANSFER request, with a request to construct user data signature,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request to construct a initiator credentials.

      - Description of states for user data integrity

- Outgoing signed data pending (STA21) - The CF shall enter in this state from STA0 and after

- the SA Service user has issued a SA-SIGNED request in order to compute and append a signature to user data,

- the CF has issued the SE-TRANSFER request to the Security Exchange Service Element (SESE), with a request for signed data construction.

- Outgoing signed data pending (STA22) - The CF shall enter in this state from STA0 and after

- the CF has received a SATR PDU containing signed user data,

- the CF has issued the SETR to the Security Exchange Service Element (SESE), with the signed user data.

    - Security ASO CF State Table

      - The Security ASO CF shall behave as if it has a control function in accordance with the state tables specified in Table 4.8-7, Table 4.8-8, and Table 4.8-9, which show diagrammatically the state transitions and actions performed by the CF in response to incoming events.

- The following conventions are used in Table 4.8-7, Table 4.8-8, and Table 4.8-9:

- Incoming events are shown in the first two columns of the state table, and are enumerated in Table 4.8-11.

- When an input event occurs and the state table indicates an action, the CF performs that action.

- Each cell in the state table shows:

- optionally, one or more predicates, denoted “pN”, where N is an integer.  The state and action which follow the predicate are only valid if the predicate is TRUE.  The inverse (logical NOT) of a predicate is indicated by the prefix “~” (tilde character).

- the new state that the CF enters after the action has been performed

- the action, if any, which the CF performs.  The possible actions are outlined in Table 4.8-12.

- Blank cells indicate error conditions.

- When an input event occurs and the state table indicates a state transition, the CF enters the new state after any associated action has been performed.

      - For the purpose of specifying CF behaviour, embedded ASEs (SESE) shall be treated as atomic entities, such that when an input event is invoked by the CF, that event is processed to completion by the ASE and the CF responds to any resulting output events from the ASE, all within the same logical processing thread.

- This provision avoids the need to specify further transient states within the CF.  It does not imply any particular implementation architecture.

      - The following combinations of input events and CF states shall be treated as error conditions:

- The occurrence of an input event other than those listed in Table 4.8-11; or

- A combination of input event and CF state which corresponds to a blank cell in Table 4.8-7, Table 4.8-8, and Table 4.8-9; or

- A combination of input event and CF state which corresponds to a cell in Table 4.8-7 and, Table 4.8-8, and Table 4.8-9 containing one or more predicates, none of which evaluates to TRUE.

      - The error handling shall result in the SA-TERMINATE indication being issued to the SA Service user and to a notification of a security event to system management.

      - In the event of a conflict between the actions implied by the state table and the text in the following paragraphs, the text shall take precedence.

    - ATN-App CF State Table in authentication process
Initiator side

| Event Source | State--> Event | STA0 Null | STA01 Out. Sign. pending | STA02 Auth. pending | STA03 Issued credential | STA04 In Sign. pending | STA05 Received Credential |
| --- | --- | --- | --- | --- | --- | --- | --- |
| From SA Service user | SA-AUTHENTICATE req | p0 and p1: STA01 SE-TRANSFER req ~p0 or ~p1: STA02 SE-TRANSFER req |  |  |  |  |  |
|  | SA-TERMINATE req |  | SE-UABORT req STA0 | SE-UABORT req STA0 | STA0 | SE-UABORT req STA0 | SE-UABORT req STA0 |
| From SESE (upper) | SE-TRANSFER ind |  |  |  |  | STA05 SETR | STA0 SA-AUTHENTICATE cnf |
|  | SE-ABORT ind |  | STA0 SE-TERMINATE ind | STA0 SE-TERMINATE ind |  | STA0 SE-TERMINATE ind | STA0 SE-TERMINATE ind |
| From SESE (lower) | SETR |  | STA02 SE-TRANSFER req | STA03 SATR |  |  |  |
| From supporting service | SATR |  |  |  | p2 and p3 and p0: STA04 SETR ~p2 and ~p3 and ~p0: STA05 SETR |  |  |

    - -

    - ATN-App CF State Table in authentication process
Receiver side

| Event Source | State--> Event | STA0 Null | STA11 In. Sign. pending | STA12 Auth. pending | STA13 Received Credential | STA14 Out Sign. pending | STA15 Sent Credentials |
| --- | --- | --- | --- | --- | --- | --- | --- |
| From SA Service user | SA-AUTHENTICATE rsp |  |  |  | p0 and p1: STA14 SE-TRANSFER req ~p0 and ~p1: STA15 SE-TRANSFER req |  |  |
|  | SA-TERMINATE req |  | SE-UABORT req STA0 | SE-UABORT req STA0 | STA0 | SE-UABORT req STA0 | SE-UABORT req STA0 |
| From SESE (upper) | SE-TRANSFER ind |  | STA12 SETR | STA13 SA-AUTHENTICATE ind |  |  |  |
|  | SE-ABORT ind |  | STA0 SE-TERMINATE ind | STA0 SE-TERMINATE ind |  | STA0 SE-TERMINATE ind | STA0 SE-TERMINATE ind |
| From SESE (lower) | SETR |  |  |  |  | STA15 SE-TRANSFER req | STA0 SATR |
| From supporting service | SATR | p2 and p3 and p0: STA11 SETR ~p2 and ~p3 and ~p0: STA12 SETR |  |  |  |  |  |

    - ATN-App CF State Table in integrity process

| Event Source | State--> Event | STA0 Null | STA21 Out. Sign. pending | STA22 Inc. Sign. pending |
| --- | --- | --- | --- | --- |
| From SA Service user | SA-SIGNED req | p1 and p0: STA21 SE-TRANSFER req |  |  |
| From SESE (upper) | SE-TRANSFER ind |  |  | STA0 SA-SIGNED ind |
|  | SE-ABORT ind |  | STA0 | STA0 |
| From SESE (lower) | SETR |  | STA0 SATR |  |
| From supporting service | SATR | p2 and p0: STA22 SETR |  |  |

    - Predicates used in Table 4.8-7, Table 4.8-8, and Table 4.8-9

| Predicate | Meaning |
| --- | --- |
| p0 | To be signed data provided. |
| p1 | Data integrity on forward path required. |
| p2 | Data integrity on return path required. |
| p3 | Signature provided. |
| p4 | Initiator of the authentication process. |

    - Incoming Event List

| Abbreviated name | Source | Description |
| --- | --- | --- |
| SA-AUTHENTICATE req | SA Service user | SA-AUTHENTICATE Request primitive issued by SA Service User |
| SA-AUTHENTICATE rsp |  | SA-AUTHENTICATE Response primitive issued by SA Service User |
| SA-TERMINATE req |  | SA-TERMINATE Request primitive issued by SA Service User |
| SA-SIGNED req |  | SA-SIGNED Request primitive issued by SA Service User |
| SATR | Supporting service | SA Transfer PDU issued by underlying service provider |
| SE- TRANSFER ind | SESE (upper service boundary) | SE-TRANSFER Indication primitive issued by SESE service |
| SE-ABORT ind |  | SE-ABORT Indication primitive issued by SESE service |
| SETR | SESE (lower service boundary) | SESE Transfer PDU issued by SESE Protocol Machine (SEPM) |

    - Outgoing Event List

| Abbreviated name | Target | Description |
| --- | --- | --- |
| SA-AUTHENTICATE ind | SA Service user | SA-AUTHENTICATE Indication primitive issued to SA Service User |
| SA-AUTHENTICATE cnf |  | SA-AUTHENTICATE Confirm primitive issued to SA Service User |
| SA-TERMINATE ind |  | SA-TERMINATE Indication primitive issued to SA Service User |
| SA-SIGNED ind |  | SA-SIGNED Indication primitive issued to SA Service User |
| SATR | Supporting service | SA Transfer PDU issued to underlying service provider |
| SE- TRANSFER req | SESE (upper service boundary) | SE-TRANSFER Request primitive issued to SESE service |
| SE-ABORT req |  | SE-ABORT Request primitive issued to SESE service |
| SETR | SESE (lower service boundary) | SESE Transfer PDU issued to SESE Protocol Machine (SEPM) |

    - SA-AUTHENTICATE Request primitive

      - When Invoked

        - When SA-AUTHENTICATE Request primitive is invoked by the SA Service user, a new instance of authentication resource is created and the CF enters in the STA0 state.

      - Action Upon Invocation

        - When the SA-AUTHENTICATE Request primitive is validly invoked, the CF shall:

        - Retrieve the security information (local private, remote public, or session keys) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

        - If data integrity is required on the forward path and if the To be signed parameter is provided,

          - Apply an initial encoding to the contents of To be signed according to the initEncRule encoding rules if this parameter exists, or to the INITIAL-ENCODING-RULES default value.

          - Build the signature for contents of To be signed (ATNSignature security exchange item)

          - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the ATNUISignature security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the ATNSignature security exchange item

- Item identifier set to the object identifier attached to the ATNSignature security exchange item.

- Start flag set to true in order to specify that this is the first item of the exchange.

- End flag set to true in order to specify that there should not be more item exchanged.

        - If data integrity is not required on the forward path,

          - Build the initiator credentials (initiatorCredentials security exchange item)

          - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the dirAuthenticationTwoWay security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the initiatorCredentials security exchange item

- Item identifier set to the object identifier attached to the initiatorCredentials security exchange item.

- Start flag set to true in order to specify that this is the first item of the exchange.

- End flag set to false in order to specify that there should be more item exchanged.

    - SA-AUTHENTICATE Response primitive

      - When Invoked

        - The SA-AUTHENTICATE Response primitive may be validly invoked by the SA Service user when the CF is the responding CF and is in STA13 state; if it is in any other state then appropriate error recovery action shall be taken.

      - Action Upon Invocation

        - When the SA-AUTHENTICATE Response primitive is validly invoked, the CF shall:

        - Retrieve the security information (local private, remote public, or session keys) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

        - If data integrity is required on the return path and if user data is provided,

          - Apply an initial encoding to the user data according to the initEncRule encoding rules if this parameter exists, or to the INITIAL-ENCODING-RULES default value.

          - Build the signature for user data (ATNSignature security exchange item)

          - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the ATNUISignature security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the ATNSignature security exchange item

- Item identifier set to the object identifier attached to the ATNSignature security exchange item.

- Start flag set to true in order to specify that this is the first item of the exchange.

- End flag set to true in order to specify that there should not be more item exchanged.

        - If data integrity is not required on the return path,

          - Build the responder credentials (responderCredentials security exchange item)

          - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the dirAuthenticationTwoWay security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the responderCredentials security exchange item

- Item identifier set to the object identifier attached to the responderCredentials security exchange item.

- Start flag set to false in order to specify that this is not the first item of the exchange.

- End flag set to true in order to specify that this is the last item of the exchange.

    - SA-TERMINATE Request primitive

      - When Invoked

        - Invocation of the SA-TERMINATE Request primitive shall be allowed when the CF is in any valid state involving authentication, except the NULL STA0 state.

      - Action Upon Invocation

        - When the SA-TERMINATE Request primitive is validly invoked, the CF shall:

        - if it is in a state where a response is expected from the SESE, as described in Table 4.8-7, Table 4.8-8, and Table 4.8-9, construct a SE-UABORT request primitive,

          - release all resources involved in the authentication process and return in NULL state.

    - SA-SIGNED Request primitive

      - When Invoked

        - When the SA-SIGNED Request primitive is invoked by the SA Service user, a new instance of communication shall be created, with its CF initially in the STA0 state.

      - Action Upon Invocation

        - When the SA-SIGNED Request primitive is validly invoked, the CF shall:

          - Retrieve the security information (local private, remote public, or session keys) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

          - Apply an initial encoding to the user data according to the initEncRule encoding rules if this parameter exists, or to the INITIAL-ENCODING-RULES default value.

          - Build the signature for contents of To be signed (ATNsigned security exchange item)

          - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the atnSignedPDU security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the ATNsigned security exchange item

- Item identifier set to the object identifier attached to the ATNsigned security exchange item.

- Start flag set to true in order to specify that this is the first item of the exchange.

- End flag set to true in order to specify that there should not be more item exchanged.

    - SATR PDU

      - When Invoked

        - The reception of SATR PDU from the supporting service is valid in any of the states as described in Table 4.8-7, Table 4.8-8, and Table 4.8-9

      - Action Upon Invocation

        - When the SATR PDU is validly received, the CF shall decode it the in order to determine the security exchange items contained in it.

        - If the SATR PDU contains the initiatorCredentials security exchange item, then the CF shall provide it to the SEPM.

        - If the SATR PDU contains the responderCredentials security exchange item, then the CF shall provide it to the SEPM.

    - SE-TRANSFER Indication primitive

      - When Invoked

        - The invocation of SE-TRANSFER Indication primitive from the SE Service provider is valid in any of the states as described in Table 4.8-7, Table 4.8-8, and Table 4.8-9

      - Action Upon Invocation

        - When the SE-TRANSFER Indication primitive is validly invoked, the CF shall:

          - If the SE-TRANSFER indication primitive contains the following parameters:

- the security exchange identifier set to the identifier of the ATNUISignature security exchange.

- the invocation identifier set to an unambiguous identifier of the exchange.

- the security exchange item containing the ATNSignature security exchange item

- the item identifier set to the object identifier attached to the ATNSignature security exchange item.

- the start flag is set to true in order to specify that this is the first request of the exchange.

- the end flag set to true in order to specify that this is the last item of the exchange.

            - Retreive the security information (remote public key, session key) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Check the integrity of the user data by checking the ATNSignature, and update the Security Requirements accordingly.

- The SIGNATURE transformation is mapped on the gulsSignatureTransformation security transformation.

            - The CF shall provide the second SETR element of the SATR to the SEPM.

          - If the SE-TRANSFER indication primitive contains the following parameters:

- the security exchange identifier set to the identifier of the dirAuthenticationTwoWay security exchange.

- the invocation identifier set to an unambiguous identifier of the exchange.

- the security exchange item containing the initiatorCredentials security exchange item

- the item identifier set to the object identifier attached to the initiatorCredentials security exchange item.

- the start flag set to true in order to specify that this is the first request of the exchange.

- the end flag is set to false in order to specify that there should be more item exchanged.

            - Retreive the security information (remote public key, session key) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Check the validity of the initiatorCredentials, and update the Security Requirements accordingly.

            - Construct a SA-AUTHENTICATE indication primitive with the following parameter values:

    - SA-AUTHENTICATE indication parameters

| Parameter Name | Ind |
| --- | --- |
| Security Requirements | Set accordingly to presence of the ATN Signature, initiatorCredentials or responderCredentials security exchange items. |

          - If the SE-TRANSFER indication primitive contains the following parameters:

- the security exchange identifier set to the identifier of the dirAuthenticationTwoWay security exchange.

- the invocation identifier set to an unambiguous identifier of the exchange.

- the security exchange item containing the responderCredentials security exchange item

- the item identifier set to the object identifier attached to the responderCredentials security exchange item.

- the start flag is set to false in order to specify that this is not the first request of the exchange.

- the end flag set to true in order to specify that this is the last item of the exchange.

            - Retreive the security information (remote public key, session key) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Check the validity of the responderCredentials, and update the Security Requirements accordingly.

            - Construct a SA-AUTHENTICATE confirm primitive with the following parameter values:

    - SA-AUTHENTICATE confirm parameters

| Parameter Name | Cnf |
| --- | --- |
| Security Requirements | Set accordingly to presence of the ATN Signature, initiatorCredentials or responderCredentials security exchange items. |

          - If the SE-TRANSFER indication primitive contains the following parameters:

- the security exchange identifier set to the identifier of the ATNUISigned security exchange.

- the invocation identifier is set to an unambiguous identifier of the exchange.

- the security exchange item contains the ATNSigned security exchange item

- the item identifier set to the object identifier attached to the ATNSigned security exchange item.

- the start flag is set to true in order to specify that this is the first request of the exchange.

- the end flag set to true in order to specify that this is the last item of the exchange.

            - Retreive the security information (remote public key, session key) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Check the integrity of the user data by checking the ATNSigned.

- The SIGNED transformation is mapped on the gulsSignedTransformation security transformation.

            - Construct a SA-SIGNED indication primitive with the following parameter values:

    - SA-SIGNED indication parameters

| Parameter Name | Ind |
| --- | --- |
| To be signed | Clear user data. |

    - SE-ABORT Indication primitive

      - When Invoked

        - The invocation of SE-TRANSFER Indication primitive from the SE Service provider is valid in any of the states as described in Table 4.8-7, Table 4.8-8, and Table 4.8-9

      - Action Upon Invocation

        - When the SE-ABORT Indication primitive is validly invoked, the CF shall:

        - Construct a SA-TERMINATE indication primitive with the following parameter values:

    - SA-TERMINATE indication parameters

| Parameter Name | Ind |
| --- | --- |
| Originator | SA Service provider |

    - SETR PDU

      - When Invoked

        - The invocation of SE-TRANSFER Indication primitive from the SE Service provider is valid in any of the states as described in Table 4.8-7, Table 4.8-8, and Table 4.8-9

      - Action Upon Invocation

        - When the SETR PDU is validly received, the CF shall:

        - if in state STA01,

            - Retrieve the security information (local private, remote public, or session keys) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Build the initiator credentials (initiatorCredentials security exchange item)

            - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the dirAuthenticationTwoWay security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the initiatorCredentials security exchange item

- Item identifier set to the object identifier attached to the initiatorCredentials security exchange item.

- Start flag set to true in order to specify that this is the first item of the exchange.

- End flag set to false in order to specify that there should be more item exchanged.

        - if in state STA02 or STA15:

          - build the SATR PDU based on the received SETRs and provide it to the SA Service provider for transfer.

        - if in state STA14 the CF shall:

            - Retrieve the security information (local private, remote public, or session keys) together with security parameters (authentication and integrity algorithms, algorithms parameters) as defined for the ATN-App AE.

            - Build the responder credentials (responderCredentials security exchange item)

            - Construct a SE-TRANSFER request service with the following parameters:

- Security exchange identifier set to the object identifier attached to the dirAuthenticationTwoWay security exchange.

- Invocation identifier set to an unambiguous identifier of the exchange.

- Security exchange item set to the responderCredentials security exchange item

- Item identifier set to the object identifier attached to the responderCredentials security exchange item.

- Start flag set to false in order to specify that this is not the first item of the exchange.

- End flag set to true in order to specify that this is the last item of the exchange.

  - ASN.1 Definitions

    - The following ASN.1 definition shall be used to specify the Security ASO protocol:

unsecuredDS		OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3)icao (27) atn-security-requirements (4)  security-services (1) unsecured-dialogue-service (1)

}

securedDS		OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27)  atn-security-requirements (4)  security-services (1) secured-dialogue (2)

}

forwardPathSecuredDS	OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1) forward-path-secured-application-dialogue (3)

}

returnPathSecuredDS	OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1) return-path-secured-application-dialogue (4)

}

securedApplication	OBJECT-IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  security-services (1)  secured-application-dialogue (5)

}

{iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  modules(3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

-- From Directory Standards: --

Name FROM InformationFramework

{joint-iso-ccitt ds(5) module(1) usefullDefinitions (0) 2}

SecurityProblem FROM DirectoryAbstractService

{joint-iso-ccitt ds (5) module (1) directoryAbstractService (2) 2}

StrongCredentials FROM DirectoryAbstractService

{joint-iso-ccitt ds (5) module (1) directoryAbstractService (2) 2}

-- From GULS Modules: -

notation FROM ObjectIdentifiers

{joint-iso-ccitt genericsULS (20 modules (1) objectIdentifiers (0)}

dirAuthenticationTwoWays FROM GulsSecurityExchanges

{joint-iso-ccitt genericsULS (20 modules (1) gulsSecurityExchanges (2) }

SECURITY-EXCHANGE {}, SE-ERROR {} FROM NOTATION

notation

IntermediateType, gulsSignedTransformation,  gulsSignatureTransformation FROM GulsSecurityTransformations

{joint-iso-itu-t genericULS (20) modules (1) gulsSecurityTransformations (3) }

initEncRules

OBJECT IDENTIFIER  {joint-iso-ccitt asn1 (1) packed-encoding (3) canonical (1) unaligned (1)}

ATN-Abstract-Syntax ABSTRACT-SYNTAX ::= { PDV-list IDENTIFIED BY ATN-abstract-syntax-object-id}

ATN-abstract-syntax-object-id	OBJECT IDENTIFIER ::= {

iso (1) identified-organisation (3) icao (27) atn-security-requirements (4) abstract-syntax (2)

}

atnEncryptedTransformation SECURITY-TRANSFORMATION ::= {

IDENTIFIER {iso(1) identified-organisation(3) icao(27) atn-security-requirements(4) security-exchanges(1) modules(3) atn-Encrypt(1)}

INITIAL-ENCODING-RULES {joint-iso-itu-t asn1 (1) packed-encoding (3) canonical (1) unaligned (1)}

XFORMED-DATA-TYPE BIT STRING

}

atnUISignature	SECURITY-EXCHANGE ::= {

SE-ITEMS	{ATNSignature}

IDENTIFIER	global : iso (1) identified-organisation (3) icao (27) atn-security-requirements (4) modules(3) securityExchanges(1) atn-Signature (1)

}

ATNSignature	SEC-EXCHG-ITEM ::= {

ITEM-TYPE	gulsSignature

ITEM-ID		1

ERRORS		{ signatureCheckFailure }

}

atnSignedPDU		SECURITY-EXCHANGE ::= {

SE-ITEMS	{ ATNSigned }

IDENTIFIER	global: iso (1) identified-organisation (3) icao (27) atn-security-requirements (4)  modules(3) securityExchanges(1) atn-SignedPDU (4)

}

ATNSigned		SEC-EXCHG-ITEM ::= {

ITEM-TYPE	gulsSigned

ITEM-ID		1

ERRORS		{ signatureCheckFailure }

}

signatureCheckFailure	SE-ERROR ::= {

PARAMETER	DirectoryAbstractService.SecurityProblem

ERROR-CODE	local : 1

}

END

-

-

-

-

-

-

-