ATNP WG3/SG3 WP16-35

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

WG3 – Applications and upper layers

SG3 – Upper Layer Architecture

Napoli, May the 17th, May the 28nd, 1999

Recovery of security errors in ULCs

Prepared by G. Mittaux-Biron

Summary

ICAO is in the process of standardising the requirements for ATN Security. This will allow the development of a secured environment of data transfer between airborne and ground ATN systems, taking into account the various local legislation constraints.

This paper presents the new behaviour of the upper layers in case of security error.

# Scope

During its last meeting in Palo Alto, WG3/SG3 decided that the security mechanisms in the upper layers should not be anymore implemented as an additional quality of service parameter based on a best effort basis, but rather as an element of the communication protocol, which would incorporate exception and errors handling.

# Background

The security in the upper layers was previously considered as an additional parameter of the quality of service provided to the ATN ASEs. This meant that any error pertaining to security (during authentication or data integrity check) would lead to an event logging by system management, and, potentially, to a downgrading of the level of security implemented by the security ASO. No decision to abort the dialogue would have been taken by the upper layers: Dialogue control function or Security ASO.

The recoveries of a security failure, or security audit on the system were delegated to system management applications.

The idea was to permit local implementations to implement their own strategy in security management: this could range from a simple mechanism where a system management application takes the automatic decision to abort any suspect dialogue at the level of the controller or of the pilot, to more complex architectures where the decision for recovery could be delegated to a centralised supervision system in charge of auditing and taking decisions on a system level basis.

# Decision

The security is now considered as part of the protocol; this means that any error related to security (failure during authentication or during data integrity check) shall lead to a set of services activation identical to those pertaining to error handling. Schematically, they will lead to an abort of the dialogue with the following characteristics:

- If the Dialogue was locally initiated, and the error is locally detected, then:

- A D-PAbort indication is returned to the Dialogue user,

- An AC-Abort request is sent to ACSE.

b) If the Dialogue was locally initiated, and the error is remotely detected, then:

- An AC-PAbort indication is received from ACSE,

- A D-PAbort indication is returned to the Dialogue user.

c) If the Dialogue was remotely initiated, and the error is locally detected, then:

- An AC-Abort request is sent to ACSE.

- If the association was already established, then a D-PAbort indication is returned to the Dialogue user,

d) If the Dialogue was remotely initiated, and the error is remotely detected, then:

- An AC-PAbort indication is received from ACSE,

- If the association was already established, then a D-PAbort indication is returned to the Dialogue user.

e) Events pertaining to security errors will be logged for system management.

# consequences

- As there is no parameter in the D-PAbort indication primitive:

- there will be no additional indication to the local user of the reason why the dialogue has been aborted and,

- no information is passed from/to the ACSE service provider when an AC-Abort request/indication primitive is activated,

- neither local or remote user of the dialogue service will be aware of the reason of the abort, which will be equivalent to an abort issued by a communication failure problem.

b) A security error on one Dialogue (in progress or already established) does not have any direct impact on other Dialogues between the same systems.

c) As it would lead to invalid security information, security related error on a CM Dialogue (in progress or already established) will prevent any subsequent opening of Dialogues for other ATN ASEs.

# Recommendation

WG3 is invited to endorse the new behaviour of the upper layers in case of security failure and the potential consequences on the ATN applications.