ATNP/WG3

WG3/16-W29

11/05/1999

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 - APPLICATIONS AND UPPER LAYERS

Naples, 18-21 May 1999 (sixteenth meeting)

Agenda item 5:   Ground-Ground Applications

REPORT ON CIDIN/ATN GATEWAY SPECIFICATION WORK

Presented by Jean-Yves Piram – Chairman of WG3/SG1

Prepared by Manfred Okle

SUMMARY

The working paper provides information on the current status of the development of SARPs and Guidance Material for the CIDIN/ATN(AMHS) Gateway. The available material is presented in the attachment.

- Introduction

At the second meeting of the ATN Panel (ATNP/2) the Application and Upper Layer Working Group (WG3) was tasked with the development of SARPs and Guidance Material for CIDIN/ATN gateway operation. The decision was made in consideration of the widespread use of the CIDIN for ATS message communication in certain regions. (In March 1999 already 25 international CIDIN centres, located in the EUR region and adjacent States, were in operation.)

This working paper informs on the technical approach taken by WG3/SG1 to meet the gateway requirement and reports on the reached status of the specification work. The available material is presented in the Attachment.

- Technical solution

- AMHS as ATN peer application for the CIDIN

The CIDIN/ATN Gateway shall support interoperability between users of the CIDIN and ATN during the transition phase from the CIDIN to ATN.

The CIDIN provides a code and byte independent connectionless transport service to its users. There may be various types of users (or CIDIN applications) each with its individual communication profile. Currently, the following CIDIN applications have been specified and are in use:

- Conveyance of AFTN formatted messages (IA-5 encoding/formatting, text length up to 1800 characters);

- OPMET data transport (up to 3,800 IA-5 characters or 15,000 octets of binary information); and

- Operator message interchange (exchange of information between operators of the CIDIN centres for co-ordination purposes – up to 1,968 IA-5 characters).

All the user data of the above listed CIDIN applications can be accommodated in AMHS (IPM) messages. Thus, the intercommunication between the CIDIN and ATN is reached by a CIDIN/AMHS Gateway. In other words, the CIDIN/AMHS Gateway is the technical solution for the required support of interworking between users of the CIDIN and ATN.

- Modelling of the CIDIN/AMHS Gateway

The functional model taken for the specification of the CIDIN/AMHS Gateway corresponds closely to that of the AFTN/AMHS Gateway. That means, at the first level of decomposition the following logical components are identified:

- CIDIN component, handling the interface to the CIDIN;

- ATN component, which allows the CIDIN/AMHS Gateway to function as an end system at the ATN or a peer system to remote AMHS components, respectively;

- Message Transfer and Control Units (MTCUs) performing bi-directional conversions between AMHS and CIDIN information objects; and

- Control Position for handling of out-of-line situations.

For each supported CIDIN application there will be a specific MTCU. This modularization allows subsetting as well as extensions with respect to regional needs.

- Component of the Extended ATS Message Service

The CIDIN/AMHS Gateway forms part of the Extended ATS Message Service and will therefore:

- make use of the forthcoming ATN directory service concerning synchronised address lookups (for address conversions) and checking on the compatibility between a given body part type / length and the addressed CIDIN application; and

- performing mapping from/to AMHS messages using the Business Class Messaging extensions.

Further studies are needed on the integration in the AMHS Systems Management (management boundary within the gateway?), support of the Security Service (probably not as CIDIN does not possess any security feature) and backward compatibility to the Basic ATS Message Service.

- Work progress

The current development of material is focused on that section of the Sub-Volume III which will provide the detailed specification of the CIDIN/AMHS Gateway as well as the accompanying Guidance Material (CAMAL). In addition, smaller amendments have to be developed for the AMHS system level SARPs in Sub-Volume III.

Due to the speciality of the subject, the WG3/SG1 agreed to a two-level approach for the review of the developed draft material:

- Reviews of submitted portions of material in the course of SG1 meetings which mainly address the functional concept, editorial principles and give advice concerning the continuation of the work; and

- detailed technical reviews in one or more special drafting group meetings when a certain degree of completion is achieved (to be done).

The up to now developed draft material is presented as Attachment. The material addresses the gateway´s logical components and such functions which are common to all types of MTCUs as logging, handling of CIDIN service confirmations and address conversions.

The work will be continued with the specification of the conversion processes to be performed in the individual MTCUs. Completion of the draft material is planned by September 1999.

- CIDIN reference documentation

The basic specifications of the CIDIN are laid down in Annex 10, Vol. III. Ongoing working group activities in the EUR region concerning CIDIN planning and implementation aspects have produced significant amendments to the Annex 10 material. The amendments are published in the EUR CIDIN Manual or provisionally in meeting reports.

By end of 1999 the CIDIN specification will be consolidated with a CIDIN Manual update. For the time being, in the draft CIDIN/AMHS Gateway material references are made in the first instance to Annex 10, Vol. III and where needed to the current EUR CIDIN Manual and supplementary documentation. The alignment with the consolidated CIDIN documentation will be done in the course of a final review by the WG3/SG1.

- Recommendation

The meeting is invited to note the reached status of work, comment on the taken approach, and give advice for the remaining work to be done by the WG3/SG1.

Attachment