ATNP/CCB WP/10-__

ATNP/WG3/SG1 WP/267

ATNP/WG3 WP/17-45

30/09/99

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL(ATNP)

CONFIGURATION CONTROL BOARD – 10th meeting

WG 3 - Applications and Upper Layers – 17th meeting

WG3/SG1 – GROUND-GROUND APPLICATIONS – 19th meeting

Gran Canaria, 28th September – 7th October 1999 (seventeenth meeting)

SME3 CCB Report

Presented by Jean-Marc Vacher (Sub-Volume 3 SME)

Summary

This paper provides a summary status of PDRs raised against the Sub-Volume III SARPs since the Phuket WGW/1 meeting.

Table of contents

1. Introduction

2. Summary of ATSMHS PDRs

3. Summary of AIDC PDRs

4. Recommendation

5. ATTACHMENT A : Details of Sub-Volume 3 PDRs

________________

# Introduction

The goal of this paper is to provide the CCB / Working Group / Subgroup with the current status of the Sub-Volume 3 PDRs.

# Summary of ATSMHS PDRs

The following table lists all PDRs raised against the ATSMHS SARPs (Doc 9705, Sub-Volume 3, Chapter 1) since their approval at the Phuket ATNP WGW/1 meeting.

| Number | Name | Status (post CCB/9) | Comments |
| --- | --- | --- | --- |
| 97060014 | ATSMHS / use of implicit-conversion flag and EITS | RESOLVED |  |
| 97060015 | ATSMHS / conversion of forwarded messages | RESOLVED |  |
| 97060016 | ATSMHS / MHS priority and ATS-Message-Priority mismatch | RESOLVED |  |
| 97060017 | ATSMHS / prohibited character check in converted AMHS messages | FORWARDED | to be addressed in Guidance Material |
| 97060018 | ATSMHS / erroneous cross-references to Sub-Volume 5 | RESOLVED |  |
| 97060019 | ATSMHS / recommendation on report generation | RESOLVED |  |
| 97100040 | ATSMHS / editorial corrections | RESOLVED |  |
| 98030005 | ATSMHS / Year 2000 dependency | RESOLVED |  |

# Summary of AIDC PDRs

The following table lists all PDRs raised against the AIDC SARPs (Doc 9705, Sub-Volume 3, Chapter 2) since their approval at the Phuket ATNP WGW/1 meeting.

| Number | Title | Status (post-CCB/9) | Comments |
| --- | --- | --- | --- |
| 97060020 | AIDC / Errors and inconsistencies in AIDC Abstract Definition | RESOLVED |  |
| 97060021 | AIDC / Provision of the Called ICAO Facility Designation when Info-transfer-request is invoked outside a dialogue | RESOLVED |  |
| 97060022 | AIDC / construction of the calling end-system AP-title | RESOLVED |  |
| 97060023 | AIDC / ASN.1 correction : Airport | RESOLVED |  |
| 97060024 | AIDC / sequencing table inconsistent with state tables | RESOLVED |  |
| 97100004 | AIDC / Definition of releaseIndicator | RESOLVED |  |
| 97100005 | AIDC / reference error in SARPs | RESOLVED |  |
| 97100015 | CPDLC/AIDC Airway Name | RESOLVED |  |
| 97100016 | CPDLC/AIDC VHF Frequency/ Frequencyvhfchannel | RESOLVED |  |
| 97100027 | AIDC / Year 2000 dependency | RESOLVED |  |
| 97100033 | AIDC / ASN.1 message type and Abort issue | RESOLVED |  |
| 97100042 | AIDC / AIDC Abstract Definition | RESOLVED |  |
| 97100043 | AIDC / Transfer Control Information parameter | RESOLVED |  |
| 97100044 | AIDC / Departure Airport | RESOLVED |  |
| 97100045 | AIDC / AircraftIdentification | RESOLVED |  |
| 97100046 | AIDC / BeaconCode | RESOLVED |  |
| 98030002 | AIDC / Control Function | RESOLVED |  |
| 98030003 | AIDC / Control Function | RESOLVED |  |
| 98040005 | all / ICAO V2.2 problems | RESOLVED |  |
| 98050019 | CPDLC/AIDC problems with ICAO V2.2 CPDLC SARPs | RESOLVED |  |

| 98090006 | AIDC / AIDC UCF Indication | WITHDRAWN |  |
| --- | --- | --- | --- |
| 98090009 | AIDC / AIDC PM variable names | RESOLVED |  |
| 99080002 | AIDC / Transfer Control protocol states | SUBMITTED SME proposal : REJECTED |  |
| 99080003 | AIDC / Provider Abort indication parameters | SUBMITTED SME proposal : ACCEPTED |  |

# Recommendation

The CCB is invited to note the information provided.

# ATTACHMENT A : Details of Sub-Volume 3 PDRs

PDR 99080002

PDR 99080003

Title : ICAO 9705 - AIDC - Transfer Control Protocol States

PDR Reference:                          990080002

Originator Reference:                   -

SARPs Document Reference:               SARPs Sub_Volume III

Status:                                 SUBMITTED

Impact:

PDR Revision Date:

PDR Submission Date:                    24/08/99

Submitting State/Organization:          CENA/CHARME Project

Submitting Author Name:                 Mathieu JEAN

Submitting Author E-mail Address:       jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                             ICAO 9705 Edition 2

SARPs Language:                         English

Summary of Defect:

Sections 3.2.6.1.28 and 3.2.6.1.29. Tables 3.2.6-4 Sections 3.2.6.1.27 and 3.2.6.1.30.

AIDC-Transfer-Control is a confirmed service.

When the AIDC-ASE protocol machine receives an indication with a AIDC-tfr-cntrl-req-apdu, the required state is the COORDINATED state and the protocol machine remains in this state.(Section 3.2.6.1.28)

In the next section (3.2.6.1.29), when the AIDC-ASE protocol machine receives the AIDC-tfr-cntrl response, the required state is the TRANSFERRING state but the protocol machine is still in the COORDINATED state.

There is the same problem on Transfer Control Request and Confirmation. When the AIDC-tfr-cntrl-req-apdu is sent, the protocol machine remains in the COORDINATED state (Section 3.2.6.1.27) but when the confirmation is received the required state is the TRANSFERRING state (Section 3.2.6.1.30).

Assigned SME: 					Sub-Volume III SME

Proposed SARPs Amendment (proposed by PDR author):

Section 3.2.6.1.27.2.1

g)  enter the TRANSFERRING state

Section 3.2.6.1.28.2.1

g) enter the TRANSFERRING state

SME Comment:

In fact, as usual in the AIDC application, the state changes of the AIDC-ASE protocol machine are performed upon sending and receiving of user confirmations, by means of the user-confirmation service.

Figure 3.2.10-19 (page 274) shows the sequence diagram concerning the Transfer-control service. State changes are performed by the AIDC-ASE:

- for the R-ATSU, upon receipt of an AIDC-ucf request primitive (see section 3.2.6.1.33.2.1 b) 2) iv) C) and the state table 3.2.6-4),

- for the C-ATSU, upon receipt of an AIDC-DATA indication with an AIDC-ucf-apdu (see section 3.2.6.1.34.2.1 c) 1) iii) C) and the state table 3.2.6-4).

The SARPs are correct with this respect; the textual description of the AIDC-ASE protocol machine, the state table and the sequence diagram are consistent. The PDR appears to be a misinterpretation of the SARPs.

SME Recommendation to CCB:   REJECT

CCB Decision:                ? (CCB/10)

Title : ICAO 9705 - AIDC - Provider Abort Indication Parameters

PDR Reference:                            99080003 Originator Reference:                   -

SARPs Document Reference:

SARPs Sub_Volume III Status:             SUBMITTED

Impact: 						C (Clarification)

PDR Revision Date:

PDR Submission Date:                     24/08/99

Submitting State/Organization:           CENA/CHARME Project

Submitting Author Name:                  Mathieu JEAN

Submitting Author E-mail Address:        jean@cenatls.cena.dgac.fr

Submitting Author Supplemental

Contact Information:

SARPs Date:                              ICAO 9705 Edition 2

SARPs Language:                          English

Summary of Defect:

Sections 3.2.6.1.33.2.1 c) 1) and 3.2.6.1.34.2.1 d) 1)

The invocation of the AIDC-pvd-abrt indication is required but the parameters are not described.

Sections 3.2.6.1.39.2.1 a)

The invocation of the AIDC-pvd-abrt is required with the AbortReason parameter set to communicationsservicefailure but the code communicationsservicefailure is unknown in the ASN1 type ProviderAbortReason.

Assigned SME: 					Sub-Volume III SME

Comments:

1/ Sections 3.2.6.1.33.2.1 c) 1) and 3.2.6.1.34.2.1 d) 1)

The action in each of these cases is to signal AIDC-pvd-abrt.indication up to the CF (thence to the AIDC-User) and AIDC-Abort.request down to the CF (thence to the peer entity).

The abort reason is a mandatory parameter for provider abort at both the AIDC-ASE service and the AIDC service. So a value must be provided. It's only optional for the AIDC-ABORT service, but it seems reasonable to

signal the same value.

Case 1 (3.2.6.1.33.2.1): AIDC-ASE-User is responding to a message other than

the one that was most recently delivered and for which a User-confirmation is awaited.

Case 2  (3.2.6.1.34.2.1): the peer ASE is responding to a message other than

the one that was most recently transmitted and for which a User-confirmation

is awaited.

Case 2 is protocol error. Case 1 is not really a protocol error and should have a separate error value.

2/ 3.2.6.1.39.2.1

The abstract value "communications service failure" is called for, which is not obviously equivalent to any one of the enumeration ProviderAbortReason. The proposal is to change this abstract-value to "provider error", which would give an equivalent view to the CF and to AIDC-user.

3/ 3.2.6.2.3.1 (Additional)

The abstract value "invalid APDU" is called for, which is not obviously equivalent to any one of the enumeration ProviderAbortReason. The proposal is here to change this abstract-value to "protocol error".

4/ 3.2.7.1.1

The insertion of the item "sequenceerror" in the ASN.1 definition is performed before the extensibility marker, because this element is not used for PDU encoding (but only for service specification). As such, it has not interoperability impact and the change in the ASN.1 will not cause any interoperability issue.

Proposed SARPs Amendment:

1/ amend 3.2.6.1.33.2.1 c) 1) and 2) to read:

"c)     if the variable vr2!= Reference ID then:

1)      invoke an AIDC-pvd-abrt indication with the AbortReason parameter set to the abstract value: sequenceerror;

2)      invoke an AIDC-ABORT request with the AbortReason parameter set to the abstract value: sequenceerror;

3)      stop all timers; and

4)      enter the IDLE state."

2/ amend 3.2.6.1.34.2.1 d) to read:

"d)	  if the variable vs2 != Reference ID , then:

1)      invoke an AIDC-pvd-abrt indication with the AbortReason parameter set to the abstract value: protocolerror;

2)      invoke an AIDC-ABORT request with the AbortReason parameter set to the abstract value: protocolerror;

3) stop all timers; and

4) enter the IDLE state."

3/ amend 3.2.6.1.39.2.1 to read:

"3.2.6.1.39.2.1       Upon the receipt of an AIDC-P-ABORT indication primitive the AIDC-ASE shall:

a) invoke an AIDC-pvd-abrt indication primitive with the AbortReason parameter set to the abstract value: providererror;

b) stop all timers; and

c) enter the IDLE state."

4/ amend 3.2.6.2.3.1 to read:

"3.2.6.2.3.1  If an APDU received is determined to be invalid, the AIDC-ASE shall:

a)      invoke an AIDC-ABORT request primitive; and

b) invoke an AIDC-pvd-abrt indication primitive with the AbortReason parameter set to the abstract value: protocolerror; and

c) enter the IDLE state."

5/ amend 3.2.7.1.1 to read:

"ProviderAbortReason ... ::= ENUMERATED

{

protocolerror  (0),

timerexpired  (1),

undefinederror  (2),

providererror  (3),

rejectedpermanent  (4),

rejectedtransient  (5),

sequenceerror  (6),

...  }"

SME Recommendation to CCB:   ACCEPT / RESOLVE

CCB Decision:                ? (CCB/10)