ATNP/WG1/Flimsy __

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 1

Rio de Janeiro, Brazil  16 - 20 March 1998

Request for IFALPA Position Paper on Context Management Implementation

Prepared by ATNP/WG3/SG2

## 1.	In order to start the Context Management (CM) data link application, the initial CM address needs to be input to the avionics.  This issue is addressed at a high level in the ICAO Manual of Air Traffic Services (ATS) Data Link Applications in the Address Management section as follows:

### Procedures need to be in place to allow dissemination of DLIC ground addresses to aircraft.  It is recommended that each ground facility supporting air-ground data link applications have DLIC addresses published in State AIPs.  This would allow look-up tables to be imbedded in the flight deck avionics implementation, reducing pilot input.

Note.— A given FIR may have multiple DLIC addresses; and more than one FIR may share the same DLIC address.

2.  	Similarly, the CM SARPs require that the initial CM address must be supplied to the avionics in some manner, but does not specify the method.  The CM guidance mentions several potential methods, and acknowledges that a standardized means should be established to provide initial identification of the CM facility designation and correlation with the corresponding presentation address.  These are given below:

- The avionics determines, based on the aircraft's position, what ground system would need to be logged on to and will perform a database or table lookup function in order to obtain the address,

- The pilot enters the facility designation or short form address of the ground system, and an on-board mechanism (such as a database or table lookup function) completes the address,

- A pre-set address in the aircraft of a ground-based CM address regional or global file server,

- A ground system (e.g. airline or service provider) supplies a locally stored address to the aircraft, and the aircraft makes it available to the dialogue service provider,

- The pilot enters the full address as listed in an AIP or like document, and

- The pilot enters a country code and a short (e.g. three letter) local address code which specifies an external addressing database, and the proper addresses are then determined by the external database and returned to the avionics.

A more detailed discussion of these methods was included in the AEEC minutes of the Data Link Users Form in Orlando, Florida, USA at the end of January 1998.  In addition, this paper will serve as the strawman for the development of the AEEC Specification 638.

3.	As implementations are designed, aircraft manufacturers must choose the method of supplying the initial CM address to the avionics.  In order to ensure acceptance by the pilot community, it is requested that IFALPA provide a position paper regarding the implementation of CM including impact on pilot workload requirements, perceived operational scenarios, and aeronautical information publication requirements (e.g. CM addresses for inclusion on Jeppesen charts) for the ATNP meeting in Utrecht, Netherlands.

- END -