Flimsy __WG2/12

Flimsy __WG3/9

Revision 1

ATNP Working Group 3

Gold Coast, Australia

TS-user Specification

This flimsy proposes certain agreements between Working Group 3 and Working Group 2 in the TS-user specification.

# Introduction

## WG 3 specifies the upper layers, which is the TS-user in terms of WG2.

# Security Label

## The Upper Layer Architecture (ULA) provides the application traffic type in order to complete the ATN CLNP security label.

## WG3 understands that the 13 octets labeled TS-user security label in Figure 1 are required to encode the one-octet Traffic Type.

## Recommendation WG3 seeks confirmation that the security label is not routinely transmitted air-ground, but rather is encoded in the SNDCF LREF information.  V, 7.5.4.5.3.a would appear to indicate this. [WG2 confirmed this.]

## Recommendation WG3 recommends that WG2 reconsider the encoding of the security label.  It is not clear that the security label with the ATN OID derives from a  CNS/ATM-1 security schema, or provides forward compatibility to a known CNS/ATM-2 schema.  [This is WG2 action 7-44 to draft a change request]

## Recommendation  WG3 recommends that WG2 incorporate the contents of the attached WG3 Flimsy in 2.7.1.1b, Table 2-2.  WG3 notes that the traffic types have been extended to A-J. [This a WG2 action 7-45 is for WG3 to provide the approved table based on the material from the ADSP WG meeting in Dakar.]

# Connectionless Transport

3.1 Recommendation WG3 wishes to stet the CLTP recommendation, as it is developing a CL ULA for the Brussels WG3 meeting. [Accepted by WG2]

# Expedited Data Option

## Reference 5.1.2 Note 1 - b)  WG3 informs WG2 that the expedited data option has been eliminated from the ATN ULA, and will not be requested in CNS/ATM-1.

# Residual Error Rate

## Recommendation Reference 5.1.2 Note 1 - c) WG3 would prefer that the note be expanded to indicate that WG2 shall support RER below that determined solely by the use of the Fletcher’s checksum.  WG3 notes that some applications will always request the low RER (checksum required) in CNS/ATM-1. [WG3 to provide text to WG2]

## Recommendation WG3 notes that applications using the ATN ULA have requested RERs of 10-6 and 10-7.  WG3 therefore asks WG2 for guidance on support  of these specified levels of RER through the use of the transport  checksum and other subnetwork-dependent mechanisims. [Sub-Volume 1 guidance material]

# ATN Naming Tree

## WG3 offers the attached ATN naming hierarchy (see figure 2) for WG2 information purposes.  WG2 has sole authority below the atn(0) arc.

# Recommendation

## It is recommended that WG2 review the material in this paper and action the Recommendations offered in 2.1.2,  2.1.3, 2.1.4, and 5.2.

|  |  |  |  |  | Value | Length |
| --- | --- | --- | --- | --- | --- | --- |
| Security Label | Parameter Code |  |  |  | 1101 0101 | 1 |
|  | Parameter Length |  |  |  | 0000 1110 | 1 |
|  | Parameter Value | Security Format |  |  | 1100 0000 | 1 |
| TS-user security label |  | Security Label | Security Reg ID |  | 0000 1100 | 1 |
|  |  |  | Security Reg ID | OID, Length, [1.3.27.0.0] | 06, 04,43,27,00,00 | 6 |
|  |  |  | Security Info Length |  | 0000 0100 | 1 |
|  |  |  | Security Info | Tag Set Name Length) | 0000 0011 | 1 |
|  |  |  |  | Tag Set Name | 0000 1111 | 1 |
|  |  |  |  | Tag Set Length | 0000 0001 | 1 |
|  |  | Traffic Type |  | Security Tag | 0000 0011 | 1 |
|  |  |  |  |  | Total: | 15 Octets |

Figure 1: Present Encoding of Security Label

All values are static in CNS/ATM-1 except the last.

Figure 2: ATN Naming Tree