EXAMPLE LIST OF SYSTEM LEVEL REQUIREMENTS

- The ATN shall provide the capability to establish a logon between the ground and aircraft systems (A/G CM )

- The ATN shall allow for the registration of the communications and Surveillance capabilities of an aircraft (A/G CM )

- The ATN shall allow for a a maximum of 256 applications to be supported (A/G CM )

- The ATN shall allow for only the aircraft to initiate the logon functions (A/G CM )

- The ATN shallow for a ground system to request an aircraft to initiate a logon function (A/G CM )

- The ATN SARP’s should provide for backward compatibility as new versions of the SARP’s are approved and promulgated (A/G, G/G)

- NOTE: For the CNS/ATM-1 Package, only the following applications will be accommodated: ADS, CPDLC,FIS(ATIS),CM, ICC)

- The ATN shall allow for the avionics to establish supporting contracts with at least four ground systems simultaneously (A/G ADS)

- The ATN shall allow for the exchange of regular messages and emergency messages (A/G , G/G)

- The ATN shall be implemented in such a way that it shall be capable of meeting the Required Communications Performance appropriate for the phase of operation (A/G ,G/G)

- The ATN shall be capable of notifying the user that information being input by the user is incompatible with that able to be processed by the system (A/G )

- Compliance with these standards is a means of assuring that the ATC system will perform its intended functions using datalink in a globally uniform and interoperable  manner (A/G)

- The ATN shall provide the capability of prioritizing the messages in accordance with defined application parameters (A/G, G/G)

- The ATN shall not have any inherent security requirements in the CNS/ATM-1 Package (ATNP WG1)

- The ATN shall notify a user if the user attempts to invoke a communications service when the user is not in an appropriate state (of readiness?) and the invocation shall be rejected (A/G)

- The ATN shall provide the media and protocols to conduct data link communications for the specified applications.(A/G, G/G)

- The ATN shall allow two Air traffic Services units to exchange Air Traffic Control information (G/G)

- The ATN shall be developed to support any timing requirements within +/- 1 second UTC (A/G,G/G)