WG3/5

Brisbane Meeting

Flimsy 5-8

Flimsy 5-8    :  Draft SARPs for ATS Message Handling Services over the ATN

Support of the ATS Message Header for the direct AMHS users

It is proposed that Table 2.4, page 2-21,  be amended as follows :

| Ref | Element | Basic ATS Message Service Support |  | Value | IA-5 Encoding |
| --- | --- | --- | --- | --- | --- |
|  |  | Orig | Rec |  |  |
| 1 | aTS-Message-Header | c1 | m1 |  |  |
| 1.1 | start-of-heading | m | m | (SOH) | (0/1) |
| 1.2 | aTS-Message-Priority | m | m |  |  |
| 1.2.1 | priority-prompt | m | m | PRI:(single space) | (5/0)(5/2)(4/9)(3/10)(2/0) |
| 1.2.2 | priority-indicator | m | m | see 2.2.3.2.1 | see 2.2.3.2.1 |
| 1.2.3 | priority-separator | m | m | (CR)(LF) | (0/13)(0/10) |
| 1.3 | aTS-Message-Filing-Time | m | m |  |  |
| 1.3.1 | filing-time-prompt | m | m | FT:(single space) | (4/6)(5/4)(3/10)(2/0) |
| 1.3.2 | filing-time | m | m | see 2.2.3.2.2 | see 2.2.3.2.2 |
| 1.3.3 | filing-time-separator | m | m | (CR)(LF) | (0/13)(0/10) |
| 1.4 | aTS-Message-Optional-Heading-Info | o | m |  |  |
| 1.4.1 | oHI-prompt | m | m | OHI:(single space) | (4/15)(4/8)(4/9)(3/10)(2/0) |
| 1.4.2 | optional-heading-information | m | m | see 2.2.3.2.3 | see 2.2.3.2.3 |
| 1.4.3 | oHI-separator | m | m | (CR)(LF) | (0/13)(0/10) |
| 1.5 | end-of-heading-blank-line | m | m | (LF) | (0/10) |
| 1.6 | start-of-text | m | m | (STX) | (0/2) |
| 2 | aTS-Message-Text | m | m | see 2.2.3.2.4 | see 2.2.3.2.4 |

c1 :	if at least one recipient is an indirect user, then m, else o

m1 :	the ATS Message User Agent is able to receive this information and to act upon its reception. The ATS Message User Agent is prohibited from rejecting the message due to absence of the element.

Guidance Material :	at the level of a UA, eg. an IPM UA as in the Basic ATS Message Service, the absence of an element which is classified as "mandatory upon reception", never generates message rejection in the base standards, since no action upon reception is specified in the base standards. It is common practice to consider that the only requirement is to the ability to display the element to the user, if present. However this in a user-interface matter and therefore out of the scope of the standard.