# APPLICATION ENTITY (AE) DESCRIPTION

## Introduction

Note. —  As indicated in Chapter 1, the AE is described in terms of the Service which it displays to the Application-user, and in terms of the Control Function (CF) which meditates the interactions of the components of the AE.

### The ATN-App AE shall exhibit external behaviour as if implemented according to the model shown in Figure 3.1, with the protocols defined in ACSE and the ATN-App ASE specifications.

Note 1. —  Figure 3.1 also indicates which section in this document describes the behaviour of the CF in response to events at the various service boundaries.

Note 2. —  The “Future ASE” component in Figure 3.1 indicates how additional functionality can be added in future packages.  ACSE provides the basic mechanisms for establishing and releasing an application association.  The service provided by the ATN-App AE represents an abstract description of the Application Programming Interface (API) seen by the Application user.  The CF of the ATN-App AE specifies how the interactions at the ATN-App AE Service boundary invoke the appropriate service primitives of the constituent ASEs, which in turn generate the actual protocol.  The CF also specifies how the constituent ASEs interact with the supporting communications service.

Figure 3.1.  Components of ATN-App AE

Note 3. —  A CF specification is not a service definition of the ATN-App AE or its components.  It only defines the actions of the CF as a result of service invocations visible to the CF.  Thus, the specification is organised around specifying the response of the CF to these inputs.  Section 3.3.2 specifies the actions that result from the inputs of the Application-user.  Section 3.3.4 and section 3.3.5 specify the actions that result from the service invocations of the ACSE component ASE.  Section 3.3.3 specifies the actions that result from the service invocations of the ATN-App ASE component ASE. Section 3.3.6 specifies the actions that result from the inputs from the supporting service.

Note 4. —  The CF specification describes the overall behaviour of the ATN-App AE. It is not a requirement that an identifiable CF entity be realised in an implementation.

Note 5. —  The CF specification in this document assumes that the embedded ASEs (ATN-App ASE and ACSE) are modelled as atomic entities, such that when an input event is invoked by the CF, that event is processed to completion by the ASE and the CF responds to any resulting output events from the ASE, all within the same logical processing thread.  This model avoids the need to specify further transient states within the CF.  It does not imply any particular implementation architecture.

Note 6. —  In the CNS/ATM-1 package, the service interface presented to the Application-user is a simple pass through to the ATN-App ASE.  That is, the Application-user passes request and response primitives directly to, and receives indication and confirmation primitives directly from, the ATN-App ASE.

Note 7. —  The CF described in this chapter supports the four air-ground SARPs defined for the CNS/ATM-1 Package.  The specification of the CF for the AIDC application is included in the AIDC SARPs.

## Application Level Naming and Context Definition

### ATN Naming Hierarchy

Note 1. —  This section assigns names, in the form of object identifiers (OIDs), to the ATN entities defined in these SARPs.

Note 2. —  ISO/IEC 8824 specifies the top of the hierarchical OID name space.  At the first level, provision is made for ISO, ITU-T and joint ISO/ITU-T sub-name spaces.  The ISO name space is further subdivided into:

standard (0)

registration-authority (1)

member-body (2)

identified-organisation (3)

Note 3. —  ICAO has requested and obtained the allocation of an International Code Designator (ICD), according to ISO 6523.  The ICD obtained, name and number “icao (27)”, uniquely identifies ICAO and allows ICAO to establish its own object identifier name space within the International Organisation arc as follows:

{ iso (1) identified-organisation (3) icao (27) }

#### Within the ICAO name space, the allocation of object identifiers for the CNS/ATM-1 Package shall follow the structure and values indicated in this section.

Note 1. — in the future, it is likely that the ATN object identifier tree will have further levels of structure, and that fully location-independent values will be assigned.

Note 2. —  The ATN naming hierarchy specified in this section is illustrated in Figure 3.2.

Figure 3.2.  ATN Naming Hierarchy

#### Immediately under the ICAO arc, the values specified in Table 3.1 shall be used to specify the next level of the naming hierarchy for the CNS/ATM-1 Package.

Table 3.1.  Top-level ICAO Identifiers

| Name and numeric value | Description |
| --- | --- |
| atn (0) | General ATN identifiers |
| atn-end-system-air (1) | ATN aircraft end systems. The following OID component beneath this arc is a 24-bit ICAO aircraft identifier |
| atn-end-system-ground (2) | ATN ground end systems. The following OID component beneath this arc is an ICAO facility designator |
| atn-ac (3) | ATN application context names |

### Application Process titles

Note. —  Application process titles for the CNS/ATM-1 Package are allocated underneath either of the Object Identifier arcs:

{ atn-end-system-air (1) } or { atn-end-system-ground (2) }.

Immediately subordinate to this arc is an arc whose value is an INTEGER derived from either the 24-bit ICAO aircraft address or the ICAO facility designator, as described in section 3.2.4.  Immediately beneath that arc is an arc whose value is determined by the category of the ATN application.  For the CNS/ATM-1 Package, only the following name and value are defined for the application category:

{ operational (0) }.

#### For the CNS/ATM-1 Package, each application category on each ATN end system shall be assigned an unambiguous application process title (AP-title).

#### The AP-title shall be an Object Identifier type (i.e. an AP-title-form2 as defined in ISO/IEC 8650-1).

#### Application Process titles for the CNS/ATM-1 Package shall be of the form:

either:

{ iso (1) identified-organisation (3) icao (27) atn-end-system-air (1) <end-system-id> (n) operational (0) }

or:

{ iso (1) identified-organisation (3) icao (27) atn-end-system-ground (2) <end-system-id> (n) operational (0) }

where:

<end-system-id> is the ICAO 24-bit address for aircraft end systems, or the ICAO facility designator for ground end systems.

(n) is an INTEGER value derived from the <end-system-id>.

Note. —  The algorithm for deriving the INTEGER n from the <end-system-id> is defined in section 3.2.4.

### Application Entity Titles

#### Each ATN application entity shall be assigned an unambiguous application entity title (AE Title).

#### For ATN, an AE Title shall be an Object Identifier type as defined in ISO/IEC 8824-1 (i.e.  an AE-title-form2 as defined in ISO/IEC 8650-1).

#### The AE Title shall be composed of an Application Process title (AP-title) and an AE-qualifier.

#### The AE-qualifier component of the AE Title shall be an INTEGER type (i.e. an AE-qualifier-form2 as defined in ISO/IEC 8650-1).

#### The AE-qualifier value arc of the AE Title object identifier for the CNS/ATM-1 Package represents the ATN application type (e.g. “ADS” or “CMA”), and shall take one of the values specified in Table 3.2.

Table 3.2.  Assigned AE-Qualifier values

| ATN ASE type | ATN AE-Qualifier name and numeric value |
| --- | --- |
| Automatic Dependent Surveillance | ADS (0) |
| Context Management Application | CMA (1) |
| Controller Pilot Data Link Communication | CPC (2) |
| Automatic Terminal Information Services (ATIS) | ATI (3) |
| Type A Gateway | GWA (4) |
| Systems Management Application (SMA) | SMA (5) |
| ATS Inter-Facility Data Communications (AIDC) | IDC (6) |
| ATS Message Server | AMS (7) |
| AFTN-AMHS Gateway | GWB (8) |
| ATS Message User Agent | AUA (9) |

#### Thus, AE-titles for the CNS/ATM-1 Package shall be of the form:

either:

{ iso (1) identified-organisation (3) icao (27) atn-end-system-air (1) <end-system-id> (n) operational (0) <ae-qualifier> (m) }

or

{ iso (1) identified-organisation (3) icao (27) atn-end-system-ground (2) <end-system-id> (n) operational (0) <ae-qualifier> (m) }

where:

<end-system-id> is the ICAO 24-bit address for aircraft end systems, or the ICAO facility designator for ground end systems.

(n) is an INTEGER value derived from the <end-system-id>

<ae-qualifier> is the name form of the AE qualifier from Table 3.2

(m) is the number form of the AE qualifier from Table 3.2.

Note. —  The algorithm for deriving the INTEGER n from the <end-system-id> is defined in section 3.2.4.

### Encoding of End System Identifiers

Note 1. —  Where the <end-system-id> appears as an INTEGER component in the Object Identifier formats specified above, the encoding of the INTEGER value is obtained as follows:

Note 2. —  For ground stations, the <end-system-id> is derived from a facility designator which is limited in length and syntax to 8 upper case alphabetic characters, e.g. “LFPODLHX”.  The syntax of the first four characters is defined in ICAO 7910 “Location Indicators”; the syntax of the next three characters is defined in ICAO 8585 “Designators for Aircraft Operating Agencies, Aeronautical Authorities and Services.”

#### For aircraft, the <end-system-id> naming arc shall be encoded as the binary value of the 24-bits comprising the ICAO aircraft identifier, expressed as an INTEGER in the range 0..(224-1).

#### For ground stations, the encoding of the <end-system-id> naming arc shall be derived from the ICAO facility designator, which is assumed to be a sequence of characters from the restricted character set (A..Z), as follows:

- Each character is encoded as a 5-bit value such that A is encoded as the binary value 00000, B is encoded as 00001, and so on up to Z which is encoded as 11001

- The <end-system-id> is then encoded as the binary value that results from the concatenation of the 5-bit character values with no padding bits, expressed as an INTEGER.

### Application Context Names

Note 1. —  The Application Context describes the ASE/ASO types which are present in the AE, including those aspects not distinguished by ASO type (e.g. version and policy aspects).  The abstract syntax of the APDUs and the control function are described in this chapter.  The Application Context name is an identifier which is used to refer to a defined Application Context.  The syntax of the Application Context name is defined in ISO/IEC 8650-1 as an Object Identifier.

Note 2. —  The application context name is used here only to distinguish between different versions of an application context within the scope of a given AE type, as identified by the AE Title.

#### For the CNS/ATM-1 package, the Application Context name shall be used to indicate the version and policy aspects relative to the AE with which it is associated.

#### Each Application Context shall be assigned an Application Context name.

#### Application Context names in the CNS/ATM-1 Package shall have the following structure:

{ iso (1) identified-organisation (3) icao (27) atn-ac (3) version-<n> (n) }

where n is an INTEGER in the range 0..255.

Note. —  The value n = 0 is reserved for use by the CF.

### Presentation Context Identification

Note. —  The Null Encoding presentation protocol option has been selected for the most efficient encoding of presentation PDUs, as defined in Chapter 5.  As a consequence, the conventional presentation protocol mechanisms which enable users of the presentation service to distinguish the presentation context of received APDUs are not available.  Therefore, this section defines an alternative, application layer, mechanism.

#### All User Data which is passed across the presentation service boundary shall be encoded using the unaligned variant of the Packed Encoding Rules (PER) for ASN.1 (ISO/IEC 8825-2).

Note. —  ISO/IEC 8823-1 specifies two choices for the encoding of User-data:

User-data ::= CHOICE {

[APPLICATION 0] 	IMPLICIT Simply-encoded-data,

[APPLICATION 1] 	IMPLICIT Fully-encoded-data  }

Simply-encoded-data ::= OCTET STRING

#### In order to be able to distinguish APDUs which are defined in different abstract syntax modules, the presentation User Data encoding shall assume the Full Encoding option of ISO/IEC 8823-1, augmented with the PER-visible constraints defined in ISO/IEC 8823-1:1994/PDAM 2 as follows:

Fully-encoded-data ::= SEQUENCE SIZE (1, ...) OF PDV-list

-- contains one or more PDV-list values

PDV-list ::= SEQUENCE

{ transfer-syntax-name		Transfer-syntax-name OPTIONAL,
  presentation-context-identifier	Presentation-context-identifier,
  presentation-data-values  CHOICE
	{ single-ASN1-type	[0] ABSTRACT-SYNTAX.&Type
		(CONSTRAINED BY {
		-- Type corresponding to presentation context identifier -- }) ,
	octet-aligned 		[1] IMPLICIT OCTET STRING,
	arbitrary 			[2] IMPLICIT BIT STRING }

-- contains one or more presentation data values from the same
	-- presentation context.

}

Presentation-context-identifier ::= INTEGER SIZE (1..127, ... )

Note. — The use of Full Encoding is specified in order to overcome the fact that: (a) the use of presentation protocol efficiency enhancements removes the ability of presentation layer to perform the necessary demarcation, and (b) the use of ASN.1 Packed Encoding Rules means that it would not have been possible to assign unique ASN.1 tag values to individual APDUs to distinguish between them, as PER does not encode tags.

#### The transfer-syntax name field in the PDV-list SEQUENCE shall not be present in the encoded presentation User Data.

#### Only the “arbitrary” (BIT STRING) choice for presentation-data-values in the PDV-list SEQUENCE shall be used in the encoded presentation User Data.

#### The values of Presentation-context-identifier which are pre-defined in Table 3.3 shall be used in the encoding of presentation User Data; the presentation-context-identifiers are not dynamically assigned by the presentation service.

Table 3.3.  Presentation Context identifiers

| Presentation-context-identifier value | Short name | Description |
| --- | --- | --- |
| 0 | acse-apdu | ACSE abstract syntax as defined in ISO/IEC 8650-1 |
| 1 | cdse-apdu | reserved for future use |
| 2 | user-ase-apdu | abstract syntax as defined in individual ATN application SARPs |
| other |  | reserved for future use |

#### Upon receiving User Data from the presentation service, the CF shall:

- decode the Fully-encoded-data and use the presentation-context-identifier value to determine the target ASE.

- if the target ASE is ACSE, decode the header of the embedded presentation-data-value to determine the APDU type, and

- pass the presentation-data-value (i.e. acse-apdu, cdse-apdu, or user-ase-apdu) to the target ASE by invoking the appropriate indication or confirmation primitive at the lower ASE service boundary, as specified in section 3.3.

#### When an ASE issues a request or response primitive at its lower service boundary which would otherwise map onto a presentation service primitive, the CF shall

- embed the User Data into a Fully-encoded-data type, using the presentation-context-identifier value corresponding to the source ASE

- pass the Fully-encoded User Data to the presentation service by invoking the appropriate primitive, as specified in section 3.3.

## Control Function Specification

### ATN-App CF State Definitions

#### The ATN-App AE shall behave as if it has a Control Function which can exist only in one of the following states:

Null  (STA0) –  This is the state of the CF when there is no association in existence.

Association Pending  (STA1) –  The CF enters this state either when the ATN-App ASE has made a request to establish a dialogue and is waiting for notification from its peer OR an indication has been received that the peer has made a request to establish a dialogue.

Data Transfer  (STA2) –  The CF enters this state once the establishment phase is complete.  An association has successfully been established and the communicating partners are free to send and receive data.

Release Pending  (STA3) –  The CF enters this state when the ATN-App ASE has requested the termination of the dialogue OR an indication has been received that the peer has made a request to terminate the dialogue.

Release Collision  (STA4) –  The CF enters this state when both communicating partners have requested the termination of the dialogue near-simultaneously.

#### CF State Table

##### The ATN-App AE CF shall behave as if it has a control function which functions in accordance with the state table specified in Table 3.4, which shows diagrammatically the state transitions and actions performed by the CF in response to incoming events.

Note. —  Incoming events are shown in the first two columns of the state table, and are enumerated in table 3.5.  Each cell in the state table shows:

- optionally, one or more predicates, denoted “pN”, where N is an integer.  The state and action which follow the predicate are only valid if the predicate is TRUE.

- the new state that the CF enters after the action has been performed

- the action, if any, which the CF performs.  The possible actions are outlined in table 3.6.

Blank cells indicate error conditions.

##### When an input event occurs and the state table indicates a state transition, the CF shall enter the new state after any associated action has been performed.

##### When an input event occurs and the state table indicates an action, the CF shall perform that action.

##### For the purpose of specifying CF behaviour, embedded ASEs (ATN-App ASE and ACSE) shall be treated as atomic entities, such that when an input event is invoked by the CF, that event is processed to completion by the ASE and the CF responds to any resulting output events from the ASE, all within the same logical processing thread.

Note. —  This provision avoids the need to specify further transient states within the CF.  It does not imply any particular implementation architecture.

##### The following combinations of input events and CF states shall be treated as error conditions:

- The occurrence of an input event other than those listed in Table 3.5; or

- A combination of input event and CF state which corresponds to a blank cell in Table 3.4; or

- A combination of input event and CF state which corresponds to a cell in Table 3.4 containing one or more predicates, none of which evaluates to TRUE.

##### The error handling shall result in the association being aborted, if one exists, and a notification being given to the Application user.

##### In the event of a conflict between the actions implied by the state table and the text in the following sections, the text shall take precedence.

Table 3.4.  ATN-App CF State Table

| Event Source | State--> Event | STA0 Null | STA1 Assoc. Pending | STA2 Data Transfer | STA3 Release Pending | STA4 Release Collision |
| --- | --- | --- | --- | --- | --- | --- |
| From ATN-App | ATN-APP function req | STA0 ATN-App ASE req | STA1 ATN-App ASE req | STA2 ATN-App ASE req | STA3 ATN-App ASE req | STA4 ATN-App ASE req |
|  | ATN-APP function rsp | STA0 ATN-App ASE rsp | STA1 ATN-App ASE rsp | STA2 ATN-App ASE rsp | STA3 ATN-App ASE rsp | STA4 ATN-App ASE rsp |
| From ATN-App ASE (upper) | ATN-APP function ind | STA0 ATN-App ind | STA1 ATN-App ind | STA2 ATN-App ind | STA3 ATN-App ind | STA4 ATN-App ind |
|  | ATN-APP function cnf | STA0 ATN-App cnf | STA1 ATN-App cnf | STA2 ATN-App cnf | STA3 ATN-App cnf | STA4 ATN-App cnf |
|  | D-START req | STA1 A-ASSOC req |  |  |  |  |
| From ATN-App ASE (lower) | D-START rsp+ |  | ~p1: STA1 A-ASSOC rsp+ |  |  |  |
|  | D-START rsp- |  | ~p1: STA1 A-ASSOC rsp- |  |  |  |
|  | D-DATA req |  |  | STA2 P-DATA req (User) | ~p2: STA3 P-DATA req (User) |  |
|  | D-END req |  |  | STA3 A-RELEASE req |  |  |
|  | D-END rsp+ |  |  |  | ~p2: STA3 A-RELEASE rsp+ |  |
|  | D-END rsp- |  |  |  | ~p2: STA3 A-RELEASE rsp- |  |
|  | D-ABORT req |  | STA1 A-ABORT req | STA2 A-ABORT req | STA3 A-ABORT req | STA4 A-ABORT req |
|  | A-ASSOCIATE ind |  | STA1 D-START ind |  |  |  |
|  | A-ASSOCIATE cnf+ |  | STA2 D-START cnf+ |  |  |  |
| From ACSE (upper) | A-ASSOCIATE cnf- |  | STA0 D-START cnf- |  |  |  |
|  | A-RELEASE ind |  |  |  | STA3 D-END ind | p1: STA4 A-RELEASE rsp+ ~p1: STA4 |
|  | A-RELEASE cnf+ |  |  |  | STA0 D-END cnf+ P-U-ABORT req | p1: STA0 D-END cnf+ P-U-ABORT req ~p1: STA4 D-END cnf+ A-RELEASE rsp+ |
|  | A-RELEASE cnf- |  |  |  | STA2 D-END cnf- |  |
|  | A-ABORT ind |  | STA0 D-ABORT ind | STA0 D-ABORT ind | STA0 D-ABORT ind | STA0 D-ABORT ind |
|  | A-P-ABORT ind |  | STA0 D-P-ABORT ind | STA0 D-P-ABORT ind | STA0 D-P-ABORT ind | STA0 D-P-ABORT ind |
|  | P-CONNECT req |  | STA1 P-CONN req |  |  |  |
|  | P-CONNECT rsp+ |  | STA2 P-CONN rsp+ |  |  |  |
| From ACSE (lower) | P-CONNECT rsp- |  | STA0 P-CONN rsp- |  |  |  |
|  | P-RELEASE req |  |  |  | STA3 P-DATA req (RLRQ) |  |
|  | P-RELEASE rsp+ |  |  |  | STA0 P-DATA req(RLRE+) | p1: STA4 P-DATA req (RLRE+) ~p1: STA0 P-DATA req (RLRE+) |
|  | P-RELEASE rsp- |  |  |  | STA2 P-DATA req (RLRE-) |  |
|  | P-U-ABORT req (data) |  | STA0 P-U-ABORT req | STA0 P-DATA req (ABRT) | STA0 P-U-ABORT req | STA0 P-U-ABORT req |
|  | P-U-ABORT req (no data) |  | STA0 P-U-ABORT req | STA0 P-U-ABORT req | STA0 P-U-ABORT req | STA0 P-U-ABORT req |
|  | P-CONNECT ind | STA1 P-CONN ind |  |  |  |  |
|  | P-CONNECT cnf+ |  | STA1 P-CONN cnf+ |  |  |  |
|  | P-CONNECT cnf- |  | STA1 P-CONN cnf- |  |  |  |
| From supporting service | P-DATA ind (RLRQ) |  |  | STA3 P-RELEASE ind | STA4 P-RELEASE ind |  |
|  | P-DATA ind (RLRE+) |  |  |  | STA3 P-RELEASE cnf+ | STA4 P-RELEASE cnf+ |
|  | P-DATA ind (RLRE-) |  |  |  | STA3 P-RELEASE cnf- |  |
|  | P-DATA ind (ABRT) |  | STA1 P-U-ABORT ind P-U-ABORT req | STA2 P-U-ABORT ind P-U-ABORT req | STA3 P-U-ABORT ind P-U-ABORT req | STA4 P-U-ABORT ind P-U-ABORT req |
|  | P-DATA ind (User) |  |  | STA2 D-DATA ind | STA3 D-DATA ind |  |
|  | P-U-ABORT ind |  | P-U-ABORT ind | P-U-ABORT ind | P-U-ABORT ind | P-U-ABORT ind |
|  | P-P-ABORT ind |  | P-P-ABORT ind | P-P-ABORT ind | P-P-ABORT ind | P-P-ABORT ind |

##### Predicates

p1:  this CF is the Initiating CF, i.e. the CF which issued the A-ASSOCIATE request primitive.

~p1:  this CF is the Responding CF, i.e. the CF which received A-ASSOCIATE indication primitive.

p2:  this CF is the Release Initiator, i.e. the CF issued an A-RELEASE request primitive.

~p2:  this CF is the Release Responder, i.e. the CF received an A-RELEASE indication primitive.

##### Input events

Table 3.5.  Incoming Event List

| Abbreviated name | Source | Description |
| --- | --- | --- |
| ATN-APP function req | upper AE service boundary | Application-specific Request primitive issued by the Application User |
| ATN-APP function rsp |  | Application-specific Response primitive issued by the Application User |
| ATN-APP function ind | ATN-App ASE (upper service boundary) | Application-specific Indication primitive issued by the Application ASE |
| ATN-APP function cnf |  | Application-specific Confirmation primitive issued by the Application ASE |
| D-START req | ATN-App ASE (lower service boundary) | D-START Request primitive issued by DS-User |
| D-START rsp+ |  | D-START Response primitive issued by DS-User, with Result = accepted |
| D-START rsp- |  | D-START Response primitive issued by DS-User, with Result = rejected (transient) or rejected (permanent) |
| D-DATA req |  | D-DATA Request primitive issued by DS-User |
| D-END req |  | D-END Request primitive issued by DS-User |
| D-END rsp+ |  | D-END Response primitive issued by DS-User, with Result = accepted |
| D-END rsp- |  | D-END Response primitive issued by DS-User, with Result = rejected |
| D-ABORT req |  | D-ABORT Request primitive issued by DS-User |
| A-ASSOCIATE ind | ACSE (upper service boundary) | A-ASSOCIATE Indication primitive issued by ACSE service |
| A-ASSOCIATE cnf+ |  | A-ASSOCIATE Confirmation primitive issued by ACSE service, with Result = accepted |
| A-ASSOCIATE cnf- |  | A-ASSOCIATE Confirmation primitive issued by ACSE service, with Result = rejected (transient) or rejected (permanent) |
| A-RELEASE ind |  | A-RELEASE Indication primitive issued by ACSE service |
| A-RELEASE cnf+ |  | A-RELEASE Confirmation primitive issued by ACSE service, with Result = affirmative |
| A-RELEASE cnf- |  | A-RELEASE Confirmation primitive issued by ACSE service, with Result = negative |
| A-ABORT ind |  | A-ABORT Indication primitive issued by ACSE service |
| A-P-ABORT ind |  | A-P-ABORT Indication primitive issued by ACSE service |
| P-CONNECT req | ACSE (lower service boundary) | P-CONNECT Request primitive issued by ACPM |
| P-CONNECT rsp+ |  | P-CONNECT Response primitive issued by ACPM, with Result = acceptance |
| P-CONNECT rsp- |  | P-CONNECT Response primitive issued by ACPM, with Result = user-rejection or provider-rejection |
| P-RELEASE req |  | P-RELEASE Request primitive issued by ACPM |
| P-RELEASE rsp+ |  | P-RELEASE Response primitive issued by ACPM, with Result = affirmative |
| P-RELEASE rsp- |  | P-RELEASE Response primitive issued by ACPM, with Result = negative |
| P-U-ABORT req (data) |  | P-U-ABORT Request primitive issued by ACPM, with the User Data parameter present. |
| P-U-ABORT req (no data) |  | P-U-ABORT Request primitive issued by ACPM, with the User Data parameter empty or absent. |
| P-CONNECT ind | supporting service | P-CONNECT Indication primitive issued by presentation service provider |
| P-CONNECT cnf+ |  | P-CONNECT Confirmation primitive issued by presentation service provider, with Result = acceptance |
| P-CONNECT cnf- |  | P-CONNECT Confirmation primitive issued by presentation service provider, with Result = user-rejection or provider-rejection |
| P-DATA ind (RLRQ) |  | P-DATA Indication primitive issued by presentation service provider, with a RLRQ APDU as User-Data |
| P-DATA ind (RLRE+) |  | P-DATA Indication primitive issued by presentation service provider, with a RLRE APDU as User-Data, with the reason field set to “normal” |
| P-DATA ind (RLRE-) |  | P-DATA Indication primitive issued by presentation service provider, with a RLRE APDU as User-Data, with the reason field set to “not-finished” |
| P-DATA ind (ABRT) |  | P-DATA Indication primitive issued by presentation service provider, with an ABRT APDU as User-Data |
| P-DATA ind (User) |  | P-DATA Indication primitive issued by presentation service provider, with an ATN-APP APDU (e.g. an ADS-ASE protocol data unit) as User-Data |
| P-U-ABORT ind |  | P-U-ABORT Indication primitive issued by presentation service provider |
| P-P-ABORT ind |  | P-U-ABORT Indication primitive issued by presentation service provider |

##### Actions taken by the CF

Table 3.6.  Outgoing Event List

| Abbreviated name | Target | Description |
| --- | --- | --- |
| ATN-App ind | upper AE service boundary | Application-specific Indication primitive mapped transparently from the upper service boundary of the ATN-App ASE. |
| ATN-App cnf |  | Application-specific Confirmation primitive mapped transparently from the upper service boundary of the ATN-App ASE. |
| ATN-App ASE req | upper ATN-App ASE service boundary | Application-specific Request primitive mapped transparently from the upper AE service boundary |
| ATN-App ASE rsp |  | Application-specific Response primitive mapped transparently from the upper AE service boundary. |
| D-START ind | DS-User | D-START Indication primitive issued. |
| D-START cnf+ |  | D-START Confirmation primitive issued, with the Result parameter set to the abstract value “accepted” |
| D-START cnf- |  | D-START Confirmation primitive issued, with the Result parameter set to the abstract value “rejected (transient)” or “rejected (permanent)”, according to the A-ASSOCIATE Confirmation primitive which was received. |
| D-DATA ind |  | D-DATA Indication primitive issued. |
| D-END ind |  | D-END Indication primitive issued. |
| D-END cnf+ |  | D-END Confirmation primitive issued, with the Result parameter set to the abstract value “accepted”. |
| D-END cnf- |  | D-END Confirmation primitive issued, with the Result parameter set to the abstract value “rejected”. |
| D-ABORT ind |  | D-ABORT Indication primitive issued. |
| D-P-ABORT ind |  | D-P-ABORT Indication primitive issued. |
| A-ASSOC req | ACSE service provider | A-ASSOCIATE Request primitive issued |
| A-ASSOC rsp+ |  | A-ASSOCIATE Response primitive issued, with Result = “accepted” |
| A-ASSOC rsp- |  | A-ASSOCIATE Response primitive issued, with Result = “rejected (transient)” or “rejected (permanent)”, according to the D-START response primitive which was received. |
| A-RELEASE req |  | A-RELEASE Request primitive issued. |
| A-RELEASE rsp+ |  | A-RELEASE Response primitive issued, with Result = “affirmative” and Reason = “normal” |
| A-RELEASE rsp- |  | A-RELEASE Response primitive issued, with Result = “negative” and Reason = “not-finished” |
| A-ABORT req |  | A-ABORT Request primitive issued. |
| P-CONN ind | lower ACSE service boundary | P-CONNECT Indication primitive invoked. |
| P-CONN cnf+ |  | P-CONNECT Confirmation primitive invoked, with the Result parameter set to “acceptance”. |
| P-CONN cnf- |  | P-CONNECT Confirmation primitive invoked, with the Result parameter set to “user-rejection”. |
| P-RELEASE ind |  | P-RELEASE Indication primitive invoked. |
| P-RELEASE cnf+ |  | P-RELEASE Confirmation primitive invoked, with the Result parameter set to “affirmative”. |
| P-RELEASE cnf- |  | P-RELEASE Confirmation primitive invoked, with the Result parameter set to “negative”. |
| P-U-ABORT ind |  | P-U-ABORT Indication primitive invoked. |
| P-P-ABORT ind |  | P-P-ABORT Indication primitive invoked. |
| P-CONN req | supporting service | P-CONNECT Request primitive issued. |
| P-CONN rsp+ |  | P-CONNECT Response primitive issued, with the Result parameter set to “acceptance”. |
| P-CONN rsp- |  | P-CONNECT Response primitive issued, with the Result parameter set to “user-rejection”. |
| P-DATA req (RLRQ) |  | P-DATA Request primitive issued. The User Data parameter contains a RLRQ APDU. |
| P-DATA req (RLRE+) |  | P-DATA Request primitive issued. The User Data parameter contains a RLRE APDU, with the reason field set to “normal”. |
| P-DATA req (RLRE-) |  | P-DATA Request primitive issued. The User Data parameter contains a RLRE APDU, with the reason field set to “not-finished”. |
| P-DATA req (ABRT) |  | P-DATA Request primitive issued. The User Data parameter contains an ABRT APDU, with a non-empty user-information field. |
| P-DATA req (User) |  | P-DATA Request primitive issued. The User Data parameter contains an ATN-App ASE APDU (e.g. an ADS-ASE protocol data unit) |
| P-U-ABORT req |  | P-U-ABORT Request primitive issued. |

### Services Invoked by the Application User

Note 1. —  This section defines the actions that result from inputs generated by the user of this ATN-App AE (see figure 3-1).

Note 2. —  The service primitives available to the Application User are specific to the ATN application.  This service is detailed in the CNS/ATM-1 package Application SARPs.

#### Application User Request and Response primitives

##### When Invoked

###### Application User Request and Response primitives may be validly invoked by the Application-user when the CF is in any state.

##### Action Upon Invocation

###### When the Application User Request or Response primitive is issued, the CF shall:

- Invoke the equivalent primitive of the ATN-App ASE service, with a one-to-one mapping of parameters; and

- Remain in its current state.

### Services Invoked by ATN-App ASE

#### ATN-App ASE Indication and Confirmation primitives

##### When Invoked

###### ATN-App ASE Indication and Confirmation primitives may be validly invoked by the ATN-App ASE when the CF is in any state.

##### Action Upon Invocation

When the ATN-App ASE Indication or Confirmation primitive is issued, the CF shall:

- Invoke the equivalent primitive of the Application-user service with a one-to-one mapping of parameters; and

- Remain in its current state.

#### D-START Request primitive

##### When Invoked

###### The D-START Request primitive may be validly invoked by the ATN-App ASE when the CF is in the NULL state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When the D-START Request is validly invoked, the CF shall:

- Retrieve the AE-qualifier as defined for the ATN-App AE,

- Construct the Application Context name, with the value of the final arc set equal to the DS-User Version Number parameter if provided, and set to zero otherwise.

- Retrieve the calling Presentation address

- Look up the called Presentation address from the Called Peer Id parameter.

- If the Calling Peer Id parameter is present, retrieve the Calling AP Title and calling AE-qualifier.

- If the Security Requirements parameter is not present, the A-ASSOCIATE parameter “ACSE Requirements” shall not be used.  If the Security Requirements parameter is present, the parameter mappings shall be as follows: ACSE Requirements parameter set to the symbolic value “authentication”; Security Requirements value mapped to the A-ASSOCIATE Authentication-value parameter.

- Construct an A-ASSOCIATE Request primitive with the following parameters:

Table 3.7

| A-ASSOCIATE Request parameter | ISO Status | ATN value |
| --- | --- | --- |
| Mode | U | Not used (default value) |
| Application Context Name | M | As derived in b) above |
| Application Context Name List | C | Not used |
| Calling AP Title | U | As derived in e) above |
| Calling AE Qualifier | U | As derived in e) above |
| Calling AP Invocation-identifier | U | Not used |
| Calling AE Invocation-identifier | U | Not used |
| Called AP Title | U | Not used |
| Called AE Qualifier | U | Not used |
| Called AP Invocation-identifier | U | Not used |
| Called AE Invocation-identifier | U | Not used |
| ACSE Requirements | U | As derived in f) above |
| Authentication-mechanism Name | U | Not used |
| Authentication-value | U | As derived in f) above |
| User Information | U | D-START User Data parameter |
| Calling Presentation Address | M | Derived as in c) above |
| Called Presentation Address | M | Derived as in d) above |
| Presentation Context Definition List | U | Not used |
| Default Presentation Context Name | U | Not used |
| Quality of Service | M | See following subsection |
| Presentation Requirements | U | Not used (default value) |
| Session Requirements | M | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | C | Not used |
| Initial Assignment of Tokens | C | Not used |
| Session-connection Identifier | U | Not used |

- Invoke the A-ASSOCIATE Request primitive

- Enter the ASSOCIATION PENDING state as an initiator CF.

##### Quality of Service parameter mappings

###### The Quality of Service parameters in D-START Request and Response primitives shall be conveyed to the ATN Internet as specified in this subsection.

###### Routing Class shall be conveyed by local means, using the values for Security Tag Value specified in Table 6-1 of Sub-Volume 5 of the CNS/ATM-1 Package SARPs.  If no value is specified in the D-START Request primitive, then the value corresponding to “No Traffic Type Policy Preference” shall be conveyed.  If no value is specified in the D-START Response primitive, then the value shall be taken to be the same as that which was passed to the DS-User in the D-START Indication primitive.

Note. —  Sub-Volume 5 states, in section 2.7.3.1, “The mechanism by which the connection initiator determines the appropriate ATN Security Label is a local matter.  For example, it may be identified by an extension to the transport service interface, be implicit in the choice of a given TSAP, or be identified using a Systems Management function.”

###### Priority shall map to the session connection priority component of the A-ASSOCIATE Quality of Service parameter, using the values for Transport Layer Priority specified in Table 2-3 of Sub-Volume 5 of the CNS/ATM-1 Package SARPs.  If no value is specified in the D-START Request primitive, then the value corresponding to “Network/systems administration” shall be conveyed.  If no value is specified in the D-START Response primitive, then the value shall be taken to be the same as that which was passed to the DS-User in the D-START Indication primitive.

Note. —  Although transport priority and network priority are semantically independent of each other, Sub-Volume 5 requires, in section 5.1.2, that the TS-user specifies the Application Service Priority, which in turn is mapped into the resulting CLNP PDUs according to Table 2-3, which defines the fixed relationship between transport priority and the network priority.

###### RER shall map to the residual error rate component of the A-ASSOCIATE Quality of Service parameter, such that if the RER has the abstract value “low”, then the residual error rate is set to zero, and if it has the abstract value “high”, then the residual error rate in the A-ASSOCIATE is set to the value 1.  If no value is specified in the D-START Response primitive, then the value shall be taken to be the same as that which was passed to the DS-User in the D-START Indication primitive.

Note. —  Sub-Volume 5 requires, in section 5.1.2, that the TS-user specifies the required residual error rate to determine whether or not the transport checksum is required.

#### D-START Response primitive

##### When Invoked

###### The D-START Response primitive may be validly invoked by the ATN-App ASE when the responder CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a D-START Response primitive is validly invoked, the CF shall exhibit the following behaviour:

- Construct the Application Context name, with the value of the final arc set equal to the DS-User Version Number parameter if provided, and set to zero otherwise.

- Retrieve the responding Presentation address

- If the Security Requirements parameter is not present, the A-ASSOCIATE parameter “ACSE Requirements” shall not be used.  If the Security Requirements parameter is present, the parameter mappings shall be as follows: ACSE Requirements parameter set to the symbolic value “authentication”;  Security Requirements value mapped to the A-ASSOCIATE Authentication-value parameter.

- Construct an A-ASSOCIATE Response primitive with the following parameters:

Table 3.8

| A-ASSOCIATE Response parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Application Context Name | M | As derived in a) above |
| Application Context Name List | C | Not used |
| Responding AP Title | U | Not used |
| Responding AE Qualifier | U | Not used |
| Responding AP Invocation-identifier | U | Not used |
| Responding AE Invocation-identifier | U | Not used |
| ACSE Requirements | C | As derived in c) above |
| Authentication-mechanism Name | U | Not used |
| Authentication-value | U | As derived in c) above |
| User Information | U | D-START User Data parameter |
| Result | M | D-START Result parameter |
| Diagnostic | U | Not used |
| Responding Presentation Address | M | Derived as in b) above |
| Presentation Context Definition Result List | C | Not used |
| Default Presentation Context Result | C | Not used |
| Quality of Service | M | As for D-START Request (see preceding section) |
| Presentation Requirements | U | Not used (default value) |
| Session Requirements | M | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | C | Not used |
| Initial Assignment of Tokens | C | Not used |
| Session-connection Identifier | U | Not used |

- If the D-START Response Result parameter has the abstract value “accepted”, invoke an A-ASSOCIATE Response primitive with the Result parameter set to “accepted”, and remain in the ASSOCIATION PENDING state.

- If the D-START Response Result parameter has the abstract value “rejected (permanent)” or “rejected (transient)”, invoke an A-ASSOCIATE Response primitive with the Result parameter set to the same abstract value, and remain in the ASSOCIATION PENDING state.

#### D-END Request primitive

##### When Invoked.

###### The D-END Request primitive may be validly invoked by the ATN-App ASE when the CF is in the DATA TRANSFER state; if it is in any other state then appropriate error recovery action shall be taken.  For example, if the CF is in the RELEASE PENDING state, then the D-END Request shall be rejected locally, with an appropriate result code.

##### Action Upon Invocation

###### When a D-END Request primitive is validly invoked, the CF shall exhibit the following behaviour:

- Construct an A-RELEASE Request primitive with the following parameter values:

Table 3.9

| A- RELEASE Request parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Reason | U | “normal” |
| User Information | U | D-END User Data parameter |

- Invoke the A-RELEASE Request primitive; and

- Enter the RELEASE PENDING state.

#### D-END Response primitive

##### When Invoked

###### The D-END Response primitive may be validly invoked by the ATN-App ASE when the responder CF is in the RELEASE PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a D-END Response primitive is validly invoked and the Result parameter has the value “accepted”, the CF shall exhibit the following behaviour:

- Construct an A-RELEASE Response primitive with parameter values as follows:

Table 3.10

| A- RELEASE Response parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Reason | U | “normal” |
| User Information | U | D-END User Data parameter |
| Result | M | “affirmative” |

- Invoke the A-RELEASE Response primitive

- Remain in the RELEASE PENDING state.

###### When a D-END Response primitive is validly invoked and the  Result parameter has the abstract value “rejected” the CF shall exhibit the following behaviour:

- Construct an A-RELEASE Response primitive with parameter values as follows:

Table 3.11

| A- RELEASE Response parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Reason | U | “not finished” |
| User Information | U | D-END User Data parameter |
| Result | M | “negative” |

- Invoke the A-RELEASE Response primitive; and

- Remain in the RELEASE PENDING state.

#### D-DATA Request primitive

##### When Invoked

###### The D-DATA Request primitive may be validly invoked by the ATN-App ASE when the CF is in the DATA TRANSFER state, or (if it is the Release Responder) in the RELEASE PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a D-DATA Request primitive is validly invoked, the CF shall exhibit the following behaviour:

- Using the definition of presentation-user-data in section 3.2.6, encode the D-DATA Request User Data parameter with presentation-context-identifier value corresponding to “user-ase-apdu”;

- Invoke a P-DATA Request primitive with the resulting encoding as User Data; and

- Remain in the same state.

#### D-ABORT Request primitive

##### When Invoked

###### The D-ABORT Request primitive may be validly invoked by the ATN-App ASE when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When a D-ABORT Request primitive is validly invoked, the CF shall exhibit the following behaviour:

- If the Originator parameter of the D-ABORT has the symbolic value “User”, then set Diagnostic to “No reason given”.  If the Originator parameter is absent or has any symbolic value other than “User”, then set Diagnostic to “Protocol error”.

- Construct an A-ABORT Request primitive with the following parameter values:

Table 3.12

| A- ABORT Request parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Diagnostic | U | derived as in a) above |
| User Information | U | D-ABORT User Data parameter, if present and not empty. |

- Invoke the A-ABORT Request primitive; and

- Remain in the same state.

### ACSE Services delivered to the CF

Note. —  This section describes events which occur at the upper service boundary of ACSE, i.e. Indication and Confirmation primitives which are generated by the ACPM and which require handling by the CF.

#### A-ASSOCIATE Indication primitive

##### When Invoked

###### The A-ASSOCIATE Indication primitive may be validly invoked by the ACSE Protocol Machine (ACPM) when the CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When an A-ASSOCIATE Indication primitive is validly invoked, the CF shall:

- If the final component of the Application Context Name parameter is non-zero, then use it as the DS-User Version Number in the D-START Indication primitive.  If it has the value zero, then omit the DS-User Version Number parameter in the D-START Indication.

- If the Calling AP Title parameter is present, extract the Calling Peer Id from it.

- If the ACSE Requirements parameter is present, and it indicates that the authentication functional unit is requested, then extract the Authentication-value parameter.

- Construct a D-START Indication primitive, with the following parameter values:

Table 3.13

| D-START Indication parameter | Value |
| --- | --- |
| Calling Peer ID | Derived as in b) above |
| DS-User Version Number | Derived as in a) above |
| Security Requirements | Derived as in c) above |
| Quality Of Service | See following subsection |
| User Data | A-ASSOCIATE User Information parameter |

- Remain in the ASSOCIATION PENDING state.

##### Quality of Service parameter mappings

###### The Quality of Service parameters in A-ASSOCIATE Indication and Confirmation primitives shall be conveyed to the DS-User as parameters of the D-START Indication and Conformation primitives, as specified in this subsection.

###### Routing Class shall be conveyed by local means, using the abstract values for Security Tag Values as specified in Table 6-1 of Sub-Volume 5 of the CNS/ATM-1 Package SARPs.

###### Priority shall be taken from the session connection priority component of the A-ASSOCIATE Quality of Service parameter, using the abstract values for Transport Layer Priority as specified in Table 2-3 of Sub-Volume 5 of the CNS/ATM-1 Package SARPs.

###### RER shall be taken from the residual error rate component of the A-ASSOCIATE Quality of Service parameter, such that if the residual error rate has a value less than 10-6 then RER is set to “low”, otherwise it is set to “high”.

#### A-ASSOCIATE Confirmation primitive

##### When Invoked

###### The A-ASSOCIATE Confirmation primitive may be validly invoked by the ACPM when the CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When an A-ASSOCIATE Confirmation primitive is validly invoked, and the  Result parameter has the abstract value “accepted” the CF shall:

- If the final component of the Application Context Name parameter is non-zero, then use it as the DS-User Version Number in the D-START Confirmation primitive.  If it has the value zero, then omit the DS-User Version Number parameter in the D-START Confirmation.

- If the ACSE Requirements parameter is present, and it indicates that the authentication functional unit is selected, then extract the Authentication-value parameter.

- Construct a D-START Confirmation primitive, with parameter values as follows:

Table 3.14

| D-START Confirmation parameter | Value |
| --- | --- |
| DS-User Version Number | Derived as in a) above |
| Security Requirements | Derived as in b) above |
| Quality Of Service | As for A-ASSOCIATE Indication (see preceding section) |
| Result | “accepted” |
| Reject Source | Not used |
| User Data | A-ASSOCIATE User Information parameter |

- Invoke the D-START Confirmation primitive

- Enter the DATA TRANSFER state as the initiator CF.

###### When an A-ASSOCIATE Confirmation primitive is validly invoked, and the Result parameter has the abstract value “rejected (permanent)” or “rejected (transient)” the CF shall:

- Extract the DS-User Version Number, if present, from the final component of the Application Context Name parameter

- If the ACSE Requirements parameter is present, and it indicates that the authentication functional unit is selected, then extract the Authentication-value parameter.

- If the A-ASSOCIATE Confirmation Result Source parameter has the abstract value “ACSE service-user” form a Reject Source parameter with value “DS user”.  If the A-ASSOCIATE Confirmation Result Source parameter has the abstract value “ACSE service-provider” or “presentation service-provider” form a Reject Source parameter with value “DS provider”.

- Construct a D-START Confirmation primitive, with parameter values as follows:

Table 3.15

| D-START Confirmation parameter | Value |
| --- | --- |
| DS-User Version Number | Derived as in a) above |
| Security Requirements | Derived as in b) above |
| Quality Of Service | As for A-ASSOCIATE Indication (see preceding section) |
| Result | “rejected (permanent)” or “rejected (transient)”, from the A-ASSOCIATE Result parameter |
| Reject Source | Derived as in c) above |
| User Data | A-ASSOCIATE User Information parameter |

- Invoke the D-START Confirmation primitive

- Enter the NULL state.

#### A-RELEASE Indication primitive

##### When Invoked

###### The A-RELEASE Indication primitive may be validly invoked by the ACPM when the CF is in the RELEASE PENDING or the RELEASE COLLISION state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When an A-RELEASE Indication primitive is validly invoked, and the CF is in the RELEASE PENDING state, it shall:

- Construct a D-END Indication primitive, with the User Data parameter set equal to the value of the User Information parameter of the A-RELEASE Indication primitive.

- Invoke the D-END Indication

- Remain in the RELEASE PENDING state.

###### When an A-RELEASE Indication primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Initiator CF, it shall:

- Construct a D-END Confirmation primitive, with the User Data parameter set equal to the value of the User Information parameter of the A-RELEASE Indication primitive, if present.  The D-END Confirmation shall not be issued to the DS-User until the orderly release procedure is complete, and an A-RELEASE Confirmation is received.

- Construct an A-RELEASE response primitive with parameter values as follows:

Table 3.16

| A- RELEASE Response parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Reason | U | “normal” |
| User Information | U | Not present |
| Result | M | “affirmative” |

- Invoke the A-RELEASE Response primitive; and

- Remain in the RELEASE COLLISION state.

###### When an A-RELEASE Indication primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Responder CF, it shall:

- Construct a D-END Confirmation primitive, with the User Data parameter set equal to the value of the User Information parameter of the A-RELEASE Indication primitive, if present.  The D-END Confirmation shall not be issued to the DS-User until the orderly release procedure is complete, and an A-RELEASE Confirmation is received.

- Remain in the RELEASE COLLISION state.

#### A-RELEASE Confirmation primitive

##### When Invoked

###### The A-RELEASE Confirmation primitive may be invoked by the ACPM when the CF is in the RELEASE PENDING or RELEASE COLLISION state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When an A-RELEASE Confirmation primitive is validly invoked, and the CF is in the RELEASE PENDING state, and the  Result parameter has the abstract value “affirmative” the CF shall:

- Construct a D-END Confirmation primitive with the following parameter values

Table 3.17

| D-END Confirmation parameter | Value |
| --- | --- |
| Result | “affirmative” |
| User Data | User Information parameter from the A-RELEASE Confirmation, if present |

- Invoke the D-END Confirmation primitive.

- Issue a P-U-ABORT request primitive, with no parameters

- Note. —  This will cause the release of the underlying transport connection.

- Enter the NULL state.

###### When an A-RELEASE Confirmation primitive is validly invoked, and the CF is in the RELEASE PENDING state, and the Result parameter has the abstract value “negative” the CF shall:

- Construct a D-END Confirmation primitive with the following parameter values

Table 3.18

| D-END Confirmation parameter | Value |
| --- | --- |
| Result | “rejected” |
| User Data | User Information parameter from the A-RELEASE Confirmation, if present |

- Invoke the D-END Confirmation primitive.

- Enter the DATA TRANSFER state.

###### When an A-RELEASE Confirmation primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Initiator CF, it shall:

- Issue the D-END Confirmation primitive, which was previously formed in response to the reception of an A-RELEASE Indication primitive, to the DS-User.

- Issue a P-U-ABORT request primitive, with no parameters.

- Note. —  This will cause the release of the underlying transport connection.

- Enter the NULL state.

###### When an A-RELEASE Confirmation primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Responder CF, it shall:

- Issue the D-END Confirmation primitive, which was previously formed in response to the reception of an A-RELEASE Indication primitive, to the DS-User.

- Construct an A-RELEASE Response primitive, with the Result parameter set to “affirmative”.

- Invoke the A-RELEASE Response.

- Remain in the RELEASE COLLISION state.

#### A-ABORT Indication primitive

##### When Invoked

###### The A-ABORT Indication primitive may be validly invoked by the ACPM when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When an A-ABORT Indication primitive is validly invoked, the CF shall:

- If the Abort Source parameter of the A-ABORT Indication is set to “ACSE service-user” and the Diagnostic parameter Is set to “No reason given”, issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to “User” and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication, if present.

- If the Abort Source parameter of the A-ABORT Indication is set to “ACSE service-user” and the Diagnostic parameter Is absent or is set to any value other than “No reason given”, then issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to “Provider” and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication, if present.

- If the Abort Source parameter of the A-ABORT Indication has the abstract value “ACSE service-provider”, then issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to the abstract value “Provider”, and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication, if present.

- Enter the NULL state.

#### A-P-ABORT Indication primitive

##### When Invoked

###### The A-P-ABORT Indication primitive may be validly invoked by the ACPM when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When an A-P-ABORT Indication primitive is validly invoked, the CF shall:

- issue a D-P-ABORT Indication primitive to the DS-User, and discard any Provider Reason parameter in the A-P-ABORT Indication; and

- Enter the NULL state.

### Services used by ACSE

Note. — ACSE, edition 2 mandates the mapping of ACSE APDUs to the underlying presentation service provider.  However, when the efficient encoding options of Session and Presentation protocols are used, only the P-CONNECT and P-DATA services are available.  Therefore, invocations of presentation service primitives by ACSE are “intercepted” by the CF and re-mapped to the “actual” presentation service as appropriate.

#### P-CONNECT Request primitive

##### When Invoked

###### The P-CONNECT Request primitive may be validly invoked by the ACPM when the CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a P-CONNECT Request primitive is validly invoked, the CF shall transparently invoke the equivalent presentation service primitive.

#### P-CONNECT Response primitive

##### When Invoked

###### The P-CONNECT Response primitive may be validly invoked by the ACPM when the CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When the P-CONNECT Response primitive is validly invoked, the CF shall:

- transparently invoke the equivalent presentation service primitive.

- If the P-CONNECT Response Result parameter has the abstract value “acceptance” then enter the DATA TRANSFER state, otherwise enter the NULL state.

#### P-U-ABORT Request primitive

##### When Invoked

###### The P-U-ABORT Request primitive may be validly invoked by the ACPM when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When a P-U-ABORT Request primitive is validly invoked, the CF shall:

  - If the P-U-ABORT request user data parameter is present, and the CF is in the DATA TRANSFER state:

    - Encode the presentation user data as indicated in section 3.2 with the P-U-ABORT user data parameter (an ABRT APDU) as the presentation data value and presentation context identifier value corresponding to “acse-apdu”.

    - Invoke a P-DATA Request primitive with the resulting encoding as User Data.

  - Otherwise, invoke a P-U-ABORT Request primitive.

- Note. — this will cause the underlying transport connection to be disconnected.

  - Enter the NULL state.

#### P-RELEASE Request primitive

Note. — ACSE, edition 2 mandates the mapping of A-RELEASE APDUs (RLRQ and RLRE) to the P-RELEASE service.  However, when the efficient encoding options of Session and Presentation protocols are used, the Session No-Orderly Release (NOR) functional unit is selected, and no mapping for the P-RELEASE service is available.  In order to provide an orderly release service, the CF re-maps invocations of the P-RELEASE service at the lower service boundary of ACSE to invocations of the P-DATA service, with the release APDUs transferred as user information.

##### When Invoked

###### The P-RELEASE Request  primitive may be validly invoked by the ACPM when the CF is in the RELEASE PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a P-RELEASE Request primitive is validly invoked, the CF shall:

- Encode the presentation user data as indicated in section 3.2 with the P-RELEASE user data parameter (a RLRQ APDU) as the presentation data value and presentation context identifier corresponding to “acse-apdu”.

- Invoke a P-DATA Request primitive with the resulting encoding as User Data; and

- Remain in the RELEASE PENDING state.

#### P-RELEASE Response primitive

##### When Invoked

###### The P-RELEASE Response primitive may be validly invoked by the ACPM when the CF is in the RELEASE PENDING or RELEASE COLLISION state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a P-RELEASE Response primitive is validly invoked, and the CF is in the RELEASE PENDING state, and the Result parameter has the abstract value “affirmative” the CF shall:

- encode the presentation user data as indicated in section 3.2 with the P-RELEASE user data parameter (a RLRE APDU) as the presentation data value and presentation context identifier corresponding to “acse-apdu”.

- Invoke a P-DATA Request primitive with the resulting encoding as User Data.

- Enter the NULL state.

###### When a P-RELEASE Response primitive is validly invoked, and the CF is in the RELEASE PENDING state, and the Result parameter has the abstract value “negative” the CF shall:

- Encode the presentation user data as indicated in section 3.2 with the P-RELEASE user data parameter (a RLRE APDU) as the presentation data value and presentation context identifier corresponding to “acse-apdu”;

- Invoke a P-DATA Request primitive with the resulting encoding as User Data; and

- Enter the DATA TRANSFER state.

###### When a P-RELEASE Response primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Initiator CF, it shall:

- Encode the presentation user data as indicated in section 3.2 with the P-RELEASE user data parameter (a RLRE APDU) as the presentation data value and presentation context identifier corresponding to “acse-apdu”;

- Invoke a P-DATA Request primitive with the resulting encoding as User Data; and

- Remain in the RELEASE COLLISION state.

###### When a P-RELEASE Response primitive is validly invoked, and the CF is in the RELEASE COLLISION state, and it is the Responder CF, it shall:

- Encode the presentation user data as indicated in section 3.2 with the P-RELEASE user data parameter (a RLRE APDU) as the presentation data value and presentation context identifier corresponding to “acse-apdu”;

- Invoke a P-DATA Request primitive with the resulting encoding as User Data; and

- Enter the NULL state.

### Supporting Services delivered to the CF

Note. —  This section specifies the mapping by the CF of presentation service indication and confirmation primitives which are invoked by the presentation service provider.

When the supporting communications service exhibits the behaviour modelled by the passing of indication or confirmation primitives to the application layer, the ATN upper layers shall exhibit the behaviour specified in the following subsections.

#### P-CONNECT Indication primitive

##### When Invoked

###### The P-CONNECT Indication primitive may be validly invoked by the supporting service when the CF is in the NULL state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a P-CONNECT Indication primitive is validly invoked, the CF shall:

- transparently invoke the equivalent presentation service primitive at the lower ACSE service boundary; and

- enter the ASSOCIATION PENDING state as the responder CF.

#### P-CONNECT Confirmation primitive

##### When Invoked

###### The P-CONNECT Confirmation primitive may be validly invoked by the supporting service when the CF is in the ASSOCIATION PENDING state; if it is in any other state then appropriate error recovery action shall be taken.

##### Action Upon Invocation

###### When a P-CONNECT Confirmation primitive is validly invoked, the CF shall:

- transparently invoke the equivalent presentation service primitive at the lower ACSE service boundary; and

- Remain in the ASSOCIATION PENDING state.

#### P-U-ABORT Indication primitive

##### When Invoked

###### The P-U-ABORT Indication primitive may be validly invoked by the supporting service when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When a P-U-ABORT Indication primitive is validly invoked, the CF shall transparently invoke the equivalent presentation service primitive at the lower ACSE service boundary, and remain in the same state.

#### P-P-ABORT Indication primitive

##### When Invoked

###### The P-P-ABORT Indication primitive may be validly invoked by the supporting service when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When a P-P-ABORT Indication primitive is validly invoked, the CF shall transparently invoke the corresponding presentation service primitive at the lower ACSE service boundary, and remain in the same state.

#### P-DATA Indication primitive

##### When Invoked

###### The P-DATA Indication primitive may be validly invoked by the supporting service when the CF is in any state, except NULL.

##### Action Upon Invocation

###### When a P-DATA Indication primitive is validly invoked, the CF shall:

  - Decode the presentation user data as indicated in section 3.2 to determine the destination ASE of the APDU, and extract the presentation data value.

  - If the destination ASE is ACSE, decode the type of APDU and behave as follows:

    - If the received APDU is RLRQ, then invoke a P-RELEASE Indication primitive at the ACSE lower service boundary, with the RLRQ as User Data.  If the CF is in the DATA TRANSFER state, then enter the RELEASE PENDING state.  If the CF is in the RELEASE PENDING state, then enter the RELEASE COLLISION state.

    - If the received APDU is RLRE, with the Reason field in the RLRE set to “not-finished”, and the CF is in the RELEASE PENDING state, then invoke a P-RELEASE Confirmation primitive at the ACSE lower service boundary, with the result parameter set to “negative”, and the RLRE as User Data; remain in the RELEASE PENDING state.

    - If the received APDU is RLRE, with the Reason field in the RLRE set to “normal”, and the CF is in the RELEASE PENDING or RELEASE COLLISION state, then invoke a P-RELEASE Confirmation primitive at the ACSE lower service boundary, with the result parameter set to “affirmative”, and the RLRE as User Data; remain in the same state.

    - If the received APDU is ABRT, then invoke a P-U-ABORT Indication primitive at the ACSE lower service boundary, with the ABRT as User Data, and issue a P-U-ABORT request to the underlying service; remain in the same state.

    - Otherwise, if the received APDU is not RLRQ, RLRE or ABRT, an error has occurred; issue a P-U-ABORT request to the supporting service and behave as if a P-U-ABORT indication had been received.

  - If the destination ASE is ATN-App ASE, and the CF is in the DATA TRANSFER or RELEASE PENDING state, then issue a D-DATA Indication primitive to the DS-User, with the APDU as the user data parameter; remain in the same state.

  - Otherwise, if the destination ASE is neither ACSE nor the ATN-App ASE, then an error has occurred; issue a P-U-ABORT request to the supporting service and behave as if a P-U-ABORT indication had been received.