# GENERAL REQUIREMENTS

## Performance Requirements

Note. — System developed to support CNS/ATM-1 Package FIS functionality will be capable of meeting the communication requirements appropriate for the phase of operation as specified in [5].

## Time Accuracy Requirements

### Absolute times which are sent as parameters over the data link shall be as accurate as the required resolution of the time parameter.

### Absolute times shall be UTC.

### Dates shall be expressed as UTC date (i.e., the day increments at 2359:59z).

### Relatives times shall be accurate to ñ 0.1 second.

### Where timestamps are used they shall consist of year, month, day and hour, minute, second.

## Security Requirements

Note — There are no internationally approved operational requirements relating to data link application security.

## Backwards Compatibility Requirements

Note.— This specification describes the version 1 of the CNS/ATM-1 Package FIS application.  Best effort will be made to ensure that subsequent versions of this protocol are backwards compatible.

### For CNS/ATM-1 the FIS-air-ASE and FIS-ground-ASE version numbers shall both be set to one.

## Error Processing Requirements

### In the event of information input by the user being incompatible with that able to be processed by the system, the user shall be notified.

### In the event of a user invoking a FIS service primitive when the FIS-ASE is not in a state specified in chapter 5, the following shall occur:

- the invocation is rejected, and

- the user is notified.