|  | ATNP/WG3/IP 12-05 17 March 1998 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Rio de Janeiro, Brazil,  16 - 20 March 1998

Information Paper

ATN System Management Material from ATNP WG1

Presented by: Tony Kerr

SUMMARY

This paper provides WG3 with a snapshot of the ATN System Management material under development by ATNP/WG1/SG3.

# 1	Introduction

During WG3 discussions of ATN Systems Management on 17th March, it was decided that the high level material being developed by WG1/SG3 would be useful background in order to put the WG3 material into context.  This Information Paper therefore includes the following attachments:

Attachment 1.  Draft change pages for the Core ATN SARPs

Attachment 2.  Draft change pages for Sub-Volume 1

Attachment 3.  Draft Guidance Material for the Core SARPs and Sub-Volume 1 Systems Management provisions.

# 2	Additional Definitions

The system management functions referred to in the attached material are

- Fault Management;

- Configuration Management;

- Accounting Management;

- Performance Management;

- Security Management.

These are defined as follows:

Fault Management.  Covers the detection and location of faults.  Provides management tools either to test systems to trace faults or to give confidence in the success of repairs which might have been carried out.

Configuration Management.  Enables the management of functions concerned with the start-up and shut-down of communication systems.  Establishes and maintains information defining the interrelationship of resources within an ATN management domain and provides information on the current state of those resources.

Accounting Management.  Allows manager applications to determine the use made of resources, and if necessary to set quotas on the extent to which a resource can be made available to a particular user in a given accounting interval.

Performance Management.  Enables the behaviour of resources and the effectiveness of communication activities to be evaluated.  Includes functions to gather statistical information, maintain and examine logs of system state histories, determine system performance under natural and artificial conditions and alter system modes of operation.

Security Management.  Provides management functions which support ATN security services by distributing security related information and alerting managers to events which could compromise security.