# MANAGEMENT INFORMATION GENERAL PROVISIONS

Editor’s note.— Provisions for encoding MO attributes in PER need to be considered.  Potentially all MOs need to be augmented with PER-visible constraints and extensibility markers.

ATN Management Information is defined by specifying:

- the managed object class definition of ATN MOs following the MO template that has been proposed for use in ATN SARPs,

- the action type operations on the attributes of ATN MOs that are available to ATN System Management.

## Symbols, abbreviations and terms

In each table, the "ISO Status" column indicates the conformance requirement as specified in the ISO/IEC base standard that defines the MO.  A hierarchy exists, so that the conformance requirements of a dependent feature only apply if the "parent" feature is supported (e.g. if an MO class is not supported, then none of the attributes will be supported, even if classified as "M").  Possible values for ISO Status are:

M - Mandatory to implement

O - Optional to implement

C - Dependent upon some Condition explained in a footnote to the table

A - Feature is ATN-specific, i.e. not present in base standard.

The "ATN Status" column indicates the conformance requirement as specified in the ATN SARPs.  Notes may be used to expand on the support requirement, e.g. to differentiate between different types of ATN system.  Possible values for ATN Status are:

M - Mandatory to implement (equivalent to a "shall" statement)

R - Recommended to implement (equivalent to a "should" statement)

O - Optional to implement (i.e. an implementation is free to implement the feature or not)

X - Prohibited to implement.

## Global Containment Tree for One System

The upper part of the global containment tree (naming hierarchy) for one system shall be as illustrated in Figure 6-X.

Note.— The subordinate nodes in the containment tree are defined in subsequent chapters.

Figure 6-X.  Global containment tree for one system

## “System” MO Classes

The generic attributes “objectClass”,  “nameBinding” and  “packages” (inherited from “top”) are implicitly included in every object class; thus they are not shown in other MO classes.

### System MO

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | system System Class is ISO/IEC 10165-2 system class. There is only one such instance of this outmost external container of all MOs. | M | M |
|  | Naming attribute | systemId | M | M |
|  | Superior in Naming Tree | <none> |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | administrativeState Indicates the permission to use the system, imposed through the management services. Can take following values: LOCKED, UNLOCKED. | GET, REPDEF | O | X |
|  | operationalState Indicates whether the system is physically installed and working, if applicable.Can take following values: DISABLED, ENABLED. | GET | M | X |
|  | supportedFeatures Identifies features within the system that are capable of being managed. | GET | O | X |
|  | systemId Naming attribute. | GET | M | X |
|  | systemTitle Used by the Manager to uniquely identify the system (Object IDentifier). | GET | M | X |
|  | usageState Indicates whether the system is actively in use, and if so, whether or not it has spare capacity for additional users. Can take one of the following values : IDLE, ACTIVE, BUSY. | GET | M | X |

#### Actions

None.

#### Notifications

| Index | Notification Name (Description) Syntax | ISO Status | ATN Status |
| --- | --- | --- | --- |
|  | stateChangeAlarm This notification is sent upon start/stop of system | O | X |

### EventForwardingDiscriminator MO

Note.— This MO is exactly as defined in the ISO standards

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | EventForwardingDiscriminator This object class allows to filter the events generated , and to decide whether they should be sent by the Agent to the Manager. [CREATE/DELETE] operations supported on the MO. There are different instances for this MO. | O | M |
|  | Naming attribute | discriminatorId | M | M |
|  | Superior in Naming Tree | system | O | M |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | activeDestination An Application Entity Title (AET) which identifies the Application Entity (AE) to which events are currently forwarded by the discriminator. | GET | O | O |
|  | administrativeState Indicates the permission to use the log, imposed through the management services. Can take following values: LOCKED, UNLOCKED. | GET SET | M | M |
|  | availabilityStatus Can take the zero value or any of the following values: IN TEST, FAILED, POWER OFF, OFF LINE, OFF DUTY, DEPENDENCY, DEGRADED, NOT INSTALLED, LOG FULL. | GET | O | O |
|  | backUpDestinationList An ordered list of AETs identifying AEs to be used as destinations if the destination specified in the destination attribute fails. This attribute is not used when the destination attribute has multiple values. | GET SET | O | O |
|  | destination Identifies the destination(s) to which the discriminator forwards event reports. May be a single AET or multiple AETs. | GET SET | O | O |
|  | discriminatorConstruct Specifies tests on the information that is sent to the manager. The discriminator construct may operate on any parameter of the information that is bound to be sent to the manager. Different profiles shall be built: - no information to be sent to Manager, except Alarms, - all information to be sent to Manager, - authorized events for Network LME plus Alarms, - authorized events for Transport LME plus Alarms, - authorized events for Application LME plus Alarms, - authorized events for System LME plus Alarms. | GET SET REPDEF | M | M |
|  | discriminatorId Naming attribute. | GET | M | M |
|  | intervalsOfDay Defines the list of time intervals for which the log will exhibit the logging-on condition. During excluded intervals, the log exhibits the logging-off condition. If not specified at creation time, its value defaults to a single interval encompassing the entire 24 hour period of a day. | GET SET REPDEF ADD REMOVE | O | O |
|  | operationalState Operational state of the eventForwardingDiscriminator, telling whether EFD is able to operate. Can only take following value: DISABLED, ENABLED. | GET | M | M |
|  | schedulerName Specifies the name of the external scheduler MO that is related to the logs. | GET | O | O |
|  | startTime Defines date and time at which an unlocked and enabled log starts functioning. If not specified at creation time, its value defaults to the time of creation, thus causing the log to function immediately. | GET SET | O | O |
|  | stopTime Defines the date and time at which the log stops functioning. If not specified at creation time, its value defaults to ‘continuous operation’, which is represented by a null value. | GET SET REPDEF | O | O |
|  | weekMask This structured attribute defines a set of mask components, each specifying a 24-hour time-of-day clock, pertaining to the selected days of the week. Defaults to a set of scheduling criteria of ‘ALWAYS ON’ at creation time. | GET SET REPDEF ADD REMOVE | O | O |

#### Actions

None.

#### Notifications

| Index | Notification Name (Description) Syntax | ISO Status | ATN Status |
| --- | --- | --- | --- |
|  | attributeValueChange Used to report change of value of one of the EFD attributes (except state attributes). | M | M |
|  | objectCreation Used to report the EFD instance creation. Contains initial attribute values. | M | M |
|  | objectDeletion Used to report the EFD instance deletion. Contains last known attribute values. | M | M |
|  | stateChange Used to report the change in the value of one or more of the state attributes of the MO. | M | M |

### Log MO

Note.— This MO is exactly as defined in the ISO standards

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | log This object class allows to filter events and to store events as log records. There are different instances for this MO. [CREATE/DELETE] operations supported on the MO. | O | M |
|  | Naming attribute | logId |  |  |
|  | Superior in Naming Tree | system |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | administrativeState Indicates the permission to use the log, imposed through the management services. Can take following values: LOCKED, UNLOCKED. | GET SET | M | M |
|  | availabilityStatus Can take the zero value or any of following values: IN TEST, FAILED, POWER OFF, OFF LINE, OFF DUTY, DEPENDENCY, DEGRADED, NOT INSTALLED, LOG FULL. | GET | M | M |
|  | capacityAlarmThreshold Specifies, as a percentage of maxLogSize, the point at which an event will be generated to indicate that a log full or log wrap condition is approaching. Support is mandatory for the halt behaviour is specified by the logFullAction attribute. | GET SET | O | O |
|  | currentLogSize Current size of the log measured in octets. | GET | O | O |
|  | discriminatorConstruct Specifies tests on the information that is to be logged, and thus decides whether events are suitable or not for storage. The discriminator construct may operate on any of the parameters of the information to be logged. A limited number of log profiles should be defined. | GET SET | M | M |
|  | intervalsOfDay Defines the list of time intervals for which the log will exhibit the logging-on condition. During excluded intervals, the log exhibits the logging-off condition. If not specified at creation time, its value defaults to a single interval encompassing the entire 24 h period of a day. | GET SET REPDEF ADD REMOVE | O | O |
|  | logFullAction Specifies the action to be taken when the maximum size of the log has been reached. Options are: wrap and halt. | GET SET | M | M |
|  | logId Naming attribute. | GET | M | M |
|  | maxLogSize Specifies the maximum size of the log measured in octets. A log may have an indetermined size. A maxLogSize of zero shall be used to specify that the log size has no predefined limit. | GET SET | O | O |
|  | numberOfRecords Current number of records contained in the log. | GET | O | O |
|  | operationalState Indicates whether the system is physically installed and working. Can take following values: DISABLED, ENABLED. | GET | M | M |
|  | schedulerName Specifies the name of the external scheduler MO that is related to the logs. | GET | O | O |
|  | startTime Defines date and time at which an unlocked and enabled log starts functioning. If not specified at creation time, its value defaults to the time of creation, thus causing the log to function immediately. | GET SET | O | O |
|  | stopTime Defines the date and time at which the log stops functioning. If not specified at creation time, its value defaults to ‘continuous operation’, which is represented by a null value. | GET SETd | O | O |
|  | weekMask This structured attribute defines a set of mask components, each specifying a 24-hour time-of-day clock, pertaining to the selected days of the week. Defaults to a set of scheduling criteria of ‘ALWAYS ON’ at creation time. | GET SET REPDEF ADD REMOVE | O | O |

#### Actions

None.

#### Notifications

| Index | Notification Name (Description) Syntax | ISO Status | ATN Status |
| --- | --- | --- | --- |
|  | attributeValueChange Used to report change of value of non-state attributes. | M | M |
|  | objectCreation Used to report the log instance creation. Contains initial attribute values. | M | M |
|  | objectDeletion Used to report the log instance deletion. Contains last known attribute values. | M | M |
|  | processingErrorAlarm Used to report a software or processing fault. Here it is used to report that the capacity alarm threshold has been reached or exceeded (see ISO 10164-6). | M | M |
|  | stateChange Used to report the change in the value of one or more of the state attributes of the MO. | M | M |

### LogRecord MO

Note.— This MO is exactly as defined in the ISO standards

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | logRecord This MO is a generic type for the logging of any type of event or alarm record; thus there are different instances for this MO. | O | M |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

### AlarmRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | alarmRecord This object class allows to log alarm notifications. [DELETE] supported on the MO. There are diferent instances for this MO (in accordance with 10164-4), such as: communicationsAlarm (many Mos are concerned), processingErrorAlarm (log is concerned), qualityofServiceAlarm (Operating System is concerned), stateChangeAlarm (System is concerned). There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | backedUpStatus When present, specifies whether the MO emitting the alarm has been backed-up, and services provided to the user have, therefore, not been disrupted. The use of this field in conjunction of the severity field forms an independent form to qualify the seriousness of the alarm and the ability of the system as a whole to continue to provide services. If TRUE, the MO emitting the alarm has been backed-up; if FALSE, it has not, which shall be a usual case. | GET | O | O |
|  | backUpObject Present when the backedUpStatus attribute is present and has the value TRUE. Specifies the MO instance that is providing back-up services for the MO about which the notification pertains. | GET | O | O |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | monitoredAttributes When present, defines one or more attributes of the MO and their corresponding values at the time of the alarm. | GET | O | O |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the correlatedNotifications attribute of other notifications. | GET | O | O |
|  | perceivedSeverity Defines six severity levels indicating how it is perceived that the capability of the MO has been affected: CLEARED, INDETERMINATE, CRITICAL, MAJOR, MINOR, WARNING . | GET | O | O |
|  | probableCause Defines further qualification as to the probable cause of the alarm. | GET | O | O |
|  | proposedRepairActions When present, used if the cause is known an the system being managed can suggest one or more solutions. | GET | O | O |
|  | specificProblems When present, identifies further refinements to the probable cause of the alarm. | GET | O | O |
|  | stateChangeDefinition When present, used to indicate a state transition associated with the alarm | GET | O | O |
|  | thresholdInfo Shall be present when the alarm is a result of crossing a threshold. It includes the identifier of the triggered threshold, the threshold level, the observed value and the alarm time. | GET | O | O |
|  | trendIndication When present, specifies the current severity trend of the MO. If present, it indicates that there is one or more outstanding alarms which have not been cleared, and belong to the same MO as the current alarm. Can take one of the following values: MORE SEVERE, NO CHANGE, LESS SEVERE. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### AttributeValueChangeRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | AttributeValueChangeRecord This managed object class allows to log attributeValueChange notifications. [DELETE] operation supported on the MO. There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | attributeIdentifierList Identifies the set of attributes whose value change is being reported | GET | O | O |
|  | attributeValueChange Definition Consists of a set of sequences of the three parameters: attribute identifier, old attribute value, new attribute value. | GET | M | M |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the correlatedNotifications attribute of other notifications. | GET | O | O |
|  | sourceIndicator When present, indicates the source of operation that led to the generation of this notification. Can take one of the following values: RESOURCE OPERATION, MANAGEMENT OPERATION, UNKNOWN. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### CommunicationsInformationRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | CommunicationsInformationRecord This managed object class allows to log communicationsInformation notifications. [DELETE] operation supported on the MO. There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | informationData Generic structure of information carried by the communicationsInformation notification. | GET | O | O |
|  | informationType The type of information held in the informationData attribute | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the correlatedNotifications attribute of other notifications. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### ObjectCreationRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | ObjectCreationRecord This managed object class allows to log objectCreation notifications. [DELETE] operation supported on the MO. There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | attributeList When present, contains a list of attributes and their values at the time the MO was created. | GET | O | O |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the correlatedNotifications attribute of other notifications. | GET | O | O |
|  | sourceIndicator When present, indicates the source of the operation that led to the generation of this notification type. It can have one of the following values: RESOURCE OPERATION, MANAGEMENT OPERATION, UNKNOWN. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### ObjectDeletionRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | ##### ObjectDeletionRecord This managed object class allows to log objectDeletion notifications. [DELETE] operation supported on the MO. There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | attributeList When present, contains a list of attributes and their values just before the MO was deleted. | GET | O | O |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the «correlatedNotifications» attribute of other notifications. | GET | O | O |
|  | sourceIndicator When present, indicates the source of the operation that led to the generation of this notification type. It can have one of the following values: RESOURCE OPERATION, MANAGEMENT OPERATION, UNKNOWN. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### StateChangeRecord MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | ##### StateChangeRecord This managed object class allows to log stateChange notifications. [DELETE] operation supported on the MO. There are different instances for this MO. |  |  |
|  | Naming attribute | logRecordId |  |  |
|  | Superior in Naming Tree | log |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | additionalInformation When present, allows the inclusion of a set of additional information in the event report. It is a series of data structures, each of which contains three items: an identifier, a significance indicator, and the problem information. The information subparameter carries information about the event. This information can be parsed if the identifier is understood. | GET | O | O |
|  | additionalText When present, allows a free form text description to be reported. | GET | O | O |
|  | attributeIdentifierList Identifies the set of attributes whose value change is being reported. | GET | O | O |
|  | correlatedNotifications When present, contains the set of notification identifiers of all notifications to which this notification is considered to be correlated. | GET | O | O |
|  | eventTime Event generation time (i.e. present in the event if available). | GET | O | O |
|  | eventType Specifies the type of event being reported. | GET | M | M |
|  | loggingTime The time at which the record was entered into the log. | GET | M | M |
|  | logRecordId Naming attribute. | GET | M | M |
|  | managedObjectClass The class of the MO in which the event occurred. | GET | M | M |
|  | managedObjectInstance The instance (DN) of the MO in which the event occurred. | GET | M | M |
|  | notificationIdentifier Contains a unique identifier for the notification, which may be present in the correlatedNotifications attribute of other notifications. | GET | O | O |
|  | sourceIndicator When present, indicates the source of the operation that led to the generation of this notification type. It can have one of the following values: RESOURCE OPERATION, MANAGEMENT OPERATION, UNKNOWN. | GET | O | O |
|  | stateChangeDefinition Consists of a set of sequences of the three parameters : attribute identifier, old attribute value, new attribute value. | GET | O | O |

#### Actions

None.

#### Notifications

None.

### PhysicalResources MO

Editor’s note.— This MO has not yet been considered in the ongoing MIB convergence process, and so is likely to change.  Detailed technical discussions are needed on whether to include MOs such as this in the standardised ATN MIB.  The attributes and notifications are merely suggestions from one project - they are included here as place-holders indicating the kind of information that might be included.

#### MO Class Support

| Index | Property | Description | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | Managed Object Class | ### PhysicalResources This MO class is used to represent the resources of the Operating System and to identify the hardware (workstation or ATSU) that needs to be monitored for ATN operation purposes. There is only one instance per managed system, locally created. Neither CREATE nor DELETE operations can be requested from a manager. Threshold values associated with cpuLoad, virtualMemoryUsage, and fileSystemUsage attributes can be set upon a manager’s request. | A | M |
|  | Naming attribute | subsystemId |  |  |
|  | Superior in Naming Tree | system |  |  |

#### Attributes

| Index | Attribute Name (Description) Syntax | Operations | ISO Status | ATN Status |
| --- | --- | --- | --- | --- |
|  | cpuLoad CPU Load (in percent). | GET | A | O |
|  | cpuLoadAlarmThreshold Used to issue an alarm whenever threshold has been reached or exceeded. | GET SET | A | O |
|  | logSizeUsage Size of log expressed as a percentage of total space allocated to logs. | GET | A | O |
|  | logSizeThreshold Usued to issue an alarm as soon as threshold has been reached concerning allocated space to logs. | GET SET | A | O |
|  | machineType Identification of the hardware supporting the system. | GET | A | O |
|  | subsystemId Naming attribute. | GET | M | M |
|  | virtual MemoryUsage Percentage of virtual memory currently being used. | GET | A | O |
|  | virtualMemoryAlarm Threshold Used to issue an alarm whenever threshold has been reached. | GET SET | A | O |

#### Actions

None.

#### Notifications

| Index | Notification Name (Description) Syntax | ISO Status | ATN Status |
| --- | --- | --- | --- |
|  | qualityofServiceAlarm Used to report alarm conditions related to the quality of service of the MO. Here it is used to report the following event thresholdCrossed: Generated whenever one of the threshold values associated with supported attributes are reached or crossed. | O | O |