D-ATIS Range and Resolution

| VARIABLES | PARAMETERS | Unit | Range/Size | Resolution |
| --- | --- | --- | --- | --- |
| Aerodrome |  | IA5 String | 4 characters |  |
| Altimeter setting | Pressure (SI) | hPA | 500-1250 | 1 |
|  | Pressure (non-SI) | inches mercury | 22.00-32.00 | 0.01 |
| Approach area MET |  | IA5 string | 1-128 characters |  |
| Arrival and/or Departure indicator |  | Enumerated list |  |  |
| ATIS designator |  | IA5 String | 1 character |  |
| Braking action | Quality | Enumerated List |  |  |
|  |  | IA5 string | 1-25 characters |  |
| Climb out area MET |  | IA5 string | 1-128 characters |  |
| Cloud amount |  | Enumerated List |  |  |
| Cloud height | Low Cloud height (SI) | Meters | 0-3000 | 30 |
|  | Low Cloud height (non-SI) | Feet | 0-10000 | 100 |
|  | High Cloud height (SI) | Meters | 3000-20000 | 300 |
|  | High Cloud height (non-SI) | Feet | 10000-60000 | 1000 |
| Cloud type |  | Enumerated List |  |  |
| Contract number |  | Integer | 1-256 | 1 |
| Contract type |  | Enumerated List |  |  |
| Day |  | Day of month | 1-31 | 1 |
| Descriptor qualifier |  | Enumerated List |  |  |
| Holding delay |  | IA5 String | 1-200 characters |  |
| Intensity qualifier |  | Enumerated List |  |  |
| Month |  | Month of Year | 1-12 | 1 |
| Other essential operational information |  | IA5 String | 1-250 characters |  |
| Present weather type |  | Enumerated List |  |  |
| Runway arresting system condition |  | Enumerated List |  |  |
| Runway arresting system location |  | Enumerated List |  |  |
| Runway designator |  | Integer | 1-36 | 1 |
| Runway letter |  | Enumerated List |  |  |
| RVR |  | Enumerated List |  |  |
|  | RVR (SI) | Meters | 0-800 | 25 |
|  |  | Meters | 800-1500 | 100 |
|  | RVR (non-SI) | Feet | 0-1000 | 100 |
|  |  | Feet | 1000-3000 | 200 |
|  |  | Feet | 3000-6000 | 500 |
| Significant runway surface conditions |  | Enumerated List |  |  |
|  |  | IA5 string | 1-256 characters |  |
| Specific ATIS instructions |  | IA5 string | 1-64 characters |  |
| Surface wind direction |  | Degrees | 10-360 | 10 |
| Surface wind speed | Wind Speed Meters (SI) | KM/Hour | 0-500 | 1 |
|  | Wind Speed Knots (non-SI) | Knots | 0-200 | 1 |
| Take-off area MET |  | IA5 string | 1-128 characters |  |
| Temperature | Temperature C (SI) | Degrees Celsius | -80 - +60 | 1 |
| Time of observation | Time hours | Hours of day | 0-23 | 1 |
|  | Time minutes | Minutes of hour | 0-59 | 1 |
| Transition Level | Flight level (SI) | Meters | 1000-20000 | 10 |
|  | Flight level (non-SI) | Feet | 3000-60000 | 100 |
| Trend type landing forecast |  | IA5 string | 1-256 characters |  |
| Type of Approach |  | Enumerated list |  |  |
|  |  | IA5 string | 1-30 |  |
| Vertical visibility | Visibility (SI) | Meters | 0-3000 | 30 |
|  | Visibility (non-SI) | Feet | 0-10000 | 100 |
|  | Not Available |  | NULL |  |
| Visibility | Visibility (SI) | Meters | 0-500 | 50 |
|  | Visibility (SI) | Meters | 500-4900 | 100 |
|  | Visibility (SI) | Kms | 5-10 | 1 |
|  | Sixteenths (non-SI) | Statute Mile | 0-3/8 | 1/16 th |
|  | Eighths (non-SI) | Statute Mile | 3/8-2 | 1/8 th |
|  | Fourths (non-SI) | Statute Mile | 2-3 | ¼ ths |
|  | Miles (non-SI) | Statute Miles | 3-15 | 1 |
|  | Fives (non-SI) | Statute Miles | 15-50 | 5 |
|  | More than 50 Statute Miles |  | NULL |  |
| Year |  | Integer | 00-99 | 1 |

Note:  Non-SI units of measurement not specified in Annex 5.

Table 6B-1:  D-ATIS Variables Range and Resolution