WP WG3/7-7

Aeronautical Telecommunication Network Panel

Working Group  3

Munich, 24-29 June 1996

Proposed ATNP/2 Working Paper

Overview of ULA SARPs and Guidance Material

(Presented by Steve Van Trees)

Abstract

WG3/SG3 describes its work as embodied in its SARPs and Guidance.

1.  Title of the SARPs produced

ATNP/WG3/SG3 has produced the Upper Layer Communication Service SARPs.  These are section 4 of the Annex 10 Appendix material.

2.  Sub-Group Chair, Editor, Contributors to the SARPs

Steve Van Trees (USA) and Bo Overgaauw (France) chaired the group between ATNP/1 and ATNP/2.  Tony Kerr (Eurocontrol) served as document editor.

Many experts contributed to the work, among them are Stephen Pearce (Australia), Frederic Picard (France), Fif Edem (SITA), and John Day, Jim Moulton, and Jim Simpkins (USA).

3.  Functional Description of the SARPs

The group produced  Standards and Recommended Practices (SARPs) for the initial version of the ATN Upper Layers (UL).  The UL specification supports all current ATN applications except the ATS Message Application defined in Section 3.  This specification is designed to optimise the use of communications bandwidth, and consequently restricts the functionality available from the OSI Session and Presentation layers.

These SARPs address the ATN requirements for Session Layer (Layer 5), Presentation Layer (Layer 6), and a part of the Application Layer (Layer 7) of the OSI reference model.  Figure 1 shows a conceptual view of the scope of these SARPs.  The remaining part of the Application Layer is the province of the individual ATN application SARPs (i.e.  the ADS, CM, CPDLC and FIS(ATIS) SARPs for air-ground applications, and the ICC (AIDC) SARPs for ground-ground applications).

Figure 1.  Conceptual view of the scope of the UL SARPs

Background

The communication aspects of the ATN applications are modelled as Application Entities (AEs).  Figure 2 illustrates an example of the application layer structure for the Air/Ground applications.

Figure 2.  Conceptual view of Application Layer

These SARPs define a profile for the protocols in the upper layers, an AE structure and a number of common application services.

Upper Layer Profile Overview

These SARPs specify a profile for the connection oriented protocols of Session layer, Presentation layer and the Association Control Service Element (ACSE).

The Session portion of the profile specified in these SARPs is based on the efficiency enhancements to the Session protocol which are standardised in ISO/IEC 8327-1 Amendment 2.

The Presentation portion of the profile specified in these SARPs is based on the efficiency enhancements to the Presentation protocol which are standardised in ISO/IEC 8823-1 Amendment 2.

As a consequence of using the Session and Presentation protocol efficiency enhancements, the protocol control information transferred by these protocols amounts to two octets in each direction during the connection establishment phase, and zero octets at all other times.

The ACSE portion of the profile specified in these SARPs is based on ISO/IEC 8650-1, including the extensibility notation as specified as Amendment 2 to that standard.

Note. — The ISO definition of the PER-visible extensibility markers is the subject of a Draft Amendment (DAM).

Application Entity (AE) Structure

The AE structure specified within these SARPs is based on the application layer structure defined in ISO/IEC 9545, where the concepts of Application Service Element (ASE), Application Service Object (ASO) and Control Function (CF) are defined.

Figure 3 shows the generic structure of an AE with arrows representing the abstract service boundaries of the various elements.  The “upper” service boundary is the abstract service provided by an ASE to its user(s).  The “lower” service boundary is the abstract service which is provided to the ASE by the CF.

The ASE is an element engineered to perform a required task.  ISO/IEC 9545 describes how two or more ASEs may be combined, together with a CF to co-ordinate their operation to form an ASO.  In turn, an ASO may be combined with other ASOs or ASEs with a CF to form larger ASOs.  The AE is the outermost ASO.

Figure 4.1-3.  Generic Application Entity structure

Application Services

These SARPs specify various abstract services.  The services are provided at abstract service boundaries.

For each of the current ATN applications a specific ASE exists, and is specified in the relevant ATN Application SARPs.  The generic name “ATN-App ASE” is used for these specific ASEs.

Figure 4 shows the AE structure which is used to model the ATN applications

Figure 4.  ATN-specific AE structure

The Dialogue Service (DS) as defined in  these SARPs is the abstract service which the ATN-App ASEs use to interact with the communication facilities defined in these SARPs.  That is, the DS is the combination of specific internal primitives made available by the CF at the lower boundary of the ATN ASE/ASO - it is the application’s “world view”.  In order to provide this service, the CF uses the services of ACSE.

4.  System Requirements (Sub-Volume 1) Fulfillment by the SARPs

The SARPs contribute to the fulfillment of the following  System Requirements:

9.  Peer Information exchange

11.  Notification of Lack of Path Availability

12.  Unambiguous Addressing

13.  Originator Identification

19.  Exchange of Address Information

20.  CM Association

21.  CM Primacy

22.  ADS Association

23.  CPDLC Association

24.  FIS (ATIS) Association

26.  ICC (AIDC)

5.  Interrelationship of the SARPs with other SARPs

The Upper Layer SARPs supports the air-ground application SARPs (context management, automatic dependent surveillance, controller-pilot datalink communication, and flight information services (auomatic terminal information service).

The Upper Layer SARPs utilizes the services of the Internet SARPs.

6.ISO Standards

The ATN is an Open Systems Interconnection (OSI) network It should be noted that WG3/SG3 invested a great deal of effort in the adaptation of OSI standards to ATN requirements.

.  ATN personnel have been involved in the ISO standardization process.

ACSE Service, 2nd Edition

ACSE Protocol, 2nd Edition

ACSE Connectionless Protocol, 2nd Edition

ISO 8649/DAM1  ACSE Service (Fast Associate)

ISO 8649/DAM2  ACSE Service (Connectionless Authentication)

ISO 8650-1/DAM1  ACSE Protocol (Fast Associate)

ISO 8650-1/DAM2  ACSE Protocol (Extensibility)

ISO 10035-1/DAM1  ACSE Connectionless Protocol (Authentication and Extensibility)

ISO 8822/DAM2  Presentation Service (Efficiency)

1SO 8823-1/DAM2  Presentation Protocol (Efficiency)

ISO 8326/DAM2  Session Service (Efficiency)

ISO 8327-1/DAM2  Session Protocol (Efficiency)

ISO PDTR 14700, OSI Efficiency

ISO DIS 8649  ACSE Service, 3rd Edition

ISO DIS 8650-1 ACSE Protocol, 3rd Edition

ISO DIS 10035-1 ACSE Connectionless Protocol, 3rd Edition