| RTCA SC-189/EUROCAE WG-53 Position Paper Air Traffic Services Safety and Interoperability Requirements |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Information |  |  |  |  |  |  |  |  |  |  |
| Position Paper Information (To be completed by Position Paper Author and attached to position paper) |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # P-PUB-04 |  |  |  |  | Revision: e.g. A A | Date: e.g. 11 Nov 94 6-Jun-97 |  |  | Comments Due: 1-Aug-97 |  |
| Author/Address/Fax/Voice/Internet: Tom Kraft, ANM-114N 1601 Lind Avenue SW Renton, WA 98055-4056 Voice: 206-227-2129 FAX: 206-227-1181 Internet: Tom.Kraft@FAA.DOT.Gov |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Author |  |  |  |  |  |
| Position Paper Title: Outline for RTCA/EUROCAE SC-189/EUROCAE WG-53 Documents |  |  |  |  |  |  |  |  |  |  |
| Abstract: This position paper provides the rationale and an overview of documents that RTCA SC-189/ EUROCAE WG-53 will produce. It includes a list of the documents together with their outlines, and the relationship of the documents to the certification of aircraft, the commissioning of air traffic services, and the operational authorizations. This position paper will be used to coordinate integration of guidance material developed by the subgroups of the RTCA SC-189/EUROCAE WG-53 for publication. |  |  |  |  |  |  |  |  |  |  |
| Action taken: |  |  |  |  |  |  |  |  |  | Resolution Required By: |
| Key words (Optional): Document Outline, RTCA SC-189/EUROCAE WG-53, Administrative |  |  |  |  |  |  |  |  |  |  |
| Status | Individual Proposal |  |  | Sub-Group Agreement |  |  | SC-189/WG-53 Agree. |  |  |  |
|  | Internal Use Only |  |  | Closed |  |  |  |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  |  |  | Fax |  |  |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  |  |  | +01-425-294-2007 |  |  |
| Martine Blaize |  | DGAC | BLAIZE_Martine@STNA.DGAC.Fr |  |  |  |  | +33-5-62-14-58-53 |  |  |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@Avions.Aerospatiale.Fr |  |  |  |  | +33-5-61-93-80-90 |  |  |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  |  |  | +01-425-227-1181 |  |  |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  |  |  | +01-425-227-1181 |  |  |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  |  |  | +33-1-40-84-1859 |  |  |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  |  |  | +01- 425-868-7297 |  |  |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  |  |  | +33-62-14-53-53 |  |  |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  |  |  | +01-703-883-1330 |  |  |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.Com |  |  |  |  | +01-425-342-6078 |  |  |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  |  |  | +33-5-61-93-07-29 |  |  |

| RTCA SC-189/EUROCAE WG-53 Comment |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Position Paper Information (To be completed by Position Paper Author and attached to position paper) |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # P-PUB-04 |  |  |  |  | Revision: e.g. A A |  | Date: e.g. 11 Nov 94 6-Jun-97 |  |  | Comments Due: 1-Aug-97 |
| Author/Address/Fax/Voice/Internet: Tom Kraft, ANM-114N 1601 Lind Avenue SW Renton, WA 98055-4056 Voice: 206-227-2129 FAX: 206-227-1181 Internet: Tom.Kraft@FAA.DOT.Gov |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Author |  |  |  |  |  |
| Position Paper Title: Outline for the RTCA/EUROCAE Documents |  |  |  |  |  |  |  |  |  |  |
| Comment Information (To be completed by Comment Author) |  |  |  |  |  |  |  |  |  |  |
| Comment Author (Include Name/Address/Voice/Fax/Internet): |  |  |  |  | Comment Date: e.g., 3 Apr 96 |  | Type of Comment (Check all that apply): Response Requested Disagreement Clarification Additional Material |  |  |  |
| Comment (Attach pages as necessary): |  |  |  |  |  |  |  |  |  |  |
| Proposed Solution (Attach pages as necessary) |  |  |  |  |  |  |  |  |  |  |
| Status of Comment/Action Taken (To be completed by position paper author or delegate) |  |  |  |  |  |  |  |  |  |  |
| Assigned to: | SG-1 | SG-2 | SG-3 | Pub |  | CAG |  | Other | Comment Number: C- |  |
| Status/Action Taken: |  |  |  |  |  |  |  |  |  |  |

3-Jun-97

Subject: Outline for RTCA/EUROCAE SC-189/EUROCAE WG-53 Documents

Author:  Tom Kraft

# Introduction.

This position paper will provide the vehicle for coordinating various RTCA/EUROCAE products for publication among the subgroups within SC-189/WG-53.  It provides rationale for the document structure by considering the use of the documents during system development, operations, and related qualification activities for implementing air traffic services supported by data communications, such as:

- aircraft certification

- ATS and communication service provider system commissioning

- operational approvals to use CNS/ATM capabilities that support new air traffic services.

The documents and their structure are oriented to facilitate these activities and provide objective guidelines that are easy to show compliance, yet allow for flexibility in the design and development of specific implementations.

# Use of the RTCA SC-189/EUROCAE WG-53 documents

These documents will ultimately be used by the aviation community to qualify CNS/ATM operational capability for providing air traffic services.  The purpose of these documents is:

- for airspace planners to define the operational environment and to ensure continued safety, performance, and interoperability.

- to identify the safety, performance, and interoperability requirements for the systems and functions that support  Air Traffic Services (ATS).

- to allocate the safety, performance, and interoperability requirements to various parts of the system or procedures that are under the jurisdiction of different organizations.

- for aviation industry to develop CNS/ATM capabilities.

- for civil aviation authorities/industry to qualify aircraft and ground data communication systems, the applications, and operations to the safety, performance, and interoperability requirements for airworthiness approval, commissioning of ground systems, and  operational approvals for airspace users, ATS providers, and communications service providers.

- for the aviation community to evaluate the modifications after initial approval to any part of the system or procedures.

# Relationship of RTCA SC-189/EUROCAE WG-53 documents to qualification activities.

The qualification activities are largely dependent on the definitions of the operational environments developed by airspace planners, regional planning groups for international airspace and States for local airspace.  Figure 1 provides an overview of the relationship of the RTCA/EUROCAE guidance documents to qualification activities.

Figure 1:  Relationship of RTCA/EUROCAE documents to qualification activities.

# Objectives for documents and their structure.

The following objectives are provided as a basis for the documents and their structure and to guide the development of the RTCA/EUROCAE publications.

- Guidelines should use terminology consistently and provide a framework that is well defined and understood.  
Rationale:  Applications of CNS/ATM technologies requires a diverse range of disciplines and specialty areas with different backgrounds.

- Guidelines should be delineated for the States and organizations developing the operational systems and procedures and provide means to ensure validity of assumptions across organizational boundaries.  
Rationale:  These documents may be invoked by various CAAs to recognize these documents as acceptable means of compliance to regulations.

- Guidelines for safety and performance should be distinguishable from guidelines for interoperability.  
Rationale:  The interoperability requirements are dependent on protocols that are used and specific applications of communication technologies.  The safety and performance requirements are derived based on characteristics of the CNS/ATM operational environment and can be specified independently of the protocols and specific applications of communication technologies.

- Requirements that are necessary to maintain an acceptable level of performance of the air traffic service should be distinguishable from performance requirements that are necessary for safety.
Rationale: Guidelines should specify minimum requirementsso that airspace planners can work with airspace users to understand the basis for performance requirements.

- Guidelines for safety and performance should be distinguishable for each of the operational objectives.  
Rationale.  SC-189/WG-53 will identify operational factors such as Dynamic Aircraft Route Planning System (DARPS), both ATS-defined and Aeronautical Operational Control (AOC)-defined, single and multiple route planning to different airspace users and, in general, reduced separation minima, and specifically apply to 50 nm and 30 nm, laterally, and 50 nm and 30 nm, longitudinally.  Guidelines should specify minimum requirements so that airspace planners can work with airspace users to determine which operational capabilities are best suited for a particular airspace and use only applicable guidelines.

- Guidelines for interoperability should be distinguishable by communication protocols and specific applications of communication technologies.
Rationale.  Airspace planners may wish to only apply certain protocols and communication technologies within their airspace.

- Guidelines for methodology or process should be distinguishable from guidelines for implementing specific operational objectives or technologies.  The methodology may include failure conditions which are specific for a given function.
Rationale.  This will facilitate the evolutionary development of the guidelines as new operational objectives are sought or new communication protocols and technologies are applied.  It will also ensure an acceptable level of consistency for establishing safety levels, yet allow for flexibility across regional planning groups where operational objectives may differ.

- Guidelines should be distinguishable from narrative and be identifiable for traceability and to facilitate compliance.  
Rationale.  This will facilitate compliance to specific guidelines that must be met when the document is used.

# List of Documents and first level outline.

The table that follows identifies the RTCA/EUROCAE publications to be developed by SC-189/WG-53.  Outlines for each of the documents are also provided.  The following points are noted:

- The main document DO-XX1/ED-X1 provides the overall concept model, definition of terms, and list of references.  This approach was used because all users of the other documents will need the main document and will avoid duplication of material because of the common use of terms and references throughout the other documents.

- All documents have a common introduction section that provides purpose, scope, relationship to other documents, and how to use this document sections.

| Doc Ref | Doc Title | Description of contents |
| --- | --- | --- |
| DO-XX1 ED-X1 | Guidelines for qualification, approval, and operation of air traffic services supported by data communications. Owner: PUB | This document refers to all the other documents and provides an overview of the processes, concepts for qualification, approval, and operation of air traffic services supported by data communications, and definition of terms and references. It provides guidelines to plan the implementation of air traffic services supported by data communications. It provides guidelines to qualify implementations to the safety and performance requirements for specific operational objectives and the interoperability requirements for specific communication protocols and technologies. It provides guidelines to facilitate continued operational safety and interoperability. The aviation community, including airspace planners, system developers, operations developers, civil aviation authorities, air traffic service providers, and communication service providers are the primary users of this document. |
| DO-XX2 ED-X2 | Operational environment template Owner: SG2. | This document provides the criteria for defining the interface between airspace planners and system developers/qualifiers. The operational environment template provides a consistent approach to defining the airspace characteristics, air traffic services, and associated functions and related performance criteria. Airspace planners and airspace users are the primary users of the template to capture characteristics of the operational environment needed by system developers/qualifiers to assess the safety, interoperability, and performance aspects of air traffic services supported by data communications. |
| DO-XX3 ED-X3 | Methods for evaluating operating concepts . Owner: SG2 and SG3 | This document provides a methodology for assessing the safety aspects of applications of air traffic services supported by data communications. The methodology provides a consistent approach to identifying and classifying hazards and evaluating failures, failure conditions, and effects on air traffic services to develop hazard mitigation strategies based on the characteristics of the operational environment. The methodology recognizes that different groups use different techniques, but provides a consistent total systems approach to establish common safety objectives that can be attained consistently without dictating a particular technique. The document may provide a generic list of failure conditions for a specific application or function. This docment will provide a methodology for allocating the end-to-end performance requirements to different parts of the system and operations. System developers, operations developers, and civil aviation authorities are the primary users of this document. |
| DO-XX4 ED-X4 | Safety and performance requirements for operational objectives supported by data communications. Owner: SG2 and SG3 | This document provides safety and performance requirements for specific operational objectives and allocates those requirements to specific parts of the total system, procedures, and airspace characteristics. The safety and performance requirements are derived from specific mitigation strategies that transcend any one part of the system, procedures, or airspace characteristics and have been agreed upon by the industry. Mitigation strategies that are within a single organizatioin are the responsibility of the organization to continue the safety assessment process to ensure adequate protection against failures in their part in accordance with established safety objectives. This document will also provide qualification means that are specific to the safety and performance requirements Operational objectives include dynamic aircraft route planning system (DARPS), both air traffic services-defined and aeronautical operational control-defined, single and multiple route planning to different airspace users and, in general, reduced separation minima, and specifically apply to 50 nm and 30 nm, laterally, and 50 nm and 30 nm, longitudinally. System developers, procedures developers, and civil aviation authorities are the primary users of this document. |
| DO-XX5 ED-X5 | Interoperability requirements for ARINC 623 ATS applications using ACARS or ARINC 622 data communications. Owner: SG1 | This document provides the interoperability requirements for ARINC 623 ATS applications using ACARS or ARINC 622 data communications. This document will also provide qualification means that are specific to the interoperability requirements for this protocol. System developers, procedures developers, and civil aviation authorities are the primary users of this document. |
| DO-XX6 ED-X6 | Interoperability requirements for non-ARINC 623 character ATS applications using ACARS or ARINC 622 data communications Owner: SG1 | This document provides the Interoperability requirements for non-ARINC 623 character ATS applications using ACARS or ARINC 622 data communications. This document will also provide qualification means that are specific to the interoperability requirements for this protocol. System developers, procedures developers, and civil aviation authorities are the primary users of this document. |
| DO-XX7 ED-X6 | Interoperability requirements for binary ATS applications using ARINC 622 data communications Owner: SG1 | This document provides the Interoperability requirements for binary ATS applications using ARINC 622 data communications. This document will also provide qualification means that are specific to the interoperability requirements for this protocol. System developers, procedures developers, and civil aviation authorities are the primary users of this document. |
| DO-XX8 ED-X8 | Interoperability requirements for ATS applications using ICAO ATN data communications Owner: SG1 | This document provides the Interoperability requirements for ATS applications using ICAO ATN data communications. This document will also provide qualification means that are specific to the interoperability requirements for this protocol. System developers, procedures developers, and civil aviation authorities are the primary users of this document. |

Outline

Guidelines for qualification, approval, and operation of air traffic services supported by data communications.

- Introduction

  - Purpose

  - Scope

  - Relationship to other documents

  - How to use this document

- A conceptual framework and terminology for qualification, approval, and operation of air traffic services supported by data communications

- Planning considerations

  - Coordination

- Definition of operational environment

  - Airspace characteristics

  - Operations description

  - Air traffic services

  - Required functional capabilities

  - Functional performance

  - Technology

- Requirements determination and substantiation

  - Safety and Performance considerations

  - Interoperability considerations

- Qualification

  - Development assurance

  - Validation

  - Verification

- Continued operational safety

  - Track the evolution of the operational systems that provide air traffic services supported by data communications

  - Report and resolve in-service problems among the participating States and organizations

- Data Items

  - Qualification/approval data

  - Continued operational safety and interoperability data

A1	Definition of terms

A2	List of references

Outline

Operational environment template

Derived from P/SG2/2, Rev 5, Characteristics of the CNS/ATM Operational Environment for Air Traffic Services (ATS) that Use Data Communications.

- Introduction

  - Purpose

  - Scope

  - Relationship to other documents

  - How to use this document

- Definition of operational environment

  - Airspace characteristics

  - Operations description

  - Air traffic services

  - Required functional capabilities

  - Functional performance (Added)

  - System Technical Characteristics

A1	Example of definition of opreational environment

Outline

Methods for evaluating operating concepts for air traffic services supported by data communications.

Derived from P/SG2/7, Rev 1, Guidelines for coordinated aircraft - ATC & telecommunication safety assessments of ATS supported by data communications, in defined CNS/ATM environments.

- Introduction

  - Purpose

  - Scope

  - Relationship to other documents

  - How to use this document

- CNS/ATM safety and performance

  - Impact of CNS/ATM on the separation between aircraft and changes of flight crew and ATC procedures

  - Impact of the ATC environment on the safety process

  - Organizational aspects of the overall safety assessment and performance allocation

- Planning the safety assessment and performance allocation coordination

  - Designers, Manufacturers, Operators, users involved in ATS systems

  - Competent authorities and agreed institutions in each domain

  - Applicable material

- Top-level safety assessment

  - Top-level hazard assessment

  - Top-level preliminary safety assessment

    - Top-level common cause analysis

  - Piece part system safety assessment

    - Functional hazard assessment

    - Preliminary system safety assessment

    - System safety assessment

- Performance allocation methodology

A1	Example of top-level safety assessment

Outline

Safety and performance requirements for operational objectives  supported by data communications.

- Introduction

  - Purpose

  - Scope

  - Relationship to other documents

  - How to use this document

- Separation minima

- Dynamic aircraft route planning system (DARPS)

  - ATS-defined DARPS

    - Single route planning

    - Multiple route planning

  - AOC-defined DARPS

    - Single route planning

    - Multiple route planning

- Primary ATS communication

- New ATS infrastructure or replacement

A1	50 nm lateral separation

A2	50 nm longitudinal separatioin

A3	30 nm lateral separation

A4	30 nm longitudinal separation

Generic Outline for interoperability requirements documents

Interoperability requirements for xxx

The following was taken from P-SG1-1, Document Outline for “Interoperability Requirements for ATS Applications Using ARINC 622 Data Communication.”

- Introduction

  - Purpose

  - Scope

  - Relationship to other documents

  - How to use this document

- Applications Description

  - CPDLC

    - Recommendations (Errors/Changes to Standards)

    - Additional Interoperability Requirements

    - Guidance Materials (Limitations, Unique Implementations)

  - ADS

    - Recommendations (Errors/Changes to Standards)

    - Additional Interoperability Requirements

    - Guidance Materials (Limitations, Unique Implementations)

  - DLIC

    - Recommendations (Errors/Changes to Standards)

    - Additional Interoperability Requirements

    - Guidance Materials (Limitations, Unique Implementations)

- Sequences of Operations

  - Connection Establishment

  - Connection Transfer

  - Connection Termination

  - CPDLC Message Exchange

  - ADS Periodic Contract

  - ADS Event Contract

  - ADS Reports

- Data Link Domain

  - Data Link Systems:  ACARS, VHF, SATCOM

  - Service Provider Networks and Internetworking

  - CRC, ACF Sections

- Validation (Propose to be a candidate for DO-XX1/ED-X1)

  - Data Link Domain, End System Communication with Service Provider, End-to-End Applications

    - Test Process, Compliance Methods, Test Facilities

    - Set of Test Cases

    - Test Traceability

A1	Cross Reference of Ground System Requirements

A2	Cross Reference of Aircraft System Requirements