| RTCA SC-189/EUROCAE WG-53 Position Paper Safety and Interoperability for Air Traffic Services Communication |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Information |  |  |  |  |  |  |  |  |  |  |
| Position Paper Number: Call 206 227 2763 to get #: Leave fax # PSG1202A |  |  |  |  | Revision: e.g. 3 A | Date: e.g. 11 Nov 94 11 Apr 97 |  |  | Comments Due: |  |
| Author/Address/Fax/Voice/Internet: Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be |  |  |  |  | Send Comments to (Include Name/Address/Fax/Voice/Internet): Rob Mead Eurocontrol, PETAL-II Rue de la Fusee 96 B-1130 Brussels Belgium Voice: +32 2 729-3082 FAX: +32 2 729-9087 Internet: mead.rob@eurocontrol.be |  |  |  |  |  |
| Position Paper Title: Common CPDLC Message Elements During Transfer of CPDLC Authority |  |  |  |  |  |  |  |  |  |  |
| Abstract: CNS/ATM-1 & FANS-A/1 Compatability Issue: FANS-1/A requires use of the End Service CPDLC element (uplink element 161) to transfer CPDLC authority (Current Data Authority) from one ATS Unit to another. This element is contained in the ICAO Manual and ATN SARPS for, with an operational intent equating to the above function. To remove a potential interoperability problem, and to ensure that the defined operational element is used rather than technical messages with no defined intent (e.g. CPDLC_end_request), CNS/ATM-1 ground systems should use element 161 during the transfer of communications process. It is therefore recommended that all CPDLC ground systems, regardless of the technical infrastructure involved comply with the following: Uplink element 161 shall be used by a CDA, either concatenated to or in place of, element 117 to end CPDLC services between the CDA and an aircraft. |  |  |  |  |  |  |  |  |  |  |
| Action taken: This paper was agreed by SG-1 at its March 1997, for consideration for FANS-1/A, Version 3.0 |  |  |  |  |  |  |  |  |  | Resolution Required By: |
| Key words (Optional): |  |  |  |  |  |  |  |  |  |  |
| Status | Individual Proposal |  |  | Sub-Group Agreement |  |  | Plenary Agreement |  |  |  |
|  | Internal Use Only |  |  | Closed |  |  | SC-189 Co-chair prop. |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |  |
| Name |  | Org | Internet |  |  |  |  | Fax |  |  |
| Dave Allen |  | BCAG | David.L.Allen@Boeing.Com |  |  |  |  | +01-425-294-2007 |  |  |
| Martine Blaize |  | DGAC | BLAIZE_Martine@STNA.DGAC.Fr |  |  |  |  | +33-5-62-14-58-53 |  |  |
| Serge Bagieu |  | Aerospatiale | Serge.Bagieu@Avions.Aerospatiale.Fr |  |  |  |  | +33-5-61-93-80-90 |  |  |
| Tom Kraft |  | FAA | Tom.Kraft@FAA.DOT.Gov |  |  |  |  | +01-425-227-1181 |  |  |
| Peter Skaves |  | FAA | Peter.Skaves@FAA.DOT.Gov |  |  |  |  | +01-425-227-1181 |  |  |
| Anne-Sophie Luce SG-1 |  | AIRSYS | LUCE@Thomson.SDC.Fr |  |  |  |  | +33-1-40-84-1859 |  |  |
| Tony Martin SG-1 |  | AJM Cnslt | AJMartin@Accessone.Com |  |  |  |  | +01- 206-868-7297 |  |  |
| Emmanuel Chenevier SG-2 |  | DGAC | CHENEVIER_Emmanuel@ccmail.DGAC.Fr |  |  |  |  | +33-62-14-53-53 |  |  |
| Elizabeth Noon SG-1 |  | FAA | elizabeth.noon@faa.dot.gov |  |  |  |  | +01-206-227-1181 |  |  |
| Mark Joseph SG-2 |  | MITRE | MJ@MITRE.Org |  |  |  |  | +01-703-883-1330 |  |  |
| Kent Loving - SG-3 |  | BCAG | Kent.Loving@Boeing.Com |  |  |  |  | +01-425-342-6078 |  |  |
| Marc Barrere - SG-3 |  | Aerospatiale | Marc.Barrere@avions.aerospatiale.fr |  |  |  |  | +33-5-61-93-07-29 |  |  |