|  | ATNP/WG3/SG1 (Ground) WPxx Version 0.1 19 June 1997 |
| --- | --- |

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 1 (Ground-Ground Applications)

Draft Guidance Material for the AIDC application for the CNS/ATM-1 Package

Prepared by: 		Ground-Ground Applications Subgroup

SUMMARY

This document is the draft of the GM for the AIDC application in the CNS/ATM-1 Package Ground-Ground Applications.

WG3 is invited to adopt this material as the GM for the AIDC application in the CNS/ATM-1 Package SARPs.

CONFIGURATION SHEET

Title: 	Draft Guidance Material for the AIDC application for the CNS/ATM-1 Package

Version :		0.1

Date:		19 June 1997

Contact:		Airservices Australia, Eurocontrol, Federal Aviation Administration, Direction Generale de l’Aviation Civile

Status:		Draft

Change History :

| Version | Description | Date |
| --- | --- | --- |
| 0.1 | Initial draft for Langen ATNP/WG3 | 28 July 2026 |

Table of Contents

3.2 AIDC Guidance Material

3.2.1 Introduction

3.2.1.1 Purpose

3.2.1.2 Scope

3.2.1.3 Overview of AIDC Application

3.2.1.4 Inter-relationship of applications

3.2.1.5 SARPs Chapter overview

3.2.1.6 References

3.2.2 Overall general functionality

3.2.2.1 Introduction

3.2.2.2 Roles of an ATSU

3.2.2.3 Phases of Coordination

3.2.2.4 AIDC Primitives

3.2.2.5 The AIDC Functional Model

3.2.2.6 Dialogues

3.2.2.7 User-confirmation primitive

3.2.2.8 Naming and addressing

3.2.2.9 Rationale for user/system functionality

3.2.2.10 Relationship to existing ground-ground mechanisms

3.2.2.11 Relationship to ATS MHS

3.2.2.12 Relationship to A-G applications

3.2.3 Service based

3.2.3.1 Service conventions

3.2.3.2 Common services

3.2.3.3 Notifying regime

3.2.3.4 Coordinating regime

3.2.3.5 Transferring regime

3.2.4 Commentary on the AIDC SARPs

3.2.4.1 SARPs 3.2.2 (Introduction)

3.2.4.2 SARPs 3.2.2 (General Requirements)

3.2.4.3 SARPs 3.2.3 (The AIDC-AE Abstract Service)

3.2.4.4 SARPs 3.2.4 (The AIDC-ASE Abstract Service)

3.2.4.5 SARPs 3.2.5 (The AIDC Control Function)

3.2.4.6 SARPs 3.2.6 (AIDC-ASE Protocol Definition)

3.2.4.7 SARPs 3.2.7 (AIDC Formal Definitions)

3.2.4.8 SARPs 3.2.8 (Communication Requirements)

3.2.4.9 SARPs 3.2.9 (AIDC-User Requirements)

3.2.5 Dimensions (size of messages)

3.2.6 Indexes / tables

3.2.7 Example scenarios

3.2.8 Example encoding

# AIDC Guidance Material

## Introduction

### Purpose

In line with normal ICAO practice, this document was developed as a companion document to the ATN Interfacility Data Communication (AIDC) SARPs. It may be read alongside the AIDC SARPs, in order to provide a greater understanding of the specification itself, or it may be read instead of the AIDC SARPs by readers who simply want to understand the purpose of the AIDC Application rather than the detail of the specification.

### Scope

This document provides guidance material for those implementing the AIDC Service. It does not define any mandatory or optional requirements for AIDC, nor does it define any recommended practices.

### Overview of AIDC Application

#### The AIDC application enables the exchange of information between ATS Units (ATSUs) for support of critical Air Traffic Control (ATC) functions, such as notification of flights approaching a Flight Information Region (FIR) boundary, coordination of boundary conditions and transfer of control and communications authority.

#### AIDC is strictly an ATC application for exchanging tactical control information between ATS Units. It does not support the exchange of information with other offices or facilities.

#### In the nature of ICAO SARPs, it is assumed that the two ATSUs are under the authority of two different States. However, a State may find it convenient to apply the AIDC SARPs for relevant communications between two ATSUs under its own authority.

### Inter-relationship of applications

#### AIDC is a ground-ground application. The AIDC SARPs do not specify any direct relationship with other ATN applications; the AIDC application is a stand-alone application which can be developed, certified, installed and operated completely independently from the other ATN applications.

#### In a scenario where all ATN applications are implemented, an operating concept can be conceived, as follows, from the perspective of a single ATSU receiving, handling, and transferring a flight:

- flight plan data is received by means of the ATS Message Handling System (ATSMHS) application;

- notification, coordination and transfer of the flight from the transferring ATSU is performed through AIDC;

- data link communication with the aircraft is established through the Context Management (CM) application;

- dialogue with the aircraft and aircrew is maintained through the Controller-Pilot Datalink Communications (CPDLC) application;

- aircraft position and intent is monitored through the Automatic Dependent Surveillance (ADS) application;

- notification, coordination and transfer of the flight to the next ATSU is performed through AIDC.

### SARPs Chapter overview

#### The remaining sections of the AIDC GM are structured as follows.

- 3.2.2 – Overall general functionality – describes generic concepts that are used throughout the AIDC SARPs and guidance material, and also covers some implementation issues that are not addressed in the SARPs.

- 3.2.3 – Service based – gives a functional breakdown of the various services that AIDC provides. It describes a peer to peer interaction, including reasons for why particular information is used or not used, and what operations on the information are expected.

- 3.2.4 – Commentary on the AIDC SARPs – clarifies any functionality that was not addressed in the previous section on a chapter by chapter basis.

- 3.2.5 – Dimensions (size of messages) – gives some sample encoding sizes for guidance on what capacities need to be allowed for in order to meet normal operational expectations.

- 3.2.6 – Indexes / tables – provides a cross-reference to the ASN.1 definitions used in the AIDC application.

- 3.2.7 – Example scenarios – gives some examples as to what typical scenarios one can expect in course of normal AIDC operation.

- 3.2.8 – Example encoding – outlines some actual sample encoding of typical AIDC messages.

### References

Editor's note.

Full references to be provided for the following.

Manual of Data Link Applications

ISO/IEC 8649

ISO/IEC 8650

ISO/IEC 10731

ISO/IEC 8822

ISO/IEC 8824-1

ISO/IEC 8825-1

ISO/IEC 8825-2

ISO/IEC 9545

ULCS GM

ULCS SARPs

AIDC SARPs

AFTN SARPs

## Overall general functionality

### Introduction

#### The ATN AIDC application is based on the concepts and requirements specified in the ICAO Manual of ATS Data Link Applications [ref. x. Part VI], developed by the ICAO ADS Panel (ADSP). The basic concepts embodied in the AIDC application are ATSU roles and phases of coordination. The AIDC application implements information exchanges that are defined for each role and phase.

#### The subgroup of ATNP/Working Group 3 which has been responsible for the production of the AIDC SARPs material has worked closely with the relevant Working Groups of the ADSP, to ensure that the development of both the operational concepts and the technical means of achieving them keep in step with each other. However, the ADSP is generally looking at a longer timescale than the current ATNP initial implementation programme, and this will inevitably mean that some elements of its work have not been incorporated into the SARPs.

#### The AIDC application fulfils the requirements for coordination and transfer of responsibility specified in the SARPs for ATC Service [ref. ICAO Annex 11 Chapter 3.6].

### Roles of an ATSU

#### An ATSU can fulfil various roles in relation to a flight from the perspective of AIDC:

- Controlling ATSU (C-ATSU) when it is the ATS Unit currently responsible for control of a flight;

- Downstream ATSU (D-ATSU) when it is responsible for control of an airspace region through or near which the aircraft will progress at some future time;

- Receiving ATSU (R-ATSU) when it is about to assume control responsibility for the flight.

#### At any time, an ATSU may fulfil different roles in relation to each flight that it is handling.

### Phases of Coordination

#### Each flight, as it proceeds from one FIR to another, goes through three phases of coordination, listed in sequential order as follows:

- Notify Phase, in which the trajectory and any changes may be conveyed to a D-ATSU from the C-ATSU prior to coordination using a Notification dialogue;

- Coordinate Phase, in which the flight’s trajectory is coordinated between a C-ATSU and one or more D-ATSUs when the flight approaches a common boundary using Coordination dialogue; and

- Transfer Phase, in which communications and executive control authority is transferred from the C-ATSU to the R-ATSU using a Transfer dialogue.

#### Notification

##### Once a flight is airborne, the trajectory may be changed by the C-ATSU. The trajectory and any changes may be sent to affected D-ATSUs using a Notification dialogue. A flight does not actually have to enter an FIR for notification to be relevant: it need only pass within the lateral separation minima associated with an FIR. Notification occurs in advance of coordination with a D-ATSU.

#### Coordination

##### The coordination phase begins when the flight is a parameter time or distance from a common boundary between FIRs controlled by two or more ATSUs. A flight does not actually have to enter an FIR for coordination to be required: it need only pass within the lateral separation minima associated with an FIR. The cleared flight trajectory and boundary crossing conditions (if the flight actually crosses the boundary) are negotiated between the ATSUs using an initial Coordination dialogue until mutually agreed upon conditions are obtained. A re-Coordination dialogue(s) provides a mechanism for modifying an existing coordination, and may be invoked while the aircraft remains in the border areas of two or more FIRs.

#### Transfer

##### Transfers of communications and control authority normally occur when the flight is about to enter a new FIR. However, these may occur at any point subsequent to initial coordination subject to approval of both ATSUs. Transfer dialogues are employed to perform these functions.

#### Regimes

##### Each of the above phases is represented in the AIDC application by a corresponding regime: Notifying, Coordinating, Transferring.

### AIDC Primitives

#### The information exchanges (messages) specified in [ref. ADS Manual Part VI] transferred by the AIDC application are defined in the AIDC SARPs in terms of communications primitives according to the accepted convention for specifying Open Systems Interconnection (OSI) communications functions [ref. ISO/IEC 10731].

#### A unique subset of AIDC primitives is associated with each of the coordination regimes.

#### Other AIDC primitives support ancillary ATC data exchanges between ATSUs, including

a)	transfer of surveillance data, including ADS and radar reports, and

b)	exchange of free-text messages.

### The AIDC Functional Model

#### Figure 3.2.2–1 shows the functional model of the AIDC Application as defined by the AIDC SARPs.

Figure 3.2.2–1: Functional Model of the AIDC Application

#### The functional elements identified in this model are the following :

- the AIDC-User, which represents the operational part of the AIDC system,

- the AIDC Application Entity (AIDC-AE), which is the component defined and specified in the AIDC SARPs, comprising

  - the AIDC Control Function (CF),

  - the AIDC Application Service Element (AIDC-ASE), and

  - the Association Control Service Element, defined in [ref. ISO/IEC 8649]

- the ATN Service Provider Interface, which represents the rest of the ATN.

#### AIDC-User

##### The AIDC-User makes use of AIDC using the communication service provided by the AIDC-AE through the AIDC-AE Service interface.

##### The structure of the AIDC-User is outside the scope of ATN SARPs. However, it can be imagined as representing the combination of ATC software and ATC controller involved in handling a flight, including in particular a flight data processing subsystem, if present.

##### The involvement of the controller reflects the need for informed human oversight of automated decision-making. This principle of human-centred automation is described in [ref. ADS Manual].

##### Agreement to participate in the AIDC Application by the AIDC-User implies its acceptance of certain constraints on its behaviour, which are specified in SARPs 3.2.9.

#### AIDC-AE

##### The specification of the AIDC-AE follows the structure laid down in the OSI Upper Layer Architecture [ref. ISO/IEC 9545], as adapted to the ATN requirements in the ULCS SARPs.

##### Following this approach, the AIDC-AE is fully specified by specifying its constituent ASEs and the AIDC-AE CF.

##### Readers are encouraged to consult the ATN ULCS SARPs and GM for detailed rationale for and explanation of the ATN Upper Layer Architecture, and for detailed implementation guidance. The present GM provides an explanation of the AIDC-AE structure that is independent of the ULCS SARPs and GM. For the reader who has assimilated the ULCS SARPs and GM, the relation of the AIDC application structure to that specified in the ULCS is explained.

#### AIDC-ASE

##### The AIDC-ASE is the element in the communication system which executes the AIDC specific protocol, enforcing the AIDC specific primitive sequencing actions, timer management, and error handling.

#### AIDC-AE CF

##### The AIDC-AE CF is responsible for mapping primitives received from one element to another within the AIDC functional model. Thus it can be seen as a 'glue' that links the AIDC-ASE and the ACSE.

##### The CF is specified in terms of the possible events that can occur. These comprise

- submission of primitives by the AIDC-User (across the AIDC service boundary),

- delivery of primitives by the ATN Service Provider (across the ATN Service Provider Interface)

- delivery of primitives by the AIDC-ASE (across the upper AIDC-ASE service boundary)

- delivery of primitives by the ACSE (across the upper ACSE service boundary)

- expiration of internal CF timers.

##### Each event results in further processing, which may include the submission or delivery of further service primitives as well as timer manipulations. The CF maintains an internal state on which the specific processing required for each event depends.

#### ATN Service Provider Interface

##### The ATN Service Provider Interface provides access to OSI layers 1-6 of the ATN supporting the AIDC application. Thus the Service Provider Interface is the presentation service boundary defined in [ref. ISO/IEC 8222].

##### The ATN Service Provider is specified in the ATN SARPs for Upper Layer Communications Service and Internet Communications Service.

##### The AIDC application uses the ATN ICS, via the ATN Service Provider Interface, to ensure that ATC information is exchanged in a reliable and timely manner between ATSUs.

#### Implementation

##### In implementing the AIDC application, the functionality described by the AIDC-AE model is to be implemented.

##### The software developed, whether it is a collection of modules, a single module, or a number of separate application processes, should, from the AIDC-User’s point of view, exhibit the behaviours defined by the model described in the SARPs.

##### SARPs requirements and recommendations on the AIDC-AE only need to be interpreted in terms of what they imply for information transferred over the ATN. Provided that the AIDC functionality is made available to the AIDC-User and the AIDC protocol, as observed at the interface between the real AIDC end system and the ATN subnetwork, conforms to the SARPs, the implementer is not constrained.

### Dialogues

#### A dialogue is a sequence of one or more related message exchange sequences. All exchanges of AIDC messages between ATSUs may be viewed as dialogues. AIDC dialogues support the following functions:

- Notification;

- Coordination;

- Transfer;

- General Information Interchange, and

- Surveillance Data Transfer.

#### Most dialogues consist of a single primitive request and response. However, coordination and transfer dialogues involve multiple primitive sequences. The structure and sequence of AIDC dialogues is discussed further in section 3.2.3.

#### An AIDC dialogue concerns a single flight. An ATSU can only maintain one AIDC dialogue per flight with another ATSU

#### A dialogue is typically initiated when one of several events, such as the following, occurs:

- proximity to an airspace boundary;

- controller action;

- a change in a flight’s cleared trajectory affecting a D-ATSU; or

- surveillance data update.

#### Use of connections

##### The AIDC service hides the use of the underlying communications service from the AIDC-User. There is a direct correspondence between a dialogue and an underlying application association, such that information dealing with a single flight is transferred over a single application association.

##### Certain AIDC primitives (which include the necessary addressing information) implicitly establish an application-association. These can be considered as initiating a dialogue. In version 1 of the AIDC SARPs, an application-association corresponds directly to a single presentation connection, session connection, and transport connection.

##### Version 1 of the AIDC application does not directly support coordination by a C-ATSU of a single flight with several D-ATSUs. When AIDC is being used to coordinate a single flight with several D-ATSUs, separate instances of the AIDC application with a correspondingdialogue exist for each pair of ATSUs together with their supporting connections.

### User-confirmation primitive

#### Many of the AIDC dialogues include the User-confirmation primitive. This is used by a responding AIDC entity to indicate either

- where the primitive is one of a sequence in a dialogue, that a preceding primitive has been delivered and is being processed, or,

- where the primitive is the last in a sequence, to acknowledge the end of a phase of the dialogue, or

- where the primitive is isolated, to acknowledge its delivery.

### Naming and addressing

#### Addressing

##### The addresses used in the AIDC application are OSI application-entity-titles, as defined in [ref. ISO/IEC 7498-3].

##### The structure of the AIDC addresses is specified in the ULCS SARPs, based on ICAO location indicators [ref. ICAO Doc 7030] of the respective ATSUs, as follows:

{ iso(1) identified-organization (3) icao (27) atn-end-system-ground (2) <end-system-id> (n) operational (0) idc (6) }

#### Version handling

##### The version of the SARPs corresponding to this GM has the AIDC-AE version number set to 1.

##### The version number is transferred in the association establishment phase as part of the application-context-name:

{ iso(1) identified-organization (3) icao (27) atn-ac (3) version1 (1)}

##### An implementation that only supports the version 1 application-context will reject an establishment attempt specifying another version.

##### An implementation that supports both version 1 and another version of the AIDC application is responsible for maintaining compatibility with implementations that only support version 1, where this is desired. For instance, an implementation supporting other versions of AIDC may detect the rejection of an establishment attempt using a version other than one and retry using the version 1 application-context.

##### Implementations using a full OSI upper-layer profile would be able to negotiate the application-context. The FastByte upper-layer profile adopted in the ATN ULCS excludes this capability.

##### The AIDC service primitives, their format and sequencing rules may all be subject to change in future versions of AIDC. The AIDC-User may have to invoke different processing logic based on the selected AIDC version and so will need to be involved in any version negotiation. If an AIDC implementation is capable of negotiating between several supported versions of the AIDC application, implementers need to consider providing both

- a means for the AIDC application to report the version of the application that is in use following a dialogue-establishing primitive (these primitives are Notify, CoordinateStart, and InfoTransfer), and

- a means by which the AIDC-User can control which of the supported versions the AIDC implementation uses.

### Rationale for user/system functionality

#### The AIDC-AE supports the dialogues needed for notification, coordination and transfer, and enforces the necessary logical and temporal constraints on the primitives in each dialogue. Thus two valid implementations of the AIDC-AE will be able to interact, passing data to each other in the correct order; they will be able to check the format of the data, ensure that it has been sent with the appropriate service primitive and ensure that the peer ASE is behaving according to the requirements in the SARPs.

#### Within the AIDC-AE, the AIDC-ASE contains all the logic specific to the AIDC application, while the AIDC-CF links it to the supporting communications services (ACSE and presentation service).

#### The SARPs define some requirements on the AIDC-User, which are required to ensure the interoperability of the two peers.

### Relationship to existing ground-ground mechanisms

#### Distribution of flight plan information

##### Messages defined in [ref. ICAO Doc 4444] are widely used to transfer flight plan information between ATSUs, typically some hours in advance of the flight. These messages are typically transferred using the Aeronautical Fixed Telecommunications Network (AFTN) [ref. ICAO Annex 10 AFTN SARPs]. Other arrangements may be used in some circumstances.

##### Distribution of flight plan information to relevant ATSUs is not essential for the use of AIDC. However, such information is typically needed in the ATSU for other reasons (eg, work scheduling, flow management). Availability of a flight plan in a D-ATSU reduces the controller's workload and avoids the need to transfer large amounts of flight plan information in the Notification phase.

##### The ATN ATSMHS Application provides another means of distributing flight plan information.

#### Notification, coordination and transfer

##### Message exchanges have been developed on a regional basis to support the notification, coordination and transfer of flights [ref. Eurocontrol OLDI Standard, NAT OLDI, …].

##### The requirements for the AIDC application were strongly influenced by the experience of developing and implementing these regional protocols. Use of AIDC as an alternative may provide benefits in the ATN era.

##### As of 1998, these regional approaches are independent of ATN. The communication mechanism supporting message exchange is provided by special point-to-point protocols, or by AFTN. Use of the ATN ICS as an alternative transport mechanism may provide benefits in the ATN era.

### Relationship to ATS MHS

#### There is no specific relationship between the AIDC application and the ATS MHS application.

#### As identified above, ATS MHS provides a possible means of distributing flight plan information to support the operation of AIDC.

### Relationship to A-G applications

#### There is no specific relationship between the AIDC application and the ATN air-ground applications.

#### The CPDLC application can be used by the C-ATSU to pass information and commands to the aircrew as needed to achieve the transfer of the flight. Integration of CPDLC with AIDC in this way may provide benefits in terms of reduced controller workload.

#### The ADS application can be used by the C-ATSU to monitor the conformance of the aircraft to the agreed coordination conditions.

## Service based

explanatory material adapted from ADSP DLA Manual chapter 6 - show how the AIDC services & protocols meet the requirements specified there.

### Service conventions

#### The AIDC service defined in SARPs 3.2.3, as an OSI service definition, is an abstract boundary in the end system. It follows the conventions for OSI services in [ref. ISO/IEC 10731].

#### There is no requirement to implement a real interface, such as a set of procedure calls, in an implementation corresponding to the AIDC service boundary.

#### However, the AIDC service boundary does represent a sensible point in an implementation where a major software interface could be defined (a better point, for instance, than the AIDC-ASE or presentation service boundaries). The AIDC service definition can be used as the basis for such a software interface, though the exact form of the interface would depend on the implementation technology. For instance, possible realisations of an AIDC software interface include

- messages sent by inter-process communication

- procedure calls

- software interrupts

- object method invocations.

#### The Eurocontrol Trials End System project [ref. xxx] has demonstrated the definition of software interfaces for ATN applications using C calls.

Editor's note

We could develop an AIDC API as an attachment?

### Common services

#### User-confirmation

##### The User-confirmation service implements the AppAccept and AppError messages defined in [ref. ADS Manual].

##### The User-confirmation service is unconfirmed: ie, the use of the service consists of the submission of User-confirmation.request by one AIDC-User and the corresponding delivery of User-confirmation.indication to the peer AIDC-User.

##### The User-confirmation service is used in all three regimes of the AIDC service, whenever an 'Information' parameter is delivered, to indicate whether or not the received parameter is valid according to the receiving AIDC-User.

##### A User-confirmation primitive may be

- positive, indicated by the Result parameter having the abstract value 'accepted', or

- negative, indicated by the Result parameter having the abstract value 'rejected'.

##### A positive User-confirmation.indication does not signify acceptance of the information (eg, it does not accept a proposal of coordination conditions). It merely indicates that the information was 'understood' by the receiving AIDC-User (eg, the flight associated with proposed coordination conditions was recognised).

##### A negative User-confirmation signifies that the information was not understood.

#### End

##### The End service implements the CoordinateCancel message defined in [ref. ADS Manual]. It may also be used outside the Coordination phase, as specified in the AIDC SARPs.

##### The End service is unconfirmed: ie, the use of the service consists of the submission of End.request by one AIDC-User and the corresponding delivery of End.indication to the peer AIDC-User.

##### The End service is used in all regimes to end a dialogue. The service can either  terminate the dialogue (implicitly indicating successful completion of the dialogue), or cancel the dialogue.

##### Use of the End service to cancel a dialogue indicates that information, proposals, and/or agreements resulting from the dialogue are no longer valid. For instance, in a Coordination or Notification dialogue, the flight may have been re-routed under the control of the C-ATSU so that it will no longer enter the airspace controlled by the D-ATSU.

#### User-abort

### Notifying regime

#### Notify

### Coordinating regime

#### Coordinate-start

#### Coordinate-negotiate

#### Coordinate-standby

### Transferring regime

#### Transfer-initiate

#### Transfer-conditions-proposal

#### Transfer-conditions-accept

#### Transfer-request

#### Transfer-control

#### Transfer-communication

#### Transfer-communication-assume

## Commentary on the AIDC SARPs

### SARPs 3.2.2 (Introduction)

### SARPs 3.2.2 (General Requirements)

Error Handling

Use of ULA in AIDC

### SARPs 3.2.3 (The AIDC-AE Abstract Service)

What is an "abstract service"

U/M, why U, M, C (cell contents), two tables

Table content, format: destination/source of the data (ASN.1) (generic)

ASN.1 versus abstract values

### SARPs 3.2.4 (The AIDC-ASE Abstract Service)

#### Function

##### The AIDC-ASE is the functional module that models the protocol associated with the AIDC application.

##### As modelled, the AIDC-ASE is only active when there is AIDC data to be transferred between the AIDC-AE peers.

##### The AIDC-ASE is not involved with the establishment of the association between AIDC-ASE peers.

#### AIDC-ASE Services

##### The abstract service boundary defined for the AIDC-ASE has two components:

- the upper service boundary, defined in SARPs 3.2.4.2, corresponds closely to the AIDC-AE abstract service defined in SARPs 3.2.3;

- the lower service boundary, defined in SARPs 3.2.4.3, is an abstraction of the supporting communications service required by the ASE.

##### The AIDC-ASE, as specified, may also be used in the specification of other applications, provided that the corresponding control function is able to coordinate other ASEs to meet the AIDC-ASE service requirements.

### SARPs 3.2.5 (The AIDC Control Function)

Explain relation to ULA

Correspondence to use of dialogue service

? issue of ACSE versions

? other things as per ULA GM

Structure of CF

#### Function

The CF modelled in the AIDC SARPs forms the basic glue which supports the other service elements modelled as part of the AIDC-AE.

The CF is responsible for the following:

- mapping primitives submitted to the CF by the AIDC-User to the primitives of the appropriate ASEs with the AIDC-AE;

- implicitly initialising the association between AIDC-AE peers;

- mapping primitives submitted by the ASEs within the AIDC-AE to other ASEs within the AIDC-AE and primitives at the ATN Service Provider interface;

- mapping of primitives submitted to the CF from the ATN Service Provider interface to the ASEs within the AIDC-AE.

#### Implementation

##### The CF is not dissimilar to the main part of a software program, if the other elements of the AIDC-AE are viewed as software modules.

##### In order to facilitate the upgrading of the AIDC application from one version to another, where protocol is added, the CF should be implemented as a module.

### SARPs 3.2.6 (AIDC-ASE Protocol Definition)

#### Protocol Description

##### The protocol description is a specification based on the functional model of the AIDC-ASE.

##### The protocol is specified in terms of the actions of the AIDC-ASE in response to the submission of service primitives at its upper service interface and the delivery of service primitives at its lower service interface.

##### For each AIDC-ASE user service primitive the protocol specifies:

- the state of the AIDC-ASE in which it is appropriate to invoke the primitive,

- the action performed by the AIDC-ASE on receipt of the primitive, and

- any conditions that need to be tested.

#### Exception Handling

#### Application Timers

#### State Table

### SARPs 3.2.7 (AIDC Formal Definitions)

#### Abstract Syntax Notation 1

##### The Abstract Syntax Notation 1 (ASN.1) is an International Standard which specifies a notation for abstract syntax definition [ref. ISO/IEC 8824-1].

##### ASN.1 is used in the AIDC SARPs to define the AIDC protocol syntax and the information transferred.

##### Thus it explicitly specifies the structure of the AIDC application protocol data units.

##### In addition, since the information transferred in AIDC APDUs is provided by and delivered through the AIDC-AE and AIDC-ASE abstract services, the ASN.1 definitions implicitly specify the abstract syntax of the parameters of these services, and hence the abstract syntax of the information passed between the AIDC-User and the AIDC-AE.

#### The AIDC ASN.1

##### Structure

###### The ASN.1 for the AIDC application is structured as follows :

- APDU definitions,

- message definitions,

- message element definitions, and

- definitions for error related types.

###### Where possible, the types reflect the definitions defined for other ATN applications to ensure consistency.

###### Throughout the ASN.1, extensibility markers (shown as …) have been placed to allow expansion of the AIDC messages and other AIDC types.

### SARPs 3.2.8 (Communication Requirements)

#### Encoding Rules

##### Data items specified using ASN.1 can be encoded systematically using a defined encoding for each primitive and constructed data type.

##### Several sets of such Encoding Rules have been standardised, of which the most widely used is the Basic Encoding Rules (BER) [ref. ISO/IEC 8825-1].

##### The ATN ULCS SARPs specify the use of the Packed Encoding Rules (PER) [ref. ISO/IEC 8825-2].

##### Encoding using PER results in much smaller encoded PDUs than using BER, but decoding PER-encoded data requires knowledge of the abstract syntax by the decoder, unlike BER.

##### PER compilers are available to generate high-level software source code based on the ASN.1 structures defined in 3.2.7 of the SARPs.

#### Quality-of-service Requirements

##### Routing Policy

##### Priority

##### Residual Error Rate

### SARPs 3.2.9 (AIDC-User Requirements)

Justify chapter 9. Chapters 3, 4 and 6 guarantee interoperability but not operational acceptability (use of service primitives, use of optional ASN.1 data, required relationships between parameters in the request and in the response.

Range and Resolution - from Manual;units.

Answers to some "whys". Justify rules beyond the ADSP instructions

Features designed to preserve an element of compatibility with previous systems.

Organisational procedures for setting up AIDC links (letter of agreement, etc);

Subsetting and compatibility issues

OLDI interworking issues?

## Dimensions (size of messages)

Largest message and APDU size + justification of upper bounds

Practicability

Current/future limitation (COTS limitations...)

Operational use (capacity, timings, ...)

Commercial considerations

Reasonable implementation. Real number given. (what happens if sender sends a message too big for the receiver to handle).

Max usable size - TBD (consult with CAAs). Is a 'shall' statement required in SARPs ?

Rate of messages

Number of connections (both ways)

Number of dialogues

## Indexes / tables

ASN.1 index (cf. CPDLC example)

## Example scenarios

User level scenarios: 	related to ADSP DLA Manual chapter 6

adapted from 'Operational requirements for initial data link'?

OLDI interworking?

## Example encoding

Based on examples from Norman Goodacre, Fred. Picard or SG3.