# FORMAL DEFINITIONS OF MESSAGES

## ADS ASN.1 Abstract Syntax

### The abstract syntax of the air-ground ADS protocol data units shall comply with the description contained in the ASN.1 module ADSAGMessageSetVersion1 (conforming to [3]), as defined in this section.

Note 1.— Different states have different requirements with respect to the units of measurement of some of the fields in the PDUs. The units described below have been defined to meet these differing requirements. Conversion to the units should be made in the transmitting application and conversion from the units should be made at the receiving application. The new units have been defined to be midway between the minimum resolution required by the two units used by different states.

Unit of altitude = 1 alt-unit = 3.02400 metres = 9.92126 feet (midway between 3 metres and 10 feet)

To convert from alt-unit to metres, multiply by	3.02400

To convert from alt-unit to feet, multiply by		9.92126

To convert from meters to alt-units, multipy by	0.33069

To convert from feet to alt-units, multipy by		0.10079

Unit of speed = 1 speed-unit = 0.9633 km/hour = 0.51980 knots (midway between half a knot and one kilometre per hour)

To convert from speed-unit to km/hour, multiply by	0.96330

To convert from speed-unit to knots, multiply by	0.51980

To convert from km/hour to speed-units, multipy by	1.03801

To convert from knots to speed-units, multipy by	1.92382

Note 2.— Where units indicate directional information, the value is given relative to true North. If magnetic information is required this will be a matter for local ground implemenation.

ADSAGMessageSetVersion1 DEFINITIONS::=

BEGIN

EXPORTS

AbortReason, ADSEmergencyReport, ADSReport, AirframeId, EventTypeReported;

-- ------------------------------------------------------------------------------------------------------------------------

-- Aircraft-generated and Ground-generated Message Choice

-- ------------------------------------------------------------------------------------------------------------------------

ADSAircraftPDUs ::= CHOICE

{

aDS-cancel-emergency-PDU		[0]	NULL,

aDS-demand-report-PDU			[1]	ADSDemandReport,

aDS-emergency-report-PDU		[2]	ADSEmergency,

aDS-event-report-PDU			[3]	ADSEventReport,

aDS-negative-acknowledgement-PDU	[4]	NegativeAcknowledgement,

aDS-noncompliance-notification-PDU	[5]	NoncomplianceNotification,

aDS-periodic-report-PDU			[6]	ADSPeriodicReport,

aDS-positive-acknowledgement-PDU	[7]	PositiveAcknowledgement,

aDS-provider-abort-PDU			[8]	AbortReason,

...

}

ADSGroundPDUs ::= CHOICE

{

aDS-cancel-all contracts-PDU		[0]	NULL,

aDS-cancel-contract-PDU			[1]	CancelContract,

aDS-cancel-emergency-acknowledgement-PDU	[2]	NULL,

aDS-demand-contract-PDU		[3]	DemandContract,

aDS-event-contract-PDU			[4]	EventContract,

aDS-modify-emergency-contract-PDU	[5]	ModifyEmergency,

aDS-periodic-contract-PDU		[6]	PeriodicContract,

aDS-provider-abort-PDU			[7]	AbortReason,

...

}

-- ------------------------------------------------------------------------------------------------------------------------

-- Ground-generated and Aircraft-generated message components - Protocol Data Units

-- ------------------------------------------------------------------------------------------------------------------------

AbortReason  ::= ENUMERATED

{

communications-service-failure (0),

unrecoverable-system-error (1),

invalid-PDU (2),

sequence-error (3),

timer-expiry (4),

cannot-establish-contact (5),

undefined-error (6),

dialogue-end-not-accepted (7),

...

}

ADSDemandReport ::= SEQUENCE

{

report					[0]	ADSReport,

positive-acknowledgement		[1]	NULL			OPTIONAL,

...

}

ADSEmergency ::= SEQUENCE

{

emergency-report				[0]	ADSEmergencyReport,

positive-acknowledgement		[1]	NULL			OPTIONAL,

...

}

ADSEventReport ::= SEQUENCE

{

event-type				[0]	EventTypeReported,

report					[1]	ADSReport,

positive-acknowledgement		[2]	NULL			OPTIONAL,

...

}

ADSPeriodicReport ::= SEQUENCE

{

report					[0]	ADSReport,

positive-acknowledgement		[1]	NULL			OPTIONAL,

...

}

CancelContract ::= ENUMERATED

{

event-contract (0),

periodic-contract (1),

...

}

DemandContract ::= SEQUENCE

{

airframe-id				[0]	NULL		OPTIONAL,

projected-profile				[1]	NULL		OPTIONAL,

ground-vector				[2]	NULL		OPTIONAL,

air-vector				[3]	NULL		OPTIONAL,

weather					[4]	NULL		OPTIONAL,

navigational-intent			[5]	ProjectionTime	OPTIONAL,

extended-projected-profile			[6]	ExtendedProjectedProfileRequest

OPTIONAL,

...

}

EventContract ::= SEQUENCE

{

-- At least one of the following optional elements must be present

lateral-deviation-change			[0]	LateralChange		OPTIONAL,

vertical-rate-change			[1]	VerticalRateChange	OPTIONAL,

altitude-range				[2]	AltitudeRange		OPTIONAL,

way-point-change			[3]	NULL			OPTIONAL,

air-speed-change				[4]	AirSpeedChange		OPTIONAL,

ground-speed-change			[5]	GroundSpeedChange	OPTIONAL,

heading-change				[6]	DegreesDirection		OPTIONAL,

extended-projected-profile-change		[7]	ExtendedProjectedProfileRequest

OPTIONAL,

fom-change				[8]	NULL			OPTIONAL,

track-angle-change			[9]	DegreesDirection		OPTIONAL,

altitude-change				[10]	AltitudeChange		OPTIONAL,

...

}

ModifyEmergency ::= ReportingInterval

NegativeAcknowledgement ::= SEQUENCE

{

request-type					RequestType,

reason						Reason

}

NoncomplianceNotification ::= CHOICE

{

demand-ncn				[0]	SET OF ReportType,

event-ncn				[1]	SET OF EventTypeContracted,

periodic-ncn				[2]	SET OF ReportTypeAndPeriod,

...

}

PeriodicContract ::= SEQUENCE

{

reporting-interval			[0]	ReportingInterval

DEFAULT {minutes-scale 5},

airframe-id-modulus			[1]	Modulus			OPTIONAL,

projected-profile-modulus			[2]	Modulus			OPTIONAL,

ground-vector-modulus			[3]	Modulus			OPTIONAL,

air-vector-modulus			[4]	Modulus			OPTIONAL,

weather-modulus				[5]	Modulus			OPTIONAL,

navigational-intent			[6]	NavigationalIntent	OPTIONAL,

extended-projected-profile-modulus	[7]	ExtendedProjectedProfileModulus

OPTIONAL,

...

}

PositiveAcknowledgement ::= RequestType

-- ------------------------------------------------------------------------------------------------------------------------

-- Reports and their components

-- ------------------------------------------------------------------------------------------------------------------------

ADSEmergencyReport ::= SEQUENCE

{

position					[0]	Position,

time-stamp				[1]	DateTimeGroup,

fom					[2]	FigureOfMerit,

airframeId				[3]	AirframeID		OPTIONAL,

ground-vector				[4]	GroundVector		OPTIONAL

}

ADSReport ::= SEQUENCE

{

position					[0]	Position,

time-stamp				[1]	DateTimeGroup,

fom					[2]	FigureOfMerit,

airframe-id				[3]	AirframeID		OPTIONAL,

projected-profile				[4]	ProjectedProfile		OPTIONAL,

ground-vector				[5]	GroundVector		OPTIONAL,

air-vector				[6]	AirVector		OPTIONAL,

weather					[7]	Weather			OPTIONAL,

short-term-intent				[8]	ShortTermIntent		OPTIONAL,

extended-projected-profile			[9]	ExtendedProjectedProfile	OPTIONAL,

...

}

AirframeID ::= BIT STRING (SIZE (24))

-- 24 bit ICAO airframe identifier

AirVector ::= SEQUENCE

{

heading					[0]	DegreesDirection 	OPTIONAL,

air-speed				[1]	AirSpeed 		OPTIONAL,

vertical-rate				[2]	VerticalRateChange	OPTIONAL

}

-- When a report provides an AirVector it should provide all the information

-- that it has available at the time.

AirSpeed ::= CHOICE

{

mach					[0]	Mach,

ias					[1]	Ias,

mach-and-ias				[2]	SEQUENCE

{

Mach,

Ias

}

}

-- When AirSpeed is returned in an ADS report, the choice of which of the above units of

-- air speed are used depends on how the aircraft is equipped and whether the aircraft is

-- flying on mach or ias at the time. The choice is made by the avionics.

ExtendedProjectedProfile ::= SEQUENCE (SIZE 1..128) OF SEQUENCE

{

way-point				Position,

time					Eta

}

FigureOfMerit ::= SEQUENCE

{

position-accuracy				PositionAccuracy,

multiple-navigational-units-operating	BOOLEAN,

acas-operational				BOOLEAN

}

PositionAccuracy ::= ENUMERATED	-- nm = nautical miles

{

complete-loss	(0),

under30nm	(1),

under15nm	(2),

under8nm	(3),

under4nm	(4),

under1nm	(5),

under-25nm	(6),  -- under 0.25 nm

under-05nm	(7)   -- under 0.05 nm

}

GroundVector ::= SEQUENCE

{

track				[0]	DegreesDirection 		OPTIONAL,

ground-speed			[1]	INTEGER (-110..4200) 		OPTIONAL,

-- units = speed-units (see Note 1)

-- range = -110 to + 4200 speed-units

vertical-rate			[2]	VerticalRateChange 		OPTIONAL

}

-- When a report provides an GroundVector it should provide all the information

-- that it has available at the time.

ProjectedProfile ::= SEQUENCE

{

next-way-point				Position,

next-time				Eta,

following-way-point			Position

}

ShortTermIntent ::= SEQUENCE

{

position					Position,

projected-time				ProjectionTime,

intermediate-intent			IntermediateIntent

}

IntermediateIntent ::= SEQUENCE  OF SIZE (0..7) SEQUENCE

{

distance				[0]	INTEGER (1..8000),

-- units = Nautical miles

-- range = 1 to 8000 nautical miles

track				[1]	DegreesDirection,

altitude				[2]	Altitude,

projected-time			[3]	ProjectionTime

}

-- IntermediateIntent indicates a set of way points between the current position and the

-- time indicated in the ShortTerm intent element.

-- When a report provides an IntermediateIntent it should provide all the information

-- that it has available at the time.

Weather ::= SEQUENCE

{

wind-speed			[0]	INTEGER (0..520) OPTIONAL,

-- units = speed-units (see Note 1)

-- range = 0 to 520 speed-units

wind-direction			[1]	INTEGER (1..360) OPTIONAL,

-- units = degrees true North

-- range= 1 to 360 degrees

temperature			[2]	INTEGER (-400..400) OPTIONAL,

-- units = 0.25 degrees Centigrate

-- range = -100 to 100 degrees C

turbulence			[3]	INTEGER (0..15) OPTIONAL

-- this is a place marker for a turbulence

-- index which is to be defined

}

-- When an ADS report provides Weather it should provide all the information

-- that the avionics has available at the time.

-- ------------------------------------------------------------------------------------------------------------------------

-- Components of Contracts

-- ------------------------------------------------------------------------------------------------------------------------

AirSpeedChange ::= CHOICE

{

mach-number-change		[0]	INTEGER (1..255),

-- units = 0.005 Mach

-- range = 0.005 to 1.275 Mach

ias-change			[1]	INTEGER (1..700)

-- units = speed-units (see Note 1)

-- range = 1 to 700 speed-units

}

AltitudeChange ::= INTEGER (1..110)

-- units = alt-units (see Note 1)

-- range = 1 alt-unit to 110 alt-units

AltitudeRange ::= SEQUENCE

{

ceiling					Altitude,

floor					Altitude

}

DegreesDirection ::= INTEGER (1..3600)

-- units = 0.1 degrees true North,

-- range = 0.1 to 360 degrees

ExtendedProjectedProfileModulus ::= SEQUENCE

{

modulus					Modulus,

extended-projected-profile-request		ExtendedProjectedProfileRequest

}

ExtendedProjectedProfileRequest ::= CHOICE

{

time-interval			[0]	INTEGER (1..80),

-- relative to current time stamp

-- units = 15 minutes

-- range =15 minutes to 20 hours

number-of-way-points		[1]	INTEGER (1..128)

}

GroundSpeedChange ::= INTEGER (0..500)

-- units = speed-units (see Note 1)

-- range = 0 speed-units to 500 speed-units

Ias ::= INTEGER (-110..2100),

-- units = speed-units (see Note 1)

-- range = -110 speed-units to +2100 speed-units

LateralChange ::= INTEGER (0..255)

-- units = 0.125 Nautical miles

-- range= 0 nautical miles to 31.875 nautical miles

Mach ::= INTEGER (0..4000)

-- units = mach 0.001

-- range = 0 mach to 4 mach

NavigationalIntent ::= SEQUENCE

{

intent-modulus				Modulus,

intent-projection-time			ProjectionTime

}

ProjectionTime ::= INTEGER (1..240)

-- units = minutes relative to current time stamp

-- range = 1 minute to 4 hours

ReportingInterval ::= CHOICE

{

seconds-scale			[0]	INTEGER (1..59),

-- units = seconds

-- range = 1 second to 59 seconds

minutes-scale			[1]	INTEGER (1..120)

-- units = minutes

-- range = 1 minute to 2 hours

}

VerticalRateChange ::= INTEGER (-3310..3310)

-- units = alt-units per minute (see Note 1)

-- range = -3310 alt-units per minute to +3310 alt-units per minute

-- ------------------------------------------------------------------------------------------------------------------------

-- Miscellaneous components

-- ------------------------------------------------------------------------------------------------------------------------

Reason ::= CHOICE

{

aDS-service-unavailable 		[0]	NULL,

cannot-meet-reporting-rate	[1]	NULL,

maximum-capacity-exceeded	[2]	GroundSystemsUsingService,

undefined-reason			[3]	NULL

...

}

GroundSystemsUsingService ::= SEQUENCE OF IA5String (SIZE(8))

-- contains a sequence of ICAO facility designators

RequestType ::= ENUMERATED

{

event-contract (0),

periodic-contract (1),

demand-contract (2),

cancel-event-contract (3),

cancel-periodic-contract (4),

modify-emergency-contract (5),

cancel-all-contracts (6),

end-forward-contract (7),

...

}

EventTypeContracted ::= ENUMERATED

{

lateral-deviation-change (0),

vertical-rate-change (1),

altitude-threshold (2),

way-point-change (3),

air-speed-change (4),

ground-speed-change (5),

heading-change (6),

extended-projected-profile-change (7),

fom-change (8),

track-angle-change (9),

altitude-change (10),

...

}

EventTypeReported ::= ENUMERATED

{

lateral-deviation-change (0),

vertical-rate-change (1),

altitude-threshold (2),

way-point-change (3),

air-speed-change (4),

ground-speed-change (5),

heading-change (6),

extended-projected-profile-change (7),

fom-change (8),

track-angle-change (9),

altitude-change (10),

baseline (11),

...

}

Modulus ::= INTEGER (1..255)

ReportType ::= ENUMERATED

{

airframe-id (0),

projected-profile (1),

ground-vector (2),

air-vector (3),

weather	(4),

short-term-intent (5),

extended-projected-profile (6),

...

}

ReportTypeAndPeriod ::= ENUMERATED

{

airframe-id (0),

projected-profile (1),

ground-vector (2),

air-vector (3),

weather	(4),

short-term-intent (5),

extended-projected-profile (6),

reporting-rate(7),

...

}

-- ------------------------------------------------------------------------------------------------------------------------

-- Common components

-- ------------------------------------------------------------------------------------------------------------------------

Eta ::= INTEGER (0..960)

-- units = 1 minute

-- range = 0 minutes to 16 hours

-- For projected profile, ETA at next way-point is relative to the current time stamp.

-- For extended projected profile, ETA at next way-point is relative to the current time stamp,

-- any following ETAs at next way-points are relative to previous way-points.

Position ::= SEQUENCE

{

latitude					Latitude,

longitude				Longitude,

altitude					Altitude

}

Latitude ::= SEQUENCE

{

sign					Sign

degrees					INTEGER (0..90),

-- units = degrees

-- range = 0 degrees to 90 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

-- If ‘degrees’ is 90, ‘minutes’ and ‘tenths-seconds’ must be 0

}

Longitude ::= SEQUENCE

{

sign					Sign

degrees					INTEGER (0..180),

-- units = degrees

-- range = 0 degrees to 180 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

-- If ‘degrees’ is 180, ‘minutes’ and ‘tenths-seconds’ must be 0

}

Sign ::= ENUMERATED

{

plus (0),

minus (1)

}

Altitude	 ::= INTEGER (-10..6640)

-- units = alt-units (see Note 1)

-- range = -10 alt-units to + 6640 alt-units

DateTimeGroup ::= SEQUENCE

{

date            Date,

time            Time

}

Date ::= SEQUENCE

{

year					Year,

month					Month,

day					Day

}

Year ::= INTEGER (1996..2095)

-- unit = year

-- range = 1996 to 2095

Month ::= INTEGER (1..12)

-- unit = month

-- range = January to December

Day ::= INTEGER (1..31)

-- unit = day

-- range = 1 to 31

Time ::= SEQUENCE

{

timeHours			[0]	TimeHours,

timeMinutes			[1]	TimeMinutes,

timeSeconds			[2]	TimeSeconds 		OPTIONAL

}

TimeHours ::= INTEGER (0..23)

-- units = hours

-- range = midnight to 23.00 (11 PM)

TimeMinutes ::= INTEGER (0..59)

-- units = minutes

-- range = 0 minutes to 59 minutes

TimeSeconds ::= INTEGER (0..59)

-- units = seconds

-- range = 0 seconds to 59 seconds

END	-- of ADSAGMessageSetVersion1

### The abstract syntax of the ground-ground ADS protocol data units shall comply with the description contained in the ASN.1 module ADSGGMessageSetVersion1 (conforming to [3]), as defined in this section.

ADSGGMessageSetVersion1 DEFINITIONS::=

BEGIN

IMPORTS

AbortReason, ADSEmergencyReport, ADSReport, AirframeId, EventTypeReported

FROM ADSAGMessageSetVersion1;

ADSGroundGroundPDUs ::= CHOICE

{

aDS-forwarded-report-PDU		[0]	ADSForwardedReport,

aDS-provider-abort-PDU			[1]	AbortReason,

...

}

ADSForwardedReport ::= SEQUENCE

{

airframeId					AirframeID,

forwardedADSReport				ForwardedReport

}

ForwardedReport	::= CHOICE

{

aDSDemandReport			[0]	ADSReport,

aDSPeriodicReport			[1]	ADSReport,

aDSEventReport				[2]	SEQUENCE

{

event-type	EventTypeReported,

aDSReport	ADSReport

},

aDSEmergencyReport			[3]	ADSEmergencyReport

}

END	-- of ADSGGMessageSetVersion1