# FIS USER REQUIREMENTS

## Introduction

Note.— Requirements imposed on FIS-users interfacing with the FIS-ASEs are presented in this chapter.

General

### General Requirements

#### When a FIS-air-user invokes FIS-demand-contract request or FIS-update-contract request and requires a particular class of communication service, it shall set the ClassOfCommunicationService parameter to be the class of communication service it requires.

#### Note 1.— When a FIS-air-user invokes FIS-demand-contract request or FIS-update-contract request and does not provide the ClassOfCommunicationService parameter, this indicates no routing preference.

Note 2.— When a FIS-air-user specifies the ClassOfCommunication Service parameter and there is a FIS contract in place, the ClassOf CommunicationService parameter is ignored.

### Timing Requirements

#### Upon receipt of a FIS-ASE service indication that requires a response, the FIS-user shall invoke the FIS-ASE service response within 1 second.

### Error Handling Requirements

#### If a FIS-user has an unrecoverable system errorit shall:

- cease the operation of all contracts with peer systems which are affected by the error,

- for each affected peer system, invoke FIS-user-abort request.

If a FIS-user receives a FIS-provider-abort indication or a FIS-user-abort indication, it shall cease operation of all FIS contracts with the peer system to which the indication is related.

### Miscellaneous Air and Ground FIS Systems Requirements

#### With the exception of the FIS-user-abort and FIS-provider-abort, the FIS-user shall respond to FIS-ASE service indications and confirmations from the FIS-ASE in the order in which they are received.

### Miscellaneous Ground FIS System Requirements

#### The FIS-ground-user shall be capable of detecting that the content of the FIS request is not valid or that the requested FIS information is not available.

#### The FIS-ground-user shall be capable of detecting that it can not continue to provide updates of the requested FIS information.

#### The FIS-ground-user shall be able to support multiple concurrent FIS contracts with a given aircraft.

### ATIS Service Requirements

#### If only the arrival ATIS or only the departure ATIS is requested, and if the FIS-ground-user only provides combined ATIS, the FIS-ground-user shall send combined ATIS with a 'combined' indication.

#### If both arrival and departure ATIS are requested, the FIS-ground-user shall:

- send a combined ATIS with a 'combined' indication, if the combined ATIS is available, or

- send an arrival ATIS and a departure ATIS if both information are available but the combined ATIS is not available, or

- send the available ATIS with an indication that the other is not available, if only one is available.

Note.— The ATIS fields should be presented in the following order:

- Airport identification,

- Departure or arrival indicator,

- ATIS Code,

- Time of Observation, if present,

- Approach Type, if present,

- Runways In Use,

- RunwayArresting System, if present,

- RunwaySurfaceConditions, if present,

- RunwayBraking Action, if present,

- Holding Delay, if present,

- Transition Level, if present,

- Other Operational Information, if present,

- Surface Wind,

- Visibility,

- RVR, if present,

- Present Weather,

- Cloud Sky and Cover,

- Air Temperature,

- Dew Point Temperature,

- Altimeter Setting,

- Significant Met Information,

- Trend Type of Landing Forecast, if present, and

- Specific ATIS Instruction, if present.

## Establishment And Operation Of a FIS Demand Contract

Note 1.— When a FIS-air-user requires to establish a FIS demand contract with a FIS-ground-user it  invokes FIS-demand-contract request.

Note 2. — Only the FIS-air-user is capable of initiating the FIS-demand-contract service.

### The FIS-air-user shall use a value for the FISContractNumber parameter of the FIS-demand-contract request which is not already in used for another FIS contract.

### The FIS-ground-user shall use the value received in the FISContractNumber parameter of the FIS-demand-contract indication for the FISContractNumber parameter of any ground-initiated request or response related to that FIS demand contract.

### The same type of FIS information (i.e. ”ATIS” for CNS/ATM-1 package) shall be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-demand-contract indication, when the FIS-ground-user is able to accept the contract in full, then:

#### If the FIS-ground-user is not able to send the FIS report within 1 seconds, it shall invoke:

- the FIS-demand-contract response with the Result parameter set to abstract value ”positive acknowledgment”, and

- invoke the FIS-report request within 10 seconds.

#### If the FIS-ground-user is able to send the FIS report within 1 seconds, it shall invoke the FIS-demand-contract response with the following parameters:

- the Result parameter set to the abstract value ”accepted”, and

- the FIS report in the FIS Information parameter.

### Upon receipt of a FIS-demand-contract indication, if the FIS-ground-user cannot comply with the demand contract request, it shall reject the contract by invoking a FIS-demand-contract response with the following parameters:

- the abstract value "rejected" as Result parameter, and

- one of the following abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined", as RejectReason parameter.

### The FIS-air-user shall be prevent from invoking a FIS-demand-contract request before the very first request (either a FIS-demand-contract or a FIS-update-contract request) has been confirmed.

### The FIS-air-user shall be allowed to reuse the value of a FISContractNumber used in a FIS-demand-contract request for a new FIS contract:

- upon receipt of a FIS-demand-contract confirmation with a Result parameter containing the abstract value ”rejected” or ”accepted”,

- upon receipt of a FIS-report indication, a FIS-user-abort indication or a FIS-provider-abort indication, or

- after invocation of a FIS-user-abort request.

## Establishment and Operation of a FIS Update Contract

Note 1.— When an FIS-air-user requires to establish a FIS update contract with an FIS-ground-user it invokes the FIS-update-contract request.

Note 2. — Only the FIS-air-user is capable of initiating the FIS-update-contact service.

### The FIS-air-user shall use a value for the FISContractNumber parameter of the FIS-update-contract request which is not already affected to another FIS contract.

### The FIS-ground-user shall use the value received in the FISContractNumber parameter of the FIS-update-contract indication for the FISContractNumber parameter of any ground-initiated request or response related to that FIS update contract.

### The same type of FIS information (i.e. ”ATIS” for CNS/ATM-1 package) shall be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-update-contract indication, if the FIS-ground-user is able to accept the contract in full then:

#### If the FIS-ground-user is not able to send the first FIS-report within 1 seconds, it shall:

- invoke the FIS-update-contract response with the Result parameter set to the abstract value ”positive acknowledgment”, and

- invoke the FIS-report request within 10 seconds.

#### If the FIS-ground-user is able to send the FIS-report within 1 seconds, it shall invoke the FIS-update-contract response with the following parameters:

- the result parameter set to the abstract value ”accepted”, and

- the FIS report in the FIS Information parameter.

#### If the FIS-ground-user does not support the update-contract function but the requested FIS report is available, it shall invoke the FIS-update-contract response with the floowing parameters:

- the Result parameter set to the abstract value ”rejected”,

- the RejectReason parameter set to the abstract value ”update contract function not supported by the FIS-ground-user”, and

- the FIS report in the FISInformation parameter.

### Upon receipt of a FIS-update-contract indication, if the FIS-ground-user cannot comply with the update contract request, it shall reject the contract by invoking a FIS-update-contract response with the following parameters:

- the abstract value "rejected" as Result parameter, and

- one of the following abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined", as RejectReason parameter.

### The FIS-air-user shall be prevent from invoking a FIS-update-contract request before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-ground-user shall invoke the FIS-report request each time the FIS information requested in the FIS-update-contract indication is updated until such time as the contract is canceled or aborted.

### The FIS-air-user shall be allowed to reuse the value of a FISContractNumber used in a FIS-update-contract request for a new FIS contract:

- upon receipt of a FIS-update-contract confirmation with a Result parameter containing the abstract value ”rejected”,

- upon receipt of a FIS-user-abort indication or a FIS-provider-abort indication,

- upon receipt of a FIS-cancel-update confirmation or a FIS-cancel-contracts confirmation,

- upon receipt of a FIS-cancel-update indication, or

- after invocation of a FIS-user-abort request.

## Air And Ground Cancellation Of a Single FIS Update Contract

Note 1. — There is no provision for the cancellation of a single FIS demand contract.

Note 2.— Both FIS-air-user and FIS-ground-user are capable of initiating the FIS-cancel-update-contract service.

### If a FIS-user requires to cancel an active FIS update contract, it shall invoke the FIS-cancel-update-contract request with the FISContractNumber parameter set to contract number identifying an active FIS update contract.

### The FIS-air-user shall be prevent from invoking a FIS-cancel-update-contract request before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### Upon receipt of a FIS-cancel-update-contract indication, the FIS-user shall cancel the FIS update contract identified by the FISContractNumber parameter.

## Air cancellation of FIS contracts of the same type

Note.— Only the FIS-air-user is capable of initiating the FIS-cancel-contracts service.

### If a FIS-air-user requires to cancel all the FIS contracts of a given type or of several types (only ”ATIS” in CNS/ATM-1 Package), it shall invoke the FIS-cancel-contracts request with the FISServiceType parameter identifying the contract type(s).

### The FIS-air-user shall be prevent from invoking a FIS-cancel-contracts request before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-air-user shall invoke the FIS-cancel-contracts request for each ground system with which it has FIS contracts.

### When the FIS-ground-user receives a FIS-cancel-contracts indication, it shall cancel any FIS demand contract and FIS update contract for the type(s) identified in the indication (e.g. ATIS) in force with that aircraft.

## FIS-user-abort

Note 1.— Both FIS-air-user and FIS-ground-user are capable of initiating the FIS-user-abort service.

Note 2.— If an active FIS-user requires to stop brutally any activity of the current FIS-ASE with the peer FIS-ASE with a possible loss of FIS messages currently in transit in the communication system, it invokes the FIS-user-abort request.

Note 3.— After the FIS-user-abort request is invoked, this FIS-user is no more considered by the FIS-AE as an active user.