|  | ATNP/WG3/SG2 (Air/Ground Application) Proposed Version 3.0 (Brussels) 22 April 1996 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 2 (Air/Ground Applications)

Part 4: Flight lnformation Services Application SARPs

Prepared by:		Air/Ground Applications Subgroup

SUMMARY

This document is the draft of the FIS Application SARPs for the ATN CNS/ATM-1 Package.

CONFIGURATION SHEET

Title :			 Draft FIS SARPs for the ATN CNS/ATM-1 Package

Version :		 Proposed 3.0

Date:

Contact:		Direction Generale de l’Aviation Civile, Airservices Australia, Eurocontrol, Federal Aviation Administration, UK National Air Traffic Services

Status:			Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 1.0 | Banff Proposed Draft SARPs |  | 6/10/1995 |
| 1.1 | Brisbane Proposed Draft SARPs |  | 5/02/1996 |
|  | - new ASN.1 (input from ADSP/WGB) | Chapter 4 |  |
|  | - new D-ABORT service provided by SG3 | Chapters 4, 5, Appendix A |  |
|  | - dialogue kept open for subsequent FIS Contracts | Chapter 5, Appendix A |  |
|  | - suppression of the D-END collision | Chapter 5, Appendix A |  |
|  | - enhancements for the FIS-cancel-contracts (old FIS-cancel-all-contracts) | Chapters 3, 5 |  |
| 2.0 | Output Brisbane Draft SARPs |  | 15/02/1996 |
|  | - Add a reference to Sub-Volume 1 | § 1.1.5 |  |
|  | - Time Requirements rewritten | § 2.2 |  |
|  | - State tables moved from Annex A to Chapter 5 | Appendix A, Chapter 5 |  |
|  | - Annex C removed from the SARPs | Appendix C |  |
|  | - Deletion of “null value for QOS” | Table 6-1 |  |
|  | - AE Qualifier set to ”ATI” | § 6.4 |  |
|  | - drfis001: error in ASN.1 | Chapter 4 |  |
|  | - drfis002: use of the default value for the Class Of Communication Service parameter. | § 5.3, table 6-1 |  |
|  | - drfis003: use of the AUTOMATIC TAGS feature in the ASN.1 alphabetising ASN.1 types | Chapter 4 |  |
|  | - drfis004: inconsistencies in table 5-4 | table 5-4 |  |
| 3.0p | Input to ATNP/WG3 at Brussels |  | 22/04/1996 |
|  | - drfis003: tags back in the ASN.1 description | § 4.1 |  |
|  | - drfis005: acceptance and refusal procedures are shifted from the ATIS level to the FIS level | § 4.1 Chapter 5 |  |
|  | - drfis006: errors in the protocol description . air UC module - reception of [FISReject] -state A-PENDING . ‘FISCancelContractAccept’ used but not defined . t-inactivity timer not used by FIS-ground-ASE . useless state LI-END-R | § 5.3.7.6.1 chapter 5 § 5.3.8.8.1, 5.3.8.8.2, 5.3.8.8.3, 5.3.8.9.1 § 5.3.12.1, table 5-10 |  |
|  | - drfis 007: invalid ‘shall’ statements removed: . redundant requirements . ”the following shall apply/occur” requirements . requirements defining protocol machine states | § 7.2.1.1, 7.2.2.2 Chapters 5 and 7 5.3.x |  |
|  | - drfis008: definition of the FISContractNumber parameter | Tables 3-1, 3-2, 3-3, 3-5 §3.5.3.1, 3.6.3.1, 3.7.2.1, 3.9.2.1 |  |
|  | - drfis009: definition of CloudSkyCoverGroup | Chapter 4 |  |
|  | - drfis010: new values for class of communication service | § 3.5, 3.6, 6.2.2.3 |  |

Table Of Contents

1. APPLICATION OVERVIEW

1.1 Introduction

1.2 Application Functionality

1.3 Functional Descriptions

2. GENERAL REQUIREMENTS

2.1 Performance Requirements

2.2 Time Accuracy Requirements

2.3 Security Requirements

2.4 Backwards Compatibility Requirements

2.5 Error Processing Requirements

3. THE ABSTRACT SERVICE

3.1 Introduction

3.2 The FIS Functional Model

3.3 The FIS-ASE Abstract Service

3.4 Conventions

3.5 FIS-demand-contract Service

3.6 FIS-update-contract Service

3.7 FIS-report Service

3.8 FIS-cancel-contracts Service

3.9 FIS-cancel-update-contract Service

3.10 FIS-user-abort Service

3.11 FIS-provider-abort Service

4. FORMAL DEFINITIONS OF MESSAGES

4.1 FIS ASN.1 Abstract Syntax

5. PROTOCOL DEFINITION

5.1 Sequence Rules

5.2 FIS Service Provider Timers

5.3 FIS-ASE Protocol Description

5.4 Exception Handling

5.5 State Tables

6. COMMUNICATION REQUIREMENTS

6.1 Encoding Rules

6.2 Dialogue Service Requirements

6.3 FIS AE Control Function Requirements

7. FIS USER REQUIREMENTS

7.1 Introduction

7.2 General

7.3 Establishment And Operation Of a FIS Demand Contract

7.4 Establishment and Operation of a FIS Update Contract

7.5 Air And Ground Cancellation Of a Single FIS Update Contract

7.6 Air cancellation of FIS contracts of the same type

7.7 FIS-user-abort