SARPs Version Number	ADS 3.0

Defect Report Number	ADS-0040

Originator	Tom Kraft/Catherine Gandolfi/Rachel Daeschler

Originator’s Reference (where applicable)	REPORT.DOC 26 July 1996

Date	29 July 1996

Location in SARPs	2.2.1.4.1.1

Status (OPEN, CLOSED or REJECTED)	OPEN

Problem (optionally including proposed solution)

The parameter ranges and resolutions differ from those in the draft ICAO Manual of ATS Data Link Applications (section 4B.1: ADS Variables Range and Resolution). In particular: Aircraft Id, Altitude, Mach airspeed, Distance, Lateral deviation change.

Proposed solution:

Resolve issue of aircraft Id - which is 2-8 characters of IA5 text in the ICAO manual, and 24 bits in the draft SARPs.

Delete:

Altitude ::= INTEGER (-10..6640)

and replace with:

Altitude ::= INTEGER (-10..10000)

Delete:

Mach ::= INTEGER (0..4000)

and replace with:

Mach ::= INTEGER (500..4000)

Delete:

IntermediateIntent ::= SEQUENCE SIZE (0..7) OF SEQUENCE

{

distance		INTEGER (1..8000),

-- units = Nautical miles

-- range = 1 to 8000 nautical miles ...

and replace with:

IntermediateIntent ::= SEQUENCE SIZE (0..7) OF SEQUENCE

{

distance		INTEGER (1..2000),

-- units = Distance units

-- range = 1 to 2000 distance units ...

Delete:

LateralChange ::= INTEGER (0..255)

-- units = 0.125 Nautical miles

-- range = 0 nautical miles to 31.875 nautical miles

and replace with:

LateralChange ::= INTEGER (1..200)

-- units = Distance units

-- range = 1 to 200 distance units

Insert in 2.2.1.4.1.1. Note 1 a definition of distance units

Agreed Solution