SARPs Version Number 				ADS ICAO V1.0

Defect Report Number 				ADS-0077

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-ads-086

Date						24 January 1997

Location in SARPs

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				11 February 1997

Fixed in 					ICAO V1.1

Problem

Due to the fact that the D-END service is hidden to the ADS-users, there is a major problem when the ADS-user receives a termination service primitive (i.e. positive response for a demand-contract, non-compliant response, negative response, a report closing a demand contract, a cancel, a cancel all or a cancel emergency) of a Demand, Event or Periodic Contract and when there is no other contract in place. In all these cases, the ADS ASE performs a D-END service hidden to the ADS-users.

In fact, the ADS-ground-user can initiate a new ADS contract on receipt of the termination service primitive. In that case, the new contract can be rejected because the D-END sequence is in progress.

There are two possible solutions for the ASE:

- defer the delivery of the termination service primitive to the ADS-user after the effective release of the dialogue (i.e. on receipt of the D-END conf). If the D-END  service is disrupted by an abort, an ADS-provider-abort indication is delivered to the user.

- memorise the second ADS contract and wait for the termination of the D-END sequence to establish this contract. If the D-END service is disrupted by an abort, and ADS-provider-abort indication is delivered to the user.

Agreed Solution

Add the following text:

2.2.1.5.3.4.5	The ground HI module shall reject requests and responses, apart from ADS-user-abort request, when the ground LI module is in the LI-G-START state or the LI-G-END state.