SARPs Version Number 				ADS 3.0 /  28-06-1996

Defect Report Number 				ADS-0060

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-ads-054

Date						18th September 1996

Location in SARPs				2.2.1.3.4, 2.2.1.3.5, 2.2.1.3.6

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				18/12/96

Fixed in

Problem

ICAO Facility Designator missing in the initial ADS requests and indication.

In section 2.2.1.3.4 (demand-contract), 2.2.1.3.5 (event-contract) and 2.2.1.3.6 (periodic-contract), the ICAO Facility Designator parameter (i.e. the calling system identifier) is not present as parameter of the request and the indication.

As a consequence, the aircraft does not know with which ground system it has ADS contract. The aircraft need this information in case a 5th ground system tries to establish an ADS-contract. In that case, the Reason of the reject contains the list of the ICAO facility designator (ASN.1 type GroundSystemsUsingService).

The proposed solution is to add the ICAO Facility Designator parameter in the ADS-demand/event/periodic requests and indications.

Agreed Solution

This has been superceded by ADS-0057.