SARPs Version Number 				ADS 3.0 /  28-06-1996

Defect Report Number 				ADS-0071

Originator					TES (EUROCONTROL)

Originator's Reference (where applicable)		tes-ads-077

Date						12 December 1996

Location in SARPs				2.2.1.5.3.10

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				18/12/96

Fixed in 					ICAO V1.1

Problem

There is no action described in the ground ADS PC module (2.2.1.5.3.10) when a t-PC-1, t-PC-2 or t-PC-3 timeout occurs.

It is proposed to add the following requirement:

- Upon expiry of the t-PC-1 timer, t-PC-2 timer or t-PC-3 timer, the ground PC module shall:

- request the ground AB module to abort with reason timer-expiry, and

- enter the PC-G-IDLE state.

Agreed Solution

The proposed solution is agreed.