SARPs Version Number 				ADS ICAO V1.0

Defect Report Number 				ADS-0082

Originator					Jane Hamelink / Dung Nguyen

Originator's Reference (where applicable)

Date						11 February 1997

Location in SARPs				2.2.1.7.4.4.6

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				11 February 1997

Fixed in 					ICAO V1.1

Problem

The ground system does not know when the aircraft received the contract request. The ground system can only understand the report on the basis of the report timestamp.

Agreed Solution

Delete:

as measured from the time of arrival of the ADS-periodic-contract indication.

and replace with

as measured from the timestamp on the ADS-report.