SARPs Version Number 				ADS ICAO V1.0

Defect Report Number 				ADS-0084

Originator					Jane Hamelink / Dung Nguyen

Originator's Reference (where applicable)

Date						11 February 1997

Location in SARPs				2.2.1

Status (OPEN, CLOSED or REJECTED)		CLOSED

Date of Resolution				11 February 1997

Fixed in 					ICAO V1.1

Problem

When an aircraft receives an ADS-event-contract indication it is able to reject some parts of the contract (using a noncompliance notification). This may be because some of the equipment that detects the event is not working. If the equipment fails once the contract has been set up, there is no way for the aircraft to tell the ground system that it can no longer fulfill the contract.

Agreed Solution

The solution is to add a new event type that means "some of the aircraft equipment that detects events may not be operational". This is to be reported on all event contracts.

Add the following to the ASN.1 definition of EventTypeReported in 2.2.1.4:

ability-to-detect-events-impaired (12),

Add the following:

2.2.1.7.3.5.14	If the ability of the aircraft to detect the occurance of events changes during the event contract to the extent that it may effect the ability of the aircraft to meet the terms of the event contract, the ADS-air-user shall invoke ADS-report request including the ability-to-detect-events-impaired element in the report details parameter.