ADS Defect Report Log

Author:	T I Maude

Date:	16 September 1996

| DR Ref. | Status | Version | Summary |
| --- | --- | --- | --- |
| ADS-0001 | CLOSED | V2.0 | Section 7.1.1 - “shall then be prohibited from” can be rewritten in a better way. |
| ADS-0002 | CLOSED | V2.0 | Section 7.1.2 - Statements made are not testable. |
| ADS-0003 | CLOSED | V2.0 | There is no clear statement as to when to provide Mach or IAS in the Air vector. |
| ADS-0004 | CLOSED | V2.0 | Throughout section 7, the style is inconsistent with section 5, i.e. If .... then the XXXX shall a).... b)..... c) .... |
| ADS-0005 | CLOSED | V2.0 | Section 7.1.5.4 and 7.1.6.3 are tautological with the statements given in section 5 |
| ADS-0006 | CLOSED | V2.0 | 7.1.6.2 is system requirement, not an interoperability requirement |
| ADS-0007 | CLOSED | V2.0 | Throughout section 7, all statements that state “If the XXX requires to ...” are not testable. |
| ADS-0008 | CLOSED | V2.0 | Throughout section 7, all statements that state that the contract should be treated as if certain elements were not present in the case of a non-compliance notification, are not clear enough. |
| ADS-0009 | CLOSED | V2.0 | Throughout section 7, there are statements that state that a parameter should not be provided. The SARPs should only state what is provided, it is implicit that, where something is not stated, it is not provided. |
| ADS-0010 | CLOSED | V2.0 | Throughout section 7, there are statements that state that certain mandatory parameters shall be provided - this is tautological with the abstract service definition in section 3. |
| ADS-0011 | CLOSED | V2.0 | Statements in section 7.3 on how the event contract works (7.3.6.x) are not clear enough. |
| ADS-0012 | CLOSED | V2.0 | Section 7.3.7 - does not allow an event report to be sent as a baseline. |
| ADS-0013 | CLOSED | V2.0 | Throughout section 7 there are statements that say a contract is implicitly cancelled - this is not clear. |
| ADS-0014 | CLOSED | V2.0 | Section 7.5.1 implicitly contains two Shall statements. |
| ADS-0015 | CLOSED | V2.0 | Section 7 contains the concept of an Emergency Mode and an Emergency Contract - these two are confusing. |
| ADS-0016 | CLOSED | V2.0 | Section 7.7.4 is tautological with statements made in section 7.6. |
| ADS-0017 | CLOSED | V2.0 | There are no statements in section 7 concerning user reaction to aborts. |
| ADS-0018 | CLOSED | V2.0 | Section 6.2.2.3 - there are now two additional classes of communication service. |
| ADS-0019 | CLOSED | V2.0 | There is no requirement to have both flight-id and airframe-id as aircraft identifiers in Chapter 4. |
| ADS-0020 | CLOSED | V2.0 | In event reports - there needs to be an ASN.1 definition for a baseline. |
| ADS-0021 | CLOSED | V2.0 | In ASN.1 - it is not clear what the years 0..99 map onto. |
| ADS-0022 | CLOSED | V2.0 | AUTOMATIC TAGS in ASN.1 are not accepted by all compilers. |
| ADS-0023 | CLOSED | V2.0 | Some ASN.1 can be rationalised by making common parameters |
| ADS-0024 | CLOSED | V2.0 | Having the forward contract module in the same ASE as the air-ground parts is not a correct use of the upper layers model. It produces a number odd requirements in the SARPs. |
| ADS-0025 | CLOSED | V2.0 | In chapter 3, ADS-demand-contract, ADS-event-contract and ADS-periodic-contract, the fact that the response and confirmation are optional is not clear. |
| ADS-0026 | CLOSED | V2.0 | The ADS-end-forward service is confirmed. The confirmation adds no value. |
| ADS-0027 | CLOSED | V2.0 | Version numbers and titles of referenced documents have changed since last version. |
| ADS-0028 | CLOSED | V2.0 | A document should be referenced that explains traffic types. |
| ADS-0029 | CLOSED | V2.0 | Chapter 1 needs operational requirements to reference for the forwarding function. |
| ADS-0030 | CLOSED | V2.0 | Various places in chapter 1 are not clear enough. |
| ADS-0031 | CLOSED | V2.3 | Following the removal of the term “NavigationalIntent” in the ASN.1, there are now two definitions of “ShortTermIntent”. |
| ADS-0032 | CLOSED | V3.0 | 2.1.1.3.1.1 has the same meaning as 2.2.1.3.2.1; 2.1.2.3.1.1 has the same meaning as 2.2.2.3.2.1 |
| ADS-0033 | CLOSED | V3.0 | Note 1 in 2.2.1.4.1.1 contains two “shoulds” - which are not permitted in notes. |
| ADS-0034 | CLOSED | V3.0 | Several comments in the ASN.1 contains “shoulds” |
| ADS-0035 | CLOSED | V3.0 | The word “must” appears in comments in ASN.1 |
| ADS-0036 | CLOSED | V3.0 | Conditions C.1 and C.2 in 2.2.1.8.1.1 contain “shall” and “must”. |
| ADS-0037 | CLOSED | V3.0 | Incorrect reference is given to section 2, instead of section 2.2.1 |
| ADS-0038 | REJECTED | V3.0 | The following acronyms are not explained the first time they are used: ETA, PDU, PER, RER, FU, RF |
| ADS-0039 | CLOSED | V3.0 | There are references to “emergency mode” which no longer exists as a concept in ADS. |
| ADS-0040 | OPEN | V3.0 | The parameter ranges and resolutions differ from those in the draft ICAO Manual of ATS Data Link Applications |
| ADS-0041 | CLOSED | V3.0 | There are no Abort timer stop events in Table 2.2.1.5-1 |
| ADS-0042 | CLOSED | V3.0 | Reference to Dialogue service is incorrect. |
| ADS-0043 | CLOSED | V3.0 | The reference to ASN.1 is incorrect. |
| ADS-0044 | CLOSED | V3.0 | The definition of AE-Qualifier serves no purpose. |
| ADS-0045 | CLOSED | V3.0 | The phrase “of functionally equivalent means to produce the same result” is superfluous. |
| ADS-0046 | CLOSED | V3.0 | Actions on unrecoverable system error should be recommendations - not requirements. |
| ADS-0047 | CLOSED | V3.0 | Incorrect reference. |
| ADS-0048 | CLOSED | V3.0 | The t-LI-1 timer is not set or tested. |
| ADS-0049 | CLOSED | V3.0 | The t-EM-4 timer on Figure 2.2.1.5-28 should be t-EM-3. |
| ADS-0050 | CLOSED | V3.0 | There is no figure to indicate a crossed ADS-cancel-emergency and ADS-modify-emergency-contract. |
| ADS-0051 | CLOSED | V3.0 | There is no stated requirement for the ASE to be able to encode and decode APDUs. |
| ADS-0052 | CLOSED | V3.0 | Reference to “user” instead of “ADS-user”. |
| ADS-0053 | CLOSED | V3.0 | There is no definition of the term “Abstract Syntax”. |
| ADS-0054 | CLOSED | V3.0 | The statement that the ASE will process primitives in the order in which they are received is superfluous, since the ULA requires this. |
| ADS-0055 | CLOSED | V3.0 | There are two stated requirements for the abstract service. |
| ADS-0056 | CLOSED | V3.0 | The definition of an Invalid PDU and Sequence error is not consistent with the other air-ground application SARPs. |