ATN AIR/GROUND APPLICATIONS

DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CM-025 |  |
| --- | --- | --- | --- |
| SARPs affected: | CM | SARPs Version/Date | 3.0 (21 June 1996) |
| Originator Name: | Thomson |  |  |
| Originator Reference: | P/C 24 |  |  |
| Date Raised: | 9 August 96 |  |  |
| Location of Defect (including Section Number): 2.1.5.4.2.1 |  |  |  |
| Summary of the Defect: There is no way to guarantee that the steps specified for an Unrecoverable System Error can be completed. This is due to the unknown level of failure in the system. |  |  |  |
| Proposed solution or assumptions made (if any): #### It is proposed to make the exception handling for Unrecoverable System Error into a recommendation instead of a requirement: #### Recommendation. — If a CM-ASE has an unrecoverable system error, the CM-ASE should: - stop all timers, - if the CM-ASE is a CM-air-ASE, create a CMAircraftMessage APDU with a cmAbortReason [undefined-error] APDU message element, - if the CM-ASE is a CM-ground-ASE, create a CMGroundMessage APDU with a cmAbortReason [undefined-error] APDU message element, - invoke D-ABORT request with: - the abstract value “provider” as the D-ABORT Originator parameter value, and - the APDU as the D-ABORT User Data parameter value, - if the CM-user is an active user, invoke CM-provider-abort service indication with the abstract value “undefined-error” as the CM-provider-abort Reason parameter value, and - enter the IDLE state. |  |  |  |
| Editors Comment: |  |  |  |
| Date of Resolution: |  |  |  |