DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CPDLC-v3-1 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | R Beppel/M. Anderson | CSC |  |
| Originator Reference: |  |  |  |
| Date Raised: | July 8 96 |  |  |
| Location of Defect (including Section Number: 2.3.7.5.1.2.3, 2.3.7.6.1 |  |  |  |
| Summary of the Defect: The user requirement allows both the air user and the ground user to initiate the basic CPDLC service. The requirements in the referenced paragraphs discuss the prevention of multiple dialogues between two users when a dialogue is already in existence. However, if both the air user and ground user initiate a start service request simultaneously then the actions discussed in these paragraphs will cause both connections to b terminated. This occurs because each initiator will view their dialogue as the first dialogue being established. This view will ultimately cause the initiator to issue abort upon receipt of the resulting start indications. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: We recognized that both the air and ground could both initiate CPDLC at the same time. The situation you describe could occur. We considered this a tail condition. This situation will be addressed in guidance material. Procedural recommendations may have to be put in place to cover this. |  |  |  |
| Date of Resolution: | 22 August 1996 No change to SARPs/CLOSED |  |  |