DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | v3-32 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | Thomson CSF |  |  |
| Originator Reference: | External P/C Report 20 |  |  |
| Date Raised: | 22 August 1996 | C-5-0005, |  |
| Location of Defect (including Section Number: Chapter 5 |  |  |  |
| Summary of the Defect: Clarification of “not permitted” and “invalid” PDU required. This specification refers to the section 2.3.5.4 (exception handling procedures) when, upon receipt of a PDU, no actions are described for the arrival of that PDU within a CPDLC-air-ASE specific state. The section 2.3.5.4.4 seems to be more relevant for the description of these exceptions. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: #### Agreed see modified text below: #### 2.3.5.3.2.2 Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CPDLC-air-ASE is in a specific state, then that PDU is considered not permitted, and exception handling procedures as described in 2.3.5.4.4 shall apply. #### 2.3.5.3.2.3 If a PDU is received that cannot be decoded, then exception handling procedures as described in 2.3.5.4.3 for invalid PDU shall apply. #### 2.3.5.3.2.4 If a PDU is not received when one is required, then exception handling as described in 2.3.5.4.3 shall apply. 2.3.5.4.4.1 If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a PDU for which action is described within a given state in 2.3.5.3, the CPDLC-air-ASE shall: #### 2.3.5.5.2.2 Upon receipt of a PDU, if no actions are described for the arrival of that PDU when a CPDLC-ground-ASE is in a specific state, then that PDU is considered not permitted, and exception handling procedures as described in 2.3.5.6.4 shall apply. #### 2.3.5.5.2.3 If a PDU is received that cannot be decoded, then exception handling procedures as described in 2.3.5.6.3 for invalid PDU shall apply. #### 2.3.5.5.2.4 If a PDU is not received when one is required, then exception handling as described in 2.3.5.6.3 shall apply. #### 2.3.5.6.4.1 If the User Data parameter of a D-START indication or D-DATA indication is a valid PDU, but is not a permitted PDU for which action is described within a given state as defined in 2.3.5.5, the CPDLC-ground-ASE shall: #### 2.3.5.6.4.2 If the User Data parameter of a D-START confirmation is a valid PDU, but is not a permitted PDU for which action is described within a given state as defined in 2.3.5.5, the CPDLC-ground-ASE shall: |  |  |  |
| Date of Resolution: | 30 September 1996, Closed, Accepted |  |  |