DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CPDLC-v3-4 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | D. Van Roosbroek |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | June 21 1996 |  |  |
| Location of Defect (including Section Number: 2.3.7.4.10.4 Insufficient Storage |  |  |  |
| Summary of the Defect: This is a special case of a general error type which is better categorised as “Insufficient Resources” to cover things like the limited number of lines on a display source. |  |  |  |
| Proposed solution or assumptions made (if any): Redefine this error as “Insufficient Resources” |  |  |  |
| Editors Comment: The name “Insufficient Storage comes from RTCA 169. We have changes the SARPs to “insufficient resources” In chapter 4: ErrorInformation ::= ENUMERATED { unrecognizedMsgReferenceNumber (0), endServiceWithPendingMsgs (1), logicalAcknowledgmentNotAccepted (2), moreThanOneNextDataAuthorityElement (3), insufficientResources (4), ... } In chapter 7: #### 2.3.7.3.10.4 Insufficient Resources ##### 2.3.7.3.10.4.1 If the CPDC-user receives a message and has insufficient resources to handle the message, the CPDLC-user shall the CPDLC-user shall: - invoke CPDLC-message request with the ERROR [errorinformation] message element with the [insufficientResources] value as the CPDLC Message parameter, and - disregard the received message. |  |  |  |
| Date of Resolution: | 28 August 1996, Accepted, Closed |  |  |