DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | CPDLC-v3-9 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | V3 |
| Originator Name: | N. Goodacre, F Picard | STEL, CENA |  |
| Originator Reference: |  |  |  |
| Date Raised: | 28 August 1996 |  |  |
| Location of Defect (including Section Number: Chapter 4 Norm , Chapter 5 Frederic |  |  |  |
| Summary of the Defect: The ASN.1 modules in all the application SARPs are incomplete. Each ASN.1 module defines two or more groups of messages for the application with a CHOICE identifier for each group. Without a CHOICE identifier at a higher level, the ASN.1 definitions do not enable a PER encoder/decoder to distinguish the group to which a given message belongs. |  |  |  |
| Proposed solution or assumptions made (if any): Add a high level CHOICE. |  |  |  |
| Editors Comment: This defect was discussed at the August Silver Spring WG3-SG2 meeting and the application SARPs had the following sentence added in the beginning of 2.3.4, instead of adding a high level CHOICE. ## Encoding/Decoding Rules ### A CPDLC-air-ASE shall be capable of encoding AircraftPDUs APDUs and decoding GroundPDUs APDUs. ### A CPDLC-ground-ASE shall be capable of encoding and decoding GroundPDUs APDUs and decoding AircraftPDUs APDUs. ### In addition clarification is required in chapter 5 a ground system must determine if a D-START indication is received form another ground system. The following paragraphs have been modified as follows: 2.3.5.5.2.1 Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address and the D-START User Data parameter contains an AircraftPDUs [StartDownMessage] APDU with the APDU-element mode “cpdlc”, the CPDLC-ground-ASE shall #### 2.3.5.5.2.2 Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is a 24 bit Aircraft Address and the D-START User Data parameter contains an AircraftPDUs [StartDownMessage]APDU with the APDU-element mode “dsc”, the CPDLC-ground-ASE shall: #### 2.3.5.5.2.3 Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is an 8 character ICAO Facility Designator and the D-START User Data parameter contains a GroundPDUs [ATCForwardMessage] APDU and the CPDLC-ground-ASE supports the CPDLC-forward service, the CPDLC-ground-ASE shall: ### 2.3.5.5.4.2 Upon receipt of a D-START indication, if the CPDLC-ground-ASE is in the IDLE state, and the abstract value of the D-START Calling Peer ID parameter is an 8 character ICAO Facility Designator and the D-START User Data parameter contains a GroundPDUs APDU and the APDU element is an ATCForwardMessage and the CPDLC-ground-ASE does not support the CPDLC-forward service, the CPDLC-ground-ASE shall: . |  |  |  |
| Date of Resolution: | August 22 1996 |  |  |