DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-1 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | TES (EUROCONTROL) | TES-cpc-053 |  |
| Originator Reference: |  |  |  |
| Date Raised: | 10 Dec 96 |  |  |
| Location of Defect (including Section Number: 2.3.3.3.7 and 2.3.3.4.7 |  |  |  |
| Summary of the Defect: 1. CPDLC SARPs are different than the other 3 applications SARPs that they require an Indication of the Class of Communication Service parameter in the CPDLC-start and DSC-start services. Is this correct, and if so include an explanation. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: Because the air can initiate a CPDLC or DSC dialogue, it is possible that the user selects or the communication system selects a Class of Communication that does not meet the ground requirements. Thus the ground system needs this piece of information as a part of determining whether or not to accept a requested CPDLC or DSC dialogue. Explanatory comments has been added to both the CPDLC-start (2.3.3.3.7) and DSC-start service (2.3.3.4.7) as follows: Note 2.— This parameter is used by the CPDLC-ground-user to determine if the Class of Communication value is acceptable for the establishment of a CPDLC dialogue. In addition, because a U to the left of an M in the CPDLC-start and DSC-start Class of Communication Service parameter is an unusual configuration the following note and requirement have been added to the CPDLC-start and DSC-start services Note 3.— The parameter indicated to the peer user is that provided by the user if spcified by the user, else the parameter indicated to the peer user is that what the dialogue provider service supplies. When this parameter is provided by the user, the same value shall be indicated to the peer user. |  |  |  |
| Date of Resolution: | 5 Feb 1997 |  |  |