DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-11 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | ADSP |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, Chapter 7 |  |  |  |
| Summary of the Defect: It is an operational requirement that in spite of open messages End service must be able to work and an abort is not sufficient. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. An End is no longer an ERROR when there are pending messages so ASN.1 has been modified: ErrorInformation ::= ENUMERATED { unrecognizedMsgReferenceNumber (0), endServiceWithPendingMsgs (1), logicalAcknowledgmentNotAccepted (21), moreThanOneNextDataAuthorityElement (3), insufficientResources (42), invalidMessageElementCombination (3), invalidMessageElement (4), ... } See attached pages for Revised chapter 7 CPDLC-end and DSC-end |  |  |  |
| Date of Resolution: |  |  |  |