DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-17 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Jane Hamelink |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, chapter 7 |  |  |  |
| Summary of the Defect: To be consistent if message element are only allowed to be sent single an error should be generated if received in combination. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. New ERROR in chapter 4: ErrorInformation ::= ENUMERATED { unrecognizedMsgReferenceNumber (0), endServiceWithPendingMsgs (1), logicalAcknowledgmentNotAccepted (21), moreThanOneNextDataAuthorityElement (3), insufficientResources (42), invalidMessageElementCombination (3), invalidMessageElement (4), ... } 2. New section added in chapter 7 from ground: 2.3.7.3.11.5 Invalid Message Element Combination 2.3.7.3.11.5.1 If a message is received containing either: - an ERROR [errorinformation] message element in combination with any other message element in a single message, - a LOGICAL ACKNOWLEDGMENT message element in combination with any other message element in a single message, or - a NEXT DATA AUTHORITY [facility] message element in combination with any other message element in a single message, the CPDLC-user shall invoke the CPDLC-message request with the ERROR [errorinformation] message element with the [invalidMessageElementCombination] value as the CPDLC Message parameter, and disregard the received message. |  |  |  |
| Date of Resolution: |  |  |  |