DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-5 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | Jane Hamelink |  |  |
| Originator Reference: |  |  |  |
| Date Raised: | Feb 97 |  |  |
| Location of Defect (including Section Number: Chapter 4, Chapter 7 |  |  |  |
| Summary of the Defect: Based on ADSP Manual changes missed in SARPs |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. UM96 ASN.1 in chapter 4 modified to be consistent with chapter 7 and the ADSP Manual -- FLY CONTINUE PRESENT HEADING 2. DM28 ASN.1 in chapter 4 modified to be consistent with chapter 7 and the ADSP Manual -- LEAVING [level] Urg(N)/Alr(L)/Resp(YN) dM28Level [28] Level, 3. DM 37 ASN.1 in chapter 4 and chapter 7 modified to be consistent with the ADSP Manual -- LEVELMAINTAINING [level] Urg(N)/Alr(M)/Resp(N) dM37Level [37] Level, 4. Facility Identification definition ASN.1 in chapter 4 modified to be consistent with ADSP Manual FacilityIdentification ::= CHOICE { facilityDesignation [0] FacilityDesignatorDesignation, facilityName [1] FacilityName } 5. UNABLE is a valid R reponse 2.3.7.3.9.14 When the CPDLC-air-user receives a message with a R RESP attribute, the only permitted responses shall be messages that contain a LOGICAL ACKNOWLEDGMENT (if required), STANDBY, ROGER, UNABLE or ERROR message element. 2.3.7.3.9.15 When the CPDLC-air-user receives a message with a R RESP attribute, the closure response message shall contain at least a ROGER, UNABLE or ERROR message element. 6. UM Message 133 changed from REPORT LEVEL to REPORT PRESENT LEVEL. Chapter 7 is correct 7. The message intent column, for DM nessages 59, 60, 61, 80 have the words “due to an emergency”. added to the end of echd intent. Range and resolution for true speed have been aligned with ADSP. SpeedTrue ::= INTEGER (7..700..2000) -- unit = Knots, Range (70..7000..2000), resolution = 10 SpeedTrueMetric ::= INTEGER (10..1370..4000) -- unit = Kilometers/Hour, Range (100..13700..4000), resolution = 10 8. All occurrances of ICAO Facility Designator changed to Facility Designation and Aircraft Identifier to Aircraft Address (chapters 3, 5, 7) Chapter 4 done previously. 9. Chapter 7 message table shown below: |  |  |  |
| Date of Resolution: | 5 Feb 1997 |  |  |

| 37 | Notification that the aircraft is maintaining the specified level. | LEVELMAINTAINING [level] | N | L | N |
| --- | --- | --- | --- | --- | --- |

SpeedTrue ::= INTEGER (7..700..2000)

-- unit = Knots, Range (70..7000..2000), resolution = 10

SpeedTrueMetric ::= INTEGER (10..1370..4000)

-- unit = Kilometers/Hour, Range (100..13700..4000), resolution = 10