DEFECT REPORT ON SARPs

| Defect Number (to be supplied by SARPs editor): |  | IV1.0_CPC-6 |  |
| --- | --- | --- | --- |
| SARPs affected: | CPDLC | SARPs Version/Date | ATNP v1 |
| Originator Name: | EOLIA | DRCPD007 |  |
| Originator Reference: |  |  |  |
| Date Raised: | Dec 1996 |  |  |
| Location of Defect (including Section Number: Chapter 4, Chapter 7 |  |  |  |
| Summary of the Defect: Need the capability for to cancel a NDA, if one has been specified. |  |  |  |
| Proposed solution or assumptions made (if any): |  |  |  |
| Editors Comment: 1. The ASN.1 has been modified to allow the NEXT DATA AUTHORITY message element (UM160) to be sent with out specifying an ICAO facility. This has been acconplished with the definition of a new ASN.1 variable: Facility. NEXT DATA AUTHORITY [facilitydesignation] Urg(L)/Alr(N)/Resp(N) uM160FacilityDesignation [160] FacilityDesignation, 2. The ASN.1 has been modified to allow a new variable where the facility need not be specified: Facility ::= CHOICE { noFacility [0] NULL, facilityDesignation [1] FacilityDesignation } 3. Chapter 7 NDA references modified: 2.3.7.2.3.3 The NEXT DATA AUTHORITY [facility designation] message element (uplink message element 160) shall be sent only as a single message element CPDLC message. 4. Cancelling action (new section): 2.3.7.4.3.1.3 If a CPDLC-message indication is received from the Current Data Authority containing only the uplink message element NEXT DATA AUTHORITY, indicating a “NULL” for the Next Data Authority the CPDLC-air-user shall do the following in the order listed: a) Check if there is a Next Data Authority already established; b) if there is, invoke CPDLC-user-abort request with the established Next Data Authority with the Reason parameter set to CPDLCUserAbortReason value [no-longer-next-data-authority], and c) cancel any existing Next Data Authority designation. Uplink table entry 160 is modified as shown below: |  |  |  |
| Date of Resolution: | 5 Feb 1997 |  |  |

| 160 | Notification to the avionics that the next data authority is the specified ATSU. | NEXT DATA AUTHORITY [facility designation] | L | N | N |
| --- | --- | --- | --- | --- | --- |