|  | ATNP/WG3/SG2 (Air/Ground Application) Version: 2.0 15th February 1996 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Sub Group 2 (Air/Ground Applications)

Draft ADS SARPs for the ATN CNS/ATM-1 Package

Part 2: ADS Application SARPs

Prepared by:		Air/Ground Applications Subgroup

SUMMARY

This document is the draft of the Automatic Dependent Surveillance application SARPs for the ATN CNS/ATM-1 Package.

CONFIGURATION SHEET

Title :		Draft SARPs for the ATN CNS/ATM-1 Package

Part 2: ADS Application SARPs

Version :	2.0

Date:

Contact:	Airservices Australia, Eurocontrol, Federal Aviation Adminstration, Direction Generale de l’Aviation Civile

Status:		Draft

Change History :

| Version | Description | Affected Parts | Date |
| --- | --- | --- | --- |
| 1.0 | Banff Proposed Draft SARPs. |  | 6 Oct 1995 |
| 1.1 | Input to ATNP/WG3 at Brisbane |  | 5 Feb 1996 |
| 2.0 | Output Brisbane Draft SARPs |  | 15 Feb 1996 |
|  | - Deletion of “null value for QOS” ............................................. | Table 6-1 |  |
|  | - Time requirements rewritten...................................................... | Chapters 2 - § 2.2 |  |
|  | - Modification of the ASN.1......................................................... - Use of the AUTOMATIC TAGS feature | Chapter 4 - § 4.1 |  |
|  | - Add ACAS and RER abbreviations, remove TCAS | § 1.1.4.1.1 |  |
|  | - Add a reference to Sub-Volume 1............................................... | Chapter 1 - § 1.1.5 |  |
|  | - Ground forwarding capability of ADS data................................ - description of the new function................................. - FD8 and FD9 added.................................................. - definition of a forward contract................................. - description of new services ....................................... - ADS-start-forward....................................... - ADS-forward-report.................................... - ADS-end-forward........................................ - impact on the other services - ADS-user-abort........................................... - ADS-provider-abort..................................... - impact on ASN.1...................................................... - Downlink PDUs become Aircraft PDUs - Uplink PDUs become Ground PDUs - 4 new PDUs are added - new figures............................................................... - ADS-start-forward and ADS-forward-report - ADS-end-forward........................................ - abort a forward contract............................... - add 2 new timers t-FC-1 and t-FC-2.......................... - ADS-ASE functional model: new FC module............ - HI module modified to handle new ADS services...... - state tables added for Ground ADS FC Module......... - AB module modified to handle request to abort from the FC module.......................................................... - Ground ADS LI module to act as a receptor side (D-START ind and D-END ind) and to handle new PDUs........................................................................ | Chapters 1, 3, 4, 5, 7 § 1.1.2.2 § 1.3.9, 1.3.10 § 1.1.4.2.1 § 3.3.2 § 3.14 § 3.15 § 3.16 § 3.17 § 3.18 § 4.1 Chapter 5 § 5.1.10 § 5.1.11 § 5.1.12 § 5.2 Table 5-1 § 5.3.1.1 Figure 5-34 § 5.3.2 § 5.3.12 § 5.3.15 § 5.3.16 |  |
|  | - State Tables moved from Appendix A to chapter 5..................... - Description of the contents of chapter 5..................... | Appendix A and Chap. 5 § 1.1.3.5 |  |

Table Of Contents

1. APPLICATION OVERVIEW	1-1

1.1 Introduction	1-1

1.2 Application Functionality	1-2

1.3 Functional Descriptions

2. GENERAL REQUIREMENTS

2.1 Performance Requirements

2.2 Time Accuracy Requirements

2.3 Security Requirements

2.4 Backwards Compatibility Requirements

2.5 Error Processing Requirements

3. THE ABSTRACT SERVICE

3.1 Introduction

3.2 The ADS Functional Model

3.3 The ADS-ASE Abstract Service

3.4 Conventions

3.5 ADS-demand-contract Service

3.6 ADS-event-contract Service

3.7 ADS-periodic-contract Service

3.8 ADS-report Service

3.9 ADS-cancel Service

3.10 ADS-cancel-all-contracts Service

3.11 ADS-emergency-report Service

3.12 ADS-modify-emergency-contract Service

3.13 ADS-cancel-emergency Service

3.14 ADS-start-forward

3.15 ADS-forward-report Service

3.16 ADS-end-forward

3.17 ADS-user-abort Service

3.18 ADS-provider-abort Service

4. FORMAL DEFINITIONS OF MESSAGES

4.1 ASN.1

5. PROTOCOL DEFINITION

5.1 Sequence Rules

5.2 ADS Service Provider Timers

5.3 ADS-ASE Protocol Description

5.4 Exception Handling

5.5 ADS ASE STATE TABLES

6. COMMUNICATION REQUIREMENTS

6.1 Encoding Rules

6.2 Dialogue Service Requirements

6.3 ADS AE Control Function Requirements

7. ADS USER REQUIREMENTS

7.1 General

7.2 Establishment and operation of a Demand Contract

7.3 Establishment and operation of an Event Contract

7.4 Establishment and operation of a Periodic Contract

7.5 Cancellation of Contracts

7.6 Establishment and Operation of Emergency Mode

7.7 Modifying an Emergency Contract

7.8 Cancellation of Emergency Mode

7.9 Establishment and operation of a Forward Contract