# FORMAL DEFINITIONS OF MESSAGES

## ASN.1

### The abstract syntax of the ADS protocol data units shall  ([52])  comply with the description contained in the ASN.1 module ADSMessageSetVersion1 (conforming to [3]), as defined in this section.

Note 1.— Different states have different requirements with respect to the units of measurement of some of the fields in the PDUs. The units described below have been defined to meet these differing requirements. Conversion to the units should be made in the transmitting application and conversion from the units should be made at the receiving application. The new units have been defined to be midway between the minimum resolution required by the two units used by different states.

Unit of altitude = 1 alt-unit = 3.02400 metres = 9.92126 feet (midway between 3 metres and 10 feet)

To convert from alt-unit to metres, multiply by	3.02400

To convert from alt-unit to feet, multiply by		9.92126

To convert from meters to alt-units, multipy by	0.33069

To convert from feet to alt-units, multipy by		0.10079

Unit of speed = 1 speed-unit = 0.9633 km/hour = 0.51980 knots (midway between half a knot and one kilometre per hour)

To convert from speed-unit to km/hour, multiply by	0.96330

To convert from speed-unit to knots, multiply by	0.51980

To convert from km/hour to speed-units, multipy by	1.03801

To convert from knots to speed-units, multipy by	1.92382

Note 2.— Where units indicate directional information, the value is given relative to true North. If magnetic information is required this will be a matter for local ground implemenation.

ADSMessageSetVersion1 DEFINITIONS AUTOMATIC TAGS::=

BEGIN

-- ------------------------------------------------------------------------------------------------------------------------

-- Aircraft-generated and Ground-generated Message Choice

-- ------------------------------------------------------------------------------------------------------------------------

ADSAircraftPDUs :: = CHOICE

{

aDS-cancel-emergency-PDU			NULL,

aDS-demand-report-PDU				ADSDemandReport,

aDS-emergency-report-PDU			ADSEmergency,

aDS-event-report-PDU				ADSEventReport,

aDS-negative-acknowledgement-PDU		NegativeAcknowledgement,

aDS-noncompliance-notification-PDU		NoncomplianceNotification,

aDS-periodic-report-PDU				ADSPeriodicReport,

aDS-positive-acknowledgement-PDU		PositiveAcknowledgement,

aDS-provider-abort-PDU				AbortReason

...

}

ADSGroundPDUs ::= CHOICE

{

aDS-cancel-all contracts-PDU			NULL,

aDS-cancel-contract-PDU				CancelContract,

aDS-cancel-emergency-acknowledgement-PDU	NULL,

aDS-demand-contract-PDU			DemandContract,

aDS-event-contract-PDU				EventContract,

aDS-modify-emergency-contract-PDU		ModifyEmergency,

aDS-periodic-contract-PDU			PeriodicContract,

aDS-forward-contract-PDU			ForwardContract,

aDS-forward-contract-response-PDU		ForwardResponse

aDS-forwarded-report-PDU			ADSForwardedReport,

aDS-end-forward-service-PDU			NULL,

aDS-provider-abort-PDU				AbortReason

...

}

-- ------------------------------------------------------------------------------------------------------------------------

-- Ground-generated and Aircraft-generated message components - Protocol Data Units

-- ------------------------------------------------------------------------------------------------------------------------

AbortReason  ::= ENUMERATED

{

communications-service-failure (0),

unrecoverable-system-error (1),

invalid-PDU (2),

sequence-error (3),

timer-expiry (4),

cannot-establish-contact (5),

undefined-error (6)

...

}

ADSDemandReport ::= SEQUENCE

{

report					ADSReport,

positive-acknowledgement		NULL			OPTIONAL

...

}

ADSEmergency ::= SEQUENCE

{

emergency-report				ADSEmergencyReport,

positive-acknowledgement		NULL			OPTIONAL

...

}

ADSEventReport ::= SEQUENCE

{

event-type				EventType,

report					ADSReport,

positive-acknowledgement		NULL			OPTIONAL

...

}

ADSPeriodicReport ::= SEQUENCE

{

report					ADSReport,

positive-acknowledgement		NULL			OPTIONAL

...

}

CancelContract ::= ENUMERATED

{

event-contract (0),

periodic-contract (1)

...

}

DemandContract ::= SEQUENCE

{

flight-id					NULL			OPTIONAL,

projected-profile				NULL			OPTIONAL,

ground-vector				NULL			OPTIONAL,

air-vector				NULL			OPTIONAL,

weather					NULL			OPTIONAL,

airframe-id				NULL			OPTIONAL,

navigational-intent			ProjectionTime		OPTIONAL,

extended-projected-profile			ExtendedProjectedProfileRequest

OPTIONAL

...

}

EventContract ::= SEQUENCE

{

-- At least one of the following optional elements must be present

lateral-deviation-change			LateralChange		OPTIONAL,

vertical-rate-change			VerticalRateChange		OPTIONAL,

altitude-range				AltitudeRange		OPTIONAL,

way-point-change			NULL			OPTIONAL,

air-speed-change				AirSpeedChange		OPTIONAL,

ground-speed-change			GroundSpeedChange		OPTIONAL,

heading-change				AngleChange			OPTIONAL,

extended-projected-profile-change		ExtendedProjectedProfileRequest

OPTIONAL,

fom-change				NULL			OPTIONAL,

track-angle-change			AngleChange			OPTIONAL,

altitude-change				AltitudeChange		OPTIONAL

...

}

ModifyEmergency ::= ReportingInterval

NegativeAcknowledgement :: = SEQUENCE

{

request-type				RequestType,

reason					Reason

}

NoncomplianceNotification ::= CHOICE

{

demand-ncn				SET OF ReportType,

event-ncn				SET OF EventType,

periodic-ncn				SET OF ReportTypeAndPeriod

...

}

PeriodicContract ::= SEQUENCE

{

reporting-interval			ReportingInterval

DEFAULT {minute-scale 5},

flight-id-modulus				Modulus			OPTIONAL,

projected-profile-modulus			Modulus			OPTIONAL,

ground-vector-modulus			Modulus			OPTIONAL,

air-vector-modulus			Modulus			OPTIONAL,

weather-modulus				Modulus			OPTIONAL,

airframe-id-modulus			Modulus			OPTIONAL,

navigational-intent			NavigationalIntent		OPTIONAL,

extended-projected-profile-modulus	ExtendedProjectedProfileModulus

OPTIONAL

...

}

PositiveAcknowledgement ::= RequestType

ForwardContract ::= SEQUENCE

{

aDSforwardedReport	ADSForwardedReport OPTIONAL

}

ADSForwardedReport ::= SEQUENCE

{

aircraftIdentifier		OCTET STRING (SIZE(3)),

forwardedADSReport	ForwardedReport

}

ForwardedReport	::= CHOICE

{

aDSDemandReport	ADSReport,

aDSPeriodicReport	ADSReport,

aDSEventReport		SEQUENCE

{

event-type	EventType,

aDSReport	ADSReport

},

aDSEmergencyReport	ADSEmergencyReport

}

ForwardResponse ::= CHOICE

{

accepted				(0),

forwardFunctionNotSupported	(1)

}

-- ------------------------------------------------------------------------------------------------------------------------

-- Reports and their components

-- ------------------------------------------------------------------------------------------------------------------------

ADSEmergencyReport :: = SEQUENCE

{

position					Position,

time-stamp				DateTimeGroup,

fom					FigureOfMerit,

flight-id					FlightID			OPTIONAL,

ground-vector				GroundVector		OPTIONAL

}

ADSReport :: = SEQUENCE

{

position					Position,

time-stamp				DateTimeGroup,

fom					FigureOfMerit,

flight-id					FlightID			OPTIONAL,

projected-profile				ProjectedProfile		OPTIONAL,

ground-vector				GroundVector		OPTIONAL,

air-vector				AirVector			OPTIONAL,

weather					Weather			OPTIONAL,

airframe-id				AirframeID			OPTIONAL,

short-term-intent				ShortTermIntent		OPTIONAL,

extended-projected-profile			ExtendedProjectedProfile	OPTIONAL

...

}

AirframeID :: = BIT STRING (SIZE (24))

-- 24 bit ICAO airframe identifier

AirVector :: = SEQUENCE

{

heading					INTEGER (1..3600) 		OPTIONAL,

-- units = 0.1 degrees true North

-- range = 0.1 degrees to 360 degrees

air-speed				AirSpeed 			OPTIONAL,

vertical-rate				INTEGER (-3310..3310)		OPTIONAL

-- units = alt-units per minute (see Note 1)

-- range = -3310 alt-units per minute to +3310 alt-units per minutes

}

-- When a report provides an AirVector it should provide all the information

-- that it has available at the time.

AirSpeed ::= CHOICE

{

mach					INTEGER (0..4000),

-- units = mach 0.001

-- range = 0 mach to 4 mach

ias					INTEGER (-110..2100),

-- units = speed-units (see Note 1)

-- range = -110 speed-units to +2100 speed-units

mach-and-ias				SEQUENCE

{

mach	INTEGER (0..4000),

ias	INTEGER (-110..2100)

}

}

-- When AirSpeed is returned in an ADS report, the choice of which of the above units of

-- air speed are used depends on how the aircraft is equipped and whether the aircraft is

-- flying on mach or ias at the time. The choice is made by the avionics

ExtendedProjectedProfile :: = SEQUENCE (SIZE 1..128) OF SEQUENCE

{

way-point				Position,

time					Eta

}

FigureOfMerit :: = SEQUENCE

{

position-accuracy				PositionAccuracy,

multiple-navigational-units-operating	BOOLEAN,

tcas-operational				BOOLEAN

}

PositionAccuracy :: = ENUMERATED	-- nm = nautical miles

{

complete-loss	(0),

under30nm	(1),

under15nm	(2),

under8nm	(3),

under4nm	(4),

under1nm	(5),

under-25nm	(6),  -- under 0.25 nm

under-05nm	(7)   -- under 0.05 nm

}

FlightID :: = IA5String (SIZE(8))

-- Field 7 of ICAO flight plan or aircraft registration number.

GroundVector :: = SEQUENCE

{

track					INTEGER (1..3600) 		OPTIONAL,

-- units = 0.1 degrees true North,

-- range = 0.1 to 360 degrees

ground-speed				INTEGER (-110..4200) 		OPTIONAL,

-- units = speed-units (see Note 1)

-- range = -110 to + 4200 speed-units

vertical-rate				INTEGER (-3310..3310) 		OPTIONAL

-- units = alt-units per minute (see Note 1)

-- range = -3310 to +3310 alt-units per minute

}

-- When a report provides an GroundVector it should provide all the information

-- that it has available at the time.

ProjectedProfile :: = SEQUENCE

{

next-way-point				Position,

next-time				Eta,

following-way-point			Position

}

ShortTermIntent:: = SEQUENCE

{

position					Position,

projected-time				INTEGER (1..240),

-- units = minutes

-- range = 1 minute to 4 hours

intermediate-intent			IntermediateIntent

}

IntermediateIntent :: = SEQUENCE  OF SIZE (0..7) SEQUENCE

{

distance					INTEGER (1..8000),

-- units = Nautical miles

-- range = 1 to 8000 nautical miles

track					INTEGER (1..3600) OPTIONAL,

-- units = 0.1 degrees true North

-- range = 0.1 degrees to 360 degrees

altitude					Altitude,

projected-time				INTEGER (1..240)

-- units = minutes

-- range = 1 minute to 4 hours

}

-- IntermediateIntent indicates a set of way points between the current position and the

-- time indicated in the ShortTerm intent element.

-- When a report provides an IntermediateIntent it should provide all the information

-- that it has available at the time.

Weather :: = SEQUENCE

{

wind-speed				INTEGER (0..520) OPTIONAL,

-- units = speed-units (see Note 1)

-- range = 0 to 520 speed-units

wind-direction				INTEGER (1..360) OPTIONAL,

-- units = degrees true North

-- range= 1 to 360 degrees

temperature				INTEGER (-400..400) OPTIONAL,

-- units = 0.25 degrees Centigrate

-- range = -100 to 100 degrees C

turbulence				INTEGER (0..15) OPTIONAL

-- this is a place marker for a turbulence

-- index which is to be defined

}

-- When an ADS report provides Weather it should provide all the information

-- that the avionics has available at the time.

-- ------------------------------------------------------------------------------------------------------------------------

-- Components of Contracts

-- ------------------------------------------------------------------------------------------------------------------------

AngleChange ::= INTEGER (1..3600)

-- units = 0.1 degrees

-- range = 0.1 degrees to 360 degrees

AirSpeedChange ::= CHOICE

{

mach-number-change			INTEGER (1..255),

-- units = 0.005 Mach

-- range = 0.005 to 1.275 Mach

ias-change				INTEGER (1..700)

-- units = speed-units (see Note 1)

-- range = 1 to 700 speed-units

}

AltitudeChange ::= INTEGER (1..110)

-- units = alt-units (see Note 1)

-- range = 1 alt-unit to 110 alt-units

AltitudeRange :: = SEQUENCE

{

ceiling					Altitude,

floor					Altitude

}

ExtendedProjectedProfileModulus ::= SEQUENCE

{

modulus					Modulus,

extended-projected-profile-request		ExtendedProjectedProfileRequest

}

ExtendedProjectedProfileRequest ::= CHOICE

{

time-interval				INTEGER (1..64),

-- relative to current time stamp

-- units = 15 minutes

-- range =15 mins to 16 hours

number-of-way-points			INTEGER (1..128)

}

GroundSpeedChange ::= INTEGER (0..500)

-- units = speed-units (see Note 1)

-- range = 0 speed-units to 500 speed-units

LateralChange ::= INTEGER (0..255)

-- units = 0.125 Nautical miles

-- range= 0 nautical miles to 31.875 nautical miles

NavigationalIntent ::= SEQUENCE

{

intent-modulus				Modulus,

intent-projection-time			ProjectionTime

}

ProjectionTime ::= INTEGER (1..255)

-- units = minutes relative to current time stamp

-- range = 1 minute to 4 hours 15 minutes

ReportingInterval ::= CHOICE

{

seconds-scale				INTEGER (1..59),

-- units = seconds

-- range = 1 second to 59 seconds

minutes-scale				INTEGER (1..120)

-- units = minutes

-- range = 1 minutes to 2 hours

}

VerticalRateChange ::= INTEGER (-3310..3310)

-- units = alt-units per minute (see Note 1)

-- range = -3310 alt-units per minute to +3310 alt-units per minute

-- ------------------------------------------------------------------------------------------------------------------------

-- Miscellaneous components

-- ------------------------------------------------------------------------------------------------------------------------

Reason ::= CHOICE

{

aDS-service-unavailable 			NULL,

cannot-meet-reporting-rate		NULL,

maximum-capacity-exceeded		GroundSystemsUsingService,

undefined-reason				NULL

...

}

GroundSystemsUsingService ::= SEQUENCE OF IA5String (SIZE(8))

-- contains a sequence of ICAO facility designators

RequestType ::= ENUMERATED

{

event-contract (0),

periodic-contract (1),

demand-contract (2),

cancel-event-contract (3),

cancel-periodic-contract (4),

modify-emergency-contract (5),

cancel-all-contracts (6),

end-forward-contract (7)

...

}

EventType ::= ENUMERATED

{

lateral-deviation-change (0),

vertical-rate-change (1),

altitude-threshold (2),

way-point-change (3),

air-speed-change (4),

ground-speed-change (5),

heading-change (6),

extended-projected-profile-change (7),

fom-change (8),

track-angle-change (9),

altitude-change (10)

...

}

Modulus ::= INTEGER (1..255)

ReportType ::= ENUMERATED

{

flight-id (0),

projected-profile (1),

ground-vector (2),

air-vector (3),

weather	(4),

airframe-id (5),

short-term-intent (6),

extended-projected-profile (7)

...

}

ReportTypeAndPeriod ::= ENUMERATED

{

flight-id (0),

projected-profile (1),

ground-vector (2),

air-vector (3),

weather	(4),

airframe-id (5),

short-term-intent (6),

extended-projected-profile (7),

reporting-rate(8)

...

}

-- ------------------------------------------------------------------------------------------------------------------------

-- Common components

-- ------------------------------------------------------------------------------------------------------------------------

Eta :: = INTEGER (0..960)

-- units = 1 minute

-- range = 0 minutes to 16 hours

-- For projected profile, ETA at next way-point is relative to the current time stamp.

-- For extended projected profile, ETA at next way-point is relative to the current time stamp,

-- any following ETAs at next way-points are relative to previous way-points.

Position :: = SEQUENCE

{

latitude					Latitude,

longitude				Longitude,

altitude					Altitude

}

Latitude :: = SEQUENCE

{

sign					Sign

degrees					INTEGER (0..90),

-- units = degrees

-- range = 0 degrees to 90 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

-- If ‘degrees’ is 90, ‘minutes’ and ‘tenths-seconds’ must be 0

}

Longitude ::= SEQUENCE

{

sign					Sign

degrees					INTEGER (0..180),

-- units = degrees

-- range = 0 degrees to 180 degrees

minutes					INTEGER (0..59),

-- units = minutes

-- range = 0 minutes to 59 minutes

tenth-seconds				INTEGER (0..599)

-- units = 0.1 seconds

-- range = 0 seconds to 59.9 seconds

-- If ‘degrees’ is 180, ‘minutes’ and ‘tenths-seconds’ must be 0

}

Sign ::= ENUMERATED

{

plus (0),

minus (1)

}

Altitude	 :: = INTEGER (-10..6640)

-- units = alt-units (see Note 1)

-- range = -10 alt-units to + 6640 alt-units

DateTimeGroup ::= SEQUENCE

{

date            Date,

time            Time

}

Date ::= SEQUENCE

{

year					Year,

month					Month,

day					Day

}

Year ::= INTEGER (0..99)

-- unit = year modulo 100

Month ::= INTEGER (1..12)

-- unit = month

-- range = January to December

Day ::= INTEGER (1..31)

-- unit = day

-- range = 1 to 31

Time ::= SEQUENCE

{

timeHours				TimeHours,

timeMinutes				TimeMinutes,

timeSeconds				TimeSeconds 			OPTIONAL

}

TimeHours ::= INTEGER (0..23)

-- units = hours

-- range = midnight to 23.00 (11 PM)

TimeMinutes ::= INTEGER (0..59)

-- units = minutes

-- range = 0 minutes to 59 minutes

TimeSeconds ::= INTEGER (0..59)

-- units = seconds

-- range = 0 seconds to 59 seconds

END	-- of ADSMessageSetVersion1