# PROTOCOL DEFINITION

## Sequence Rules

Note 1.— The following figures define the valid sequences of primitives that is possible to invoke during the operation of the FIS application. They show the relationship in time between the service request and the resulting indication, and if  applicable, the subsequent response and the resulting confirmation.

### Only the sequence of primitives, except abort primitives, described below shall  ([45])  be permitted.

Note 1.— All abort primitives may interrupt and terminate any of the normal message sequences outline below.

### With the permissible exception of abort primitives, the FIS-air-ASE and the FIS-ground-ASE shall  ([46])  process primitives in the order in which they are received.

Note 1.— This ensures that the FIS-ASE  will, with the exception of aborts, guarantee message sequencing.

### FIS-demand-contract

Note 1.— The following sequence of messages, shown in  Figure 5-1, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Demand Contrat is either accepted or rejected.

T

I

M

E

Figure 5-1: use of FIS-demand-contract with no dialogue existing with contract accept or contract reject

Note 2.— The following sequence of messages, shown in Figure 5-2, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the FIS demand contract is either accepted or rejected.

Figure 5-2: use of FIS-demand-contract with dialogue existing with contract accept, or contract reject

Note 3.— The following sequence of messages, shown in Figure 5-3, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and a FIS report is sent by the FIS-ground-user after a positive acknowledgment.

Figure 5-3:: Use of FIS-demand-contract service  with no existing dialogue and with a positive ackowledgment

Note 4.— The following sequence of messages, shown in Figure 5-4, occurs when the FIS-demand-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and a FIS report is sent by the FIS-ground-user after a positive acknowledgment.

Figure 5-4: Use of FIS-demand-contract with existing dialogue and positive acknowledgment

### FIS-update-contract

Note 1.— The following sequence of messages, shown in Figure 5-5, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, the Update Contract is accepted. The cancellation is air-initiated.

(*) : timer set only for positive acknowledgment

Figure 5-5: Use of FIS-update-contract with no existing dialogue and with contract accept and with air-initiated cancellation.

Note 2.— The following sequence of messages, shown in  Figure 5-6, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE,  the Update Contract is accepted. The cancellation is ground-initiated.

(*) : timer set only for positive acknowledgment

Figure 5-6 : Use of FIS-update-contract with no existing dialogue and with contract accept and with ground-initiated cancellation.

Note 3.— The following sequence of messages, shown in  Figure 5-7, occurs when the FIS-update-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Update Contract is accepted. The cancellation is ground initiated.

(*): timer set only for positive acknowledgment

Figure 5-7:  use of FIS-update-contract with existing dialogue and with contract accept and with ground-initiated cancellation.

Note 4.— The following sequence of messages, shown in  Figure 5-8, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the Update Contrat is rejected.

Figure 5-8: Use of the FIS-update-contract with no existing dialogue and with contract reject.

Note 5.— The following sequence of messages, shown in Figure 5-9, occurs when the FIS-update-contract service is initiated by the FIS-air-user, at least one FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE, and the update contract is rejected.

Figure 5-9: Use of FIS-update-contract with existing dialogue and with contract reject

### FIS-cancel-update-contract

Note 1.— The following sequence of messages, shown in Figure 5-10, occurs when the FIS-cancel-update-contract service is initiated at the same time by both FIS-air-user and FIS-ground-user. If the cancelled contract is the last FIS Contract in operation, the dialogue is released.

Figure 5-10: Use of FIS-cancel-update by both FIS-users without other FIS Contract in place

Note 2.— The following sequence of messages, shown in  Figure 5-11, occurs when the FIS-update-contract service is initiated by the FIS-air-user, no FIS Contract already exists between the FIS-air-ASE and the FIS-ground-ASE and the ground-initiated cancellation is requested during contract establishement phase.

Figure 5-11 : Use of FIS-update-contract with no existing dialogue and with ground-initiated cancellation during contract establishment phase.

### FIS-cancel-contracts

Note 1.— The following sequence of messages, shown in Figure 5-12, occurs when the FIS-cancel-contracts service is initiated by the FIS-air-user, and all FIS contracts in place are impacted by the cancellation (this is always the case for CNS/ATM-1 Package).

Figure 5-12: Use of FIS-cancel-contracts with all contracts in place cancelled

### FIS-user Abort

Note 1.— The following sequence of messages, shown in Figure 5-13, occurs when the FIS-user-abort service is initiated by the FIS-air-user.

Figure 5-13: User of FIS-user-abort (air-initiated)

Note 2.— The following sequence of messages, shown in Figure 5-14, occurs when the FIS-user-abort service is initiated by the FIS-ground-user.

Figure 5-14: Use of FIS-user-abort   (ground-initiated )

### FIS-provider-abort Service

Note 1.— The following sequence of messages, shown in Figure 5-15, occurs when the dialogue service provider aborts.

Figure 5-15: Use of FIS-provider-abort  service (dialogue service provider aborts)

### FIS-ASE abort

Note 1.— The following sequence of messages, shown in Figure 5-16, occurs when the FIS-air-ASE aborts.

Figure 5-16: Use of  FIS-provider-abort  service (FIS-air-ASE aborts)

Note 2.— The following sequence of messages, shown in Figure 5-17, occurs when the FIS-ground-ASE aborts.

Figure 5-17: Use of FIS-provider-abort  service (FIS-ground-ASE aborts)

## FIS Service Provider Timers

Note 1.— Table 5-1 lists the time constraints related to the FIS application.  Each time constraint requires a timer to be set in the FIS protocol machine.

### The FIS-ASEs shall  ([47])  measure the time between the initial event and the corresponding final event.

### If the maximum time is exceeded before the final event has occurred, the following shall  ([48])  occur:

#### For all timers but the inactivity timer, the FIS ASEs shall  ([49])  take appropriate action as defined in 5.4.1.1.

#### For the inactivity timer, the FIS-ASE shall  ([50])  stop the dialogue by using the D-END service.

#### The action shall  ([51])  be taken immediately after the maximum time has expired.

Note.— The t-inactivity timer is an airborne timer value set on configuration basis.

| FIS Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| FIS- demand- | t-DC-1 | 6 minutes | FIS-demand-contract request | FIS-demand-contract confirmation |
| contract | t-DC-2 | 9 minutes + 1 sec + 10 sec | FIS-demand-contract request | FIS-report indication (in case a FIS-demand-contract ind with result = positive ack has been received) |
| FIS- update- | t-UC-1 | 6 minutes | FIS-update-contract request | FIS-update-contract confirmation |
| contract | t-UC-2 | 9 minutes + 1 sec + 10 sec | FIS-update-contract request | FIS-report indication (in case a FIS-update- update-contract ind with result = positive ack has been received) |
| FIS- cancel- update | t-UC- 3 | 6 minutes | FIS-cancel-update-contract request | FIS-cancel-update-contract confirmation |
| FIS-cancel- contracts | t-CL-1 | 6 minutes | FIS-cancel-contracts request | FIS-cancel-contracts confirmation |
| General | t-LI-1 | 6 minutes | D-END request | D-END confirmation |

Table 5-1:  FIS Application Timers

## FIS-ASE Protocol Description

### Description

#### Functional Model

Note 1.— The FIS-ASE is functionally made of 6 modules, as shown in Figure 5-17:

- the High Interface module (HI module). This module interfaces with the ASE-user through the abstract service interface as defined in chapter 3.

- the FIS Demand Contract module (DC module) : one DC module manages a single FIS Demand Contract.

- the FIS Update Contract module (UC module) : one UC module manages a single FIS Update Contract,

- the FIS CanceL contracts module  (CL module) : the CL module processes the termination of all contracts of the same type (ie ATIS) still in operation,

- the FIS Abort module (AB module): the AB module handles aborts in case of unrecoverable error, and

- the Low Interface module (LI module). This module interfaces the Dialogue Service Provider on behalf of the DC, UC and AB modules. It performs the multiplexing of FIS Contracts on a single dialogue.

Note 2.— This functional architecture allows simplification of the description of the protocol handled by the FIS-ASE. It does not constrain the implementation architecture.

Figure 5-18 : Functional Architecture of the FIS-ASE

Note 1.— The following subsections describe the actions of the individual modules in both the air and ground systems. Appendix A contains state tables for the individual modules.

### In the following sections, if no actions are described for an FIS-service primitive in a particular state, then the invocation of that service primitive shall  ([52])  be prohibited in that state.

### In the following sections, if no actions are described for the arrival of an APDU when in a particular state, then exception handling procedures as described in section 5.2 shall  ([53])  apply.

### High Interface module

#### Upon receipt of a FIS-ASE service primitive, the HI module shall ([54])  :

- if the primitive is a FIS-demand-contract request or a FIS-update-contract request primitive, reject the primitive if the Contract Number parameter corresponds to an existing FIS Contract,

- if the primitive is not a FIS-demand-contract request or a FIS-update-contract request primitive, reject the primitive if the Contract Number parameter does not correspond to an existing FIS Contract,

- pass the primitive to the relevant DC, UC, CL or AB module, as shown in Table 5-2.

| Primitive Name | Module |
| --- | --- |
| FIS-demand-contract request | air DC |
| FIS-update-contract request | air UC |
| FIS-cancel-update-contract request | air UC or ground UC |
| FIS-user-abort request | AB |
| FIS-demand-contract response | ground DC |
| FIS-update-contract response | ground UC |
| FIS-cancel-contracts request | air CL |
| FIS-report request | ground UC or ground DC |

Table 5-2: Request and response primitive to module mapping

#### Upon receipt of a request to invoke a service primitive by one of the modules in the FIS-ASE as shown in figure 5-3, and if the FIS-user is active, the HI module shall  ([55])  do so.

| Primitive Name | module |
| --- | --- |
| FIS-demand-contract cnf | air DC |
| FIS-update-contract cnf | air UC |
| FIS-cancel-update-contract ind | air UC or ground UC |
| FIS-cancel-update-contract cnf | air UC or ground UC |
| FIS-report ind | air DC or air UC |
| FIS-user-abort ind | AB |
| FIS-provider-abort ind | AB |
| FIS-demand-contract ind | ground DC |
| FIS-update-contract ind | ground UC |
| FIS-cancel-all-contracts ind | ground CL |
| FIS-cancel-all-contracts cnf | air CL |

Table 5-3  Indication and confirmation primitives to module mapping

### Air FIS DC module

#### The air FIS DC module shall  ([56])  be in one of the following states:

- DC-A-IDLE,

- DC-A-PENDING, or

- DC-A-ACTIVE.

#### On initiation, the air FIS DC module shall  ([57])  be in the DC-A-IDLE state.

#### Upon receipt of a FIS-demand-contract request, the following shall  ([58])  apply:

##### If in the DC-A-IDLE  state, the  DC module shall ([59])  :

- start timers t-DC-1 and t-DC-2,

- create a FISDownlinkAPDU [FISRequest] APDU with a demand contract APDU-element based on the FIS ContractNumber and FIS Contract Details parameters,

- request the LI module to send the APDU to the FIS-ground-ASE identified by the received ICAO Facility Designator parameter, and with the Class Of Communication Service parameter if specified by the user or the abstract value ”null” if not specified by the user,  and

- enter the DC-A-PENDING state.

#### Upon receipt of a [FISAccept] APDU with an accept APDU-element, the following shall  ([60])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([61])  :

- stop timers t-DC-1 and t-DC-2 timer ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "accepted",

  - the FIS Information parameter containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the DC-A-IDLE state.

#### Upon receipt of a [FISAccept] APDU containing a positive acknowledgment  messsage element, the following shall  ([62])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([63])  :

- stop timer t-DC-1,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "positive acknowledgement"

- enter the DC-A-ACTIVE state.

#### Upon receipt of a [FISReject] APDU, the following shall  ([64])  apply:

##### If in the DC-A-PENDING state, the module shall ([65])  :

- stop timers t-DC-1  and t-DC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-demand-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "rejected", and

  - the RejectReason parameter containing the information which has been received  as the ATISRejectReason APDU-element in the APDU.

- enter the DC-A-IDLE state.

#### Upon receipt of a [FISReport] APDU, the following shall  ([66])  apply:

##### If in the DC-A-ACTIVE state, the module shall ([67])  :

- stop timer t-DC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the DC-A-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([68])  apply:

##### If not in the DC-A-IDLE state, the DC module shall ([69])  :

- stop any timers,

- enter the DC-A-IDLE state.

##### If in the DC-A-IDLE, the DC module shall  ([70])  ignore the request.

#### Upon expiration of the t-DC-1 timer, the following shall  ([71])  apply:

##### If in the DC-A-PENDING state, the DC module shall ([72])  :

- stop timer t-DC-2,

- request the AB module to abort with reason timer-expiration, and

- enter the DC-A-IDLE state.

#### Upon expiration of the t-DC-2 timer, the following shall  ([73])  apply:

##### In in the DC-A-ACTIVE state, the DC module shall ([74])  :

- request the AB module to abort with reason timer-expiration, and

- enter the DC-A-IDLE state.

### Ground FIS DC module

#### The ground FIS DC module shall  ([75])  be in one of the following states:

- DC-G-IDLE,

- DC-G-PENDING, or

- DC-G-ACTIVE.

#### On initiation, the ground FIS DC module shall  ([76])  be in the DC-G-IDLE state.

#### Upon receipt of a [FISRequest] APDU with a demand contract messsage element, the following shall  ([77])  apply:

##### If in the DC-G-IDLE state, the DC module shall ([78])  :

- request the HI module to send a FIS-demand-contract indication to the FIS-ground-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Contract Details parameter, containing the information which has been received as the [FISRequestData] APDU-element in the APDU, and

- enter the DC-G-PENDING state.

#### Upon receipt of a FIS-demand-contract response with the Result parameter containing the abstract value "accepted", the following shall  ([79])  apply:

##### If in the DC-G-PENDING state,  the DC module shall ([80])  :

- create a FISUplinkAPDU [FISAccept] APDU with an accept APDU-element based on the FIS Contract Number and FIS Information parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt of a FIS-demand-contract response with the Result parameter containing the abstract value "positive acknowledgement", the following shall  ([81])  apply:

- create a FISUplinkAPDU [FISAccept] APDU with a positive acknowledgement APDU-element based on the FIS Contract Number parameter,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-ACTIVE state.

#### Upon receipt of a FIS-demand-contract  response with the Result parameter containing the abstract value "rejected", the following shall  ([82])  apply:

- create a FISUplinkAPDU [FISReject] APDU with an other reason APDU-element based on FIS Contract Number  and the RejectReason parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt of a FIS-report request, the following shall  ([83])  apply:

##### If in the DC-G-ACTIVE state, the DC module shall ([84])  :

- create a FISUplinkAPDU [FISReport] APDU based on the FISContract Number and the FIS Information parameters, and

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the DC-G-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([85])  apply:

##### If not in the DC-G-IDLE state, the DC module shall  ([86])  enter the DC-G-IDLE state.

##### If in the DC-G-IDLE state, the DC module shall  ([87])  ignore the request.

### Air FIS UC module

#### The air FIS UC module shall  ([88])  be in one of the following states:

- UC-A-IDLE,

- UC-A-PENDING,

- UC-A-ACTIVE, or

- UC-A-CANCEL

#### On initiation, the air FIS UC module shall  ([89])  be in the UC-A-IDLE state.

#### Upon receipt of a FIS-update-contract request, the following shall  ([90])  apply:

##### If in the UC-A-IDLE state, the UC module shall ([91])  :

- start timers t-UC-1 and t-UC-2,

- create a FISDownlinkAPDU [FISRequest] APDU with an update contract message lement based on the FIS Contract Number and FIS Contract Details parameters,

- request the LI module to send the APDU to the FIS-ground-ASE identified by the received ICAO Facility Designator parameter, with the Class Of Communication Service  parameter if specified by the user or the abstract value ”null” if not specified by the user, and

- enter the UC-A-PENDING state.

#### Upon receipt of a [FISAccept] APDU containing an accept  APDU-element, the following shall  ([92])  apply:

##### If in the UC-A-PENDING state, the UC module shall ([93])  :

- stop timers t-UC-1 and t-UC-2,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "accepted",

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU, and

- enter the UC-A-ACTIVE state.

#### Upon receipt of a [FISAccept] APDU containing a positive acknowledgement APDU-element, the following shall  ([94])  apply:

##### If in the UC-A-PENDING state, the UC module shall ([95])  :

- stop timer t-UC-1,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "positive acknowledgement",

- enter the UC-A-ACTIVE state.

#### Upon receipt of a [FISReject] APDU, the following shall  ([96])  apply;

##### If in the UC-A-PENDING state, the UC module shall ([97])  :

- stop timers t-UC-1 and t-UC-2 ,

- if there is no other FIS contract in place, start t-inactivity timer,

- request the HI module to send a FIS-update-contract confirmation to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the Result parameter, containing the abstract value "rejected", and

  - the RejectReason parameter containing the information which has been received  as the ATISRejectReason APDU-element in the APDU.

- enter the UC-A-IDLE state.

#### Upon receipt of a [FISReport] APDU, the following shall  ([98])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([99])  :

- if timer t-UC-2 is set, stop timer t-UC-2 ,

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU,

- remain in the UC-A-ACTIVE state.

##### If in the UC-A-CANCEL state, the UC module shall ([100])  :

- request the HI module to send a FIS-report indication to the FIS-air-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Information parameter, containing the information which has been received as the FISReportData APDU-element in the APDU.

- remain in the UC-A-CANCEL state.

#### Upon receipt of a FIS-cancel-update-contract request, the following shall  ([101])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([102])  :

- if timer t-UC-2 set, stop timer t-UC-2 ,

- create a FISDownlinkAPDU [FISCancelUpdateContract]  APDU based on the received FIS Contract Number parameter, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- start the t-UC-3 timer, and

- enter the UC-A-CANCEL state.

##### If in the UC-A-PENDING state and a dialogue is fully established, the UC module shall ([103])  :

- if timer t-UC-2 set, stop timer t-UC-2 ,

- create a FISDownlinkAPDU [FISCancelUpdateContract]  APDU based on the received FIS Contract Number parameter, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- start the t-UC-3 timer, and

- enter the UC-A-CANCEL state.

#### Upon receipt of a [FISCancelUpdateContract] APDU, the following shall  ([104])  apply:

##### If in the UC-A-ACTIVE state, the UC module shall ([105])  :

- if timer t-UC-2 is set, stop timer t-UC-2 ,

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-air-user with the FIS ContractNumber  parameter as received in the APDU,

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-A-IDLE state.

##### If in the UC-A-PENDING state, the UC module shall ([106])  :

- stop timers t-UC-1 and t-UC-2 ,

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-air-user with the FIS ContractNumber  parameter as received in the APDU,

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-A-IDLE state.

##### If in the UC-A-CANCEL state (ie collision of cancel-contract-update requests), the module shall ([107])  :

- create a FISDownlinkAPDU [FISCancelUpdateAccept] APDU based on the FISContractNumber as received in the APDU, and

- request the LI module to send the APDU to the FIS-ground-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- remain in the UC-A-CANCEL state.

#### Upon receipt of a [FISCancelUpdateAccept] APDU, the following shall  ([108])  apply:

##### If in the UC-A-CANCEL state, the module shall ([109])  :

- stop the t-UC-3 timer,

- request the HI module to send a FIS-cancel-update-contract confirmation to the FIS-air-user with the FIS Contract Number parameter as received in the APDU,and

- enter the UC-A-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([110])  apply:

##### If not in UC-A-IDLE state, the UC module shall ([111])  :

- stop any timers, and

- enter the UC-A-IDLE state.

##### If in UC-A-IDLE state, the UC module shall  ([112])  ignore the request.

#### Upon expiration of the t-UC-1 timer, the following shall  ([113])  occur:

##### If in UC-A-PENDING state, the UC module shall ([114])  :

- stop timer t-UC-2,

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

#### Upon expiration of the t-UC-2 timer, the following shall  ([115])  occur:

##### If in UC-A-ACTIVE state, the UC module shall ([116])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

#### Upon expiration of the t-UC-3 timer, the following shall  ([117])  occur:

##### If in UC-A-CANCEL state, the UC module shall ([118])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-A-IDLE state.

### Ground FIS UC module

#### The ground FIS UC module shall  ([119])  be in one of the following states:

- UC-G-IDLE,

- UC-G-PENDING,

- UC-G-ACTIVE, or

- UC-G-CANCEL.

#### On initiation, the ground FIS UC module shall  ([120])  be in the UC-G-IDLE state.

#### Upon receipt of a [FISRequest] APDU containing an update contract APDU-element, the following shall  ([121])  apply:

##### If in the UC-G-IDLE state, the UC module shall ([122])  :

- request the HI module to send a FIS-update-contract indication to the FIS-ground-user with the following parameters:

  - the FIS Contract Number parameter as received in the APDU,

  - the FIS Contract Details parameter, containing the information which has been received as the FISRequestData APDU-element in the APDU, and

- enter the UC-G-PENDING state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "accepted", the following shall  ([123])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([124])  :

- create a FISUplinkAPDU [FISAccept] APDU with an accept  APDU-element based on the FIS contract Number and FIS Information parameters,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-ACTIVE state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "positive acknowledgement", the following shall  ([125])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([126])  :

- create a FISUplinkAPDU [FISAccept] APDU with a positive acknowledgement  APDU-element based on the FIS Contract Number parameter,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-ACTIVE state.

#### Upon receipt of a FIS-update-contract response with a Result parameter containing the abstract value "rejected", the following shall  ([127])  apply:

##### If in the UC-G-PENDING state,  the UC module shall ([128])  :

- create a FISUplinkAPDU [FISReject] APDU based on:

  - the FIScontractNumber and FISInformation parameters if the RejectReason parameter contains the abstract value "update contract function not supported", or

  - the FIScontractNumber and RejectReason parameters if the RejectReason parameter does not contain the abstract value "update contract function not supported",

- request the LI module to send the APDU to the FIS-air-ASE,

- enter the UC-G-IDLE state.

#### Upon receipt of a FIS-report request, the following shall  ([129])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([130])  :

- create a FISUplinkAPDU [FISReport] APDU based on the FIS Contract Number  and FIS Information parameters,

- request the LI module to send the APDU to the FIS-air-ASE,

- remain in the UC-G-ACTIVE state.

#### Upon receipt of a FIS-cancel-update-contract request, the following shall  ([131])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([132])  :

- create a FISUplinkAPDU [FISCancelUpdateContract] APDU based on the FISContractNumber parameter,

- start timer t-UC-3,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-CANCEL state.

##### If in the UC-G-PENDING state, the UC module shall ([133])  :

- create a FISUplinkAPDU [FISCancelUpdateContract] APDU based on the FISContractNumber parameter,

- start timer t-UC-3,

- request the LI module to send the APDU to the FIS-air-ASE, and

- enter the UC-G-CANCEL state.

#### Upon receipt of a [FISCancelUpdateContract] APDU, the following shall  ([134])  apply:

##### If in the UC-G-ACTIVE state, the UC module shall ([135])  :

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-ground-user with the FIS ContractNumber as received in the APDU,

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

##### If in the UC-G-PENDING state, the UC module shall ([136])  :

- request the HI module to send a FIS-cancel-update-contract indication to the FIS-ground-user with the FIS ContractNumber as received in the APDU,

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

##### If in the UC-G-CANCEL state (ie collision of cancel-contract-update requests), the UC module shall ([137])  :

- create a FISUplinkAPDU [FISCancelContractAccept] APDU based on the FIS ContractNumber as received in the APDU,

- request the LI module to send the APDU to the FIS-air-ASE,

- if there is no FIS Contract any more, start the t-inactivity timer, and

- enter the UC-G-IDLE state.

#### Upon receipt of a [FISCancelContractAccept] APDU, the following shall  ([138])  apply:

##### If in the UC-G-CANCEL state, the UC module shall ([139])  :

- stop timer t-UC-3,

- request the HI module to send send a FIS-cancel-update-contract confirmation to the FIS-ground-user with the FIScontractNumber parameter as received in the APDU,

- start the t-inactivity timer, and

- enter the UC-G-IDLE state.

#### Upon receipt on a request from the AB module to stop operation, the following shall  ([140])  occur:

##### In not in UC-G-IDLE state, the UC module shall  ([141])  enter the UC-G-IDLE state.

##### If in UC-G-IDLE state, the UC module shall  ([142])  ignore the request.

#### Upon expiration of the t-UC-3 timer, the following shall  ([143])  occur:

##### If in UC-G-CANCEL state, the UC module shall ([144])  :

- request the AB module to abort with reason timer-expiration, and

- enter the UC-G-IDLE state.

### Ground and Air AB module

Note 1.— All statements in this section apply to both the FIS ground AB module and the FIS air AB module.

#### Upon receipt of a request to abort from the HI, LI, DC or UC modules, the AB module shall ([145])  :

- request DC and UC modules to stop operation,

- request the HI module to send a FIS-provider-abort indication primitive to the FIS-user with the reason parameter set to value supplied by the module requesting the abort,

- create a FISDownlinkAPDU or FISUplinkAPDU [FISAbort] APDU, with the FISProtocolErrorDiag element set to the value supplied by the module requesting the abort,

- request the LI module to send a D-ABORT request with the originator parameter set to the abstract value « provider ».

#### Upon receipt of a HI request to send a FIS-user-abort request, the AB module shall ([146])  :

- request DC and UC modules to stop operation, and

- request the LI module to send a D-ABORT request with the originator parameter set to the abstract value « user ».

#### Upon receipt of a LI request to deliver a D-ABORT indication, the AB module shall ([147])  :

- request DC and UC modules to stop operation,

- if the originator parameter contains the abstract value « user », request the HI module to send a FIS-user-abort indication primitive to the FIS-user.

- if the originator parameter contains the abstract value « provider », request the HI module to send a FIS-provider-abort indication primitive to the FIS-user with the FISProtocolErrorDiag element of the received APDU as AbortReason parameter.

#### Upon receipt of a LI request to deliver a D-P-ABORT indication, the AB module shall ([148])  :

- request DC and UC modules to stop operation,

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "communication system failure" as AbortReason parameter.

### Air Cancel (CL) module

#### Upon receipt of a HI request to send a FIS-cancel-contracts request, if there are DC and UC modules handling contracts of the types specified in the FIS Service Type parameter, the CL module shall ([149])  :

- request these DC and UC modules to stop operation for the types of contracts concerned by the cancellation as specified in the FIS Service Type parameter,

- create a FISDownlinkAPDU  [FISCancelAllContract] APDU based on the FIS Service Type parameter,

- request the LI module to send the APDU to the peer FIS-ASE, and

- start the t-CL-1 timer.

#### Upon receipt of [FISCancelContractsAccept] APDU, the CL module shall ([150])  :

- stop the t-CL-1 timer,

- request the HI module to send a FIS-cancell-contracts confirmation primitive to the FIS-air-user with the types of contracts concerned by the cancellation specified in the [FISCancelAllContractAccept] APDU as FISServiceType parameter,

- start the t-inactivity timer if there is no other FIS contract in place.

#### Upon expiration of the timer t-CL-1, the CL module shall  ([151])  request the AB module to abort with reason timer-expiration.

### Ground Cancel (CL) module

#### Upon receipt of [FISCancelAllContract] APDU, the CL module shall ([152])  :

- request DC and UC modules to stop operation for the types of contracts concerned by the cancellation as specified in the [FISCancelAllContract] APDU,

- request the HI module to send a FIS-cancell-contracts indication primitive to the FIS-user with the types of contracts concerned by the cancellation specified in the [FISCancelAllContract] APDU as FISServiceType parameter,

- create a FISDownlinkAPDU  [FISCancelAllContractAccept] APDU based on the FIS Service Type parameter,

- request the LI module to send the APDU to the peer FIS-ASE.

### Ground and Air Low Interface module

Note 1.— All statements in this section apply to both the FIS ground LI module and the FIS air LI module.

Note 2.— Table 5-4 specifies the type of the APDUs the LI module can send to the peer FIS-ASE.

| APDU | Type |
| --- | --- |
| FISRequest | initial |
| FISAccept | normal |
| FISReject | normal |
| FISReport | normal |
| FISCancelUpdateContract | normal |
| FISCancelUpdateAccept | normal |
| FISCancelContracts | normal |
| FISCancelContractsAccept | normal |

Table 5-4 : Types of the FIS-ASE APDUs

Note3.— Table 5-5 specifies the recipient of APDU received by the LI module.

| APDU Name | module |
| --- | --- |
| FISRequest | ground DC, ground UC |
| FISCancelUpdateContract | air UC, ground UC |
| FISCancelUpdateAccept | air UC, ground UC |
| FISAccept | air DC, air UC |
| FISReject | air DC, air UC |
| FISReport | air DC, air UC |
| FISCancellContracts | ground CL |
| FISCancellContractsAccept | air CL |

Table 5-5 :PDU to module mapping

#### The LI module shall  ([153])  be in one of the following states:

- LI-IDLE,

- LI-START-I (initiator of the D-START req),

- LI-START-R (receptor of the D-START ind),

- LI-DIALOGUE,

- LI-END-I (initiator of the D-END req), or

- LI-END-R (receptor of the D-END ind).

#### On initiation, the LI module shall  ([154])  be in the LI-IDLE state.

#### When requested by a DC or UC module to send an initial APDU to the peer ASE, the following shall  ([155])  apply:

##### if in the LI-IDLE state, the LI module shall ([156])  :

- invoke the D-START request primitive with the following parameters:

  - the ICAO Facility Designator provided by the DC or UC module as the CalledPeer Id parameter,

  - as components of the Quality Of Service parameter:

    - the Routing Class based on the Class of Communication Service as provided by the DC or UC module,

    - the Application Service Priority as defined in section 6.3.1,

    - the RER as defined in section 6.3.2.

  - the APDU as User Data parameter,

- enter the LI-START-I state.

##### if in the LI-DIALOGUE state, the LI module shall ([157])  :

- invoke the D-DATA request primitive with the APDU as User Data parameter, and

- remain in the LI-DIALOGUE state.

#### When requested by a DC or UC module to send a normal APDU to the peer ASE, the following shall  ([158])  apply:

##### if in the LI-DIALOGUE state, the LI module shall ([159])  :

- invoke the D-DATA request primitive with the APDU as User Data parameter,  and

- remain in the LI-DIALOGUE state.

##### if in the LI-START-R state, the LI module shall ([160])  :

- invoke the D-START response primitive with the following parameters:

  - the abstract value "accepted" as the Result parameter, and

  - the APDU as the User Data parameter,

- enter the LI-DIALOGUE state.

#### When requested by the AB module send a D-ABORT request, the following shall  ([161])  apply:

##### In not in the LI-IDLE state, the LI module shall ([162])  :

- invoke the D-ABORT request primitive with the APDU as UserData parameter and the value supplied by the AB module as originator parameter.

- enter the LI-IDLE state.

##### In in the LI-IDLE state, the LI module shall  ([163])  ignore the request.

#### Upon receipt of a D-START indication with the UserData parameter containing a valid APDU, the following shall  ([164])  apply :

##### If in the LI-IDLE state, the LI module shall ([165])  :

- pass the APDU to the relevant DC or UC module, as defined in Table 5-5, and

- enter the LI-START-R state.

#### Upon receipt of a positive D-START confirmation with a UserData  parameter containing a valid APDU, the following shall  ([166])  apply:

##### If in the LI-START-I state, the LI module shall ([167])  :

- pass the APDU to the relevant DC or UC module, as defined in Table 5-5,

- enter the LI-DIALOGUE state.

#### Upon receipt of a negative D-START confirmation (ie Result  contains the abstract value "rejected(transient)") with the RejectSource containing the abstract value "DS provider", the following shall  ([168])  apply:

##### If in the LI-START-I state, the LI module shall ([169])  :

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "cannot establish contact with the peer" as AbortReason parameter.

- enter the LI-DIALOGUE state.

#### Upon receipt of a negative D-START confirmation (ie Result  contains the abstract value "rejected(transient) or "rejected(permanent)") with the RejectSource containing the abstract value "DS user", the following shall  ([170])  apply:

##### If in the LI-START-I state, the LI module shall ([171])  :

- request the HI module to send a FIS-provider-abort indication to the FIS-user with the abstract value "contact refused by the the peer" as AbortReason parameter.

- enter the LI-DIALOGUE state.

#### Upon receipt of a D-DATA indication with a UserData parameter containing a valid APDU, the following shall  ([172])  apply:

##### In the LI-DIALOGUE, LI-END-I or LI-END-R state, the LI module shall ([173])

- identify the module from the Contract Number parameter of the APDU,

- pass the APDU to the relevant module, as defined Table 5-5, and

- remain in the same state.

#### Upon receipt of a D-END indication, the following shall  ([174])  apply:

##### If in state LI-DIALOGUE, the LI module shall ([175])  :

- invoke the D-END response primitive with the abstract value "accepted" as Result  parameter, and

- enter the LI-IDLE state.

#### Upon receipt of a D-END confirmation with a Result parameter containing the abstract value "accepted", the following shall  ([176])  apply:

##### If in the LI-END-I state, the LI module shall  ([177])  enter the LI-IDLE state.

#### Upon receipt of a D-ABORT indication with the UserData  parameter containing a valid APDU, the LI module shall ([178])  :

- forward the primitive to the AB Module, and

- enter the LI-IDLE state.

#### Upon receipt of a D-P-ABORT indication, the LI module shall ([179])  :

- forward the primitive to the AB module, and

- enter the LI-IDLE state.

#### Upon receipt of an indication that the t-inactivity timer has expired, the following shall  ([180])  apply:

##### In in LI-DIALOGUE state, the LI module shall ([181])  :

- start the timer t-LI-1,

- invoke the D-END request primitive, and

- enters the  LI-END-I state

## Exception Handling

### Timer Expiration

#### When any of the timers in any of the modules reaches the maximum time stated in section 5.2, the module shall  ([182])  request the AB module to abort with reason "timer expiration".

### Unrecoverable internal error

#### When any module has an unrecoverable system error, the module shall  ([183])  request the AB module to abort with reason "unrecoverable internal error".

### Protocol Error

#### When the LI module detects that a dialogue service indication or confirmation does not contain an APDU, the module shall  ([184])  request the AB module to abort with the reason "protocol error".

### Sequence Error

#### When a PDU is passed to a module for which there are no instruction in section 5.3 (ie the PDU has arrived out of sequence), the module shall  ([185])  request the AB module to abort with the reason "sequence error".

### Decoding Error

#### When the LI module detects a error when decoding an APDU, the LI module shall  ([186])  request the AB module to abort with the reason "decoding error".

### Invalid FIS Contract Number

#### When the LI module detects that the FIS Contract Number in the decoded APDU is invalid (ie the ContractNumber in a non  initial APDU is not already in use or the ContractNumber in an initial APDU is already in use), the UC or DC module shall  ([187])  request the AB module to abort with the reason "invalid contract number".

## State Tables

Note 1.— This section defines the state tables for modules of  both FIS-air-ASE and FIS-ground-ASE.

Note 2.— If the state tables shown in this section conflict with the textual statements made elsewhere in chapter 5, the textual statements take precedence.

### Conventions for state tables

Note 1.— The following notation conventions apply for the following state tables :

- Module States are identified as follows : <function name>-<location>-<state>.

- Events between a source module and a recipient module are identified as shown in the following figure:

Figure 5-1: Functional Model for ASE state tables

### High Interface (HI) Module

Note 1.— As the HI Module provides the ASE-users with a straightforward pass-through service to the DC, UC, CL and AB Modules, state tables related to the HI Modules are not described.

Note 2.— FIS-xxx request and response primitives are mapped onto DC-xxx, UC-xxx, CL-xxx or AB-xxx request and response primitives; DC-xxx, UC-xxx, CL-xxx or AB-xxx indication and confirmation primitives are mapped onto FIS-xxx indication and confirmation primitives, with the following exception:

- When AB-user-abort indication or AB-provider-abort indication are received by the HI Module from the AB Module, the corresponding FIS-user-abort indication or FIS-provider-abort indication is only generated to the FIS-user when the FIS-user is active.

### Air Demand Contract (DC) Module

Note. — Table 5-6 reflects the possible states of the air DC Module.

| State Þ | DC-A-IDLE | DC-A-PENDING | DC-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses from HI module |  |  |  |
| DC-demand-contract req | LI-Send-Initial [FISRequest-DemandContract] APDU Start t-DC-1, t-DC-2 èDC-A-PENDING | not permitted | not permitted |
| FIS APDUs from LI module |  |  |  |
| [FISReport] APDU | cannot occur | AB-abort ("sequence error") èDC-A-IDLE | stop t-DC-2 if last FIScontract start t-INACTIVITY DC-report ind èDC-A-IDLE |
| [FISAccept-accept] APDU | cannot occur | stop t-DC-1, t-DC-2 if last FISContract start t-INACTIVITY DC-demand-contract cnf èDC-A-IDLE | AB-abort ("sequence error") èDC-A-IDLE |
| [FISAccept-positive acknowledgment ] APDU | cannot occur | stop t-DC-1 DC-demand-contract cnf èDC-A-ACTIVE | AB-abort ("sequence error") èDC-A-IDLE |
| [FISReject] APDU | cannot occur | stop t-DC-1, t-DC-2 if last FIScontract start t-INACTIVITY DC-demand-contract cnf èDC-A-IDLE | AB-abort ("sequence error") èDC-A-IDLE |
| Requests from AB module |  |  |  |
| DC-stop-operation | do nothing ! | stop t-DC-1, t-DC-2 èDC-A-IDLE | stop t-DC-2 èDC-A-IDLE |

| State Þ | DC-A-IDLE | DC-A-PENDING | DC-A-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Timer expiration |  |  |  |
| t-DC-2 | cannot occur | cannot occur | AB-abort ("timer expiration") èDC-A-IDLE |
| t-DC-1 | cannot occur | Stop t-DC-2 AB-abort ("timer expiration") èDC-A-IDLE | cannot occur |

Table 5-6:  FIS-demand-contract air based state table

### Ground Demand Contract (DC) Module

Note. — Table 5-7 reflects the possible states of the ground DC Module.

| State Þ | DC-G-IDLE | DC-G-PENDING | DC-G-ACTIVE |
| --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |
| Primitive Requests and Responses from HI Module |  |  |  |
| DC-demand-contract rsp (accepted) | not permitted | LI-Send-Normal [FISAccept-accept] APDU èDC-G-IDLE | not permitted |
| DC-demand-contract rsp (rejected) | not permitted | LI-Send-Normal [[FISReject] APDU èDC-G-IDLE | not permitted |
| DC-demand-contract rsp (positive acknowledgment) | not permitted | LI-Send-Normal [FISAccept-positive-ack] APDU èDC-G-ACTIVE | not permitted |
| DC-report req | not permitted | not permitted | LI-Send-Normal [FISReport] APDU èDC-G-IDLE |
| APDU from LI Module |  |  |  |
| [FISRequest-DemandContract] APDU | DC-demand-contract ind èDC-G-PENDING | AB-abort ("sequence error") èDC-G-IDLE | AB-abort ("sequence error") èDC-G-IDLE |
| APDU from AB Module |  |  |  |
| DC-stop-operation | do nothing ! | èDC-G-IDLE | èDC-G-IDLE |

Table 5-7 :  FIS-demand-contract Ground based state table

### Air Update Contract (UC) Module

Note. — Table 5-8 reflects the possible states of the air UC Module.

| State Þ | UC-A-IDLE | UC-A-PENDING | UC-A-ACTIVE | UC-A-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses from HI |  |  |  |  |
| UC-update-contract req | LI-Send-Initial [FIS-Request-UpdateContract] APDU Start t-UC-1, t-UC-2 èUC-A-PENDING | not permitted | not permitted | not permitted |
| UC-cancel-update-contract req | not permitted | if dialogue not established not permitted else stop t-UC-1, t-UC-2 LI-Send-Normal [FISCancelUpdate Contract] APDU start t-UC-3 èUC-A-CANCEL | if set, stop t-UC-2 LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-A-CANCEL | not permitted |
| FIS uplink PDUs from LI |  |  |  |  |
| [FISAccept-accept] APDU | cannot occur | stop t-UC-1, t-UC-2 UC-update-contract cnf èUC-A-ACTIVE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |
| [FISAccept-positive acknowledgment] APDU | cannot occur | stop t-UC-1 UC-update-contract cnf èUC-A-ACTIVE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |

| State Þ | UC-A-IDLE | UC-A-PENDING | UC-A-ACTIVE | UC-A-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| [FISReject] APDU | cannot occur | stop t-UC-1, t-UC-2 if last FIScontract start t-INACTIVITY UC-update-contract cnf èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE |
| [FISReport] APDU | cannot occur | AB-abort ("sequence error") èUC-A-IDLE | If set, stop t-UC-2 UC-report ind èUC-A-ACTIVE | UC-report ind èUC-A-CANCEL |
| [FISCancelUpdateContract] APDU | cannot occur | UC-cancel-update-contract ind LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-IDLE | If set, stop t-UC-2 UC-cancel-update-contract ind LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-IDLE | /* collision */ LI-Send-Normal [FISCancelUpdateAccept] APDU if last FIS Contract start t-inactivity èUC-A-CANCEL |
| [FISCancelUpdateAccept] APDU | cannot occur | AB-abort ("sequence error") èUC-A-IDLE | AB-abort ("sequence error") èUC-A-IDLE | stop t-UC-3 UC-cancel-update-contract cnf èUC-A-IDLE |
| Request from AB module |  |  |  |  |
| UC-stop-operation | do nothing ! | stop t-UC-1, t-UC-2 èUC-A-IDLE | stop t-UC-2 èUC-A-IDLE | èUC-A-IDLE |
| Timer Expiration |  |  |  |  |
| t-UC-1 | cannot occur | Stop t-UC-2 AB-abort ("timer expiration") èUC-A-IDLE | cannot occur | cannot occur |
| t-UC-2 | cannot occur | cannot occur | AB-abort ("timer expiration") èUC-A-IDLE | cannot occur |
| t-UC-3 | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") èUC-A-IDLE |

Table 5-8:  FIS-update-contract air based state table

### Ground Update Contract (UC) Module

Note. — Table 5-9 reflects the possible states of the ground UC Module.

| State Þ | UC-G-IDLE | UC-G-PENDING | UC-G-ACTIVE | UC-G-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| Primitive Requests and Responses from HI |  |  |  |  |
| UC-update-contract rsp (accepted) | not permitted | LI-Send-Normal [FISAccept-accept] APDU èUC-G-ACTIVE | not permitted | not permitted |
| UC-update-contract rsp (rejected) | not permitted | LI-Send-Normal [FISReject] APDU èUC-G-IDLE | not permitted | not permitted |
| UC-update-contract rsp (processing) | not permitted | LI-Send-Normal [FISAccept-processing] APDU èUC-G-ACTIVE | not permitted | not permitted |
| UC-report req | not permitted | not permitted | LI-Send-Normal [FISReport] APDU èUC-G-ACTIVE | not permitted |
| UC-cancel-update-contract req | not permitted | LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-G-CANCEL | LI-Send-Normal [FISCancelUpdateContract] APDU start t-UC-3 èUC-G-CANCEL | not permitted |

| State Þ | UC-G-IDLE | UC-G-PENDING | UC-G-ACTIVE | UC-G-CANCEL |
| --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |
| FIS downlink PDUs from LI |  |  |  |  |
| [FISRequest-UpdateContract] APDU | UC-update-contract ind èUC-G-PENDING | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE |
| [FISCancelUpdateContract] APDU | cannot occur | AB-abort ("protocol error") è UC-A-IDLE | UC-cancel-update-contract ind LI-Send-Normal [FISCancelContractAccept] APDU è UC-A-IDLE | /* collision */ LI-Send-Normal [FISCancelContractAccept] APDU è UC-A-IDLE |
| [FISCancelUpdateAccept] APDU | cannot occur | AB-abort ("protocol error") è UC-G-IDLE | AB-abort ("protocol error") è UC-G-IDLE | stop t-UC-3 FIS-cancel-update-contract cnf èUC-G-IDLE |
| Request from AB module |  |  |  |  |
| UC-stop-operation | do nothing ! | è UC-G-IDLE | è UC-G-IDLE | è UC-G-IDLE |
| Timer Expiration |  |  |  |  |
| t-UC-3 | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") è UC-G-IDLE |

Table 5-9 :  FIS-update-contract ground based state table

### Air and Ground Low Interface (LI) Module

Note. — Table 5-10 reflects the possible states of the air and ground LI Modules.

| State Þ | LI-IDLE | LI-START-I | LI-DIALOGUE | LI-START-R | LI-END-I | LI-END-R |
| --- | --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |  |
| Primitive Requests and Responses from DC, UC or AB Module |  |  |  |  |  |  |
| LI-Send-Initial (APDU) | D-START req è LI-START- I | not permitted | D-DATA req | not permitted /* only air-initiated */ | not permitted | cannot occur |
| LI-Send-Normal (APDU) | not permitted | not permitted | D-DATA req | D-START rsp è LI-DIALOGUE | not permitted | not permitted |
| Primitive Requests from AB Module |  |  |  |  |  |  |
| AB_D-ABORT req | /* do nothing */ | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE | D-ABORT req è LI-IDLE |
| Timer Expiration |  |  |  |  |  |  |
| t-LI-1 | cannot occur | cannot occur | cannot occur | cannot occur | AB-abort ("timer expiration") èLI-IDLE | cannot occur |
| t-INACTIVITY | cannot occur | cannot occur | start t-LI-1 D-END req è LI-END-I | cannot occur | cannot occur | cannot occur |

| State Þ | LI-IDLE | LI-START-I | LI-DIALOGUE | LI-START-R | LI-END-I | LI-END-R |
| --- | --- | --- | --- | --- | --- | --- |
| Event ß | (Initial State) |  |  |  |  |  |
| Dialogue Service primitives from DSP |  |  |  |  |  |  |
| D-START ind | Pass APDU è LI-START- R | cannot occur | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf positive (result=accepted) | cannot occur | Pass APDU è LI-DIALOGUE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf negative (result=rejectedtransient & source=DS provider) | cannot occur | AB-dialogue-abort ("cannot establish contact with the peer") è LI-IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-START cnf negative (result=rejectedtransient & source=DS user) | cannot occur | AB-dialogue-abort ("contact refused by the peer") è LI-IDLE | cannot occur | cannot occur | cannot occur | cannot occur |
| D-DATA ind | cannot occur | cannot occur | Pass APDU | cannot occur | Pass APDU | Pass APDU |
| D-END ind | cannot occur | cannot occur | if APDU Pass APDU to UCmod è LI-END-R else D-END rsp positive è LI-DLE | cannot occur | cannot occur | cannot occur |
| D-END cnf positive | cannot occur | cannot occur | cannot occur | cannot occur | è LI-IDLE | cannot occur |
| D-ABORT ind | cannot occur | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE | AB_D-ABORT ind è LI-IDLE |
| D-P-ABORT ind | cannot occur | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE | AB_D-P-ABORT ind è LI-DLE |

Table 5-10 : LI Module state table

### Air and Ground Abort (AB) Modules

Note. — Table 5-11 reflects the possible states of the air and ground AB Modules.

| AB module state Þ | AB-IDLE |
| --- | --- |
| Event ß |  |
| Primitive Requests from DC and UC Module |  |
| AB-abort (reason) | UC-stop-operation (all FIS services) DC-stop-operation AB-provider-abort ind (reason) AB-D-ABORT req (« provider »,[ FISAbort] APDU) |
| Primitive Indications and APDUs from LI Module |  |
| AB_D-P-ABORT ind (reason) | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-provider-abort ind (reason) |
| AB_D-ABORT ind (« provider »,APDU) | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-provider-abort ind (APDU.AbortReason) |
| AB_D-ABORT ind (« user ») | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-user-abort ind () |
| Primitive Requests from HI Module |  |
| AB-user-abort req | UC-stop-operation (all FIS services) DC-stop-operation (all FIS services) AB-D-ABORT req (« user ») |

Table 5-11 : AB Module state table

### Air Cancel (CL) Module

Note. — Table 5-12 reflects the possible states of the air CL Module.

| CL module state Þ | CL-IDLE |
| --- | --- |
| Event ß |  |
| Primitive Request from the HI module |  |
| CL-cancel-contracts req (types of FIS service) | if UC or DC exist with these types of FIS service UC-stop-operation (type of FIS service) DC-stop-operation (type of FIS service) LI-Send-Normal [FISCancelContracts] APDU start t-CL-1 |
| APDUs from LI Module |  |
| [FISCancelContractsAccept] APDU | stop t-CL-1 CL-cancel-contracts cnf if last FIS Contract, start t-inactivity |
| Timer Expiration |  |
| t-CL-1 | AB-abort (« timer expiration ») |

Table 5-12 : Air CL Module state table

### Ground Cancel (CL) Module

Note. — Table 5-13 reflects the possible states of the ground CL Module.

| CL module state Þ | CL-IDLE |
| --- | --- |
| Event ß |  |
| APDUs from LI Module |  |
| [FISCancelContracts] APDU | UC-stop-operation (type of FIS service) DC-stop-operation (type of FIS service) CL-cancel-contracts ind LI-Send-Normal [FISCancelContractsAccept] APDU |

Table 5-13 : Ground CL Module state table