# FIS USER REQUIREMENTS

## Introduction

Note 1.—  Requirements imposed on FIS-user interfacing with the FIS-ASEs are presented in this chapter.

Note 2.— Where reference is made to the ”FIS-user” this implies both the FIS-air-user and the FIS-ground-user.

## General

### General Parameter Requirements

#### When a FIS-air-user invokes one of the FIS-ASE primitives described in chapter 3, it shall  ([194])  provide a user-defined reference for the FIS contract.

#### When a FIS-ground-user invokes one of the FIS-ASE primitives described in chapter 3, it shall  ([195])  provide a user-defined reference for the FIS contract.

When a FIS-air-user invokes FIS-demand-contract request or FIS-update-contract request and has no preference for the class of communication service to be used, or there is an existing FIS contract in place and the FIS-air-user requires the existing class of communication service parameter, it shall  ([196])  not provide class of communication service parameter.

### Timing Requirements

#### Upon receipt of a FIS-ASE service indication that requires a response, the FIS-ground-user shall  ([197])  invoke the FIS-ASE service response within 1 seconds.

### Error Handling

#### If the FIS-air-user has an unrecoverable system error, and if it is able to do so, it shall ([198])  :

- for each FIS-ground-user that the aircraft has contracts with and which is effected by the error, invoke the FIS-user-abort request, and

- inform its end user.

#### If the FIS-ground-user has an unrecoverable system error, and if it is able to do so, it shall ([199])  :

- for each aircraft that the FIS-ground-user has contracts with and which is effected by the error, invoke the FIS-user-abort request, and

- inform its end user.

#### If the FIS-ground-user receives a FIS-provider-abort indication or FIS-user-abort indication, it shall ([200])  :

- cease operation of all contracts with that air system, and

- inform its end user.

#### If the FIS-air-user receives a FIS-provider-abort indication or a FIS-user-abort indication, it shall ([201])  :

- cease operation of all contracts with that FIS-ground-user, and

- the FIS-air-user shall  ([202])  inform the end user.

### Miscellaneous Air FIS System Requirements

#### The FIS-air-user shall  ([203])  be able to retrieve the naming information required to establish a contract with a ground FIS system.

#### The FIS-air- user shall  ([204])  respond to indications and confirmations from the FIS-air-ASE in the order in which they are invoked.

### Miscellaneous Ground FIS System Requirements

#### The FIS-ground-user shall  ([205])  be capable of detecting that the content of the FIS request is not valid or that the requested FIS information is not available.

#### The FIS-ground-user shall  ([206])  be capable of detecting that it can not continue to provide updates of the requested FIS information.

#### The FIS-ground-user shall  ([207])  be able to support multiple concurrent FIS Contracts  with a given aircraft.

#### The FIS-ground user shall  ([208])  respond to indications and confirmations from the FIS-ground-ASE in the order in which they are invoked.

### ATIS service requirements

#### If the arrival ATIS or departure ATIS is requested, and if the FIS-ground-user only provides combined ATIS, the FIS-ground-user shall  ([209])  send combined ATIS with a 'combined' indication.

#### If both arrival and departure ATIS are requested, the FIS-ground-user shall ([210])  :

- send a combined ATIS with a 'combined' indication, if the combined ATIS is available, or

- send an arrival ATIS and a departure ATIS if both information are available but not the combined ATIS, or

- send the available ATIS with an indication that the other is not available, if only one is available.

#### The ATIS fields shall  ([211])  be presented to the pilot in the following order:

- Airport identification,

- Departure or arrival indicator,

- ATIS Code

- Time of Observation, if present,

- Approach Type, if present,

- Runways In Use,

- RunwayArresting System, if present,

- RunwaySurfaceConditions, if present,

- RunwayBraking Action, if present,

- Holding Delay, if present,

- Transition Level, if present,

- Other Operational Information, if present,

- Surface Wind,

- Visibility,

- RVR, if present,

- Present Weather,

- Cloud Sky and Cover,

- Air Temperature,

- Dew Point Temperature,

- Altimeter Setting,

- Significant Met Information,

- Trend Type of Landing Forecast, if present, and

- Specific ATIS Instruction, if present.

## Establishment and operation of a FIS Demand Contract

### If the FIS-air-user requires to establish a demand contract with the FIS-ground-user it shall  ([212])  invoke FIS-demand-contract request primitive.

### Only the FIS-air-user shall  ([213])  be capable of initiating the FIS-demand-contract service.

### The FIS-air-user shall  ([214])  use a value for the ContractNumber parameter of the FIS-demand-contract request primitive which is not already affected to another FIS contract.

### The FIS-ground-user shall  ([215])  use the value received in the ContractNumber parameter of the FIS-demand-contract indication primitive for the ContractNumber parameter of any ground-initiated request or response primitive (ie FIS-demand-contract response and FIS-report request).

### The same type of FIS Information (ie ”ATIS” for CNS/ATM-1 package) shall  ([216])  be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-demand-contract indication primitive, if the FIS-ground-user is able to accept the contract in full, the following shall  ([217])  apply:

#### If the FIS-ground-user is not able to send the FIS report within 1 seconds, it shall  ([218])  invoke the FIS-demand-contract response primitive with the result parameter set to abstract value ”positive acknowledgment” within 1 seconds and invoke the FIS-report request primitive within 10 seconds.

#### If the FIS-ground-user is able to send the FIS report within 1 seconds, it shall  ([219])  invoke the FIS-demand-contract response primitive with the result parameter set to the abstract value ”accepted” and the FIS report in the FIS Information parameter.

### Upon receipt of a FIS-demand-contract indication primitive, if the FIS-ground-user cannot comply with the demand contract request, it shall  ([220])  reject the contract by invoking an FIS-demand-contract response primitive with the following parameters:

- the abstract value "rejected"  as result  parameter, and

- one of the following  abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined" , as reject reason  parameter.

### The FIS-air-user shall  ([221])  be prevent from invoking a FIS-demand-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract request) has been confirmed.

### The FIS-air-user shall  ([222])  be allowed to reuse the value of a FIS Contract Number used in a FIS-demand-contract request primitive for a new FIS Contract:

- upon receipt of a FIS-demand-contract confirmation primitive with a Result parameter containing the abstract value ”rejected” or ”accepted”,

- upon receipt of a FIS-report indication, a FIS-user-abort indication or a FIS-provider-abort indication primitive,

- after invocation of a FIS-user-abort request primitive.

## Establishment and operation of a FIS Update Contract

### If the FIS-air-user requires to establish an update contract with the FIS-ground-user it shall  ([223])  invoke the FIS-update-contract request primitive.

### Only the FIS-air-user shall  ([224])  be capable of initiating the FIS-update-contact service.

### The FIS-air-user shall  ([225])  use a value for the ContractNumber parameter of the FIS-update-contract request primitive which is not already affected to another FIS contract.

### The FIS-ground-user shall  ([226])  use the value received in the ContractNumber parameter of the FIS-update-contract indication primitive for the ContractNumber parameter of any ground-initiated request or response primitive (ie FIS-update-contract response and FIS-report request).

### The same type of FIS Information (ie ”ATIS” for CNS/ATM-1 package) shall  ([227])  be identified by the FIS-air-user and FIS-ground-user when invoking the FIS-demand-contract request and response primitives.

### Upon receipt of a FIS-update-contract indication primitive, if the FIS-ground-user is able to accept the contract in full the following events shall  ([228])  occur:

#### If the FIS-ground-user is not able to send the first FIS-report within 1 seconds,  then it shall  ([229])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”positive acknowledgment”  within 1 seconds and invoke the FIS-report request primitive within 10 seconds.

#### If the FIS-ground-user is able to send the FIS-report within 1 seconds, then it shall  ([230])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”accepted” and the FIS report in the FIS Information parameter.

#### If the FIS-ground-user does not support the update-contract function but the FIS report is available, it shall  ([231])  invoke the FIS-update-contract response primitive with the result parameter set to the abstract value ”rejected”, the reject reason parameter set to the abstract value ”update contract function not supported by the FIS-ground-user” and the FIS report in the FIS Information parameter.

### Upon receipt of a FIS-update-contract indication primitive, if the FIS-ground-user cannot comply with the update contract request, it shall  ([232])  reject the contract by invoking an FIS-update-contract response with the following parameters:

- the abstract value "rejected"  as result  parameter, and

- one of the following  abstract values "cannot comply", "FIS service not available", "error detected in the FIS request" or "undefined" , as reject reason  parameter.

### The FIS-air-user shall  ([233])  be prevent from invoking a FIS-update-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-ground-user shall  ([234])  invoke the FIS-report request primitive each time the FIS information requested in the FIS-update-contract indication primitive is updated until such time as the contract is canceled or aborted.

### The FIS-air-user shall  ([235])  be allowed to reuse the value of a FIS Contract Number used in a FIS-update-contract request primitive for a new FIS contract:

- upon receipt of a FIS-update-contract confirm primitive with a Result parameter containing the abstract value ”rejected”,

- upon receipt of a FIS-user-abort indication or a FIS-provider-abort indication primitive,

- upon receipt of a FIS-cancel-update confirm or a FIS-cancel-contracts confirm primitive,

- upon receipt of a FIS-cancel-update indication primitive, or

- after invocation of a FIS-user-abort request.

## Air and ground cancellation of a single update contract

Note 1. — There is no provision for the cancellation of a single demand contract.

### If a FIS-user requires to cancel an active update contract, it shall  ([236])  invoke the FIS-cancel-update-contract request primitive with contractnumber parameter set to contract number identifying an active update contract.

### Both FIS-air-user and FIS-ground-user shall  ([237])  be capable of initiating the FIS-cancel-update-contract service.

### The FIS-air-user shall  ([238])  be prevent from invoking a FIS-cancel-update-contract request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### Upon receipt of a FIS-cancel-update-contract indication, the FIS-user shall  ([239])  cancel the update contract identified by the ContractNumber parameter.

## Air cancellation of FIS contracts of the same type

### If a FIS-air-user requires to cancel all the FIS contracts of a given type or of several types (only ”ATIS” in CNS/ATM-1 Package), it shall  ([240])  invoke the FIS-cancel-contracts request primitive with the FISServiceType parameter identifying the contract type(s).

### Only the FIS-air-user shall  ([241])  be capable of initiating the FIS-cancel-contracts service.

### The FIS-air-user shall  ([242])  be prevent from invoking a FIS-cancel-contracts request primitive before the very first request (either a FIS-demand-contract or a FIS-contract-request) has been confirmed.

### The FIS-air-user shall  ([243])  invoke the FIS-cancel-contracts request primitive for each ground system  with which it has FIS contracts.

### When the FIS-ground-user receives a FIS-cancel-contracts indication, it shall  ([244])  cancel any demand contract and update contract for the type(s) identified in the indication (eg ATIS) in force with that aircraft.

## FIS-user-abort

### If an active FIS-user requires to stop brutally any activity of the current FIS-AE with the peer FIS-AE with a possible loss of FIS messages currently in transit in the communication system, it shall  ([245])  invoke the FIS-user-abort request primitive.

Note 1.— This FIS-user is no more considered by the FIS AE as an active user.

### Both FIS-air-user and FIS-ground-user shall  ([246])  be capable of initiating the FIS-user-abort service.