# Transport Service and Protocol Specification

## General

### Overview

#### The COTP (Connection Oriented Transport Protocol ) shall be used to provide an end-to-end reliable data transfer service between transport service users on two ATN End Systems.

#### In ATN End Systems, the implementation of the COTP shall conform to ISO/IEC 8073 and the mandatory requirements given in this Chapter.

#### The CLTP (Connectionless Mode Transport Protocol) shall be used to provide a Connectionless data transfer service between Transport Service users on two ATN End Systems.

#### In ATN End Systems, the implementation of the CLTP shall conform to ISO/IEC 8602 and the mandatory requirements given in this chapter.

Note.— The transport protocols specified for use in ATN End Systems provide both Connection Mode and Connectionless Mode communication services. The implementation and use of a particular mode of the Transport Layer service depends on the requirements of the application(s) supported by a given ATN End System.

### Transport Service Description

Note 1. — When the TS-user requires use of the connection mode transport protocol the TS-user will provide the following information to the TS-provider on a per Transport Connection basis:

- called and calling TSAP address;

- whether or not the expedited data option is required;

- the required residual error rate (RER) to determine whether use or non-use of the transport checksum is allowed;

- the Application Service Priority to be mapped into the resulting CLNP NPDUs according to Table 1.2-2;

- the ATN Security Label specifying the ATN Traffic Type, i.e.

- ATN Operational Communications;

- ATN Administrative Communications;

- General Communications;

- ATN Systems Management Communications.

Note 2. — In the case where the Traffic Type specified is ATN Operational Communications the TS-user will additionally provide the traffic category, i.e. Air Traffic Services Communications (ATSC) or Aeronautical Operational Control (AOC).

Note 3. — In the case of the ATSC traffic category  the TS-user will further specify the required ATSC  Class as defined in Table 1.2-1, or no traffic type policy preference.

Note 4. — In the case of the AOC traffic category  the TS-user will further specify the subnetwork preference (including no preference).

Note 5. — The ATN Traffic Types and their associated traffic categories are specified in Table 5.6-1. The encoding of the ATN Security Label is specified in Figure 5.6-1 and 5.6.2.2.2 bullet b).

Note 6. — The TS-user is not required to specify any other Transport Service Quality of Service parameters.

### Transport Service Access Point Addresses

#### A TSAP address shall comprise two elements, a NSAP address and a TSAP selector.

#### The NSAP address and the TSAP selector shall conform to the provisions in 5.4.

### Exchange of Transport-Selector parameters

Note 1. — TSAP Selectors are transmitted in Calling and Called Transport-Selector parameters in COTP, and in Source and Destination Transport-Selector parameters in CLTP.

#### The transport entity shall support Transport-Selector parameters to accommodate the ATN TSAP selector syntax and encoding requirements as specified in 5.4.

#### Recommendation.— The transport entity should support remote Transport-Selector parameters of variable size from 0 up to 32 octets using any encoding and any value.

Note 2. — The absence of a Calling and Called Transport-Selector assumes the Network Address alone unambiguously defines the Transport Address.

#### In COTP, on receipt of CR (Connection Request) TPDU, the absence of a Calling or Called Transport-Selector shall be treated as equivalent to a zero length Calling or Called Transport-Selector.

#### The absence of a Calling or Called Transport-Selector in a received CC (Connection Confirm) TPDU shall indicate that Calling or Called Transport-Selector is equivalent to the corresponding parameter specified in the sent CR TPDU.

#### When present in a received CC TPDU, Calling and Called Transport-Selector parameters shall be identical in length and value to the corresponding parameter specified in the sent CR TPDU.

#### In CLTP, on receipt of UD (User Data) TPDU, the absence of a Source or Destination Transport-Selector shall be treated as equivalent to a zero length Source or Destination Transport-Selector.

## Connection Mode Transport Layer Operation

### Connection Mode Transport Service Primitives

Note 1. — For the purpose of describing the notional interfaces between different OSI protocol layers, each protocol layer is assumed to provide a service to the next higher protocol layer. The assumed service provided by the ATN transport layer to its user is described in ISO/IEC 8072.

Note 2. — ATN Applications may specify their use of the COTP implemented in ATN End Systems using the Transport Service specified in ISO/IEC 8072, including use of ATN priority, and security parameters as defined in this specification.

Note 3. — There is no requirement to implement the service specified in ISO/IEC 8072 as a software interface.

### ATN Specific Requirements

#### ATN End Systems shall implement the ISO/IEC 8073 Class 4 transport protocol in order to provide Connection Mode communications over the ATN Internet.

#### The COTP shall operate using the CLNS (Connectionless Network Service) as specified in Chapter 5.6.

Note.— TPDUs (Transport Protocol Data Units) are sent via the N-UNITDATA Request primitive.

#### The transport entity shall not concatenate TPDUs from TCs with different transport priorities or different Security Labels.

#### Recommendation.— The Selective Acknowledgement mechanism should be used for conservation of bandwidth by preventing retransmission of correctly received out-of-sequence TPDUs.

#### Recommendation.— The Request of Acknowledgement mechanism should be used to reduce AK traffic.

#### Recommendation. — The maximum TPDU size should be at least 1024 octets.

Note 1. — This is to support efficient transmission of anticipated application data exchanges.

#### Recommendation.— The Transport Layer should propose a TPDU size of at least 1024 octets.

#### Recommendation.— The Transport Layer should use the TPDU size parameter rather than the preferred maximum TPDU size parameter.

#### Recommendation.— Implementations of the ATN Transport Layer should propose use of normal format in the CR TPDU.

#### Recommendation.— The Extended format should only be proposed when explicitly necessary to meet application Quality of Service requirements.

Note 2.— Because the increased TPDU size resulting from use of extended data TPDU numbering may be more inefficient, this option is used on a TC only when absolutely required.

#### Recommendation.— The Transport Layer should accept non-use of checksum when proposed in a CR TPDU.

#### Recommendation. — The following timers and variables should be configurable on a TC basis:

- the local retransmission timer (T1),

- the acknowledgement timer (AL),

- the window update timer (W),

- the inactivity timer (I),

- the frozen reference time (L),

- the maximum retransmission number (N),

- the persistence timer (R).

#### When intended for operation over Air/Ground subnetworks, transport protocol implementations shall support the minimum - maximum ranges for COTP timer values as presented in Table 5.5-1.

##### Recommendation.— Nominal values indicated in Table 5.5-1 should be used.

##### Recommendation.— The assignment of optimized values for timers and parameters other than the nominal values indicated in Table 5.5-1 should be based on operational experience.

#### Recommendation.— When intended for operation exclusively over Ground/Ground subnetworks, implementations of transport protocol timer values should be optimized to ensure interoperability.

| Name | Description | Minimum Value | Nominal Value | Maximum Value |
| --- | --- | --- | --- | --- |
| MRL, MLR | NSDU Lifetime, seconds | 26 | 400 | 600 |
| ERL, ELR | Maximum Transit Delay, seconds | 1 | 100 | 150 |
| AL, AR | Acknowledgement Time, seconds | 1 | 20 | 400 |
| T1 | Local Retransmission Time, seconds | 12 | 221 | 300 |
| R | Persistence Time, seconds | 1 | 443 | 2710 |
| N | Maximum Number of Transmissions | 1 | 3 | 10 |
| L | Time bound on reference and/or sequence numbers, seconds | 160 | 1263 | 3000 |
| I | Inactivity Time, seconds | 600 | 4500 | 6000 |
| W | Window Time, seconds | 160 | 4000 | 6000 |

Table 5.5-1

Note 1.— In Table 5.5-1, the subscripts “R” and “L” refer to “remote” and “local” respectively.  The variable ERL, for example, refers to the maximum transit delay from the remote entity to the local entity.  The variable ELR  is the maximum transit delay from the local entity to the remote entity.  It is assumed that these values may be different.

Note 2.— Several of the timers and variables listed in Table 5.5-1 are not directly configurable, but may be determined based on the values of other timers and variables.  These computed values are:

T1 = (ELR + ERL + AR + x)

R = (T1 * (N-1) + x)

L = (MLR + MRL + R + AR)

W = (I - ELR - offset)

x = Local processing time

offset = Unanticipated delay exceeding ELR values

### Connection Mode Transport Quality of Service

#### Connection Mode Transport Priority

##### The Transport Layer shall allow a TC (Transport Connection)  priority in the range [0 - 14].

##### The Transport Layer shall not alter the proposed TC priority specified by the TS-user.

##### The Transport Layer shall treat all connections without expressed priority as being at the default TC priority.

##### The default TC priority shall be the lowest priority, i.e. priority [14].

##### When a TS-user specifies a TC priority, the relationship between this TC priority and the CLNP priority shall be as specified in Table 1.2-2.

#### Connection Mode Transport Security

Note.— The ATN security mechanism does not make use of the ISO/IEC 8073 Protection parameter. The support of the Protection parameter is therefore optional.

##### The Transport Layer shall allow a TS-user to specify a Security Label for a transport connection. The transport security procedure shall be implemented as specified in 5.2.7.3.1.

##### The Security Label format shall be according to 5.2.7.1. The Transport Layer shall not alter the Security Label specified by the TS-user.

Note . — When no Security Label is present, a « General Communications » traffic type is implied. In this case, CLNP NPDUs are generated without the Security parameter.

### Encoding of Transport Protocol Data Units

#### General

##### The encoding of TPDUs shall conform to ISO/IEC 8073 for the COTP.

#### Encoding of the Acknowledgment Time Parameter

##### In ATN compliant systems, the acknowledgement time parameter of the CR and CC TPDUs shall be encoded as follows:

| Parameter Code: | 1000 0101 |
| --- | --- |
|  | Note 1. — This is identical to the ISO/IEC 8073 standard parameter |
| Parameter Length: | 2 or 3 octets. |
| Parameter Value: | Acknowledgment Timer (AL) value expressed in milliseconds (per ISO/IEC 8073 standard) |

Note 2 .— This enhancement is in response to the unique requirements of the aeronautical environment which may require longer acknowledgment times than foreseen in ISO/IEC 8073.

Note 3. — Initial values of these timers may depend upon the subnetwork, traffic type and routing policy requirements expressed in the associated ATN Security Label.

Note 4. — In cases where the AL value is expressed in 2 octets (less than 65536 milliseconds), the ATN implementation will behave in compliance with the ISO/IEC 8073 standard.

Note 5. — Implementors are advised to permit systems administrators to readily specify initial values.

### Transport Layer Congestion Avoidance

#### General

Note 1.— The congestion avoidance mechanisms in the Transport Layer make use of the notification by the Network Layer of Congestion Experienced flags in received NPDUs. This mechanism allows transport entities to reduce the window, i.e. the number of DT TPDUs allowed to be sent without acknowledgement, when the proportion of NPDUs indicating congestion reaches a certain threshold.

Note 2.— This congestion information consists of the total length of the sequence of NPDUs forming the associated NSDU, and the number of NPDUs of that sequence that had their congestion experienced flag set upon reception.

Note 3.— Transport Congestion Avoidance measures are applicable to the Connection Mode  transport service only.

##### The transport entity shall implement the congestion avoidance algorithm defined in this section.

##### This algorithm shall be applied for each transport connection individually.

#### Advertised Window

##### General

###### A receiving transport entity shall provide the sending transport entity with the lower window edge and the size of the advertised window (W) by using the explicit flow control mechanisms specified in ISO/IEC 8073.

Note.— The advertised window is the window advertised by the receiver of the data to the sender of the data. It indicates the number of DT TPDUs that the receiver is willing to accept.

##### Initialisation of the Advertised Window

###### The initial value of the window W0 that is advertised to the sending transport entity shall have a locally configurable value.

###### This initial window shall be sent to the sending transport entity in the first CDT field transmitted.

#### Receiving Transport Entity Congestion Avoidance

##### General

###### Congestion avoidance shall be performed within repeated update phases.

###### Each update phase shall terminate with the possible advertisement of a new window size to the sending transport entity.

##### Start of Update Phase

###### An update phase of the advertised window shall start after the receiving transport entity has advertised a new value of the window Wnew to the sending transport entity.

##### Ignoring Congestion Information

###### After having advertised a new window size, the receiving transport entity shall ignore congestion information coming from the Network Layer, until it has received W (i.e. the « old » advertised window size) further DT-TPDUs. It then shall enter the sampling sub-phase.

###### When the sending transport entity advertises the initial window size W0, it shall set W to 0.

##### Sampling Congestion Information

###### The receiving transport entity shall maintain a count N equal to the total number of NPDUs that convey DT-TPDUs, and a count NC equal to the number of such NPDUs that had their congestion experienced flag set upon reception.

###### Upon entering the sampling sub-phase, these counts shall be reset to zero.

###### These counts shall be updated upon receipt of a DT-TPDU using the congestion information supplied by the Network Layer.

###### The sampling sub-phase shall end as soon as the transport entity has received Wnew DT-TPDUs within the sampling sub-phase. The end of the sampling sub-phase also terminates the update phase.

##### Action Upon the End of the Update Phase

###### The receiving transport entity shall take the following actions at the end of each update phase:

- If the count NC is less than l % of the count N, the receiving transport entity shall increase the size of the advertised window by adding  up to a maximum based on the local buffer management policy. Otherwise, it shall decrease the size of the advertised window by multiplying it by b. If the result of this multiplication has a decimal part, the new window size shall be the truncated to its integer value. The size of the advertised window shall not go to a value smaller than 1.

- The counts N and NC shall be reset to 0.

- The new window size shall be transmitted to the sending transport entity in accordance with the explicit flow control mechanisms specified in ISO/IEC 8073.

Note.— This procedure does not explicitly require the reduction of the upper window edge, as it is possible to gradually reduce the credit window.

#### Recommended Algorithm Values

##### Recommendation.— The value settings defined in the following table should be implemented and configurable by a System Manager:

Table 5.5-2. Congestion Avoidance algorithm values

| Name | Description | Recommended value/range |
| --- | --- | --- |
| b | Window decrease factor | 0.75 to 0.95 |
|  | Window increase amount | 1 |
| W0 | Initial window | 1 |
| l | Congestion ratio | 50 % |

### Use of the ATN Network Service

Note.— This section specifies how the COTP operates over the CLNS provided by the ATN Network Layer.

#### Use of the N-UNITDATA Request

##### General

###### The Transport Layer shall use the N-UNITDATA Request primitive, as defined in ISO/IEC 8073, to transmit TPDUs.

Note.— The way the parameters are exchanged between the transport entity and the Network Service is a local matter.

###### The length indication given to the network service shall be equal to the length of the TPDU(s).

Note.— The maximum size of each TPDU is restricted to the locally defined maximum NSDU size.

##### NS-user-data

Note.— Transport entities transmit TPDUs as NS-user-data of the N-UNITDATA Request primitive.

##### Network Service Access Point Addresses

Note.— The Transport Layer has knowledge of the source and destination address parameters only as octet strings.

##### Network Quality of Service

###### General

The COTP shall use the network QoS parameters as defined in the sections below.

###### Network Layer Priority

The COTP shall use the network priority parameter to indicate the relative priority of a NSDU.

When a transport priority has been specified, the value of network priority shall be determined based on the transport connection priority, as defined in Table 1.2-2.

If the Transport Layer supports levels of TC priority numerically greater than 14, TPDUs associated with the TC shall be transmitted using a network priority level of zero.

Note.— As specified in ISO/IEC 8073, the Transport Layer priority level zero is highest. ISO/IEC 8473 specifies zero as the lowest network priority and fourteen as the highest. Table 1.2-2 defines the required  mapping between these two schemes for use by ATN systems.

###### Network Layer Security

Note.— The use of the Network Layer security is specified in 5.2.7.3.1.

#### Use of the N-UNITDATA Indication

##### General

###### The Transport Layer shall be capable of receiving TPDUs from the ATN network service using the N-UNITDATA indication primitive, as defined in ISO/IEC 8073.

Note.— The way the parameters are exchanged between the transport entity and the Network Service is a local matter.

##### NS-user-data

Note.— Transport entities receive TPDUs as NS-user-data of the N-UNITDATA Indication primitive.

### Connection Mode Transport APRL

#### Mandatory and Optional Functions

##### General

Note. — The requirements for the COTP are provided in the form of an ATN Protocol Requirements List (APRL). The APRL has been prepared using the PICS (Protocol Implementation Conformance Statement) proforma provided with ISO/IEC 8073.

###### An implementation of the ISO/IEC 8073 Transport Protocol shall be used in an ATN End System if and only if its PICS is in compliance with the APRL provided with these SARPs.

##### Protocol Implementation

###### Classes Implemented

| Index | Class | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| C0 | Class 0 | 14.2 | O.1 | O |
| C1 | Class 1 | 14.4 | C0:O | O |
| C2 | Class 2 | 14.2 | O.1 | O |
| C3 | Class 3 | 14.3 | C2:O | O |
| C4 | Class 4 operation over CONS | 14.3 | C2:O | O |
| C4L | Class 4 operation over CLNS | 14.3 | C2:O | M |

###### Specific ATN Requirements

| Index | Feature | SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| ATN1 | Support of Congestion Avoidance Procedures? | 5.5.2.5 | M |
| ATN2 | Transport to Network Priority Mapping? | 5. | M |
| ATN3 | Support of ATN Security Label? | 5. | M |
| ATN4 | Timer Settings on a per Transport Connection basis? | 5. | M |
| ATN5 | Enhanced encoding of Acknowledgment Time Parameter? | 5. | M |

##### Initiator/Responder Capability for Protocol Classes 0-4

| Index |  | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| IR1 | Initiating CR TPDU | 14.5 a) | O.2 | M |
| IR2 | Responding to CR TPDU | 14.5 a) | O.2 | M |

##### Supported Functions

###### Supported Functions for Class 4 (C4 or C4L::)

Mandatory Functions for Class 4

| Index | Function | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| T4F1 | TPDU transfer | 6.2 | M | M |
| T4F2 | Segmenting | 6.3 | M | M |
| T4F3 | Reassembling | 6.3 | M | M |
| T4F4 | Separation | 6.4 | M | M |
| T4F5 | Connection establishment | 6.5 | M | M |
| T4F6 | Connection refusal | 6.6 | M | M |
| T4F7 | Data TPDU numbering (normal) | 6.10 | M | M |
| T4F8 | Retention and acknowledgement of TPDUs (AK) | 6.13.4.1 | M | M |
| T4F9 | Explicit flow control | 6.16 | M | M |
| T4F10 | Checksum | 6.17 | M | M |
| T4F11 | Frozen references | 6.18 | M | M |
| T4F12 | Retransmission on time-out | 6.19 | M | M |
| T4F13 | Resequencing | 6.20 | M | M |
| T4F14 | Inactivity control | 6.21 | M | M |

Mandatory Functions for Operation over Connectionless Network Service

| Index | Function | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| T4F23 | Transmission over CLNS | 6.1.2 | M | M |
| T4F24 | Normal release when operating over CLNS (explicit) | 6.7.2 | M | M |
| T4F25 | Association of TPDUs with transport connections when operating over CLNS | 6.9.2 | M | M |
| T4F26 | Expedited data transfer when operating over CLNS (Network normal) | 6.11.2 | M | M |
| T4F27 | Treatment of protocol errors when operating over CLNS | 6.22.2 | M | M |

ISO/IEC 8073 Optional Functions

| Index | Feature | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| T4F28 | Data TPDU numbering (extended) | 6.10 | O | O |
| T4F29 | Non-use of checksum | 6.17 | O | M |
| T4F30 | Concatenation | 6.4 | O | O |
| T4F31 | Retention and acknowledgement of TPDUs Use of selective acknowledgement | 6.13.4.4 | O | O |
| T4F32 | Retention and acknowledgement of TPDUs Use of request acknowledgement | 6.13.4.3 | O | O |

##### Supported TPDUs

| Index | TPDUs |  |  | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- | --- | --- |
| ST1 | CR | supported on transmission | 13.1 | IR1:M | M |
| ST2 | CR | supported on receipt | 13.1 | IR2:M | M |
| ST3 | CC | supported on transmission | 13.1 | IR2:M | M |
| ST4 | CC | supported on receipt | 13.1 | IR1:M | M |
| ST5 | DR | supported on transmission | 13.1 | IR2:M | M |
| ST6 | DR | supported on receipt | 13.1 | IR1:M | M |
| ST7 | DC | supported on transmission | 13.1 | C4L:M | M |
| ST8 | DC | supported on receipt | 13.1 | C4L:M | M |
| ST9 | DT | supported on transmission | 13.1 | M | M |
| ST10 | DT | supported on receipt | 13.1 | M | M |
| ST11 | ED | supported on transmission | 13.1 | C4L:M | MO |
| ST12 | ED | supported on receipt | 13.1 | C4L:M | MO |
| ST13 | AK | supported on transmission | 13.1 | C4L:M | M |
| ST14 | AK | supported on receipt | 13.1 | C4L:M | M |
| ST15 | EA | supported on transmission | 13.1 | C4L:M | MO |
| ST16 | EA | supported on receipt | 13.1 | C4L:M | MO |
| ST19 | ER | supported on receipt | 13.1 | M | M |

Note.— The following table states for which classes, if any, ER TPDU is supported on transmission:

| Index | Class | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| SER4L | ER support on transmission of Class 4 over CLNS | 6.22.2 | O | O |

##### Supported Parameters of Issued TPDUs

###### Parameter Values for CR TPDU (C4L::)

| Index | feature | ISO/IEC 8073 Reference | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| ICR1 | Bits 8 and 7 in the additional options selection parameter of a CR TPDU set to zero? | 13.3.4 g) | M | M |

If the preferred class in the CR is 2,3 or 4:

| Index | feature | ISO/IEC 8073 Reference | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| ICR2 | Is class 0 always offered as an alternative class? | 14.4 | O | X |

###### Supported parameters for Class 4 TPDUs (C4L::)

Optional Parameters for a Connection Request TPDU

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4CR7 | Called Transport-Selector | 13.3.4 a) | O | M |
| I4CR8 | Calling Transport-Selector | 13.3.4 a) | O | M |
| I4CR9 | TPDU size | 13.3.4 b) | O | O |
| I4CR10 | Version Number | 13.3.4 d) | O | O |
| I4CR11 | Protection parameters | 13.3.4 e) | O | O |
| I4CR12 | Additional option selection | 13.3.4 g) | O | M |
| I4CR13 | Throughput | 13.3.4 k) | O | O |
| I4CR14 | Residual error rate | 13.3.4 m) | O | O |
| I4CR15 | Priority | 13.3.4 n) | O | M |
| I4CR16 | Transit delay | 13.3.4 p) | O | O |
| I4CR17 | Acknowledgement time | 13.3.4 j) | O | M |
| I4CR18 | Preferred maximum TPDU size | 13.3.4 c) | O | O |
| I4CR19 | Inactivity timer | 13.3.4 r) | O | M |

Optional Parameters for a Connection Confirm TPDU

Note. — According to ISO, the following parameters are optional if a CC TPDU is issued in class 4:

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4CC6 | Called Transport-Selector | 13.4.4 | O | M |
| I4CC7 | Calling Transport-Selector | 13.4.4 | O | M |
| I4CC8 | TPDU size | 13.4.4 | O | O |
| I4CC9 | Protection parameters | 13.4.4 | O | O |
| I4CC10 | Additional option selection | 13.4.4 | O | M |
| I4CC11 | Acknowledgement time | 13.4.4 | O | M |
| I4CC12 | Throughput | 13.4.4 | O | O |
| I4CC13 | Residual error rate | 13.4.4 | O | O |
| I4CC14 | Priority | 13.4.4 | O | M |
| I4CC15 | Transit delay | 13.4.4 | O | O |
| I4CC16 | Preferred maximum TPDU size | 13.4.4 | I4CR18:O | O |
| I4CC17 | Inactivity timer | 13.4.4 | O | M |

Note.— The support of T4F26 implies that the Additional Options Selection parameter is mandatory.

Optional Parameter for a Disconnect Request TPDU

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4DR4 | Additional information | 13.5.4 a) | O | O |

Mandatory Parameter for a Data TPDU

Note.— According to ISO, the following parameter is mandatory in a DT TPDU if request of acknowledgement has been selected.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4DT4 | Request of acknowledgement | 13.7.3 b) | T4F32:M | T4F32:M |

Optional Parameter for an Acknowledgement TPDU

Note.— According to ISO, an AK TPDU containing flow control information will be transmitted if an AK TPDU is received under the conditions specified in ISO/IEC 8073 12.2.3.9. The following parameter is mandatory for ATN compliant systems if an AK TPDU is issued in Class 4.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4AK4 | Flow control confirmation | 13.9.4 c) | O | M |

Use of the Subsequence Number Parameter in the Acknowledgement TPDU

Note.— According to ISO, if an implementation can reduce credit and does so in the manner outlined in ISO/IEC 8073 12.2.3.8.2 then the subsequence number in AK TPDU is mandatory.

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4AK5 | Subsequence number | 13.9.4. b) | O | M |

Use of the Selective Acknowledgement Parameter in the Acknowledgement TPDU

Note.— According to ISO, the following parameter is optional in an AK TPDU if selective acknowledgement has been negotiated.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4AK6 | Selective acknowledgement parameters | 13.9.4. d) | T4F31:O | T4F31:O |

Optional Parameters for an Error TPDU

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| I4ER3 | Invalid TPDU | 13.12.4 a) | O | O |

##### Supported parameters for received TPDUs

Note.— ISO/IEC 8073 requires implementations to be capable of receiving and processing all possible parameters for all possible TPDUs, depending upon the class and optional functions implemented.

###### TPDUs in Class 4 (C4L::)

Note.— According to ISO, if use of checksum has been selected then it is mandatory to process a checksum parameter in the following TPDUs.

| Index | TPDU | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| R4CCch | CC TPDU | 13.4.4 | M | M |
| R4DRch | DR TPDU | 13.5.4 b) | M | M |
| R4DCch | DC TPDU | 13.6.4 | M | M |
| R4DTch | DT TPDU | 13.7.4 | M | M |
| R4EDch | ED TPDU | 13.8.4 | M | M |
| R4AKch | AK TPDU | 13.9.4 a) | M | M |
| R4EAch | EA TPDU | 13.10.4 | M | M |
| R4ERch | ER TPDU | 13.12.4 b) | M | M |

##### User Data in Issued TPDUs

###### Class 4 (C4 or C4L::)

| Index | User Data | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| D4ICR | User data of up to 32 octets in a CR with preferred class 4 ? | 13.3.5 | M | M |
| D4ICC | User data of up to 32 octets in a CC ? | 13.4.5 | M | M |
| D4IDR | User data of up to 64 octets in a DR ? | 13.5.5 | M | M |

##### User Data in Received TPDUs

| Index | User Data | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| DRCC | 32 octets of user data in a CC TPDU ? | 13.4.5 | IR1:M | IR1:M |
| DRDR | 64 octets of user data in a DR TPDU ? | 13.5.5 | IR1:M | IR1:M |
| DRCR | 32 octets of user data in a CR TPDU ? | 13.3.5 | IR2:M | IR2:M |

##### Negotiation

Note.— If an option is not returned in the CC, it is considered to have been refused. This allows compatible negotiation between versions of the ISO/IEC 8073 transport protocol.

###### Class Negotiation - Initiator

| Index | Feature | ISO/IEC 8073 References | ATN Supported Value |
| --- | --- | --- | --- |
| NC | The preferred class in the CR TPDU may contain any of the classes supported by the implementation | 6.5.5 j) | Class 4 |

Note 1.— Negotiation of other protocol classes is out of scope. If this is the only profile supported then it is not possible to negotiate any other protocol class.

Note 2.— The table below specifies valid alternative classes.

| Index | Preferred class | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Values |
| --- | --- | --- | --- | --- |
| NAC5 | Class 4 over CLNS | 6.5.5 j) | None | None |

Note 3.— The class cannot be negotiated since Class 4 is the only class allowed over CLNS.

###### Class negotiation - responder side

| Index | Preferred class | ISO/IEC 8073 References | ISO Allowed Responses | ATN Supported Values |
| --- | --- | --- | --- | --- |
| RC4 | What classes can you respond with if CR proposes only class 4? | 6.5.4 j) Table 3 | 2,4 or connection refused depending on classes supported | Class 4 |
| RC4a | What classes can you respond with if CR proposes class 4 as preferred class and the alternative class parameter is present? | 6.5.4 j) Table 3 | 0,1,2,3,4 or connection refused depending on classes supported and coding of alternative class | Class 4 |

Note.— This table does not preclude connection refusal for other reasons.

###### TPDU Size Negotiation

| Index | TPDU size | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| TS1 | If maximum TPDU size is proposed in a CR TPDU then the initiator shall support all TPDU sizes from 128 octets to the maximum proposed | 14.6 e) | I4CR9:M | I4CR9:M |
| TS2 | If the preferred maximum TPDU size parameter is used in a CR TPDU then the initiator shall support all TPDU sizes, except 0, that are multiples of 128 octets up to the preferred maximum proposed | 14.6 e) | I4CR18:M | I4CR18:M |

| Index | TPDU size | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Values |
| --- | --- | --- | --- | --- |
| TS3 | What is the largest value of the preferred maximum TPDU size parameter in a CR TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |
| TS4 | What is the largest value of the preferred maximum TPDU size parameter in a CC TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |

Note.— An implementation of the Transport Layer can support a preferred maximum TPDU size larger than 1024 octets.

| Index | TPDU size | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Values |
| --- | --- | --- | --- | --- |
| T4S1 | What is the largest value of the maximum TPDU size parameter in a CR TPDU with preferred class 4? | 14.6 e) | One of 128, 256, 512, 1024, 2048, 4096, 8192 | One of 128, 256, 512, 1024, 2048, 4096, 8192 |
| T4S2 | What is the largest value of the maximum TPDU size parameter which may be sent in the CC TPDU when class 4 is selected? | 14.6 e) | 128, 256, 512, 1024, 2048, 4096, 8192 | 128, 256, 512, 1024, 2048, 4096, 8192 |

###### Use of Extended Format

| Index | Extended format | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Value |
| --- | --- | --- | --- | --- |
| NEF3 | What formats can you propose in the CR TPDU in class 4? | 6.5.5 n) | normal, extended | normal,extended |
| NEF6 | What formats can you select in CC when extended has been proposed in CR in class 4? | 6.5.5 n) | normal, extended | normal,extended |

Note.— This table does not preclude proposal of the extended format.

###### Expedited data Transport service

| Index | Expedited data | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| TED1 | Is the expedited data indication supported in CR and CC TPDU? | 6.5.5 r) | M | MO |

Note.— Expedited data is proposed using the Additional Options Parameters in the CR and CC TPDUs.

###### Non-use of Checksum (C4L and T4F29::)

| Index | Non-use of checksum | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Values |
| --- | --- | --- | --- | --- |
| NUC1 | What proposals can you make in the CR? | 6.5.5 p) | non-use, use | non-use, use |
| NUC2 | What proposals can you make in CC when non-use of checksum has been proposed in CR? | 6.5.5 p) | non-use, use | non-use, use |

Note 1.— A Transport Layer is able to propose either use or non-use of checksum in a CR TPDU.

Note 2.— The term ”non-use” means that the Transport Layer may respond accepting non-use of checksum. A Transport Layer may also respond with use of checksum if non-use has been proposed.

###### Use of selective acknowledgement

| Index | Selective Acknowledgement | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| USA1 | Is use of selective acknowledgement proposed in CR TPDUs ? | 6.5.5 s) | O | O |
| USA2 | Is use of selective acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 s) | O | O |

###### Use of Request Acknowledgement

| Index | Request of Acknowledgement | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| ROA1 | Is use of request of acknowledgement proposed in CR TPDUs ? | 6.5.5 t) | O | O |
| ROA2 | Is use of request of acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 t) | O | O |

##### Error Handling

Note.— Using Class 4 over CLNS, a TPDU with an invalid checksum will be discarded.

###### Action on Detection of a Protocol Error

| Index | Item | ISO/IEC 8073 References | ISO Allowed Values | ATN Supported Values |
| --- | --- | --- | --- | --- |
| PE4L | Class 4 over CLNS | 6.22.2.3 | C4L: ER, DR, Discard | C4L: ER, DR, Discard |

Note.— The choice of action (DR, Discard) is an implementation choice and may depend on the type of error encountered.

###### Actions on receipt of an invalid or undefined parameter in a CR TPDU

| Index | Event | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| RR1 | A parameter not defined in ISO/IEC 8073 shall be ignored | 13.2.3 | M | M |
| RR2 | An invalid value in the alternative protocol class parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR3 | An invalid value in the class and option parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR4 | On receipt of the additional option selection parameter bits 8 to 7, and bits 6 to 1 if not meaningful for the proposed class, shall be ignored | 13.3.4 g) | M | M |
| RR6 | On receipt of the class option parameter bits 4 to 1 if not meaningful for the proposed class shall be ignored | 13.3.3 | M | M |

| Index | Event | ISO/IEC 8073 Reference | ISO Allowed Value | ATN Supported Value |
| --- | --- | --- | --- | --- |
| RR7 | What action is supported on receipt of a parameter defined in ISO 8073 (other than those covered above) and having an invalid value ? | 13.2.3 | Ignore, Protocol Error | Ignore, Protocol Error |

Note.— The choice of action (Ignore, Protocol error) is an implementation choice and may depend on the type of error encountered.

###### Actions on receipt of an invalid or undefined parameter in a TPDU other than a CR TPDU

| Index | Event | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| U11 | A parameter not defined in ISO/IEC 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U12 | A parameter which has an invalid value as defined in ISO/IEC 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U13 (class 4 only) | A TPDU received with a checksum which does not satisfy the defined formula shall be discarded | 6.17.3 | M | M |

##### Class 4 Timers and Protocol Parameters

| Index |  | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| TA1 | T1 (Local Retransmission) | 12.2.1.1.4 | M | M |
| TA2 | N (Maximum Transmission) | 12.2.1 | M | M |
| TA3 | IL (Local Inactivity Time) | 12.2.1.1.7 | M | M |
| TA4 | W (Window Update) | 12.2.1 | M | M |
| TA5 | L (Frozen Reference Time) | 12.2.1.1.6 | M | M |

| Index |  | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| ATN-TA1 | R (Persistence) | 12.2.1.1.5 | O | O |
| ATN-TA2 | MLR (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA3 | MRL (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA4 | ELR (Maximum Transit Delay) | 12.2.1.1.2 | O | O |
| ATN-TA5 | ERL (Maximum Transit Delay) | 12.2.1.1.2 | O | O |
| ATN-TA6 | AL (Acknowledgement Time) | 12.2.1.1.3 | O | M |
| ATN-TA7 | AR (Acknowledgement Time) | 12.2.1.1.3 | O | M |
| ATN-TA8 | IR (Remote Inactivity Time) | 12.2.1.1.7 | O | M |

Note. — According to ISO, the following applies to an implementation under test (IUT):

| Index |  | ISO/IEC 8073 References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| OT9 | Does IUT support optional timer TS2 when operating in class 4? | 6.22.2.3 | O | O |

## Connectionless Mode Transport Protocol Operation

### Connectionless Mode Transport Protocol Overview

#### ATN End Systems shall implement the ISO/IEC 8602 transport protocol in order to provide Connectionless Mode communications over the ATN Internet.

Note.— The ATN CLTS model conforms to the service model defined in ISO/IEC 8072.

#### The CLTP shall operate over the CLNS provided by the ATN Network Layer, according to the provisions in 5.5.3.5.

### Connectionless Mode Transport Service Primitives

Note 1. — For the purposes of describing the notional interfaces between different OSI protocol layers, each protocol layer is assumed to provide a service to the next higher protocol layer. The assumed service provided by the ATN Transport Layer to its user is described in ISO/IEC 8072.

Note 2. — ISO/IEC 8072 limits CL user-data to a maximum of 63488 octets per TSDU.

Note 3. — There is no requirement to implement the service specified in ISO/IEC 8072 as a software interface.

#### T-UNITDATA Request

##### The source and destination Transport Addresses shall conform to the ATN Transport Layer Addressing provisions as specified in Chapter 5.4.

#### T-UNITDATA Indication

Note. — All of the associated parameter values are equal to the values passed to the TS provider via the T-UNITDATA Request primitive, except possibly the QoS parameter values.

### ATN Connectionless Mode Transport Quality of Service Parameters

#### General

##### The Transport Layer shall support the use  of checksums on a per TSDU (Transport Service Data Unit) basis.

Note . — The actual use of this feature will be dependant upon the application’s requirements.

#### Priority

##### The transport entity providing the connectionless mode transport service shall allow a TS-user to specify TSDU priority in the range [0-14].

Note.— The CLTP itself does not support a priority field in the TPDU.

#### Security

##### The transport entity providing the connectionless mode transport service shall allow a TS-user to specify the ATN Security Label in the T-UNITDATA request.

Note.— The CLTP itself does not support a security parameter field in the TPDU.

### Encoding of Transport Protocol Data Units

#### The encoding of TPDUs shall conform to ISO/IEC 8602 for the CLTP.

### Use of the ATN Network Service

Note.— This section specifies how the CLTP operates over the CLNS provided by the ATN Network Layer.

#### Use of the N-UNITDATA Request

##### General

###### The Transport Layer shall use the N-UNITDATA Request Primitive, as defined in ISO/IEC 8602, to transmit TPDUs.

##### NS-user-data

Note.— Transport Entities transmit TPDUs as NS-user-data of the N-UNITDATA Request primitive.

##### Network Service Access Point Addresses

Note. — The Transport Layer has knowledge of source and destination address parameters only as octet strings.

##### Network Quality of Service

###### General

The transport entity providing the connectionless mode transport service shall use the network QoS parameters as defined in the sections below.

###### Network Layer Priority

The transport entity providing the connectionless mode transport service shall use the network priority parameter to indicate the relative priority of an NSDU. The NSDU priority shall be determined from the TSDU priority, using the mapping given in Table 1.2-2.

###### Network Layer Security

The transport entity providing the connectionless mode transport service shall use the Security Label provided in the T-UNITDATA request as the value of the N-UNITDATA security parameter.

#### Use of the N-UNITDATA Indication

Note.— Following ISO/IEC 8602, the Transport Layer receives TPDUs from the Network Layer-provided N-UNITDATA Indication primitive.

##### Network Quality of Service

###### To meet the ISO/IEC 8072 service specification, the transport entity providing the connectionless mode transport service shall translate the received NSDU priority to the TSDU priority using the mapping shown in Table 1.2-2.

### ATN Connectionless Mode Transport APRL

Note. — The requirements for the CLTP are provided in the form of an APRL. The APRL has been prepared using the PICS proforma provided with ISO/IEC 8602.

#### General

##### An implementation of the ISO/IEC 8602 Transport Protocol shall be used in an ATN End System if and only if its PICS is in compliance with the APRL provided with these SARPs.

#### Protocol Implementation

| Item |  | Protocol Function Support |  | ISO/IEC 8602 Reference |  | ISO Status |  | ATN Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NS |  | Network service selection |  | 5.3.2.2 |  | M |  | M |
| AM |  | Address mapping |  | 5.3.2.3 |  | M |  | M |
|  |  | PDU Support |  |  |  |  |  |  |
| UD1 |  | Unitdata PDU supported on transmission |  | 6.1.3 |  | M |  | M |
| UD2AM |  | Unitdata PDU supported on reception |  | 6.1.3 |  | M |  | M |
|  |  | Parameters of the Unitdata PDU on Transmission |  |  |  |  |  |  |
| TpTc |  | <t> TPDU UD Checksum |  | 6.2.4.1 |  | O |  | M |
| TpTs | <t> TPDU UD Source Transport-Selector |  | 6.2.4.1 |  | M |  | M |  |
| TpTd |  | <t> TPDU UD Destination Transport-Selector |  | 6.2.4.1 |  | M |  | M |
| TpTu |  | <t> TPDU UD User Data |  | 6.2.4.1 |  | M |  | M |
|  |  | Parameters of the Unitdata PDU on Reception |  |  |  |  |  |  |
| TpRc |  | <r> TPDU UD Checksum |  | 6.2.4.2 |  | M |  | M |
| TpRs |  | <r> TPDU UD Source Transport-Selector |  | 6.2.4.2 |  | M |  | M |
| TpRd |  | <r> TPDU UD Destination Transport-Selector |  | 6.2.4.2 |  | M |  | M |
| TpRu |  | <r> TPDU UD User Data |  | 6.2.4.2 |  | M |  | M |
|  |  | Service Support |  |  |  |  |  |  |
| CL |  | Connectionless Mode Network Service |  | 6.2 |  | M |  | M |