# Specification of Subnetwork Dependent Convergence Functions

## Introduction

Note 1.— The purpose of a Subnetwork Dependent Convergence Function (SNDCF) is to provide the connectionless SN-Service assumed by the ATN Internet Protocols over real subnetworks.

Note 2.— The Subnetwork Service (SN-Service) provided by an SNDCF and as specified in this Chapter is provided to the ISO/IEC 8473 Internetwork Protocol and the ISO/IEC 9542 End System to Intermediate System Protocol entities.

Note 3.— The ATN Internetwork Layer, including CLNP and the routing protocols that support it, assumes this common connectionless service to be provided by all subnetworks providing communications between ATN systems.

Note 4.— Figure 5.7-1 illustrates the relationships between the SNDCFs defined in this chapter, the SN-Service that they provide to CLNP and ES-IS, and the underlying subnetworks.

Note 5.— There is no requirement to implement this service as a software interface.

Service Provided by the SNDCF

Table 5.7-1 SN-Services and Associated Parameters

| Parameter | SN-UNITDATA Request | SN-UNITDATA Indication |
| --- | --- | --- |
| SN-Source-Address | Mandatory | Mandatory |
| SN-Destination-Address | Mandatory | Mandatory |
| SN-Priority | Optional | Optional |
| SN-Quality-of-Service | Optional | Optional |
| SNS-Userdata | Mandatory | Mandatory |

Note 1. — This section specifies the assumed service provided internally by the SNDCF for the purpose of conveying Network Data PDUs between Network Entities.

Note 2. — The service to support SN-Service-Users is defined by the primitives in Table 5.7-1.

### Subnetwork Service Primitive Parameters

Note. — The following sections specify the Subnetwork Service primitive parameters.

#### Subnetwork Point of Attachment (SNPA) Addresses

Note. — The SN-Source-Address and SN-Destination-Address parameters specify the points of attachment to a public or private subnetwork(s). The SN-Source-Address and SN-Destination-Address addresses include information denoting a particular underlying subnetwork, as well as addressing information for systems attached directly to that subnetwork. SNPA values for a particular subnetwork are those specified and administered by the authority responsible for administration of that subnetwork.

#### SN-Priority

Note 1.— The SN-Priority parameter indicates the relative importance of the associated SNS-Userdata parameter, and may influence the order in which the SNS-Userdata are transferred via the real underlying subnetwork service.

Note 2.— SN-Priority values are in the range zero to fourteen, with higher values indicating higher priorities.

Note 3. — If no SN-Priority is indicated, the value zero is assumed to be the default.

Figure 5.7-1. Relationship of SNDCFs to SN-Service and underlying Subnetworks

Note 4. — Further requirements related to subnetwork priority are specified in 5.2.8.5.

#### Subnetwork Quality of Service (SNQoS)

Note 1. — The use of the SN-Quality-of-Service parameter is optional, and depends on the needs of the SN-Service-User.

Note 2. — Associated with each connectionless-mode transmission, certain measures of quality of service are requested when the SN-UNITDATA primitive action is initiated. These requested measures (or parameter values and options) are based on a priori knowledge of the service available from the subnetwork. Knowledge of the nature and type of service available is typically obtained prior to an invocation of the underlying connectionless-mode service and the information passed is a local matter.

#### Subnetwork Service Userdata

Note 1.— The SNS-Userdata contains the ISO/IEC 8473 or ISO/IEC 9542 NPDU that has to be conveyed between adjacent network entities.

Note 2. — The SNS-Userdata is an ordered multiple of octets, and is transferred transparently between the subnetwork points of attachment specified in the SNS primitive.

## SNDCF for ISO/IEC 8802-2 Broadcast Subnetworks

### The SNDCF for use with ISO/IEC 8802-2 Broadcast Subnetworks shall be implemented according to ISO/IEC 8473-2.

## SNDCF for the Common ICAO Data Interchange Network (CIDIN)

### General Considerations

Note 1.— The ISO/IEC 8473 Protocol assumes a Connectionless underlying subnetwork service.  CIDIN provides a Connectionless Mode Service which is already very close to what is required by this protocol in the ATN.

Note 2.— This SNDCF maps to level 4 of CIDIN as specified in Annex 10, Volume III.

#### The SNDCF for CIDIN shall be as specified in the following sections.

### SN-UNITDATA Request and Indication Primitives

#### These primitives shall correspond to the request to send a CIDIN message at a CIDIN entry centre and the reception of a CIDIN message at a CIDIN exit centre respectively.

#### CIDIN messages shall be sent with the "no acknowledgement" option.

Note.— CIDIN messages requested to be transported to exit addresses which are not reachable are discarded in the entry centre.

### SN Source Address

#### This address shall correspond to a CIDIN entry address in the Entry Address item.

### SN Destination Address

#### This address shall correspond to a CIDIN exit address in an Exit Address item.

### SN Quality of Service

#### A priori values for transit delay, protection against unauthorized access, cost determinants and residual error probability shall be entered as management data in the ATN system.

### SN Priority

#### The mapping between SN Priority and the CIDIN Subnetwork Priority shall be entered as management data in the ATN system.

### SNS-Userdata

#### SNS-Userdata shall be conveyed as the contents of the CIDIN message which is transported transparently by CIDIN.

Note.— The coding of the CIDIN message is code and byte independent.

## SNDCF for ISO/IEC 8208 General Topology Subnetworks

### Over ISO/IEC 8208 General Topology Subnetworks, the subnetwork service described in 5.7.2 shall be provided using either the SNDCF for ISO/IEC 8208 General Topology Subnetworks as specified in ISO/IEC 8473-3, or the Mobile SNDCF specified below in 5.7.6.

Note.— Although most ATN Ground Systems are generally expected to use the ISO/IEC 8473-3 specified SNDCF over ISO/IEC 8208 General Topology Subnetworks, Ground Systems may specify the use of the Mobile SNDCF, in order to improve the bandwidth utilisation over fixed ISO/IEC 8208 subnetworks.

### Recommendation.— Implementations using the Mobile SNDCF as specified in 5.7.6 and the LREF Compression Procedure for Ground/Ground communications, should also use the LREF optional local reference cancellation mechanism.

## SNDCF for ISO/IEC 8208 Mobile Subnetworks

### General

#### Over ISO/IEC 8208 Mobile Subnetworks, the subnetwork service described in 5.7.2 shall be provided using the SNDCF for ISO/IEC 8208 Mobile Subnetworks as specified below.

Note 1.— The SNDCF specified below is only applicable when providing the SN-UNITDATA service to ISO/IEC 8473, ISO/IEC 9542, ISO/IEC 11577 and ISO/IEC 10589 Network Layer protocols. Unpredictable behavior may result if used to support other Network Layer Entities.

Note 2.— This SNDCF supports the following Data Compression Procedures:

- Local Reference (LREF) Compression;

- The ICAO Address Compression Algorithm;

- Compression of the data stream according to ITU-T Recommendation V.42bis.

Note 3.— Optional features of LREF Compression provide for "Local Reference Cancellation" and for "maintenance of the Local Reference Directory".  The mechanism for maintaining the Local Reference Directory requires the support of the ISO/IEC 8208 Fast Select Facility.

Note 4.— A Subnetwork Connection Group is the set of virtual circuits simultaneously active between the same pair of DTEs, and which use the same subnetwork priority level, the same Data Compression Mechanisms and options, and share the same Local Reference Directory as defined in 5.7.6.3.1.

Note 5. — If a Subnetwork Connection Group already exists with the same remote DTE and the same compression mechanisms  but with a different priority than the one used by the newly established virtual circuit, this circuit may not use the Local Reference Directory of this group, as packets will not travel at the same speed on two circuits which have different priorities.

Note 6.— The supported Data Compression Mechanisms and their options are negotiated when each virtual circuit used by the SNDCF is established.  As a result of this negotiation, the virtual circuit is placed into a new Subnetwork Connection Group or is inserted in an existing Subnetwork Connection Group.  Negotiated Data Compression mechanisms and their options are applied on a Subnetwork Connection Group basis.

#### All ATN Intermediate Systems using Mobile ISO/IEC 8208 subnetworks for communication with other Intermediate Systems shall implement the LREF compression procedure.

#### Recommendation.— Implementations using this SNDCF for Air/Ground communications should only implement the LREF optional facility for local reference cancellation when the lifetime of the virtual circuits is of the same order as the flight time.

### Call Setup

#### Calling DTE Procedures

##### General

###### A Virtual Circuit shall be established either:

- dynamically, on receipt of a SN-UNITDATA request and when the SNDCF lacks a suitable virtual circuit to the NPDU’s destination supporting the required priority and QoS, or

- by the explicit intervention of Systems Management, identifying the destination SNDCF's SNPA address, priority and QoS.

###### When it has been determined that a virtual circuit is to be made available, the calling SNDCF shall establish the virtual circuit using the procedures specified in ISO/IEC 8208.

###### An ISO/IEC 8208 CALL REQUEST packet shall be sent to the DTE Address specified as the SN-Destination-Address, with the following optional user facilities and CCITT-specified DTE facilities.

Note.—  Other optional user facilities and CCITT-specified DTE facilities may be required by subnetworks. The use of these facilities is a local matter.

###### The Call Request user data shall be formatted as specified in 5..

##### The Priority Facility

###### The mapping of ATN network layer priorities to ATN mobile subnetwork priorities shall be as defined in 1.3.8 for those mobile subnetworks subject to ICAO standards.

###### For mobile subnetworks not subject to ICAO standards, the Priority Facility shall be used if the subnetwork provider supports prioritisation of Virtual Circuits and specifies the mapping of Network Service to Subnetwork Service priorities.

###### The priority value passed in the SN-UNITDATA request or indicated by the System Manager shall be mapped to priority of data on a connection, as specified by the Subnetwork Provider.

###### If the priority to gain a connection and/or priority to keep a connection is conveyed within the ISO/IEC 8208 Facility Parameter Field, these priorities shall be consistent with the priority of data on a connection, and set according to the Subnetwork Provider’s guidelines.

Note 1.—  The SNDCF is assumed to know, a priori, if a given subnetwork supports prioritisation of virtual circuits, the number of discrete priority levels supported and the relationship between the subnetwork priority and SNSDU priority.

Note 2.— The mapping between SNSDU priority and subnetwork priority is specified separately for each subnetwork type.

##### The Non-Standard default packet size Facility

###### Non-standard default packet size Facility shall be used and the value requested set to the maximum supported by the subnetwork.

##### The Fast Select Facility

###### The Fast Select Facility shall be used if supported by all Subnetwork Provider(s) in the DTE-DTE virtual path.

Note .— Airborne routers are assumed to have a priori knowledge of Fast Select support (or lack thereof) along the DTE-DTE virtual path based on each individual destination air/ground router´s DTE address.

###### No restriction on response shall be indicated.

Note 1.— This permits the responding DTE accept the call and to return up to 128 octets of user data.

Note 2.— If Fast Select is not supported, the Compression Procedures can only be negotiated by successive attempts to establish the virtual circuit requesting different combinations of Compression Procedures.

##### Call Request User Data

Note .— Call Request User Data is used to indicate which Compression Procedures are offered by the calling DTE. When the Fast Select Facility is used, Call Accept User Data is then used to indicate which Compression Procedures are accepted by the Called DTE.

###### The call user data format shall be as illustrated in Figure 5.7-2.

###### The first octet of the call user data (the Subsequent Protocol Identifier (SPI)) shall be set to Binary [1100 0001] to indicate that the virtual circuit is to be used to provide the underlying service by this SNDCF.

Note.— ISO TR 9577 provides the international register for SPI values. The value binary [1100 0001] has not been assigned by the ISO Technical Report and it is unlikely that it will be.

###### The value of the second octet (length indicator) shall be an unsigned binary number giving the number of octets in the SNDCF parameter block (from version number field up to and including (if present) the maximum number of directory entries field).

The third octet is the SNDCF version indicator and shall be set to [0000 0001] to indicate this version of the SNDCF protocol.

Figure 5.7-2 Format for Call Request User Data

###### The fourth and fifth octets shall provide the Subnetwork Connection Reference (SNCR).

###### The value encoded in this field shall be the number of virtual circuits currently established between the calling and called DTEs (i.e. the originating and the responding SNDCFs) at this call priority.

Note.— The use of the SNCR is specified in ISO/IEC 8473 for use in call collision resolution over ISO/IEC 8208 subnetworks.

###### The sixth octet shall indicate the compression techniques offered by the calling DTE, according to Table 5.7-2.

###### LREF Compression shall always be offered.

Note 1.— This specification mandates the use of the LREF Compression algorithm.  This may not be true in future versions of this specification.  Hence procedures are specified to negotiate the use of the LREF Compression on a per virtual circuit basis.

Note 2.— The decision as regards which options to offer out of those supported is otherwise a local matter.

Note 3.— Multiple compression procedures may be offered.

Table 5.7-2 Compression Options Offered Parameter

| bit number | option |
| --- | --- |
| bit 8 | Spare (S) |
| bit 7 | ICAO Address Compression Algorithm (ACA) |
| bit 6 | V.42 BIS |
| bit 5 | Maintenance/Initiation of Local Reference Directory option (M/I) |
| bit 4 | Spare (S) |
| bit 3 | Spare (S) |
| bit 2 | Local Reference (LREF) option |
| bit 1 | Local Reference Cancellation Option (CAN) supported |

###### Bit 1 of octet 6 shall only be set if bit 2 is also set.

Note. — At most, one of the the ACA or V.42bis compression algorithms can be used. However, both can be offered in the Call Request Packet when Fast Select is in use, but only one can be accepted in the Call Accept Packet.

###### Both ACA and V.42bis shall not be simultaneously offered if the Fast Select Facility is not in use.

###### When the LREF Compression algorithm is offered, i.e if bit 2 in octet six is set, then the seventh and eight octets (Maximum Directory Entries) shall indicate the maximum number of directory entries supported for the local reference (minimum size 128), as an unsigned even number.

###### The M/I bit shall be set to one by the calling SNDCF in the Call Request Packet when the calling SNDCF has identified a Subnetwork Connection Group with the called DTE, with the requested subnetwork priority and Data Compression mechanisms and options, to request that the newly established circuit shares the Local Reference Directory associated with this group.

###### The request for Local Reference directory maintenance shall only be used when the Call Request uses the Fast Select Facility and when bit 2 of the Compression Options Parameter (Local Reference compression) is set to one.

###### When the LREF compression algorithm is not offered, the seventh octet shall be the first octet of the User Data field.

###### When the LREF compression algorithm is offered , the ninth octet shall be the first octet of the User Data field.

Note.— When the fast select facility is available, the User Data field may be used to convey the ISO/IEC 9542 ISH PDU as part of the routing initiation sequence.

##### Receipt of “Call Confirm Packet”

###### Fast Select Facility In Use

When an ISO/IEC 8208 Call Confirm Packet is received from the Called DTE and the Fast Select Facility is in use, then the Calling DTE shall inspect the Call Confirm User Data (see 5.7.6.2.2.4) in order to determine which of the offered Compression Procedures have been accepted.

If the called SNDCF has accepted the call indicating that an offered Compression Procedure is not supported, then the Calling SNDCF shall maintain the virtual circuit and shall not apply this compression procedure.

If the M/I bit is set to zero in the Call Confirm User Data, then a new Subnetwork Connection Group shall be created and the newly established virtual circuit becomes the first member of that Group.

If the M/I bit is set to one in the Call Confirm User Data and the M/I bit in the preceding Call Request had also been set to one, then the newly established virtual circuit shall be inserted into the Subnetwork Connection Group identified when the Call Request was issued.

If the M/I bit is set to one in the Call Confirm User Data, and M/I bit had been set to zero in the preceding Call Request, then this is an error condition, and the call shall be cleared with an ISO/IEC 8208 Cause Code of zero, and a diagnostic code of 242 (Disconnection - incompatible information in user data).

If the length indicator in the SNDCF header of the received Call Confirm Packet exceeds two, and if the User Data field is a recognized NPDU SPI, then the User Data field contains an NPDU and the calling SNDCF shall pass this NPDU in an SN-UNITDATA indication to the appropriate ATN SN-Service user.

###### Fast Select Facility not in Use

When an ISO/IEC 8208 Call Confirm Packet is received from the Called DTE and the Fast Select Facility is not in use, then the Calling DTE shall assume that all of the offered Compression Procedures have been accepted.

##### Call Rejection by the DCE or Called DTE

###### General

Recommendation. When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic value indicating that the proposed LREF directory is too big (see Table 5.7-3), then the call should be re-attempted with the minimum directory size (see 5.7.6.3.1.3).

Note.— This is to ensure that the call is not rejected again due to the requested directory size being too big.

If the diagnostic indicates Call Collision resolution then no further attempt shall be made to re-establish the call.

Recommendation.  In all other cases, the problem should be reported to a System Manager.

Note.— Any further attempts to establish the virtual circuit are a local matter.

###### Fast Select Facility Requested

When a DCE or DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating Fast Select not Subscribed or Fast Select Acceptance Not Subscribed, then the call shall be re-attempted but without requesting the Fast Select Facility.

Note.— Some Network Service Providers may indicate the non availability of the Fast Select Facility via other diagnostic codes.

###### Fast Select Facility not in Use

Note.— In this case, when rejection by the called DTE indicates that the reject reason is due to an offered compression procedure not being supported, then the call is re-attempted without offering the rejected procedure. This is the only negotiation procedure possible when Fast Select is not available.

When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating LREF Compression not supported (see Table 5.7-3), the call shall be re-attempted without offering LREF Compression.

When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating Local Reference Cancellation not Supported (see Table 5.7-3), the call shall be re-attempted without offering Local Reference Cancellation.

When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating ACA compression not Supported (see Table 5.7-3), the call shall be re-attempted without offering the ACA.

When a DTE originated ISO/IEC 8208 Call Clearing Packet is received with a diagnostic indicating V.42bis compression not Supported (see Table 5.7-3), the call shall be re-attempted without offering V.42bis compression.

#### Called DTE Procedures

##### Incoming Call Processing

###### When an ISO/IEC 8208 Incoming Call Packet is received, the called SNDCF first shall check for a call collision.

###### If the SNDCF has an outstanding Call Request to the same DTE Address, specified as the calling DTE in this Incoming Call Packet, and the call priority and SNCR are identical, then a call collision has occurred, and the call collision resolution procedures specified in ISO/IEC 8473-3 shall be invoked to resolve the call collision.

###### The called SNDCF shall then determine whether to accept the call.

###### The call shall be rejected if any of the following conditions are true:

- The proposed ISO/IEC 8208 facility is not available;

- The proposed priority is not supported;

- The Fast Select Facility was not selected in the Incoming Call Packet and an offered compression algorithm is not supported;

- The format of the call user data is invalid;

- The version number is not supported;

- The Local Reference compression is offered and the called SNDCF does not support the proposed directory size;

- Local Policy does not permit communication with the calling DTE.

###### The call shall then be rejected using a Call Clearing Packet, with the appropriate diagnostic code, as listed in Table 5.7-3.

###### If the call is to be accepted then the Called SNDCF shall perform the ISO/IEC 8208 procedures associated with accepting a call.

##### Call Acceptance with the Fast Select Facility in Use

###### The combination of compression techniques acceptable to the SNDCF, out of those offered by the Calling SNDCF, shall be indicated by including the one-octet field shown in Figure 5.7-3, in the ISO/IEC 8208 Call Accept User Data.

###### The Called SNDCF shall not accept both the ACA and V.42bis compression algorithms simultaneously.

###### If the M/I bit is set to one in the Call Request User Data and,

- there is one and only one existing Subnetwork Connection Group with the calling DTE with the same Data Compression Procedures and options as indicated in the Call Request User Data, and the requested priority, and

- it is acceptable to share the Local Reference Directory associated with this Subnetwork Connection Group with this virtual circuit,

then the virtual circuit shall be inserted in this Subnetwork Connection Group and the M/I bit set to one in the Call Accept User Data.

###### Otherwise, a new Subnetwork Connection Group shall be created, with this virtual circuit as the first member of the group and the M/I bit set to zero in the Call Accept User Data.

Note. — By setting the M/I bit to zero, the responding SNDCF can refuse to maintain the Local Reference directory from the old virtual circuit to the new virtual circuit.  This will result in an additional Subnetwork Connection Group and , as long as one or more exists, in all further Local Reference directory maintenance requests to be rejected.

###### If the length indicator in the SNDCF header of the received incoming Call Packet exceeds seven (or nine if Local Reference Compression is offered), and if the User Data field is a recognized NPDU SPI, then the User Data field contains an NPDU and the called SNDCF shall pass this NPDU in an SN-UNITDATA indication to the appropriate ATN SN-Service user.

##### Call Acceptance without the Fast Select Facility in Use

###### If Fast Select is not in use then a call shall only be accepted if all offered compression procedures and facilities are acceptable, and the proposed LREF directory size can be supported.

Note. —  Call rejection is specified above in 5.

##### Call Accept User Data

Note.— User Data can only be present in the Call Accept packet if the Fast Select Facility is available and has been selected in the Call Request.

When Fast Select is available and has been selected in the Call Request, then a Call Accept User Data shall be present in the Call Accept packet.

Figure 5.7-3 Format for Call Accept User Data

###### The Call Accept User Data format shall be as illustrated in Figure 5.7-3.

###### The first octet of the Call Accept User Data shall identify the compression procedure(s) accepted by the called DTE.

Note. — The bit fields have the same semantics as the ones used for the sixth octet of the Call Request User Data.

###### The Second octet of the Call Accept User Data shall be the first octet of the User Data field.

Note.— The  User Data field may be used to convey the ISO/IEC 9542 ISH PDU as part of the routing initiation sequence.

#### Data Transfer Phase

##### During the data transfer phase of a virtual circuit established by this SNDCF, the Compression Procedures accepted by the called DTE shall be applied to each NPDU transferred over the virtual circuit.

Note.— NPDUs are queued for transfer as a result of an SN-UNITDATA.request. Received NPDUs are passed to the SN-Service user by an SN-UNITDATA.indication.

##### The order in which concurrently applied Compression Procedures and ISO/IEC 8208 segmentation are applied shall be as follows:

- If the LREF compression algorithm is used, it shall be applied to the ISO/IEC 8473 PDU first;

- If either of the ACA or V.42bis compression algorithms is used, it shall be applied after LREF compression and before M-bit segmentation;

- Finally, if the ISO/IEC 8208 M-bit sequencing procedures are required due to the size of the PDU, then these shall be applied.

##### This sequence shall be inverted on the receiving end as follows:

- If M-bit -segmentation has been applied, then reassembly of the NPDU from the received ISO/IEC 8208 Data Packets shall be done first;

- If either of the ACA or V.42bis compression algorithms is used the corresponding decompression algorithm shall be applied after M-bit segmentation and before LREF compression;

- Finally if the LREF compression is used, the LREF decompression algorithm shall then be applied.

#### Call Clearing

##### The SNDCF shall clear a virtual circuit when:

- System Management requests call clearing, or

- On the expiration of a timeout period following the transmission or receipt of SN-UNITDATA, or

- If the resources are required by another virtual circuit with a higher priority.

##### Items b) or c) above shall only apply to those virtual circuits that have been established following an SN-UNITDATA.request.

##### When it has been determined that a virtual circuit is to be cleared, the SNDCF shall invoke the ISO/IEC 8208 functions associated with call clearing.

##### All packets subsequently received other than a Clear Confirm or a Clear Indication shall be ignored.

##### The same actions shall apply to the receipt of a Clear Indication.

##### The Clearing Cause octet in the ISO/IEC-8208 Cause/Diagnostic field shall be set to [1000 0000].

##### The reason for clearing the call shall be placed in the Diagnostic field.

Note 1.— The diagnostic values listed in Table 5.7-3 shall be used when an SNDCF rejects a Call Request or clears a virtual circuit.

Note 2.— If a virtual connection is cleared due to a network problem, the SNDCF may attempt to re-establish the connection before the associated forwarding information is removed from Network Layer routing tables. The selective re-establishment of X.25 connections may be based on the originating Clearing Cause and Diagnostic Codes.

### Local Reference Compression Procedures

#### Local Directory Initialization

##### Both calling and called SNDCFs shall create a local directory to be associated with each newly established Subnetwork Connection Group.

##### This directory shall consist of entries numbered from zero to a maximum of 32767, each entry consisting of:

- A pair of NSAP Addresses, known as the inward and outward NSAP Addresses respectively;

- The ISO/IEC 8473 protocol version number;

- The value of the security options parameter which may be empty.

##### The directory shall be initially empty. The Mobile SNDCF shall support a minimum directory size of 128 entries.

Action following an SN-UNITDATA Request

Table 5.7-3 Diagnostics values for ATN call clearing

|  | Hexadecimal value | Decimal value | Clearing Cause |
| --- | --- | --- | --- |
| 1 | 1111 1001 | 249 | Connection Rejection - unrecognized protocol identifier in user data |
| 2 | 1000 0000 | 128 | Version number not supported |
| 3 | 1000 0001 | 129 | Length field invalid |
| 4 | 1000 0010 | 130 | Call Collision Resolution |
| 5 | 1000 0011 | 131 | Proposed Directory Size too large |
| 6 | 1000 0100 | 132 | Local Reference Cancellation Not Supported |
| 7 | 1000 0101 | 133 | Received DTE refused, received NET refused or invalid NET selector |
| 8 | 1000 0110 | 134 | Invalid SNCR field |
| 9 | 1000 0111 | 135 | ACA compression not supported |
| 10 | 1000 1000 | 136 | LREF compression not supported |
| 11 | 1000 1111 | 143 | V42bis compression not supported |
| 12 | 1111 0000 | 240 | System lack of resources |
| 13 | 0000 0000 | 0 | Cleared by System Management |
| 14 | 1001 0000 | 144 | Idle Timer expiration |
| 15 | 1001 0001 | 145 | Need to re-use the circuit |
| 16 | 1001 0010 | 146 | By local means (to be used for system local error) |
| 17 | 1001 0011 | 147 | Invalid SEL field value in received NET |

##### General

###### On receipt of a SN-UNITDATA request the SNDCF shall identify an appropriate virtual circuit to the subnetwork user associated with the SN-Destination-Address, and which satisfies the PDU Priority and Security requirements, and queue the accompanying PDU (i.e. the user data associated with the SN-UNITDATA request) for transfer over that virtual circuit.

###### If there is no virtual circuit which satisfies the PDU Priority and Security requirement, then the SNDCF shall try to establish a virtual circuit with the requested PDU Security and priority.

###### If a suitable virtual circuit can be established, then the PDU shall be queued for transfer over the newly established virtual circuit. If no such virtual circuit can be established, then if an existing virtual circuit associated with the SN-Destination-Address provides an adequate level of security and priority, the PDU shall be queued for transfer over the existing virtual circuit.

###### Otherwise, the PDU shall be discarded.

Note 1.— The opening of an additional virtual circuit for this purpose may be inappropriate in certain cases. For example, opening an additional virtual circuit via a single frequency VDL subnetwork or via the Mode S subnetwork will not necessarily result in increased capacity.

Note 2.— The maintenance of the minimum QoS level includes ensuring that the number of local references that are required to support the number of data streams multiplexed over a given virtual circuit does not exceed the number available.

###### If no virtual circuit exists to the SN-Destination-Address, and the circuit is not classified as dynamically assigned by the ISO/IEC 10589 (IS-IS) routing protocol or under a static routing regime, then the SN-UNITDATA shall be discarded, with an error report sent to a System Manager.

Note.— Virtual Circuits between Intermediate Systems and between Intermediate Systems and End Systems are initially established by procedures associated with the specific routing procedures employed. If no such virtual circuit has been established, or may be established under the routing procedures, then no route exists and hence it is an error if an attempt is made to send a PDU over such a route.

##### Identification of Network Layer Protocol

###### Prior to transmission of an SN-UNITDATA SN-Userdata parameter over a virtual circuit, the SNDCF shall inspect the initial octet of the SN-Userdata parameter (Initial Protocol Identifier (IPI)) to identify the Network Layer protocol contained within the SN-UNITDATA request.

###### If the IPI contains binary [1000 0001] indicating ISO/IEC 8473, then the procedures in 5.7.6.3.2.3 shall be performed.

###### If the IPI contains binary [1000 0010] indicating ISO/IEC 9542 (ES-IS), binary [1000 0011] indicating ISO/IEC 10589 (IS-IS), or binary [0100 0101] indicating ISO/IEC 11577 (NLSP), then the packet shall be sent unchanged over the virtual circuit, using the M-bit segmentation mechanism, if the packet is larger than the maximum length of user data permitted for the virtual circuit.

###### If the IPI contains any other value, the SN-UNITDATA request shall be discarded, and an error sent to a System Manager.

Note.— The IPI designating the ISO/IEC 11577 has been included in the set of allowed IPIs in order to preserve the possiblity for use of this protocol in the future. However, at the time of publication of this specification, no ATN Security Protocol Architecture has been defined. Thus, this inclusion of the NLSP IPI in the allowed IPI set does not indicate that NLSP will be incorporated into the future ATN security architecture.

##### Identification of Option Parameter and Local Directory Look-up

###### The options part of the ISO/IEC 8473 NPDU header contained in the SN-Userdata shall then be inspected. If one of the following is true:

- Source Routing option is present,

- Recording of Route option is present,

- QoS Maintenance option is anything other than the globally unique format,

- padding option is present,

- priority option is present with a value greater than 14,

- an unknown parameter is present,

then the SN-Userdata shall be sent unchanged over the virtual circuit using M-bit segmentation procedures as appropriate.

###### Otherwise, the local directory associated with the virtual circuit shall then be interrogated to determine if an entry exists such that:

- the inward NSAP Address is equal to the PDU's source NSAP Address;

- the outward NSAP Address is equal to the PDU's destination NSAP Address;

- a security parameter is present with the same value as that contained in the PDU header, if present, and otherwise absent;

- the same ISO/IEC 8473 version number as is present in the PDU header.

###### If an entry is found, then the NPDUshall be sent in the compressed form constructed according to 5., using the local directory entry number as the local reference.

###### If no entry is found, then a new directory entry shall be created and the SN-Userdata shall be modified as specified in 5.7.6.3.2.4.

##### Establishing a New Local Reference

###### A new directory entry shall be created containing the NPDU source NSAP Address as the inward NSAP Address, and the NPDU destination NSAP Address as the outward NSAP Address.

###### The value of the protocol version number, and the security parameter, if present, shall also be placed in this entry.

###### The entry number shall have the lowest possible entry number that has not previously been used for the local directory associated with this virtual circuit, and shall be in the range [0..63] or [128..16447] if the SNDCF is the initiator of the first virtual circuit in a Subnetwork Connection Group, or [64..127] or [16448..32767], if the SNDCF is the responder for such a virtual circuit.

###### When a directory size greater than 128 but less than 32767 has been negotiated, then the highest local reference that the initiator may allocate shall be:

127 + (n -128) / 2

and the highest local reference that the responder may allocate shall be

16447 + (n -128) / 2

where 'n' is the agreed maximum directory size.

###### If a directory full condition occurs then, as a local matter, either the PDU shall be sent unmodified over the virtual circuit or the virtual circuit shall be reset.

Note.— A user generated Network Reset results in the total clearing of the directory which then permits the assignment of an unused local reference.

###### Recommendation.— When this SNDCF is used for Air/Ground communication or when the local reference cancellation option is available for use, then the PDU should be sent unmodified over the virtual circuit.

###### The PDU, which may be either a DT PDU or an ER PDU, shall have an additional options field added to the PDU header.

###### This option parameter shall have local significance only (i.e. is only of interest to the sending and receiving SNDCFs), and is called the Local Reference.

###### This Local Reference option parameter shall be included as the first parameter in the Option Part of the DT or ER PDU header.

###### This option shall be specified as follows:

| Parameter Code: | [0000 0101] |
| --- | --- |
| Parameter Length: | variable |
| Parameter Value: | the entry number of the local directory entry created above and expressed as an unsigned integer. |

Note . — The entry number is therefore assigned as a so called Local Reference.

###### The Checksum, Length Indicator, and Segment Length fields of the PDU header shall be modified to reflect the insertion of the new options field, and any changes to the length of the source and destination address.

###### The Total Length, if present, shall be left unmodified.

##### Reference Cancellation Option

###### When the optional Local Reference Cancellation facility is implemented, and both SNDCFs using a virtual circuit have indicated that they support this facility, then the SNDCF shall monitor the number of local references on each virtual circuit which it has both assigned and are in use.

###### When the number of such local references on a given virtual circuit exceeds a System Manager specified threshold, then the local reference cancellation procedures specified in 5. shall be invoked, in order to ensure that the number of unused local references in the range in which the SNDCF is permitted to assign local references, is at least equal to a System Manager specified target.

##### Transfer of the Modified ISO/IEC 8473 PDU

###### The modified ISO/IEC 8473 NPDU (i.e. the NPDU with the added Local Reference Option) shall be inserted in the User Data field of an ISO/IEC 8208 Data packet and shall be sent over the virtual circuit, using the ISO/IEC 8208 M-bit segmentation procedure if appropriate.

#### Compression of SN-Userdata

##### General

###### An Initial DT NPDU shall be compressed according to the procedures specified in 5..

###### A Derived DT NPDU shall be compressed according to the procedures specified in 5..

###### An ER NPDU shall be compressed according to the procedures specified in 5..

##### Initial DT PDU Compression

###### General

Note.— An Initial DT PDU is an ISO/IEC 8473 DT PDU that either contains no Segmentation Part in its PDU header or contains a Segmentation Part with a Segment Offset value that equals zero and the Segment Length is equal to the Total Length.

The original Initial DT PDU shall be compressed into the Compressed Initial Data PDU as shown in Figure 5.7-4.

The fields of the Compressed Initial Data PDU shall be set as follows.

###### Type Field

The PDU Type field value shall be set according to the values of the original Initial DT PDU ER, SP and More Segments (MS) flags as defined in Table 5.7-4.

PDU Priority Field

Table 5.7-4 Initial DT PDU PDU Type codes

| PDU Type Values | CLNP NPDU ER Value | CLNP NPDU SP Value | CLNP NPDU MS Value |
| --- | --- | --- | --- |
| 0 0 0 0 | 0 | 0 | 0 |
| 0 0 0 1 | 0 | 1 | 0 |
| 0 0 1 0 | 1 | 0 | 0 |
| 0 0 1 1 | 1 | 1 | 0 |

The PDU Priority field value shall be set to the lowest four bits of the original PDU Priority parameter value field, if the Priority option is present, and set to zero otherwise.

###### PDU Lifetime Field

The PDU Lifetime field value shall be set to the eight bits of the original NPDU lifetime field.

###### P bit Field

The P field value shall be set to one if the original uncompressed PDU contained the priority option. This field shall be set to zero otherwise.

###### Q bit Field

The Q field value shall be set to one if the original uncompressed PDU contained the QoS Maintenance option.

This field shall be set to zero otherwise.

###### R bit Field

The R field value shall be set to one if the original uncompressed PDU contains a non-zero checksum.

This field shall be set to zero otherwise.

###### S/T, CE , T/C, E/T, and E/C Fields

The values of these fields shall be set to bits 5 through 1 of the QoS parameter value option field of the original PDU, if the Quality of Service maintenance option is present.

The S/T field shall be set to the value of bit 5 of the Quality of Service Maintenance parameter value field, if present (i.e. sequencing vs. transit delay) and set to zero otherwise.

The CE  field shall be set to the value of bit 4 in the Quality of Service Maintenance parameter value field.

The T/C field shall be set to the value of bit 3 in the Quality of Service Maintenance parameter value field.

Figure 5.7-4 Compressed Initial and Derived PDU Formats

The E/T field shall be set to the value of bit 2 in the Quality of Service Maintenance parameter value field.

The E/C field shall be set to the value of bit 1 in the Quality of Service Maintenance parameter value field.

###### EXP, Local-REF/A and Local-REF/B Fields

If the value of the Local Reference determined according to the procedure specified in 5. is less than 128, then the EXP field shall be set to zero.

In this case, only the Local-REF/A field shall be present in the PDU.

The Local-REF/A field value shall be set to the value of the Local Reference encoded as an unsigned integer.

If the value of the Local Reference is greater than or equal to 128, the EXP field shall be set to one, and both Local-REF/A and Local-REF/B fields shall be present in the PDU.

The Local Reference shall be encoded as a 15 bit unsigned integer, with the least significant eight bits placed in the Local-REF/B field, and the most significant seven bits placed in the Local-REF/A field.

###### PDU Identifier

If the Initial DT PDU allows segmentation (SP Flag is set to one), then the PDU Identifier field shall be included in the Compressed Initial Data PDU.

The PDU Identifier field shall contain the Data Unit Identifier as provided in the segmentation part of the Initial DT PDU.

If the Initial DT PDU does not allow segmentation (SP Flag is set to zero), then this field shall not be included in the Compressed Initial Data PDU.

###### PDU Segment Offset

This field shall not be present in the Compressed Data PDU for an Initial DT PDU.

Note. — The segment offset of an Initial DT PDU is always zero and is a priori known by the receiving SNDCF.

###### PDU Total Length

This field shall not be present in the Compressed Data PDU for an Initial DT PDU.

Note. — The Total Length field value of an Initial DT PDU is the length of the entire PDU in octets. This value is identical to the value of the Segment Length field for an Initial DT PDU and both values may be recalculated by the receiving SNDCF.

###### Network Service Data Unit Field

This field shall contain the Data Part of the original Initial DT PDU.

##### Derived DT PDU Compression

###### General

The original Derived DT PDU shall be compressed into the Compressed Derived Data PDU as shown in Figure 5.7-4.

The fields of the Compressed Derived Data PDU shall be set as defined in the following sections.

###### Type Field

The PDU Type field value shall be set according to the values of the original NPDU ER, SP and MS flags as defined in Table 5.7-5.

PDU Priority Field

Table 5.7-5 Derived PDU Type Codes

| PDU Type Values | CLNP NPDU ER Value | CLNP NPDU SP Value | CLNP NPDU MS Value |
| --- | --- | --- | --- |
| 0 1 1 0 | 0 | 1 | 0 |
| 0 1 1 1 | 0 | 1 | 1 |
| 1 0 0 1 | 1 | 1 | 0 |
| 1 0 1 0 | 1 | 1 | 1 |

This field shall be set as defined in 5..

###### PDU Lifetime Field

This field shall be set as defined in 5..

###### P bit Field

This field shall be set as defined in 5..

###### Q bit Field

This field shall be set as defined in 5..

###### S/T, CE , T/C, E/T, and E/C Fields

These fields shall be set as defined in 5..

###### EXP, Local-REF/A and Local-REF/B Fields

These fields shall be set as defined in 5..

###### PDU Identifier Field

The PDU Identifier field value shall be set to the Data Unit Identifier contained in the segmentation part of the original Derived DT PDU header.

###### PDU Segment Offset Field

The PDU Segment Offset field value shall be set to the Segment Offset value contained in the segmentation part of the original Derived DT PDU header.

###### PDU Total Length Field

The PDU Total Length field value shall be set to the value of the Total Length field contained in the Segmentation Part of the original Derived DT PDU.

##### Error Report PDU Compression

###### General

The original ER PDU shall be compressed into the Compressed Error Report PDU as shown in Figure 5.7-5.

The fields of the Compressed Error Report PDU shall be set as defined in the following sections.

###### PDU Type Field

The PDU Type field value shall be set to [1101].

###### PDU Priority Field

This field shall be set as defined in 5..

###### PDU Lifetime Field

This field shall be set as defined in 5..

###### P bit Field

This field shall be as defined in 5..

###### Q bit Field

This field shall be set as defined in 5..

###### S/T, CE , T/C, E/T and E/C Fields

These fields shall be set as defined in 5..

EXP, Local-REF/A, Local-REF/B Fields

Figure 5.7-5 Compressed Error Report PDU

These fields shall be set as defined in 5..

###### Discard Reason Field

This field shall be set to the value of the Reason for Discard Parameter Value field contained in the original NPDU header.

###### Header of Discarded NPDU Field

This field shall contain the value of the Error Report Data Part if provided in the original Error Report PDU.

###### Transfer of Compressed ISO/IEC 8473 PDUs

The compressed ISO/IEC 8473 NPDU (i.e. Compressed Initial Data PDU, Compressed Derived Data PDU, or Compressed Error Report PDU) shall be inserted in the User Data field of an ISO/IEC 8208 Data packet and shall be sent over the virtual circuit, using the ISO/IEC 8208 M-bit segmentation procedure if appropriate.

#### Processing of Packets Received from the Subnetwork Service Provider

Note . — The following sections specify the processing of packets received from the Subnetwork Service provider.

##### Initial Processing of NPDU

###### On receipt of an incoming packet received from a virtual circuit, the SNDCF shall inspect the first octet to determine the Network Layer Protocol ID or the compressed PDU type (see Table 5.7-6).

- If this value is set to [1000 0001] indicating that the NPDU is an ISO/IEC 8473 NPDU with an uncompressed header, then the NPDU shall be processed according to 5..

- If the first octet indicates either ISO/IEC 9542 (ES-IS), ISO/IEC 11577 (NLSP) or ISO/IEC 10589 (IS-IS), the SNDCF shall generate an SN-UNITDATA.indication with the NPDU as its SN-Userdata parameter, and the SN-Source-Address and SN-Destination-Address parameters set to the remote and local DTE addresses for the virtual circuit over which the NPDU was received.

- If the value of the first four bits of the first octet is in the range binary [0000] to binary [0011] then the PDU is a compressed ISO/IEC 8473 Initial DT PDU which shall be decompressed using the procedures specified in 5..

- If the value of the first four bits of the first octet is in the range binary [0110] to binary [1010] (excluding 1000) then the PDU is a compressed ISO/IEC 8473 Derived PDU, which shall be decompressed using the procedures specified in 5..

- If the value of the first four bits of the first octet is binary [1101] then the PDU is a compressed ISO/IEC 8473 Error PDU, which shall be decompressed using the procedures specified in 5..

- If the value of the first four bits of the first octet is binary [1110] then the PDU is an SNDCF Error Report, which shall be processed according to the procedures of 5., and no SN-UNITDATA.indication generated.

- If the value of the first four bits of the first octet is binary [0100] or binary [0101], then the PDU is respectively, a local reference cancellation request or response, which shall be processed according to the procedures of 5. and no SN-UNITDATA.indication generated.

In all other cases, the PDU shall be discarded and an SNDCF Error Report Generated (see 5.).

Table 5.7-6 Mapping between Compressed PDU Type Fields and Uncompressed PDU Types

| Compressed PDU Type Field | Uncompressed PDU Type |
| --- | --- |
| [0000] - [ 0011] | Compressed Initial DT PDU |
| [0110] - [0111] [1001] - [1010] | Compressed Derived DT PDU |
| [1101] | Compressed Error Report PDU |
| [1110] | SNDCF Error Report |
| [0100] | Cancellation Request PDU |
| [0101] | Cancellation Accept PDU |

##### Incoming ISO/IEC 8473 PDU with Uncompressed Header

###### General

If the received NPDU is an ISO/IEC 8473 NPDU then the options part shall be inspected for the options field containing the local reference.

###### Processing of Unmodified ISO/IEC 8473 PDUs

If the local reference option is not present, then the SNDCF shall generate a SN-UNITDATA indication with the NPDU as its SN-Userdata, and the SN-Source-Address and SN-Destination-Address parameters set to the remote and local DTE addresses for the virtual circuit over which the NPDU was received.

###### Processing of Modified ISO/IEC 8473 PDUs

If the Local Reference option is present, it shall be removed, and the checksum and PDU header length indication and segment length shall be modified to reflect this removal.

If a Local Reference options field is present, then the local directory associated with the virtual circuit over which the PDU was received shall be inspected for the presence of the corresponding entry.

If no such entry is present, and the value of the Local Reference is in the range within which the remote SNDCF is permitted to create local directory entries, then the entry shall be created, and:

- The value of the inward NSAP Address set to the PDU's destination NSAP Address,

- The value of the outward NSAP Address set to the NSAP's source NSAP Address, and

- The values of the Version Number and Security Parameter, set to the corresponding values in the PDU header.

An SNDCF Error Report (see 5.) shall be generated if the value of the Local Reference is not within the range within which the remote SNDCF is permitted to create local directory entries, or is greater than the maximum negotiated when the call was established.

Otherwise, the local directory entry shall be compared with the received PDU. If:

- The inward NSAP Address does not match the destination NSAP Address, or

- The outward NSAP Address does not match the source NSAP Address, or

- The Version Number does not match the Version Number present in the directory entry, or

- The value of the Security options parameter does not match the value in the directory, or is not correspondingly absent, then

an SNDCF Error Report shall be generated and returned over the same virtual circuit as the PDU was received.

The SNDCF shall then generate a SN-UNITDATA.indication with the NPDU as its SN-Userdata, and the SN-Source-Address and SN-Destination-Address parameters set to the remote and local DTE addresses for the virtual circuit over which the NPDU was received.

##### Incoming Compressed Data PDU

###### General

If the most significant four bits of the first octet of a received PDU (i.e. the PDU Type field) are in the range [0000] to [0011] binary, excluding [1000], then the packet is a compressed ISO/IEC 8473 Initial DT NPDU.

If the PDU Type field of a received compressed PDU is in the range [0110] to [1010] binary, then the PDU is a compressed ISO/IEC 8473 Derived DT NPDU.

Upon receipt, the SNDCF shall examine and validate the Local-REF in the compressed PDU.

The value of the Local Reference shall be extracted from the compressed header and the corresponding entry in the local directory located.

If no entry exists corresponding to the Local-REF present in the PDU, then an SNDCF Error Report shall be generated and returned over the same virtual circuit as the PDU was received, and the PDU shall be discarded.

If the Local-REF is valid, the original uncompressed NPDU shall be recreated by the procedures defined in 5. through 5..

The SNDCF then shall generate a SN-UNITDATA.indication with the SN-Source Address and SN-Destination Address parameters set to the remote and local DTE addresses for the virtual circuit over which the NPDU was received, and the SN-Userdata shall be set to the uncompressed DT NPDU.

###### Fixed Part

Note 1. — The Fixed Part of the NPDU header consists of the Network Layer Protocol Identifier, Length Indicator, Version/Protocol Identifier Extension, PDU Lifetime, SP flag, MS flag, E/R Flag, Type, Segment Length and Checksum fields as defined in ISO/IEC 8473.

Note 2.— If the EXP field is set to zero, the Local Reference is the seven bit integer value of the Local-REF/A field. If the EXP field is set to one, the Local Reference value consists of the fifteen bit unsigned integer as stored with the least significant eight bits placed in the Local-REF/B field, and the most significant seven bits placed in the Local-REF/A field.

Network Layer Protocol Identifier

This field shall be set to binary [1000 0001] to identify this Network Layer Protocol as ISO/IEC 8473.

Length Indicator

This field shall be set to the length of the uncompressed NPDU header in octets.

Version/Protocol Identifier Extension

The Version/Protocol Identifier Extension field shall be set to the values provided in the corresponding entry of the local directory.

PDU Lifetime

The eight bits of the PDU Lifetime field shall be set to the eight bits of the PDU Lifetime field of the Compressed Data PDU.

Segmentation Permitted, More Segments, Error Report Flags

The values of these flags shall be derived from the value of the Protocol ID field and Type field of the Compressed Data PDU.

These flag values shall be determined according to Table 5.7-4 for an Initial Data PDU and Table 5.7-5 for a Derived Data PDU.

Type Code

This field shall be set to binary [11100] to indicate a DT PDU.

Segment Length

This field shall indicate the entire length in octets of the PDU, including both header and data.

The value of this field shall be computed by the SNDCF.

For an Initial DT NPDU, the value of this field shall be identical to the value of the Total Length field located in the Segmentation Part of the header.

PDU Checksum

The value of this field shall be set to zero if the R bit in the compressed header is zero.

Otherwise, a Checksum field shall be recomputed.

Note.— For the DT PDU, this includes the segmentation and options part (if present). For the Error Report PDU, this includes the reason for discard field as well.

###### Address Part

Note.— The Address Part consists of the Destination Address Length Indicator, Destination Address, Source Address Length Indicator and Source Address as defined in ISO/IEC 8473.

Destination and Source Address Length Indicators and Addresses

The Source and Destination NSAP addresses shall be set to the values provided in the corresponding entry of the local directory for the Local Reference number calculated.

The source NSAP Address shall be set to the value of the outward NSAP Address, and the destination NSAP Address set to the value of the inward NSAP Address.

The Length fields shall contain the length of each address in octets.

###### Segmentation Part

General

If the ISO/IEC 8473 SP field is set to one, then the Segmentation Part shall be generated.

The Segmentation Part shall consist of the Data Unit Identifier, Segment Offset, and Total Length field as defined in ISO/IEC 8473.

Data Unit Identifier

This field shall contain the value of the PDU Identifier field as provided in the compressed DT PDU.

Segment Offset

For an Initial DT PDU, this field shall be set to zero.

For a Derived DT PDU, this field shall be set to the PDU Segment Offset field as provided in the compressed DT PDU.

PDU Total Length

For a Derived DT PDU, this field shall contain the value of the PDU Total Length field as provided in the Compressed DT PDU.

For an Initial PDU, the entire length of the PDU in octets shall be calculated by the SNDCF and stored in this field.

###### Options Part

General

If the Q bit field is set to one, the Globally Unique QoS option shall be recreated according to 5..

If the Security option is present in the local reference directory entry, the Security option shall be recreated according to 5..

If the P bit field is set to one, the Priority option shall be recreated according to 5..

Priority

For the Priority option, the Parameter Code shall be set to binary [1100 1101] and the Parameter Length set to one octet.

The four most significant bits of the Parameter Value shall be set to zero, and the four least significant bits set to the PDU Priority field as provided in the compressed DT PDU.

Quality of Service Maintenance

For the Quality of Service Maintenance option, the Parameter Code shall be set to binary [1100 0011], the Parameter Length set to one octet.

The high order two bits of the Parameter Value shall be set to binary [11] to indicate Globally Unique, bit 6 shall be set to zero, and bits 5 through one set to the S/T, CE , T/C, E/T and E/C fields respectively as provided in the compressed Data PDU.

Security

This field shall be set to the value of the Security parameter contained in the corresponding Local Reference directory entry.

###### Data Part

The Data Part shall be copied from the Compressed Data PDU data part.

##### Incoming Compressed Error Report PDU

###### General

The original uncompressed header shall be recreated as defined in the following sections.

Note.— If the four most significant bits of the first octet (the PDU Type Field) of a received packet are [1101] then the packet is a compressed ISO/IEC 8473 ER NPDU.

###### Fixed Part

The Fixed Part of the ER PDU shall be composed in the same manner as defined in 5. except for the Type Code which shall be set to binary [00001] to indicate an ER PDU, and for the SP and MS flags which shall be set to zeros.

###### Address Part

The Address Part of the ER PDU shall be composed in the same manner as defined in 5..

###### Options Part

The Options Part of the ER PDU shall be composed in the same manner as defined in 5. for an Initial DT PDU.

###### Reason for Discard

To compose this field, the Parameter Code shall be set to binary [1100 0001], the Parameter Length set to two octets, and the Parameter Value set to the Discard Reason field as provided in the Compressed Error Report PDU.

###### Error Report Data Part

If the Compressed Error Report PDU contains the Header of Discarded NPDU field, then the Error Report Data Part shall be set to the value of the Header of Discarded NPDU field.

##### Incoming SNDCF Error Report

###### On receipt of an SNDCF Error Report, the virtual circuit shall be reset (see 5.7.6.3.7), unless the reason is "compressed PDU with unrecognized local reference".

###### In this case, the directory entry corresponding to the local reference returned in the SNDCF Error Report shall be reset to the unused state.

###### Recommendation .— The error should be notified to Systems Management.

Note.— If the four most significant bits of the first octet (the PDU Type field) of an incoming packet are set to [1110], then a SNDCF Error Report has been received (see 5.).

#### SNDCF Error Report

##### The SNDCF Error Report is a packet format unique to the Mobile SNDCF, and shall be used to report errors in the use of local references as specified below.

##### The SNDCF Error Report PDU shall be constructed as follows:

- The most significant four bits (PDU Type) of the first octet are set to binary 1110, while the least significant four bits are set to 0000.

- The second octet is a discard reason encoded as an unsigned integer, with the following reason codes defined in the Table 5.7-7:

Table 5.7-7 SNDCF Error Report Diagnostic Codes

| Code | Reason |
| --- | --- |
| [0000 0000] | Compressed NPDU with unrecognized Local Reference |
| [0000 0001] | Creation of directory entry outside of sender's permitted range |
| [0000 0010] | Directory entry exists |
| [0000 0011] | Local Reference greater than maximum value accepted. |
| [0000 0100] | Data Unit Identifier missing when SP=1. |
| [0000 0101] | Reserved. |
| [0000 0111] | Compressed ISO/IEC 8473 PDU with unrecognized Type. |
| [0000 1000] | Local Reference Cancellation Error |

- The Local Reference contained in the PDU for which the error is being reported is placed in the remaining octet(s) of the SNDCF Error Report PDU Header, unless the reason is Local Reference Cancellation Error, when the SNDCF Error Report shall consist of three octets only, and the third octet shall contain the Cancellation Reference of the invalid Cancellation Request PDU.

##### The data portion of the SNDCF Error Report shall be used to return a copy of the PDU in error, similar to the ISO/IEC 8473 Error Report PDU.

##### The Error Report PDU shall be sent as an ISO/IEC 8208 DATA packet(s) and, if needed, segmented using the M-bit procedures.

#### Local Reference Cancellation Option

##### General

Note.— When the implementation of this option has been agreed by both SNDCFs using a virtual circuit during the call setup procedures, then the following procedures may be used to selectively cancel one or more Local References, i.e. make them available for re-use. An SNDCF may only request the cancellation of Local References which are within the range in which it is permitted to assign Local References.

###### When an SNDCF invokes the procedures for Local Reference cancellation it shall format a Cancellation Request PDU, as specified below, and send the PDU to the other SNDCF over the virtual circuit to which it applies.

###### A Cancellation Request PDU shall be retransmitted periodically until it is acknowledged by a cancellation accept PDU, or an SNDCF Error Report PDU is received indicating an error in the request.

###### When a Cancellation Accept PDU is received, the corresponding directory entries shall be cleared, and the Local References therefore become available for re-use.

###### When an SNDCF receives a Cancellation Request PDU, it shall first check to ensure that the local references identified in the PDU are within the range in which the sending SNDCF is permitted to assign local references.

###### If any one of them is not, then an SNDCF error report shall be returned, and the request ignored.

###### Otherwise, the directory entries corresponding to the indicated local references shall be cleared, and a cancellation accept PDU be formatted and returned, in order to accept cancellation of these local references.

##### The Cancellation Request PDU

###### The PDU format shall be as illustrated in Figure 5.7-6. The first octet shall be set to [0100 0000]. The remainder of the PDU shall consist of:

- A Cancellation Reference expressed as a one octet unsigned integer, and which uniquely identifies this Cancellation Request within the context of the virtual circuit.

Note.— In most cases uniqueness will be assured if the reference is implemented as a sequence number starting at zero and incremented by one (modulo 256), each time a Cancellation Request is sent.

- A length octet (L1) given as an unsigned integer (0 to 255), which indicates the length in octets of the set of individual Local References to cancel.

- One or more Local References expressed as one or two octets each, as appropriate, and encoded in successive octets, with the total number of octets containing such local references given by L1.

- A length octet (L2) given as an unsigned integer (0 to 255), which indicates the length in octets of the set of inclusive Local Reference ranges to cancel.

- One or more pairs of Local Reference ranges expressed as one or two octets each, as appropriate, and encoded in successive octets, with the total number of octets containing such Local References given by L2.

| PDU Type |  | Unused |
| --- | --- | --- |
| Cancellation Reference |  |  |
| L1 |  |  |
| EXP | Local-REF/A |  |
| Local-REF/B |  |  |
| . . . |  |  |
| L2 |  |  |
| EXP | Local-REF/A |  |
| Local-REF/B |  |  |
| . . . |  |  |

Figure 5.7-6 Cancellation Request PDU

###### In each of the above cases, if the value of a local reference is less than 128, then bit eight of the first octet in which it is encoded shall be set to zero, and the remaining seven bits set to the value of the Local Reference encoded as an unsigned integer.

###### The extended Local Reference octet shall not be present.

###### Otherwise, bit eight shall be set to one, and the remaining seven bits and the next octet set to the value of the Local Reference encoded as a 15 bit unsigned integer, with the least significant eight bits placed in the extended Local Reference octet, and the most significant seven bits placed in the first octet.

Note.— This format allows for the Local References to be cancelled, to be expressed as either a set of individual references, or a set of inclusive ranges of individual references, or both.

##### The Cancellation Accept PDU

###### The PDU format shall be as illustrated in Figure 5.7-7.

###### The first octet shall be set to binary [0101 0000], and the second octet set to the Cancellation Reference of the Cancellation Request which is being accepted.

| PDU Type | Unused |
| --- | --- |
| Cancellation Reference |  |

Figure 5.7-7 Cancellation Accept PDU

#### Call Reset Provisions

##### If at any time, a Reset Indication is received indicating a DCE originated reset, then this shall be confirmed and all other procedures associated with the Call Reset performed.

Note. —There is otherwise no impact on this SNDCF.

##### If the Reset Indication indicates a DTE user originated reset then, additionally, the directory associated with the virtual circuit shall be cleared to its initial state.

#### Call Clearing and LREF Procedures

##### When a virtual circuit has been terminated and the corresponding Subnetwork Connection Group is now empty, then the Local Reference Directory associated with this group shall be discarded.

### ATN NSAP Compression Algorithm (ACA)

#### General Overview

##### When negotiated in the Mobile SNDCF Call establishment phase, the optional ATN NSAP Compression Algorithm (ACA) shall be applied as follows:

- the compression processing (5.) to data octets being output to the subnetwork, and

- the decompression processing (5.) to data octets input from the subnetwork.

#### Address Length Determination

##### The address length for the address or address prefix to be compressed shall be extracted from the octet preceding the AFI octet in the uncompressed data stream.

##### If the extracted length lies in the range 7 through 20, the extracted length shall be used as the address "octet length" and the address length type shall be indicated as "normal".

##### If the extracted length lies in the range 56 through 160 and is an integral multiple of 8, the extracted length shall be divided by 8 to compute the length in octets of the address prefix and the address length type shall be indicated as "IDRP".

##### If the extracted length does not lie in either of these ranges, the input data does not form a compressible ATN NSAP address and the ACA shall not further process the current data as a compressible ATN NSAP address.

##### The octet length for ACA compressed address prefixes shall be encoded in the first header octet LEN/SEL subfield and the FP subfield shall be set to one.

##### If the octet length for the ACA compressed address is 20 (indicating a full address instead of a prefix) the FP subfield shall be set to zero.

##### The explicit address length octet shall be removed as part of the ACA compression processing.

Note 1.— No length octet is required for compressed ACA addresses. All information concerning address length and the presence or length of variable-length fields is contained in the header octets.

Note 2.— The shortest ATN NSAP address prefix that can be compressed is 7 octets and the length of a full ATN address is 20 octets.

Note 3.— Address lengths for normal addresses and prefixes are expressed in octet units. The address lengths for IDRP addresses and prefixes are expressed in bit units (even though the address lengths are always in full octets).

Note 4.—The IDRP subfield in the first header octet indicates whether the expanded address used octet or bit length units. Internal (compressed) addresses assume octet lengths for encoding.

#### Compressed Address Structure

##### General

###### An ACA compressed address or address prefix shall consist of the following components in the order shown in Table 5.7-8:

Table 5.7-8 Compressed NSAP Address Format

| Name | Length (octets) | Reference |
| --- | --- | --- |
| Address Marker | 2 | 5.7.6.4.4 |
| Header Octet 1 | 1 | 5.7.6.4.3.2.2 |
| Header Octet 2 | 1 | 5.7.6.4.3.2.3 |
| Compressed ADM | 2 or 3 | 5.7.6.4.3.3 |
| Variable Fields | 0 to 14 | 5.7.6.4.3.4 |

The coding and use of each component shall be as defined below.

Note.— Multi-octet uncompressed ATN address fields (ADM, ARS, LOC, and SYS) are processed from left to right, i.e. from most-significant to least-significant octet.

##### Address Header Octets

###### General

Two header octets shall begin each compressed address or address prefix.

All bits of these header octets shall be set to zero unless otherwise specified in the following subparagraphs.

Bits in each header octet shall be assigned from the high-order (most-significant or left-most).

Note.— The value of the first header octet is never zero for any compressed address. This prevents confusing a compressed address with an embedded address marker (5.).

###### First Header Octet

General

The first header octet of a compressed address shall be subdivided into four subfields as follows:

Table 5.7-9.

| Name | Length (bits) | Comments |
| --- | --- | --- |
| IDRP | 1 | Units of address length |
| FP | 1 | Full address or prefix |
| LEN/SEL | 3 | Address length or SEL code |
| CVER | 3 | Compressed VER value |

The coding and use of each subfield shall be as defined below

IDRP Subfield

If the address length determination process (5., 5.) indicates that the address to be compressed expresses length in octet units, the IDRP subfield shall be set to zero.

If the address expresses length in bit units (i.e. IDRP address), the IDRP subfield shall be set to one.

FP Subfield

The FP subfield shall be set to one if the address to be compressed is an address prefix.

The FP subfield shall be set to zero if the address to be compressed is a full address (i.e. its octet length is 20).

LEN/SEL Subfield

If the address to be compressed is an address prefix (the FP subfield is set to one), the LEN/SEL subfield shall be set to the the prefix length encoded using the encodings in Table 5.7-10.

Table 5.7-10.

| Length | Encoding | Comments |
| --- | --- | --- |
| -- | 0 | reserved |
| 7 | 1 | end with ADM |
| 8 | 2 | end with RDF |
| 11 | 3 | end with ARS |
| 13 | 4 | end with LOC |
| 19 | 5 | end with SYS |
| -- | 6, 7 | unassigned |

If the length is not found in this encoding table then the input data does not form an ATN NSAP address prefix that can be compressed and the address prefix shall not be further processed.

If the address to be compressed is a full address (the FP subfield is set to zero), the LEN/SEL subfield shall be set to the encoded value of the address SEL field (5.4.2.4.3) using encodings in Table 5.7-11.

Table 5.7-11.

| SEL | Encoding | Comments |
| --- | --- | --- |
| -- | 0 | reserved |
| 00 hex | 1 | NET |
| fe hex | 2 | NET of an airborne router not supporting IDRP |
| -- | 3, 4, 5, 6 | unassigned |
| -- | 7 | other SEL codes |

If the SEL field value in the address to be compressed is not one of the table entries above, the LEN/SEL encoding value shall be set to 7.

Note .— A LEN/SEL subfield value of zero is not allowed in either encoding to insure that the first header octet can never have the value [00] hexadecimal. Hence, no compressed address can be confused with an embedded address marker (5.).

CVER Subfield

If the value of the VER field in the address is in the range [01- 07], [41- 47], [81- 87], or [c1- c7], then the CVER subfield shall be set to the low-order 3 bits of the VER value.

If the value of the VER field in the address is not in one of the above ranges, then the CVER subfield shall be set to zero.

Note.— The encoding of the version numbers in ATN addresses is defined in 5.4.2.4.3.4.

###### Second Header Octet

General

The second header octet of a compressed address shall be subdivided into 8 subfields as follows:

Table 5.7-12.

| Name | Length (bits) | Comments |
| --- | --- | --- |
| ADMF | 1 | Flag - compressed ADM value |
| T/I | 1 | ATSC/AINSC |
| F/M | 1 | Fixed/Mobile |
| ARSD | 1 | Flag - defaulted ARS value |
| LOCD | 1 | Flag - defaulted LOC value |
| SYS6 | 1 | Flag - octet 6 of SYS = 0 |
| SYS5 | 1 | Flag - octet 5 of SYS = 0 |
| SYS4 | 1 | Flag - octet 4 of SYS = 0 |

The encodings and use of each subfield shall be as defined below.

ADMF Subfield

The ADMF subfield shall be set to one if the ADM value in the address to be compressed may be encoded into two octets using the identifier metacharacter syntax (5.4.2.3.7).

The ADMF subfield shall be set to zero if the ADM value in the address to be compressed cannot be expressed using the identifier metacharacter syntax.

Note.— The ADM value can be compressed if each of its three octets contain a character from one of the following character classes:

a) An upper-case letter "A-Z"

b) A decimal digit "0-9"

c) The "@" character.

T/I Subfield

The T/I subfield shall be set to zero if the VER value in the address to be compressed lies in the ranges [01]-[3f] or [41]-[7f], indicating that the address is in the AINSC domain.

The T/I subfield shall be set to one if the VER value in the address to be compressed lies in the ranges [81]-[bf] or [c1]-[ff], indicating that the address is in the ATSC domain.

If the VER value in the address to be compressed is either [00], [40], [80], or [c0], then the T/I subfield shall be set to zero.

Note.— The encoding of the VER field in an ATN address is defined in 5.4.2.4.3.4.

F/M Subfield

The F/M subfield shall be set to zero if the VER value in the address to be compressed lies in the ranges [01]-[3f] or [81]-[bf], indicating that the address is a fixed system.

The F/M subfield shall be set to one if the VER value in the address to be compressed lies in the ranges [41]-[7f] or [c1]-[ff], indicating that the address is a Mobile system.

Note.— The values [00], [40], [80] and [c0] are not used in the VER field of an ATN address (see 5.4.2.4.3.3)

ARSD Subfield

The ARSD subfield shall be set to zero if the ARS value in the address to be compressed is not the default value ([000001] hexadecimal) or if the address prefix to be compressed does not include an ARS field.

The ARSD subfield shall be set to one if the ARS value in the address to be compressed has the default value (5.4.2.4.6.5).

LOCD Subfield

The LOCD subfield shall be set to zero if the LOC value in the address to be compressed is not the default value ([0001] hexadecimal) or if the address prefix to be compressed does not include a LOC field. The LOCD subfield shall be set to one if the LOC value in the address to be compressed has the default value (5.4.2.4.7.5).

SYS6 Subfield

The SYS6 subfield shall be set to zero if the value of the high-order (6th) octet of the SYS field in the address to be compressed is zero or if the address prefix to be compressed does not include a SYS field.

The SYS6 subfield shall be set to one if the value of the high-order (6th) octet of the SYS field in the address to be compressed is nonzero.

SYS5 Subfield

The SYS5 subfield shall be set to zero if the value of the second to high-order (5th) octet of the SYS field in the address to be compressed is zero or if the address prefix to be compressed does not include a SYS field.

The SYS5 subfield shall be set to one if the value of the second to high-order (5th) octet of the SYS field in the address to be compressed is nonzero.

SYS4 Subfield

The SYS4 subfield shall be set to zero if the value of the third to high-order (4th) octet of the SYS field in the address to be compressed is zero or if the address prefix to be compressed does not include a SYS field.

The SYS4 subfield shall be set to one if the value of the third to high-order (4th) octet of the SYS field in the address to be compressed is nonzero.

##### Compressed ADM Field

###### If the ADM field value of the address to be compressed follows the syntax of an identifier (5.4.2.3.7) then the compressed ADM field shall consist of two octets and shall contain the encoded value of the identifier obtained by applying the definitions in 5.4.2.3.7 to the ADM field value.

###### If the ADM field value of the address to be compressed does not follow the identifier syntax then the compressed ADM field shall consist of three octets and shall contain the 3-octet ADM value unchanged.

Note.— The value of the ADMF subfield in the second header octet indicates whether the compressed ADM field has the 2-octet (compressed) or 3-octet (uncompressed) format.

##### Variable Fields

###### The variable fields shall have a minimum length of 0 octets and a maximum length of 13 octets. Variable field data octets shall be concatenated when required in the order that their fields occur in the ATN address (Figure 5.7-3) as follows:

- VER value (if > 7), 1 octet

- ARS value (if not default), 3 octets

- LOC value (if not default), 2 octets

- SYS octet 6 value (if nonzero), 1 octet

- SYS octet 5 value (if nonzero), 1 octet

- SYS octet 4 value (if nonzero), 1 octet

- SYS octets 3-1, 3 octets

- SEL value (if not defined in 5.4.2.4.9.3), 1 octet

###### The ACA compression of address prefixes shall omit those variable fields b) through h) which are not present in the uncompressed address prefix.

#### Compressed Address Marker

##### General

###### The ACA shall prefix each compressed address or address prefix with an address marker.

###### The address marker shall consist of two octets with the value [55aa] hexadecimal.

###### The ACA shall process the case of the address marker value occuring in the input octet stream as defined in 5. below.

##### Normal Address Case

###### In the case of a normal compressed address or address prefix, the header octets of the compressed address format (5.) shall follow the address marker.

Note.— The first header octet of a compressed address can never have the value [00]. This distinguishes the normal address case from the embedded address case.

##### Embedded Address Marker Case

###### If two octets with the value of an address marker occur in data, a padding octet with value [00] hexadecimal shall be inserted into the data stream following the embedded address marker octets.

Note 1.—The likelihood of embedded address markers in the input data stream is very low. When they occur, however, the ACA algorithm must add the extra padding octet. Hence, it is possible (although highly unlikely) for the ACA to expand data.

Note 2.— The design of the ACA requires that the first header octet of a compressed address can never have the value [00] hexadecimal. Hence, the first header octet of a compressed address cannot be confused with the padding octet of an embedded address marker.

#### Compression Algorithm

##### General

###### The ACA shall perform compression by replacing ATN addresses or address prefixes identified in the input octet stream with compressed, encoded equivalents as defined below.

###### The format of a compressed address shall be as defined in 5..

###### Each compressed address shall be prefixed with a compressed address marker (5.).

###### Any embedded address markers found in the input octet stream shall be padded with a null-value octet (5.).

###### The overall logic flow of the ACA compression processing shall be as defined in 5..

##### Address Encoding Process

###### General

The process of encoding an ATN address or address prefix into the ACA compressed format (5.) shall be performed using the sequence of steps defined in this paragraph.

The steps shall be performed in the order they are listed.

If any step of the encoding process fails, the ACA compression processing shall not consider the current input octets as an address and shall continue with the compression logic.

###### Encoding Address Length

Determination of the length in octets of an address to be compressed shall be performed as defined in 5.7.6.2.

If the address length is of type "normal", the IDRP subfield in the first header octet shall be set to zero.

Otherwise, the IDRP subfield shall be set to one.

If the octet length of the address is 20 (indicating a full ATN address), the FP subfield in the first header octet shall be set to zero.

If the octet length of the address is less than 20 (indicating an address prefix), the FP subfield shall be set to one and the address length shall be encoded in the LEN/SEL subfield of the first header octet according to the table in 5..

If the address length is not found in the length table, the encoding process shall halt and the current input octet string shall not be treated as an ATN address.

###### Encoding the AFI and IDI Fields

No encoding shall be performed on the constant values of the address AFI and IDI fields.

These fields shall be omitted from the compressed address encoding.

###### Encoding the VER Field

If the VER value in the address to be compressed lies within the range [01]-[3f], the T/I subfield in the second header octet shall be set to zero and the F/M subfield in the second header octet shall be set to zero.

If the VER value lies within the range [01]-[07], then the low-order 3 bits of the VER value shall be stored in the CVER subfield of the first header octet.

If the VER value lies in the range [08]-[3f], then the CVER subfield shall be set to zero and the VER value octet shall be concatenated to the variable field of the encoded address.

If the VER value in the address to be compressed lies within the range [41]-[7f], the T/I subfield in the second header octet shall be set to zero and the F/M subfield in the second header octet shall be set to one.

If the VER value lies within the range [41]-[47], then the low-order 3 bits of the VER value shall be stored in the CVER subfield of the first header octet.

If the VER value lies in the range [48]-[7f], then the CVER subfield shall be set to zero and the VER value octet shall be concatenated to the variable field of the encoded address.

If the VER value in the address to be compressed lies within the range [81]-[bf], the T/I subfield in the second header octet shall be set to one and the F/M subfield in the second header octet shall be set to zero.

If the VER value lies within the range [81]-[87], then the low-order 3 bits of the VER value shall be stored in the CVER subfield of the first header octet.

If the VER value lies in the range [88]-[bf], then the CVER subfield shall be set to zero and the VER value octet shall be concatenated to the variable field of the encoded adress.

If the VER value in the address to be compressed lies within the range [c1]-[ff], the T/I subfield in the second header octet shall be set to one and the F/M subfield in the second header octet shall be set to one.

If the VER value lies within the range [c1]-[c7], then the low-order 3 bits of the VER value shall be stored in the CVER subfield of the first header octet.

If the VER value lies in the range [c8]-[ff], then the CVER subfield shall be set to zero and the VER value octet shall be concatenated to the variable field of the encoded adress.

If the VER value is either [00], [40], [80], or [c0], the encoding process shall halt and the current input octet string shall not be treated as an ATN address.

###### Encoding the ADM Field

If the three octets of the ADM field in the address to be compressed do not follow the rules for Identifier Syntax (5.4.2.3.7), the ADMF subfield in the second header octet shall be set to zero and the three octets of the ADM field value shall be concatenated to the compressed ADM of the encoded address.

If the ADM field value does follow the Identifier Syntax rules, the ADMF subfield shall be set to one and the two-octet compressed ADM value (5.) shall be concatenated to the compressed ADM of the encoded address.

###### Encoding the RDF Field

If the address length indicates an address prefix whose length is less than or equal to 7, no RDF field value shall be encoded and the encoding process shall halt.

If the RDF value in the address to be compressed is not [00], the encoding process shall halt and the current input octet string shall not be treated as an ATN address.

###### Encoding the ARS Field

If the address length indicates an address prefix whose length is less than or equal to 8, no ARS field value shall be encoded and the encoding process shall halt.

If the ARS value of the address to be compressed has the default value ([000001] hexadecimal), the ARSD subfield in the second header octet shall be set to one.

If the ARS value of the address to be compressed is not default, the ARSD subfield shall be set to zero and the three octets of the ARS value shall be concatenated to the variable field data of the encoded address.

###### Encoding the LOC Field

If the address length indicates an address prefix whose length is less than or equal to 11, no LOC field value shall be encoded and the encoding process shall halt.

If the LOC value of the address to be compressed has the default value ([0001] hexadecimal), the LOCD subfield in the second header octet shall be set to one.

If the LOC value of the address to be compressed is not default, the LOCD subfield shall be set to zero and the two octets of the LOC value shall be concatenated to the variable field data of the encoded address.

###### Encoding the SYS Field

If the address length indicates an address prefix whose length is less than or equal to 13, no SYS field value shall be encoded and the encoding process shall halt.

If the high-order (6th) octet of the SYS field of the address to be compressed has a nonzero value, the SYS6 subfield in the second header octet shall be set to zero and the value of the SYS field octet shall be concatenated to the variable field data of the encoded address. Otherwise, the SYS6 subfield shall be set to one.

If the second to high-order (5th) octet of the SYS field of the address to be compressed has a nonzero value, the SYS5 subfield in the second header octet shall be set to zero and the value of the SYS field octet shall be concatenated to the variable field data of the encoded address.

Otherwise, the SYS5 subfield shall be set to one.

If the third to high-order (4th) octet of the SYS field of the address to be compressed has a nonzero value, the SYS4 subfield in the second header octet shall be set to zero and the value of the SYS field octet shall be concatenated to the variable field data of the encoded address.

Otherwise, the SYS4 subfield shall be set to one.

The three remaining octets of the SYS field shall be concatenated to the variable field data of the encoded address.

###### Encoding the SEL Field

If the address length indicates an address prefix whose length is less than or equal to 19, no SEL field value shall be encoded and the encoding process shall halt.

Since the address length indicates a full ATN address, the FP subfield in the first header octet shall be set to zero.

The SEL value shall be encoded into the LEN/SEL subfield in the first header octet according to the table in 5..

If the SEL value is not one of the table entries, the LEN/SEL subfield shall be set to 7 and the SEL value octet shall be concatenated to the variable field data of the encoded address.

##### Compression Logic Flow

###### The ACA compression logic shall process octets sequentially from the uncompressed data input stream.

###### For each input octet, a test shall be performed to determine if the current octet and the subsequent octets form an ATN address or address prefix.

###### If they do form an ATN address, the ACA shall attempt to encode the address into the compressed address format (5.) as defined in the steps of 5..

###### If the encoding process is successful, a compressed address marker (5.) shall be output to the compressed octet stream followed by the compressed address octets.

###### The compression processing shall then continue with the next uncompressed data octet not a part of the address just processed.

###### If the encoding process fails, or if the current octet does not begin an ATN address, the ACA processing shall check at the current uncompressed octet position in the input data stream for an embedded address marker (5.).

###### If an embedded address marker is found, the ACA shall copy the address marker octets to the compressed output octet stream. A padding zero-valued octet shall be output as well as the address marker.

###### The compression processing shall then continue with the next uncompressed data octet not a part of the embedded address marker.

###### If neither an ATN address or embedded address mark is found, the ACA shall copy the current uncompressed input octet to the compressed output octet stream and shall continue processing with the next sequential input octet.

Note.— Since the ACA compression logic may not recognize the appearance of an ATN address or prefix in the data stream until after the uncompressed length octet has been processed (the length octet precedes the fixed-value ATN AFI and IDI fields that distinguish an ATN address), the ACA compression process will need to be able to recall the value of the previous input octet during compression processing. Hence, a one-octet "backup" may be necessary in the implementation of the ACA compression logic.

#### Decompression Algorithm

##### General

###### The ACA shall perform decompression by replacing compressed ATN addresses or address prefixes in the ACA compressed format (5.) with their expanded equivalent as defined below. Address markers and padding octets shall be removed from the data stream during ACA decompression processing.

###### The overall logic flow of the ACA decompression processing shall be as defined in 5..

##### Address Decoding Process

###### General

The process of decoding a compressed ATN address or address prefix from the ACA compressed format (5.) shall be performed using the sequence of steps defined in the following paragraphs.

The steps shall be performed in the order listed below. The expanded address or prefix shall include the decoded address length octet and the decoded 7-20 address octets.

###### Decoding Address Length

If the FP subfield in the first header octet is zero, the octet length of the compressed address shall be set to 20 (a full ATN address).

Otherwise, the octet length of the compressed address prefix shall be decoded from the LEN/SEL subfield in the first header octet according to the table in 5..

The address octet length shall be used in the further decoding process steps.

If the IDRP subfield in the first header octet is zero, the output address length shall be the address octet length.

Otherwise, the output address length shall be 8 times the address octet length.

Note.— The address octet length is an internal variable used in the decoding process. The output length prefixed to the expanded address after the decoding process is completed is either the same as the octet length (normal case) or 8 times the octet length (IDRP case, length in bits).

###### Decoding the AFI and IDI Fields

The AFI field of the decoded address shall be set to its constant value of [47] hexadecimal.

The IDI field of the decoded address shall be set to its constant value of [0027] hexadecimal.

###### Decoding the VER Fields

If the CVER subfield in the first header octet is zero, the VER octet shall be extracted from the next octet in the variable field of the compressed address.

If the CVER subfield is non-zero, then the VER field value in the expanded address shall be computed as follows:

- If the T/I subfield in the second header octet is zero and the F/M subfield in the second header octet is zero, then the VER field value shall be set to the CVER value.

- If the T/I subfield in the second header octet is zero and the F/M subfield in the second header octet is one, then the VER field value shall be set to the CVER value plus 64.

- If the T/I subfield in the second header octet is one and the F/M subfield in the second header octet is zero, then the VER field value shall be set to the CVER value plus 128.

- If the T/I subfield in the second header octet is one and the F/M subfield in the second header octet is one, then the VER field value shall be set to the CVER value plus 192.

###### Decoding the ADM Fields

If the ADMF subfield in the second header octet is set to zero, the three octets of the ADM field shall be extracted from the next three octets in the variable field data.

Otherwise, the ADM field value shall be decoded from the compressed ADM which is extracted from the next two octets in the variable field data of the compressed address.

The decoding of the compressed ADM value shall be performed as defined in 5.4.2.3.7.

###### Decoding the RDF Fields

The RDF field in the expanded address shall be set to zero.

###### Decoding the ARS Fields

If the address length indicates an address prefix whose length is less than or equal to 8, no ARS field value shall be decoded and the decoding process shall halt.

If the ARSD subfield in the second header octet of the compressed address is set to one, the expanded ARS field shall be set to the default value ([000001] hexadecimal).

Otherwise, the expanded ARS field value shall be extracted from the next three octets in the variable field data of the compressed address.

###### Decoding the LOC Fields

If the address length indicates an address prefix whose length is less than or equal to 11, no LOC field value shall be decoded and the decoding process shall halt.

If the LOCD subfield in the second header octet of the compressed address is set to one, the expanded LOC field shall be set to the default value ([0001] hexadecimal).

Otherwise, the expanded LOC field value shall be extracted from the next two octets in the variable field data of the compressed address.

###### Decoding the SYS Fields

If the address length indicates an address prefix whose length is less than or equal to 13, no SYS field value shall be decoded and the decoding process shall halt.

If the SYS6 subfield in the second header octet has the value one, the high-order (6th) octet of the expanded SYS field shall be extracted from the next octet in the variable data field of the compressed address.

Otherwise, the high-order (6th) octet of the expanded SYS field shall be set to zero.

If the SYS5 subfield in the second header octet has the value one, the second to high-order (5th) octet of the expanded SYS field shall be extracted from the next octet in the variable data field of the compressed address. Otherwise, the second to high-order (5th) octet of the expanded SYS field shall be set to zero.

If the SYS4 subfield in the second header octet has the value one, the third to high-order (4th) octet of the expanded SYS field shall be extracted from the next octet in the variable data field of the compressed address.

Otherwise, the third to high-order (4th) octet of the expanded SYS field shall be set to zero.

The remaining three octets of the expanded SYS field shall be extracted from the next three octets in the variable data field of the compressed address.

###### Decoding the SEL Fields

If the address length indicates an address prefix whose length is less than or equal to 19, no SEL field value shall be decoded and the decoding process shall halt.

If the FP subfield in the first header octet has the value zero (indicating a full ATN address), then the value of the SEL field shall be decoded from the LEN/SEL subfield in the first header octet.

If the value of the LEN/SEL subfield lies in the range 1-2 the SEL value shall be decoded using the SEL encoding table in 5..

If the LEN/SEL subfield encoding has the value 7, the SEL field value shall be extracted from the next octet in the variable data field of the compressed address.

Note.— Only a full ATN address (not a prefix) includes a SEL field.

##### Decompression Logic Flow

###### The ACA decompression logic shall process octets sequentially from the compressed data input stream.

###### If the octet at the current input position and the next octet do not form a compressed address marker (5.), the current input octet shall be copied to the decompressed output octet stream and decompression processing shall continue with the next input octet.

###### When a compressed address marker is found in the input octet stream, the decompression processing shall examine the value of the next octet beyond the address marker.

###### If the value of this octet is zero (indicating an embedded address mark (5.), the compressed address marker octets shall be copied to the decompressed output octet stream and the zero-value octet shall be dropped from the output stream.

###### If the value is nonzero (indicating a compressed ATN address), the compressed address shall be decoded according to 5..

###### The decoded address octets shall be copied to the decompressed octet output stream and decompression processing shall continue with the next input octet beyond those that formed the compressed ATN address.

###### The compressed address marker octets shall not be copied to the output.

## ATN SNDCF Protocol Requirements List

### Conformance

#### An implementation of the ATN Mobile SNDCF shall be used in ATN Airborne and Air/Ground Routers if and only if its PICS is in compliance with the APRLs given in 5.7.7.8.

#### An implementation of the ISO/IEC 8802 SNDCF shall be used in ATN End Systems and Routers if and only if its PICS is in compliance with the APRLs given in 5.7.7.2.

#### An implementation of the SNDCF for General Topology ISO/IEC 8208 Subnetworks shall be used in ATN End Systems and Routers if and only if its PICS is in compliance with the APRLs given in 5.7.7.4.

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8802-2 Subnetworks - Functions

| Item | Function | ISO/IEC 8473-2 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| S802SNUD | Is subnetwork user data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M |
| S802SNTD | Is Transit Delay determined by the SNDCF prior to the processing of User Data ? | 5.2 | M | M |

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8802-2 Subnetworks - Multi Layer Dependencies

| Item | Dependency | ISO/IEC 8473-2 Reference | ATN Support |
| --- | --- | --- | --- |
| S802SSg-r | <r> Maximum SN data unit size (RX) | 5.2 | >=512 |
| S802SSg-s | <s> Maximum SN data unit size (TX) | 5.2 | >=512 |

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8208 Subnetworks - Functions

| Item | Function | ISO/IEC 8473-3 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| XSNUD | Is Subnetwork User Data of at least 512 octets transferred transparently by the SNDCF ? | 5.2 | M | M |
| XSNTD | Is Transit Delay determined by the SNDCF prior to the processing of user data ? | 5.2 | M | M |
|  | Call Setup Considerations | 5.31 |  |  |
|  | Is a new call setup: |  |  |  |
| XCalla | a. when no suitable call exists ? | 5.3.1 a. | O.3 | O.3 |
| XCallb | b. when queue threshold reached ? | 5.3.1 b. | O.3 | O.3 |
| XCallc | c. by systems management ? | 5.3.1 c. | O.3 | O.3 |
| XCalld | d. when queue threshold reached and timer expires ? | 5.3.4 | O.3 | O.3 |
| XCalle | e. by other local means ? | 5.3.1 | O.3 | O.3 |
|  | Call clearing considerations Are calls cleared: | 5.3.2 |  |  |
| XClra | a. when idle timer expires | 5.3.2 a., 5.3.4 | O | O |
| XClrb | b. when need to re-use circuit | 5.3.2 b. | O | O |
| XClrc | c. by systems management | 5.3.2 c. | O | O |
| XClrd | d. by provider ? | 5.3.2 d. | M | M |
| XClrer | e. by other local means ? | 5.3.2 | O | O |
| XPD | X.25 Protocol Discrimination | 5.3.3 | M | M |
| XVCC | Resolution of VC collisions | 5.3.5 | M | M |
| XMCR | Multiple VCs responding | 5.3.6 | M | M |
| XMCI | Multiple VCs initiating | 5.3.6 | O | O |
| Xpri | X.25 Priority procedure | 5.3.7 | O | M |

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8208 Subnetworks - X.25 Call User Data

| Item | Parameter | ISO/IEC 8473-3 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| PD-s | <s> Protocol Discriminator | 5.3.3 | M | M |
| PD-r | <r> Protocol Discriminator | 5.3.3 | M | M |
| LI-s | <s> Length Indication | 5.3.6 | XMCI:M | XMCI:M |
| LI-r | <r> Length Indication | 5.3.6 | M | M |
| Ver-s | <s> SNCR Version | 5.3.6 | XMCI:M | XMCI:M |
| Ver-r | <r> SNCR Version | 5.3.6 | M | M |
| SNCR-s | <s> SNCR Value | 5.3.6 | XMCI:M | XMCI:M |
| SNCR-r | <r> SNCR Value | 5.3.6 | M | M |

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8208 Subnetworks - ISO/IEC 8208 SNDCF Timers

| Item | Timer | ISO/IEC 8473-3 Reference | Status | Values | ATN Support |
| --- | --- | --- | --- | --- | --- |
| XIDL | X25 VC Idle | 5.3.4 | XClra:O | Any | XClra:O |
| XNVC | additional VC | 5.3.4 | O | Any | M |

### Subnetwork Dependent Convergence Functions SNDCF for use with ISO/IEC 8208 Subnetworks - SNDCF Multi Layer Dependencies

| Item | Dependency | ISO/IEC 8473-3 Reference | Status | Values Supported |
| --- | --- | --- | --- | --- |
| XSSg-r | <r> Maximum SN data unit size (Rx) | 5.2 | >=512 | >=512 |
| XSSg-s | <s> Maximum SN data unit size (Tx) | 5.2 | >=512 | >=512 |

| Item | Dependency | ISO/IEC 8473-3 Reference | Status | ATN Support |
| --- | --- | --- | --- | --- |
| Xvc | X.25 Virtual call service | 5.3.8 | M | M |
| Xdt | X.25 Data transfer | 5.3.8 | M | M |
| Xfc | X.25 flow control procedures | 5.3.8 | M | M |
| Xfrp | X.25 flow control + reset packets | 5.3.8 | M | M |
| Xccp | X.25 call setup and clear packets | 5.3.8 | M | M |
| Xdp | X.25 DTE and DCE data packets | 5.3.8 | M | M |
| Xrs | X.25 restart procedures | 5.3.8 | M | M |
| XDct | X.25 DCE timeouts | 5.3.8 | M | M |
| XDtT | X.25 time limits | 5.3.8 | M | M |
| Xpco | X.25 network packet coding | 5.3.8 | M | M |
| Xfcn | X.25 flow control parameter negotiation | 5.3.8 | O | O |
| Xtd | X.25 transit delay selection and negotiation | 5.3.8 | O | O |
| Xtc | X.25 throughput class negotiation | 5.3.8 | O | O |
| Xoth | Other X.25 elements | 5.3.8 | O | O |

### ATN Requirements for Mobile SNDCFs

Note. — This section specifies the requirements for the Mobile SNDCF in Airborne and Air/Ground Routers.

#### Major Capabilities

| Item | Capability | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| *mcNego | Negotiation of Compression Algorithm | 5.7.6.2 | M |
| *mcLocRef | Local Reference Header Compression | 5.7.6.3 | M |
| *mcCan | Local Reference Cancellation | 5.7.6.3.6 | O |
| McM/I | Local Reference directory maintenance | 5.7.6.3 | Snvdl:M ^Snvdl:O |
| *mcACA | ICAO Address Compression Algorithm (ACA) | 5.7.6.4 | O |
| mcV42 | V.42bis Compression | 5.7.6.2 | O |

Note. — Snvdl is true when the VDL SNDCF is implemented.

#### Call Setup and Clearing Procedures

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| csDynam | Dynamic Call Setup | 5.7.6.2.1.1.1 | M |
| csSys | Call Setup by Systems Management | 5.7.6.2.1.1 | O |
| csDef | Use of non-standard Default packet size | 5.7.6.2.1.3 | M |
| csFast | Use of Fast Select | 5.7.6.2.1.4 | M |
| csM/I | Local Reference directory maintenance request/acceptance | 5.7.6.2.1.5.13 5.7.6.2.2.2.3 | ^csFast: - McM/I:M |
| csOther | Use of other optional User Facilities and CCITT-specified DTE facilities | 5.7.6.2.1.1.3 | O |
| csCol | Call Collision Resolution | 5.7.6.2.2.1.2 | M |

Note. — Fast Select only required if supported by subnetwork.

continued..

Call Setup and Clearing Procedures continued..

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| csAcp | Call Acceptance Procedures | 5.7.6.2.1.6 | M |
| csRej | Call rejection Procedures | 5.7.6.2.1.7 | M |
| csOrd | Order of compression Procedures | 5.7.6.2.3.2 | M |
| csDiag | Use of call rejection diagnostic codes | 5.7.6.2.1.7.3 | M |
| csReset | Call Clearing Procedures | 5.7.6.2.4 | M |

#### Negotiation of Compression Algorithm

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| caMaxd | Indication of the maximum of directories entries in the call user Data | 5.7.6.2.1.5.11 | mcNego:O |

#### Local Reference Header Compression

| Item | Function | ATN SARPs Reference |  |  | ATN Support |
| --- | --- | --- | --- | --- | --- |
| lrVC | Opening additional virtual circuits | 5.7.6.3.2.1.2 |  |  | M |
| *lrDirSize | Local Directory with more than 128 entries | 5.7.6.3.1 |  |  | O |
| lrProt | Identification of Network Layer Protocol | 5.7.6.3.2.2 |  |  | M |
| lrMod | Processing of SN-UnitData Requests | 5.7.6.3.2 |  |  | M |
| lrEst | Establishment of new local reference | 5.7.6.3.2.4 |  |  | M |
| lrTransfer | Transfer of modified ISO 8473 PDU | 5.7.6.3.2.6 |  |  | M |
| lrInitial | Initial DT PDU Compression | 5.7.6.3.3.2 |  |  | M |
| lrDerived | Derived DT PDU Compression | 5.7.6.3.3.3 |  |  | M |
| *lrError-s | Generation of Error PDU Compression | 5.7.6.3.3.3 |  |  | M |
| lrDiscard | Compression of discarded PDU encapsulated within Error PDU | 5.7.6.3.3.4 |  |  | lrError-s:M |
| lrCompTr | Transfer of compressed PDUs | 5.7.6.3.3.4.11 |  | M |  |
| lrReceived | Processing of received PDUs | 5.7.6.3.4 | M |  |  |
| lrUncomp-r | Processing of received uncompressed PDUs | 5.7.6.3.4.2 | M |  |  |
| LrReset | Purging directories entries on Reset | 5.7.6.3.7 | mcLocRef:M |  |  |
| lrUnMod-r | Processing of received unmodified PDUs | 5.7.6.3.4.2.2 | M |  |  |
| lrComp-r | Processing of received compressed data PDUs | 5.7.6.3.4.3 | M |  |  |
| lrError-r | Processing of received compressed Error PDUs | 5.7.6.3.4.4 | M |  |  |
| lrSNDCFerr-s | Generation of SNDCF Error Report | 5.7.6.3.5 | M |  |  |
| lrSNDCFerr-r | Processing of received SNDCF Error Report | 5.7.6.3.4.5 | M |  |  |

#### Local Reference Cancellation

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| lrcMgmt | Management of local references | 5.7.6.3.2.5 | mcCan:M |
| lrcRequest-s | Generation of Cancellation Request PDU | 5.7.6.3.6 | mcCan:M |
| lrcRequest-r | Processing of incoming Cancellation Request PDU | 5.7.6.3.6 | mcCan:M |
| lrcReliable | Reliable transfer of Cancellation Request | 5.7.6.3.6 | mcCan:M |
| lrcAccept-s | Generation of Cancellation Accept PDU | 5.7.6.3.6 | mcCan:M |
| lrcAccept-r | Processing of incoming Cancellation Accept PDU | 5.7.6.3.6 | mcCan:M |

#### ICAO Address Compression Algorithm

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| acOut | Compression of outgoing PDUs | 5.7.6.4.1 | mcACA:M |
| acIn | Decompression of incoming PDUs | 5.7.6.4.1 | mcACA:M |
| acAddr | Address Length Determination | 5.7.6.4.2 | mcACA:M |
| acComp | Compression of NSAP Addresses and address prefixes | 5.7.6.4.5 | mcACA:M |
| acDecomp | Decompression of NSAP Addresses and address prefixes | 5.7.6.4.6 | mcACA:M |

#### PDU Formats

##### Call Request User Data

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| crLen | Length Indicator | 5.7.6.2.1.5.3 | M |
| crVersion | Version Indicator | 5.7.6.2.1.5.4 | M |
| crSNCR | Subnetwork Connection Reference (SNCR) | 5.7.6.2.1.5.5 | M |
| crComp | Offerred Compression Techniques | 5.7.6.2.1.5.7 | M |
| crDir | Maximum Directory Size Note.— Dynamically, this field is only generated if Local Reference Compression is offered. | 5.7.6.2.1.5.11 | M |
| crAdd-s | Additional User Data on send | 5.7.6.2.1.5.12 | O |
| crAdd-r | Additional User Data on receive | 5.7.6.2.1.5.12 | O |
| MaxDir | Maximum number of directory entries supported | 5.7.6.2.1.5.7 | ³128 |

##### Call Accept User Data

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| caComp | Offerred Compression Techniques | 5.7.6.2.2.4.3 | mcNego:M |
| caAdd-s | Additional User Data on send | 5.7.6.2.2.4.4 | mcNego:O |
| caAdd-r | Additional User Data on receive | 5.7.6.2.2.4.4 | mcNego:O |

##### Modified ISO/IEC 8473 NPDU

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| npLocRef-s | Local Reference Option field | 5.7.6.3.2.3 | M |

##### Compressed Initial PDU

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| inType | PDU Type | 5.7.6.3.3.2.2 | M |
| inPri | Priority | 5.7.6.3.3.2.3 | M |
| inLifetime | Lifetime | 5.7.6.3.3.2.4 | M |
| inFlags | Flag Bits | 5.7.6.3.3.2.5 to 5.7.6.3.3.2.9 | M |
| inLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.9 | M |
| inLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 | lrDirSize:M ^lrDirsize:X |
| inPDUId | PDU Identifier | 5.7.6.3.3.2.10 | M |
| inNSDU | User Data | Figure 5.7-4 | M |

##### Compressed Derived PDU

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| drType | PDU Type | 5.7.6.3.3.3.2 | M |
| drPri | Priority | 5.7.6.3.3.3.3 | M |
| drLifetime | Lifetime | 5.7.6.3.3.3.4 | M |
| drFlags | Flag Bits | 5.7.6.3.3.3.5 to 5.7.6.3.3.3.8 | M |
| drLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.8 | M |
| drLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.8 | lrDirSize:M ^lrDirsize:X |
| drPDUId | PDU Identifier | 5.7.6.3.3.3.9 | M |
| drSegOff | Segment Offset | 5.7.6.3.3.3.10 | M |
| drTotalLen | Total Length | 5.7.6.3.3.3.11 | M |
| drNSDU | User Data | Figure 5.7-4 | M |

##### Compressed Error PDU

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| erType | PDU Type | 5.7.6.3.3.4.2 | M |
| erPri | Priority | 5.7.6.3.3.4.3 | M |
| erLifetime | Lifetime | 5.7.6.3.3.4.4 | M |
| erFlags | Flag Bits | 5.7.6.3.3.4.5 to 5.7.6.3.3.4.8 | M |
| erLocRef | Local Reference (1 octet) | 5.7.6.3.3.2.8 | M |
| erLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.8 | lrDirSize:M ^lrDirsize:X |
| erReason | Discard Reason | 5.7.6.3.3.4.9 | M |
| erNSDU | Compressed Header of discarded PDU | 5.7.6.3.3.4 | M |

##### SNDCF Error Report PDU

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| sfType | PDU Type | 5.7.6.3.5 | M |
| sfReason | Discard Reason | 5.7.6.3.5 | M |
| sfLocRef | Local Reference | 5.7.6.3.5 | M |
| sfLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 | lrDirSize:M ^lrDirsize:X |

##### Cancellation Request

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| cqType | PDU Type | 5.7.6.3.6 | mcCan:M |
| cqRef | Cancellation Reference | 5.7.6.3.6 | mcCan:M |
| cqLocRef | Local Reference | 5.7.6.3.6 | M |
| cqLocRef2 | Local Reference (2 octet) | 5.7.6.3.3.2.9 | lrDirSize:M ^lrDirsize:X |

##### Cancellation Accept

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| ccType | PDU Type | 5.7.6.3.6 | mcCan:M |
| ccRef | Cancellation Reference | 5.7.6.3.6 | mcCan:M |