# Systems Management Provisions

## Introduction

### Recommendation.— ATN managed resources should be grouped into Management Domains in order to assign responsibility for control of the resources.

### Recommendation.— States and Organizations should assign an administrative authority to establish and maintain the respective management  of each of their Management Domains, and to manage the transfer of control of resources from one Management Domain to another.

Note.— The definition and implementation of a global ATN Systems Management solution may be specified in future amendments to these SARPs. Currently:

- No exchange of Systems Management information is required between routers of different Administrative Domains.

- No exchange of Systems Management information is required by means of a management protocol over the Air/Ground links. This does not preclude the exchange of routing information, by means of routing information exchange protocols.

- The exchange of Systems Management information within an Administrative Domain is considered a local matter and can be achieved by any means deemed appropriate.