WG-B/SGB-1  Flimsy 5

ATNP WG-B Position on

Liaison statement from AMCP Working Group M to ATNP WG A.

AMCP WG M considered during its meeting from 12 - 19 December 2000 in Malmo, Sweden, the proposals for a ATN subnetwork priority mapping for VDL Modes 3 and 4.

AMCP Working Group M wishes to make the following comments on the proposed table:

a.	The table includes the term AMSS-1 to refer to the first generation Aeronautical Mobile Satellite Service.

This note is confusing since it would imply that there are other generation aeronautical mobile satellite services referenced in the SARPs. This is not the case. The note should be deleted and the reference to AMSS-1 needs to be amended into AMSS.

ATNP WG-B Position:  Agree to revise priority table:  propose to revise table to label 3rd column AMSS, delete reference to Note 4, delete note 4 and renumber subsequent notes.

b.	The note "restricted" (Note 6) has been used in the table to identify message categories of which the use of particular sub-networks may not be allowed in certain States and/or regions based on ITU frequency allocation. Both the note and its usage are erroneous and based upon a wrong interpretation of the relevant ITU provisions. In this context it should be noted that also Note 3 ("not allowed") is unclear to what it is referencing. Assumed is that this note refers to regulations in the SARPs that restrict the use of the mobile subnetworks. These are contained (with the exception of the AMSS) in Annex 10, Volume II. Similar provisions are also included in the ITU Radio Regulations as indicated in Note 7. It might be useful to align the two notes.

ATNP WG-B Position:  Do not agree to revise Note 6.  This was previously coordinated with AMCP and the term restricted was selected based on AMCP inputs that there are a few exceptions to the ITU prohibition of allowing anything other than communications related to safety and regularity of flight from being provided on the VHF comm. band.

Note 7 gives the suggestion that ITU frequency allocations (assumed is here that the ATNP means "ITU Radio Regulations" as the priority of communications in the aeronautical mobile service is not part of the table of frequency allocation but is regulated in chapter S VIII of the Radio Regulations. This chapter is dealing with aeronautical services) regulate on a regional or national basis the use of UN-charter communications (ITU priority 7, ATN network priority 3), State/Government communications (not clear is the difference between State and government communications, the ITU Radio Regulations only refer to "Government messages for which priority has been expressly requested; the ITU priority for these messages is 8 and the ATN priority 2). Normal priority communications is not a category that is included in the Radio Regulations; however, the ITU refers to a category "service communications relating to the working of the telecommunication service or to communications previously exchanged". Finally, the ITU Radio Regulations provide for the category "other communications" in category 10, which includes public correspondence. Please note that Aeronautical administrative messages are not part of the ITU provisions and fall in the category "public correspondence" for the use of the air-ground sub-network.

The terms "urgent-priority administrative communications", "high-priority administrative communications", normal-priority communications" and "low-priority communications" do not appear in the Radio Regulations. As such, these categories should be removed from the table since also Annex 10 (Volume II, Chapter 3, on the Aeronautical Mobile Service does not include these messages. These types of messages, as well as public correspondence (or the so-called aeronautical passenger communications) are not allowed in ANY band allocated to the aeronautical mobile service and therefore the table 1-3 should read "not allowed" against VDL Mode 2, VDL Mode 3, VDL Mode 4 and HFDL for these categories. Further, it is not clear where Annex 10 distinguishes between normal and high-priority communications in the air-ground communications.

The Radio Regulations explicitly do not allow for public correspondence in the bands allocated to the aeronautical mobile service. However, there are not such provisions for the categories 7 - 9. Therefore, these categories have to be accommodated in the air-ground sub-networks.

The table is unclear on the category "airline administrative messages"

Airline administrative messages are not allowed on the air-ground network, however, ICAO and ITU have made provisions for Flight Regularity or Aeronautical Operational Control (AOC) communications and these should be referenced in the table

ATNP WG-B Position:  Reject changes to the definition of ATN priority levels.  These priority levels have previously been the subject of extensive coordination with AMCP and are already included in Annex 10.  The defined ATN priority levels are for the ATN internet which must serve both mobile and fixed service communications.  The ATN priority levels are generally a superset of the ITU defined priority levels for mobile communications.  It is up to the mobile subnetwork SARPs to comply with the ITU radio regulations for use of the r.f. channel (i.e., define a compliant link layer priority scheme).  The ATN internet priority levels as currently defined in Annex 10 are sufficient to allowing use of mobile subnetworks that provide a link layer priority scheme compliant the ITU radio regulations.

c.	Note 7 is indicating that the VDL Mode 4 sub-network has only been specified to support surveillance applications (e.g. ADS).

It is felt that this information is inappropriate in the ATN part of Annex 10. Annex 10 already specifies that VDL Mode 4 SARPs apply to surveillance applications (e.g. ADS-B and ADS-C) and it is undesirable to repeat typical VDL SARPs and related material in the ATN section of Annex 10 or the ATN Manual.

ATNP WG-B Position:  Agree to reword note 7 to read:  VDL Mode 4 provides mobile subnetwork support for surveillance applications (e.g., ADS)

d.	Finally, is it observed that SSR Mode S is making use of spectrum that has been allocated by the ITU to the Aeronautical Radionavigation Service. Therefore, the Radio Regulations do not allow this system to be used for mobile communications and, in principle, the whole of column "SSR Mode S" should, with the exception of the reference to ADS-C. However, in practice, the provisions of the Radio Regulations seem to be ignored by aviation in this case. No opinion is expressed here about this practice.

ATNP WG-B Position:  No change required.

Table 1-3. Mapping of ATN network priority to mobile subnetwork priority

| Message categories | ATN network layer priority | Corresponding mobile subnetwork priority (see Note 5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AMSS-1 (see Note 4) | VDL Mode 2 | VDL Mode 3 | VDL Mode 4 | SSR Mode S | HFDL |
| Network/systems management | 14 | 14 | see Note 1 | 3 | high see Note 7 | high | 14 |
| Distress communications | 13 | 14 | see Note 1 | 2 | high see Note 7 | high | 14 |
| Urgent communications | 12 | 14 | see Note 1 | 2 | high see Note 7 | high | 14 |
| High-priority flight safety messages | 11 | 11 | see Note 1 | 2 | high see Note 7 | high | 11 |
| Normal-priority flight safety messages | 10 | 11 | see Note 1 | 2 | high see Note 7 | high | 11 |
| Meteorological communications | 9 | 8 | see Note 1 | 1 | medium see Note 7 | low | 8 |
| Flight regularity communications | 8 | 7 | see Note 1 | 1 | medium see Note 7 | low | 7 |
| Aeronautical information service messages | 7 | 6 | see Note 1 | 0 | medium see Note 7 | low | 6 |
| Network/systems administration | 6 | 5 | see Note 1 | 0 | medium see Note 7 | low | 5 |
| Aeronautical administrative messages | 5 | 4 | restricted - see Note 1 | restricted | low restricted see Note 7 | not allowed | 4 restricted |
| <unassigned> | 4 | not assigned | not assigned - see Note 1 | not assigned | not assigned see Note 7 | not allowed | not assigned |
| Urgent-priority administrative and U.N. Charter communications | 3 | 3 | restricted - see Note 1 | restricted | low restricted see Note 7 | not allowed | 3 restricted |
| High-priority administrative and State/Government communications | 2 | 2 | restricted - see Note 1 | restricted | low restricted see Note 7 | not allowed | 2 restricted |
| Normal-priority administrative communications | 1 | 1 | restricted - see Note 1 | restricted | low restricted see Note 7 | not allowed | 1 restricted |
| Low-priority administrative communications & Aeronautical Passenger Communications | 0 | 0 | restricted - see Note 1 | restricted | low restricted see Note 7 | not allowed | 0 restricted |
| Note 1.— VDL Mode 2 has no specific subnetwork priority mechanisms. Note 2.— The AMSS SARPs specify mapping of message categories to subnetwork priority without explicitly referencing ATN network layer priority. Note 3.— The term “not allowed” means that only communications related to safety and regularity of flight are authorized to pass over this subnetwork as defined in the subnetwork SARPs. Note 4. — The term AMSS-1 refers to the first generation Aeronautical Mobile Satellite Service. Note 5. — Only those mobile subnetworks are listed for which subnetwork SARPs exist and for which explicit support is provided by the ATN Boundary Intermediate System technical provisions. Note 6. — The term “restricted” means for this message category the use of this subnetwork may not be allowed in certain States and/or regions based on ITU radio frequency spectrum allocation. Note 7. — The VDL Mode 4 subnetwork has only been specified to support surveillance applications (e.g., ADS). |  |  |  |  |  |  |  |